TITLE 35:  ENVIRONMENTAL PROTECTION

SUBTITLE C:  WATER POLLUTION

CHAPTER II:  ENVIRONMENTAL PROTECTION AGENCY

PART 373

THIRD STAGE TREATMENT LAGOON EXEMPTIONS

SUBPART A:  INTRODUCTION

Section

373.101   Purpose

373.102   Definition of a Third Stage Treatment Lagoon Facility

373.103   Eligibility

SUBPART B:  FACILITY INFORMATION

Section

373.201   Application Requirement

373.202   Facility Description

373.203   Population Equivalent Requirements

373.204   Sufficient Isolation Requirements

373.205   Useful Life

SUBPART C:  STREAM INFORMATION

Section

373.301   Critical Length

373.302   Stream Description

373.303   Smaller Facilities

373.304   Stream Assimilative Capacity

373.305   Model Limitations

APPENDIX A      Maximum Critical Length

APPENDIX B      Modified Streeter-Phelps Equation

APPENDIX C      Hydraulic Parameters

APPENDIX D      Model Limitations

AUTHORITY:  Implementing and authorized by Section 39(a) of the

Environmental Protection Act (Ill. Rev. Stat. 1989, ch. 111 1/2, par.

1039(a)).

SOURCE:  Emergency rule adopted December 2, 1974, amended rule filed March

1, 1976, effective March 11, 1976; rules repealed, new rules adopted at 8

Ill. Reg. 3286, effective March 7, 1984; amended at 12 Ill. Reg. 3472,

effective January 22, 1988; amended at 14 Ill. Reg. 18289, effective

October 30, 1990.

SUBPART A:  INTRODUCTION

<BSection 373.101  Purpose>>

This Part describes information required in an application for lagoon

exemption effluent limitations allowed by 35 Ill. Adm. Code 304.120(a) and

(c).  An approved lagoon exemption is required before the appropriate

limitations can be specified in a National Pollution Discharge Elimination

System (NPDES) permit.

<BSection 373.102  Definition of a Third Stage Treatment Lagoon Facility>>

A third-stage treatment lagoon is defined as a waste treatment facility

employing an aerated or nonaerated waste stabilization lagoon alone or in

conjunction with one or more additional treatment units in which the units

are incapable of producing an effluent quality equal to or better than

otherwise applicable effluent limitations of 35 Ill. Adm. Code 304.120 and

water quality standards of 35 Ill. Adm. Code 302.

(Source  Amended at 12 Ill. Reg. 3472, effective January 22, 1988)

<BSection 373.103  Eligibility>>

In accordance with the requirements of 35 Ill. Adm. Code 304.120(a) and

(c), third stage treatment lagoon facilities must, in order to be eligible

for consideration for a lagoon exemption, be properly constructed pursuant

to 35 Ill. Adm. Code 370, and properly maintained and operated; cannot

alone or in combination with other sources cause a violation of the

applicable dissolved oxygen water quality standard of 35 Ill. Adm. Code

302.206; and must qualify under one of the following categories:

a)  Any wastewater treatment works with an untreated waste load of

less than 2,500 Population Equivalents (P.E.), which is

sufficiently isolated so that combining with other sources to

aggregate 2,500 P.E. or more is not practicable.

b)  Any wastewater treatment works in existence and employing third

stage treatment lagoons on January 1, 1986, whose untreated waste

load is 5,000 P.E., or less and sufficiently isolated that

combining to aggregate 5,000 P.E. or more is not practicable.

c)  Any wastewater treatment works with an untreated waste load of

5,000 P.E., or less, which has reached the end of its useful life

(see Section 373.205) by January 1, 1987, and is sufficiently

isolated that combining to aggregate 5,000 P.E. or more is not

practicable.

d)  Any wastewater treatment works with an untreated wasteload of

5,000 P.E. or less which has not reached the end of its useful

life and which has received an adjusted standard determination

from the Board that it qualifies for a lagoon exemption. Such a

Board determination will only be made in an adjusted standard

proceeding, held in accordance with Section 28.1 of the

Environmental Protection Act (Ill. Rev. Stat. 1989, ch. 111 1/2,

par. 1028.1) and applicable by procedures set forth by 35 Ill.

Adm. Code 106.

1)  In an adjusted standard proceeding the Board may determine

that the petitioning wastewater treatment source qualifies

for a lagoon exemption if the wastewater treatment works

proves that it is so situated that a land treatment system is

not a suitable treatment alternative. Factors relevant to a

suitability finding may include the following:  cost;

influent character; geographic characteristics; climate; soil

conditions; hydrologic conditions; and the availability of

irrigable land.

2)  For the purpose of this subsection (d), a land treatment

system is a wastewater treatment system which does not

directly discharge treated effluent to water of the State but

instead uses the treated effluent to irrigate terrestrial

vegetation.

(Source:  Amended at 14 Ill. Reg. 18289, effective October 30, 1990)

SUBPART B:  FACILITY INFORMATION

<BSection 373.201  Application Requirement>>

In order to have the effluent limits stated in 35 Ill. Adm. Code 304.120(a)

and (c) included in an NPDES permit, the discharger shall submit a written

application to the Agency containing the information required by this Part.

The application shall be on a form provided by the Agency.

<BSection 373.202  Facility Description>>

The application shall describe the facility including, at a minimum:

a)  location,

b)  design and operating data,

c)  untreated waste load and effluent quality,

d)  construction permit and NPDES permit information, and

e)  useful life of the facility being replaced, if applying per

Section 373.103(c), and

f)  any proposed expansion or upgrading program.

(Source:  Amended at 12 Ill. Reg. 3472, effective January 22, 1988)

<BSection 373.203  Population Equivalent Requirements>>

a)  If applying per Section 373.103(a), the applicant shall show that

the facility is presently receiving an untreated organic waste

load of less than 2,500 P.E.

b)  If applying per Section 373.103(b) or (c), the applicant shall

show that the facility is presently receiving an untreated organic

waste load of 5000 P.E. or less.

c)  If the facility is treating a waste load of less than the

applicable P.E. limit as specified in Section 373.103 but is

designed to treat a greater load, this regulation may be applied

until the facility begins to treat the allowed maximum waste load.

A specific deadline which estimates when this loading limit will

be reached shall be included in the permit.

(Source:  Amended at 12 Ill. Reg. 3472, effective January 22, 1988)

<BSection 373.204  Sufficient Isolation Requirements>>

The applicant shall show that the facility is sufficiently isolated so that

combining with other sources to aggregate 2,500 P.E. or more if applying

per Section 373.103(a), or more than 5,000 P.E. if applying per Section

373.103(b) or (c), is not practical.

a)  If the facility is not located in a designated facilities planning

area the facility will be considered isolated.

b)  If the facility is located in a designated facilities planning

area which has less than the allowable total wastewater load, it

will be considered isolated.  If load forecasts suggest that the

P.E. limit will be met within the design life of the facility, the

lagoon exemption will be granted for only a fixed period of time.

At each permit renewal, the condition shall be reevaluated based

on actual load increases.

c)  If the facility is located in a designated facilities planning

area with more than the allowable total wastewater load, the

exemption shall be denied unless one or more of the following

conditions are met:

1)  The facility is a permanent feature of an approved facilities

plan for the area.

2)  The facility is an interim feature of an approved facilities

plan, in which case the exemption shall be granted with

termination conditions as provided for in the plan.

3)  The facility was not addressed in an approved facilities

plan, and all units of government which might be considered

capable of providing service have indicated in writing their

inability to do so on equitable terms.

4)  There is no completed and approved facilities plan for the

area and the discharge is not located within the area of

zoning control of any municipality or within any sewer

district or any other unit of government capable of providing

sewer service.  In this case, the isolation criteria shall be

satisfied on an interim basis pending completion and approval

of the facilities plan for the area.  In such instances, a

lagoon exemption may be issued  for a specified time period,

subject to review and extension or termination after approval

of the completed facility plan.

(Source:  Amended at 12 Ill. Reg. 3472, effective January 22, 1988)

<BSection 373.205  Useful Life>>

Applicants wishing to qualify for an exemption under Section 373.103(c)

must complete a Useful Life evaluation.  This evaluation must demonstrate

that the existing facility has exceeded its useful life.  Useful life is

considered to be the time span over which a wastewater treatment facility

can be expected to be economically operated and maintained.  Useful life is

distinctly different from design life.  Publicly owned treatment works

constructed with the assistance of a federal or state construction grant

must, at the minimum, be operated and maintained for the design life of the

project (typically 20 years).  The Useful Life requirement is intended to

prevent the premature retirement of facilities capable of meeting the more

stringent effluent limits of 10mg/1 BOD and 12 mg/1 total suspended solids.

The evaluation is also intended to assess the Useful Life of individual

components of the existing facility, so that any salvageable components are

incorporated into the proposed facility for which an exemption is requested

should it prove to be cost-prohibitive to continue to meet the more

stringent limits.  This ensures that the highest degree of treatment

possible is provided, in the most cost-effective manner.  Applicants will

provide the following information:

a)  Determine the structural integrity of the individual units in the

existing facility.

b)  Review the operations and maintenance record for past performance.

c)  Relate Subsections (a) and (b) to the expected life cycle for the

individual units.  USEPA provides the following general guidelines

for life cycle:  conveyance structures (piping) -- 50 years;

process equipment -- 15 to 20 years; buildings and concrete

tanks -- 30 to 50 years; auxiliary equipment -- 10 to 15 years.

d)  Determine the present worth cost to continue use of the existing

facility over a 5, 10 and 20 year planning period using standard

engineering economic analysis.  Sunk costs are not included in

this analysis.  Relate this cost to user charges.

e)  Describe alternatives to using the existing facility, ranging from

addition of one or more lagoon cells in conjunction with upgrading

of existing facilities to the construction of an entirely new

lagoon system.  Determine the present worth costs of these

alternatives over the same planning horizons.  Relate these costs

to user charges.

(Source:  Added at 12 Ill. Reg. 3472, effective January 22, 1988)

SUBPART C:  STREAM INFORMATION

<BSection 373.301  Critical Length>>

The applicant shall provide information about the stream to which the

facility discharges.  This information shall describe the stream for its

critical length as determined by the procedure outlined in Appendix A.

<BSection 373.302  Stream Description>>

The applicant shall provide the following information at a minimum:

a)  The name of the receiving stream and the progression of higher

order streams it flows into, up to and including the major river

basin;

b)  The location of the point of discharge by county and United States

Geological Society (USGS) coordinates;

c)  A copy of the most recent 7.5 or 15 minute USGS topographic map,

showing the entire critical stream length, the point of discharge

for which the exemption is being requested and the discharge point

of other dischargers if known;

d)  A description of the stream's physical characteristics including

substrate, channel obstructions, bank condition, and degree of

meandering.  This description shall also include a statement of

the presence or absence of sludge or organic deposits of unnatural

origin in amounts that are likely to elevate sediment oxygen

demand above background levels for similar streams in the

vicinity; and

e)  The name, location, design average flow rate and NPDES permit

limitations of other wastewater sources (if any) which may

influence the critical length of the receiving stream.

<BSection 373.303  Smaller Facilities>>

a)  Facilities which meet the following conditions need not perform

the analysis required by Section 373.304:

1)  The facility must be designed for a waste load less than 750

P.E., and

2)  The critical length of stream below the discharge is not

subject to significant sediment oxygen demand from bottom

deposits as described in Section 373.302(d).

b)  After review of the application for smaller facilities the Agency

shall require further stream study if warranted by local

conditions including:

1)  the impact of other dischargers on the stream,

2)  the stream use designation or stream segment classification,

or

3)  other characteristics that would limit the assimilative

capacity of the stream.

<BSection 373.304  Stream Assimilative Capacity>>

a)  Unless conditions as described in Appendix D indicate otherwise,

the applicant shall use the Modified Streeter-Phelps Equation

contained in Appendix B to predict the influence of the treated

wastewater discharge on the dissolved oxygen profile of the

receiving stream.

b)  The critical conditions for estimating the stream assimilative

capacity shall include:

1)  the 7-day 10-year low flow value,

2)  ambient BOD concentration,

3)  ambient total ammonia nitrogen concentration,

4)  ambient dissolved oxygen concentration,

5)  the lagoon system discharge at its design average flow rate

and design effluent quality,

6)  expected maximum stream temperature, and

7)  other hydraulic parameters as described in Appendix C.

<BSection 373.305  Model Limitations>>

If the limiting factors described in Appendix D are present in the stream,

the exemption shall be denied.

<BSection 373.APPENDIX A  Maximum Critical Length>>

The maximum critical length of a receiving stream is the distance

(downstream from the wastewater source) required at 7-day 10-year low flow

to reestablish an instream BOD5 of 5 mg/l.  The amount of time required to

reestablish the 5 mg/l BOD 5 is termed the critical time of travel.  The

maximum critical length can be estimated by computing the maximum critical

time of travel, which can be approximated using the following equation:

tc <Plesser than>> -1/Kc ln (5)/Ef

ln = natural logarithm function

Ef = BOD5 initially present in the stream

Kc = carbonaceous decay constant

tc = maximum critical time of travel

This relationship assumes that BOD5 decays according to ordinary first

order reaction kinetics.

<BSection 373.APPENDIX B  Modified Streeter-Phelps Equation>>

The Modified Streeter-Phelps Equation mathematically defines the

relationship between carbonaceous oxygen demand, nitrogenous oxygen demand,

natural stream reaeration and the dissolved oxygen deficit as a function of

time:

D = KcLac/K2-Kc (e -Kct -e -k2t)

+ KnLan/K2-Kn (e -Kn(t-tO) -e -K2 (t-tO))

+Dae -K2t

Definition and discussion of terms:

a)  D = Dissolved oxygen deficit; units = mg/l; defined as the

difference between the dissolved oxygen concentration at

saturation and the actual instantaneous dissolved oxygen

concentration at time t, i.e.,

D = D.O.(sat) - D.O.(actual).  From this relation, the stream

dissolved oxygen concentration can be computed for various

times-of-travel (t's) downstream and plotted on a graph of t vs.

D.O.(actual).

b)  Kc = carbonaceous decay constant; units = 1/day; this constant

describes the rate at which carbonaceous BOD is utilized in a

stream.  Its value may be determined experimentally for a specific

effluent and a specific stream.  The actual value of Kc depends

essentially upon the origin and strength of the wastewater,  the

type of treatment that wastewater has undergone, as well as

various stream characteristics.

The following guidelines may be used for selection of a Kc value

for various applications:

1)  Effluents containing up to and including 10 mg/l BOD5:  0.10

2)  Effluents containing between 10 and 30 mg/l BOD5:  0.30

3)  Virtually all effluents may be tested using an appropriate

experimental procedure for a more precise determination of

Kc.

c)  Lac = ultimate carbonaceous demand; units = mg/l; this term may be

calculated once the BOD5 and Kc are known by use of the following

equation:

Ef

Lac = <P                >>

1 - e -5Kc

In this equation, Ef is the treatment works effluent BOD5.

d)  K2 = stream reaeration constant; units = 1/day; this constant

describes the rate at which atmospheric oxygen diffuses into the

water of a flowing stream.  Its value depends upon the hydraulic

and geometric properties of the stream in question.  Many

investigators have developed equations to predict K2.  The

equation given below has been shown to yield results which best

fit the field observations of many researchers over a wide variety

of stream types:

K2 = (110.5H = 0.5832V2)<P(SV)0.375>>

H2

In this equation, "H" is average depth of flow in feet, "V" is

stream average velocity in feet per second, and "S" is the

dimensionless parameter, stream slope, ft./ft.  Velocity and

average depth of flow may not be estimated but must be field

measured at the 7-day 10-year low flow stream condition or

computed from field measurements of stream geometry (cross

sections and slopes using ordinary principles of open-channel

hydraulics).  Significant changes in stream geometry will change

average velocity and average depth of flow.  K2 must be computed

for each stream segment as defined in Appendix C.

e)  e = the Naperian logarithm base, dimensionless; e = 2.71828...

f)  t =  time; units = days.

g)  Kn = nitrogenous decay constant; units = 1/day; this constant

describes the rate at which nitrogenous BOD is utilized in a

stream.  Its value may be determined experimentally for a specific

effluent and a specific stream.  Previous experimental work has

established a range of typical values for Kn of 0.25 to 0.37 per

day with an average of 0.29 per day.  It should be noted that the

higher values of Kn yield generally more conservative results when

applied to the Streeter-Phelps Equation.

h)  Lan = ultimate nitrogenous demand; units = mg/l; this term may be

calculated, once the initial ammonia nitrogen concentration is

established, by use of the following formula:

Lan = 4.57 (Amm-N concentration in mg/l).

i)  tO = nitrogenous lag time; units = days; when a waste contains

both carbonaceous and nitrogenous oxygen demand, there is usually

a time lag before the onset of nitrogenous oxygen demand.  The

time lag may typically vary from 0-10 days with its actual value

dependent upon the complex chemical characteristics of the waste

as well as various stream characteristics.  The value of tO may be

experimentally determined where effluent or stream field

measurements are practicable.  In the case of well nitrified

effluents, the value of t0 may generally be considered to be less

than 1 day.  Note that for t less than t0 the nitrogenous term,

<PKnLan>> (e-Kn(t-t0) -e-K2 (t-t0))

K2-Kn

does not enter into the calculation of D.

j)  Da - initial dissolved oxygen deficit; units = mg/l.  Determined

by subtracting assumed effluent dissolved oxygen concentration of

6.0 mg/l from dissolved oxygen saturation value at the expected

maximum stream temperature.

Temperature Adjustments

KC,K2,Kn and Lac are temperature dependent quantities.  The values

calculated in accordance with the above are 20 degree Celsius values.

Since the saturation D.O. decreases with increasing temperature, it will be

necessary to adjust the parameters KC,K2,Kn and Lac to reflect the expected

maximum stream temperature condition.  In the equations listed below, T is

the expected maximum stream temperature in degrees Celsius.

a)  Kc(T) = Kc x 1.047(T-20)

b)  K2(T) = K2 x 1.024(T-20)

c)  Kn(T) = Kn x 1.047(T-20)

d)  Lac(T) = Lac (0.02T + 0.6)

Since the time of the year at which the 7-day 10-year low flow

occurs typically varies from stream to stream, it is not possible

to prescribe a uniform maximum temperature adjustment throughout

the state.  The maximum temperature should be ascertained from

field measurements in the stream at the time of year at which

7-day 10-year low flow is expected to occur.  IEPA ambient water

quality monitoring network data are available for making such

determination.  This data may be obtained by contacting the

Division of Water Pollution Control.

(Source:  Amended at 8 Ill. Reg. 3286, effective March 7, 1984)

@R+R

<BSection 373.APPENDIX C  Hydraulic Parameters>>

In order to utilize the modified Streeter-Phelps dissolved oxygen model

specified in Section 373.304, it is necessary to determine specific

hydraulic parameters including mean stream depth, mean stream velocity and

time of travel.  These factors can be estimated for the critical 7-day

10-year low flow condition through basic open-channel hydraulic

calculations (Manning Equation) for each stream segment.

The critical stream length must be divided into one or more segments of

uniform hydraulic, geometric and water quality characteristics.  The

characteristics of importance in this analysis include volumetric flow

rate, average stream velocity, depth and width of flow, stream slope,

channel geometry, and BOD and total ammonia nitrogen concentrations.  As a

rule of thumb, therefore, a new segment should begin where:

a)  there is a wastewater discharge to the stream,

b)  the stream channel undergoes a change in slope or cross sectional

geometry, or

c)  stream flow increases through addition of another point source

discharge, or confluence with another stream with non-zero 7-day

10-year low flow.

@R+R

<BSection 373.APPENDIX D  Model Limitations>>

The Modified Streeter-Phelps Equation used in this Part should not be

construed as a precise predictive model but rather as a means of

demonstrating adequate receiving stream assimilative capacity under

estimated worst-case conditions.  Even for a general application of this

type, there are inherent limitations to the model that will preclude its

utilization in some instances.

The modified Streeter-Phelps Equation constitutes a steady state, one

dimensional model and as such is limited in its application to free flowing

stream systems that can be reasonably well represented as steady state, one

dimensional systems.  Once mixing of the wastewater discharge and natural

stream flow is accomplished, there should be minimal vertical or lateral

variation (throughout a given stream cross section) of key factors such as

waste concentration, temperature, and carbonaceous and nitrogenous bacteria

population density.  In most free flowing streams in Illinois (even with

predominantly pool/riffle morphology) this should not be a limitation.

However, lakes, ponds, stream segments impounded by man-made structures or

natural impediments, streams with unstable channel characteristics, swamps

and marshes are all examples of systems that may not be successfully

modeled with the Modified Streeter-Phelps Equation.  Likewise, systems with

wide variations in temperature, flowrate or organic loading may not reach a

steady state condition conducive to model application.  The applicant

should be aware of these limitations when applying the model to a

particular situation.

@R