ILLINOIS POLLUTION CONTROL BOARD

December 19, 2002

| IN THE MATTER OF: WATER QUALITY TRIENNIAL REVIEW: AMENDMENTS TO 35 ILL. ADM. CODE 302.105, 302.208(e)-(g), 302.504(a), 302.575(d), 309.141(h); and PROPOSED 35 ILL. ADM. CODE 301.267, 301.313, 301.413, 304.120, and 309.157 | ) ) ) ) ) ) ) ) | R02-11 (Rulemaking - Water) |
| --- | --- | --- |

Adopted Rule.  Final Notice.

OPINION AND ORDER OF THE BOARD (by G.T. Girard, M.E. Tristano):

On November 9, 2001, the Illinois Environmental Protection Agency (Agency) filed a rulemaking proposal (Prop.) to amend the Board’s water regulations at 35 Ill. Adm. Code 302.208(e)-(g), 302.504(a), 302.575(d), 303.444, 309.141(h) and to add new sections at 35 Ill. Adm. Code 301.267, 301.313, 301.413, 304.120, and 309.157.  The proposed rule will update the Board’s regulations pursuant to the State’s triennial review of water regulations.

On December 6, 2001, the Board accepted this matter for hearing.

The Board held two hearings prior to first notice.  Board Hearing Officer Marie Tipsord conducted hearings on January 29, 2002, in Chicago, and on March 6, 2002, in Springfield.  At those two hearings, the Agency, Galesburg Sanitary District, the Environmental Groups, Illinois Association of Wastewater Agencies (IAWA), and Rock River Water Reclamation District testified.  The Board received 21 public comments prior to first notice.  On June 20, 2002, the Board proceeded to first notice with the rule.  26 Ill. Reg. 9573, 9580, 9602, and 9609.

After first notice the Board held one additional hearing and received 7 public comments.  The hearing before Board Hearing Officer Tipsord was held on July 25, 2002, in Chicago, with testimony from the Agency and the Environmental Groups.

On October 17, 2002, the Board proposed for second notice the amendments to the Board’s water rules.  The Board directed that the rules be submitted to the Joint Committee on Administrative Rules (JCAR) for second-notice review.  On November 19, 2002, JCAR issued a certification of no objection to the rule.

Today the Board adopts these amendments to the Board’s water quality rules.  The Board has made minor non-substantive changes in response to requests by JCAR.  The following opinion will explain the proposal background, summarize the first-notice proposal, summarize the second-notice proposal, and, finally, discuss the economic reasonableness and technical feasibility of the rule.

# BACKGROUND

States are required to revise and update their water quality standards pursuant to the Federal Water Pollution Control Act (33 U.S.C. §§ 1251-1387 (1987)) (Clean Water Act).  Prop. at 7.  The update is necessary to ensure that the water quality standards protect public health and welfare, enhance the quality of water, and promote the purposes of the Clean Water Act.  Id.  This process is called a triennial water quality standards review.  Id. citing 33 U.S.C. §1313(c)(1).  One element in the triennial water quality standards review is the refining of numeric standards based on the best available current knowledge.  Id.  The Agency filed a proposal on November 9, 2001, which revised the water quality standards based on revised federal policy and new scientific information collected over the years.  Prop. at 8.

# FIRST-NOTICE PROPOSAL

As a part of the triennial review, the Board sent to first notice a rule which proposed changes in five areas of the State’s regulations.  First, the proposed rule amends new aquatic life acute and chronic numeric General Use Water Quality Standards and Lake Michigan Water Quality Standards for benzene, ethyl benzene, toluene, and xylene (BETX).  Second, the proposed rule revises General Use Water Quality Standards for zinc, and nickel.  Third, the proposed rule changes the General Use Water Quality Standards for metals from total to dissolved form.  Fourth, the proposed rule corrects the Lake Michigan water rules adopted in Conforming Amendments for the Great Lakes Initiative:  35 Ill. Adm. Code 302.101; 302.105; 302.Subpart E; 303.443 And 304.222, R97-25, (Dec. 18, 1997).  Fifth, the proposed rule allows the Agency to use five-day carbonaceous biochemical oxygen demand (CBOD5) instead of five-day biochemical oxygen demand (BOD5) for measuring compliance with Section 304.120 (35 Ill. Adm. Code 304.120) in National Pollutant Discharge Elimination System (NPDES) permits.

Three areas of concern emerged prior to first notice.  Those three areas were:  (1) whether the Board should require the Agency to provide implementation rules regarding hardness, reasonable potential testing, dissolved oxygen and the metals translator prior to proceeding to first notice with the Agency’s proposal; (2) whether the Board should adopt the revised cyanide standard proposed by the Agency; and (3) whether compliance with the BOD5 effluent limits in Section 304.120 should be determined by measuring CBOD5.  After careful consideration of the comments the Board addressed each of those concerns in the first-notice opinion and order.  The Board also asked that the parties provide additional comments on the areas of concern during the first-notice comment period.

# Draft Implementation Rules

The Board proceeded to first notice even though the Agency did not provide draft implementation rules.  The Board noted that in general, seeing implementation procedures for the water quality standards would be important and the Board’s hearing officer had asked the Agency to provide the Board with copies of the implementation rules as a part of the Agency’s comments.  Tr.2 at 149.  While it would be helpful to know the implementation procedures in developing comprehensive water quality regulations, in this proceeding the Board found that the Agency has sufficient federal guidance and experience to develop implementation procedures which ensure that water quality standards are protective of aquatic life.  Water Quality Triennial Review:  Amendments To 35 Ill. Adm. Code 302.105, 302.208(e)-(g), 302.504(a), 302.575(d), 309.141(h); and Proposed 35 Ill. Adm. Code 301.267, 301.313, 301.413, 304.120, and 309.157 R02-11 (June 20, 2002), slip op. 7-8.

Cyanide

At first notice, the Board decided not to proceed with the revised cyanide standards proposed by the Agency.  The Agency proposed an acute standard of 49 µg/L and a chronic standard of 9.9 µg/L which is more liberal than the current acute standard of 22 µg/L and chronic standard of 5.2 µg/L.  The Board was convinced by the comments and testimony that changing the cyanide standard was not warranted at this time.  The Agency based the proposed amendment of the cyanide standard on the assumption that Illinois does not have native cold-water species of fish outside of Lake Michigan.  However, the Illinois Department of Natural Resources (IDNR) has indicated that stocked cold-water species have reproduced in Illinois.  This is information that the Agency did not have when considering the relaxation of the standard for cyanide (see Tr.1 at 62).  The Agency also testified that a cool-water species (a sculpin) is present in Illinois, but those streams “are not now thought to contain significant amounts of cyanide,” and the Board’s antidegradation rules can be used to evaluate the streams.  Tr.2 at 141.  The Board noted in the first notice-opinion, that while antidegradation evaluations provide additional protection to a water body in a permitting context, such an evaluation should not be used as a justification to relax water quality standards.  Water Quality Triennial Review:  Amendments To 35 Ill. Adm. Code 302.105, 302.208(e)-(g), 302.504(a), 302.575(d), 309.141(h); and Proposed 35 Ill. Adm. Code 301.267, 301.313, 301.413, 304.120, And 309.157 R02-11 (June 20, 2002), at slip op. 9.

The Board also expressed concern with the lack of information regarding mussels in Illinois.  At this time there are no studies that either the Agency or the participants are aware of which review the effect of cyanide toxicity on mussels.  The Agency, in proposing the change, relied on the fact that the United States Environmental Protection Agency (USEPA) did not use mussel data.  However, the Board opined that USEPA’s lack of information is not scientific support for relaxing the standard.  The Board stated that there are many endangered and threatened species of mussels in Illinois and no evidence to establish that relaxation of the standard will have no effect on those species.  Furthermore, the Board noted that the relaxation of the standard would not help any Illinois dischargers.  Water Quality Triennial Review:  Amendments To 35 Ill. Adm. Code 302.105, 302.208(e)-(g), 302.504(a), 302.575(d), 309.141(h); and Proposed 35 Ill. Adm. Code 301.267, 301.313, 301.413, 304.120, And 309.157 R02-11 (June 20, 2002), at slip op. 9.

CBOD5

The Board proceeded to first notice with a provision to allow the use of CBOD5 instead of BOD5 for measuring compliance with Section 304.120 (35 Ill. Adm. Code 304.120) in NPDES permits.  The Board noted that the Environmental Groups’ primary concern is that using CBOD5 to measure the efficiency of wastewater treatment facilities may not be fully protective of dissolved oxygen levels in Illinois waters, because CBOD5 does not measure nitrogenous oxygen demand.  However, the Board found that the evidence in the record (such as IAWA’s Exhibits 14, 15 and 16) document that the BOD5 test in many cases does not accurately represent wastewater treatment efficiency or the actual oxygen demand experienced in the receiving stream.  The Board further found that the record supported the Agency’s position that combined effluent testing for CBOD5 and ammonia nitrogen provides a more representative measure of treatment efficiency.  Water Quality Triennial Review:  Amendments To 35 Ill. Adm. Code 302.105, 302.208(e)-(g), 302.504(a), 302.575(d), 309.141(h); and Proposed 35 Ill. Adm. Code 301.267, 301.313, 301.413, 304.120, and 309.157, R02-11 (June 20, 2002), at slip op. 11.

SECOND NOTICE PROPOSAL

In the first notice opinion, the Board sought additional comments on:  (1) whether the Board should require the Agency to provide implementation rules regarding hardness, reasonable potential testing, dissolved oxygen and the metals translator prior to proceeding to first notice with the Agency’s proposal; (2) whether the Board should adopt the revised cyanide standard proposed by the Agency; and (3) whether compliance with the BOD5 effluent limits in Section 304.120 should be determined by measuring CBOD5.  During the first notice comment period, the Board only received comments on two issues:  (1) the Board’s decision not to amend the cyanide standards; and, (2) the Board’s decision to allow the use of CBOD5 to measure compliance with the BOD5 standard from Section 304.120 (35 Ill. Adm. Code 304.120) in NPDES permits.

The Agency submitted testimony and comments in support of the adoption of cyanide standards as originally proposed by the Agency (PC 27).  The Metropolitan Water Reclamation District (MWRD) filed a comment in support of the Agency’s proposal to change the cyanide standard (PC 23), while the IDNR filed a comment opposing a change to the cyanide standard (PC 22).  The Environmental Groups filed a comment supporting the Board’s first-notice proposal declining to change the cyanide standard and suggesting that the Board adopt a new USEPA approved test method capable of testing cyanide below the current standard.  The Environmental Groups also asked the Board to reexamine the issue of using of CBOD5 to meet the BOD5 requirement from Section 304.120 (35 Ill. Adm. Code 304.120) in NPDES permits (PC 25).  The IAWA continued to support using CBOD5 to meet the BOD5 requirement from Section 304.120 (35 Ill. Adm. Code 304.120) in NPDES permits.  No additional comments were received regarding the Agency’s implementation procedures.  The following discussion will briefly summarize the Board’s position regarding the remaining issues (cyanide and CBOD5).

# Cyanide

First regarding the new USEPA approved test method, OIA-1677, capable of testing for cyanide below the current standard, the Agency stated that the new method has sufficient benefits over the current weak acid dissociable (WAD) test method, including less interference and lower detection limit.  PC 27 at 5.  The Agency recommended that the new cyanide test method be referenced along with the WAD test method in the Board’s cyanide standard.  PC 27 at 5.  The Agency stated that although most commercial laboratories in Illinois are not equipped to run the new test, given an option to use the new test method, dischargers may begin to demand that the laboratories use the new method instead of the WAD test method.

The Board agreed with the Environmental Groups and the Agency that the new test method has significant benefits over the current WAD test method.  However, the Board found no need to amend the water quality regulations in order to encourage dischargers/laboratories to use the new test method to show compliance with the cyanide water quality standard.  In this regard, the Board noted that the current analytical testing requirements for both the General Use waters, and the Lake Michigan waters allow the use of test methods that are consistent with the USEPA's current manual of practice or with other procedures acceptable to USEPA and the Agency.  See 35 Ill. Adm. Code 301.104 and 302.550.  Since the new cyanide test method, OIM-1677, has been approved by the USEPA, the Agency may require the dischargers/laboratories to use the new cyanide test method to show compliance or notify the dischargers/laboratories that the new cyanide test method is the preferred method.

As to the actual cyanide standard, the Board was unconvinced by the additional comments of the Agency and the MWRD that the relaxation of the cyanide standard should be proposed at this time.  The Board found insufficient evidence to support changing to an acute standard of 49 µ/L and a chronic standard of 11 µ/L rather than the current acute standard of 22 µ/L and chronic standard of 5.2 µ/L for cyanide.

CBOD5

The Board acknowledged the additional comment on the issue of using CBOD5 rather than BOD5 in NPDES permits.  However, the Board found that the additional comments were not sufficient to warrant a change in the Board’s position.  Therefore, the Board did not make any changes to the first-notice proposal concerning the use of CBOD5 rather than BOD5 to measure compliance with Section 304.120 (35 Ill. Adm. Code 304.120) in NPDES permits.

# ECONOMIC REASONABLENESS AND TECHNICAL FEASIBILITY OF THE PROPOSAL

At first notice, the Board found that the rule was economically reasonable and technically feasible.  The Board has received no additional comments discussing economic reasonableness and technical feasibility of the proposed rule.  On March 12, 2002, pursuant to Section 27(b) of the Act (415 ILCS 5/27(b) (2000)), the Board requested that the Department of Commerce and Community Affairs (DCCA) conduct an economic impact study on the proposed rule.  The request letter referenced a letter, dated March 10, 2000, from DCCA.  The March 10, 2000 letter informed the Board that DCCA would not be doing economic impact studies.  At the July 25, 2002 hearing the Board provided copies of the DCCA letter and the Board’s March 12, 2002 letter.  The Board received no comments on the letter.

# CONCLUSION

The Board today proposes for final notice amendments to the Board’s water quality rules.  The Board is adopting the rule with only minor changes to the second-notice proposal suggested by JCAR.

# ORDER

The Board directs that the Clerk to file the following adopted rule with the Secretary of State for publication in the Illinois Register for final notice and adoption in the Illinois Administrative Code.

TITLE 35:  ENVIRONMENTAL PROTECTION

SUBTITLE C:  WATER POLLUTION

CHAPTER I:  POLLUTION CONTROL BOARD

PART 301

INTRODUCTION

Section

| 301.101 | Authority |
| --- | --- |
| 301.102 | Policy |
| 301.103 | Repeals |
| 301.104 | Analytical Testing |
| 301.105 | References to Other Sections |
| 301.106 | Incorporations by Reference |
| 301.107 | Severability |
| 301.108 | Adjusted Standards |
| 301.200 | Definitions |
| 301.205 | Act |
| 301.210 | Administrator |
| 301.215 | Agency |
| 301.220 | Aquatic Life |
| 301.221 | Area of Concern |
| 301.225 | Artificial Cooling Lake |
| 301.230 | Basin |
| 301.231 | Bioaccumulative Chemicals of Concern |
| 301.235 | Board |
| 301.240 | CWA |
| 301.245 | Calumet River System |
| 301.250 | Chicago River System |
| 301.255 | Combined Sewer |
| 301.260 | Combined Sewer Service Area |
| 301.265 | Construction |
| 301.267 | Conversion Factor |
| 301.270 | Dilution Ratio |
| 301.275 | Effluent |
| 301.280 | Hearing Board |
| 301.285 | Industrial Wastes |
| 301.290 | Institute |
| 301.295 | Interstate Waters |
| 301.300 | Intrastate Waters |
| 301.301 | Lake Michigan Lakewide Management Plan |
| 301.305 | Land Runoff |
| 301.310 | Marine Toilet |
| 301.311 | Method Detection Level |
| 301.312 | Minimum Level |
| 301.313 | Metals Translator |
| 301.315 | Modification |
| 301.320 | New Source |
| 301.325 | NPDES |
| 301.330 | Other Wastes |
| 301.331 | Outlier |
| 301.335 | Person |
| 301.340 | Pollutant |
| 301.341 | Pollutant Minimization Program |
| 301.345 | Population Equivalent |
| 301.346 | Preliminary Effluent Limitation |
| 301.350 | Pretreatment Works |
| 301.355 | Primary Contact |
| 301.356 | Projected Effluent Quality |
| 301.360 | Public and Food Processing Water Supply |
| 301.365 | Publicly Owned Treatment Works |
| 301.370 | Publicly Regulated Treatment Works |
| 301.371 | Quantification Level |
| 301.372 | Reasonable Potential Analysis |
| 301.373 | Same Body of Water |
| 301.375 | Sanitary Sewer |
| 301.380 | Secondary Contact |
| 301.385 | Sewage |
| 301.390 | Sewer |
| 301.395 | Sludge |
| 301.400 | Standard of Performance |
| 301.405 | STORET |
| 301.410 | Storm Sewer |
| 301.411 | Total Maximum Daily Load |
| 301.413 | Total Metal |
| 301.415 | Treatment Works |
| 301.420 | Underground Waters |
| 301.421 | Wasteload Allocation |
| 301.425 | Wastewater |
| 301.430 | Wastewater Source |
| 301.435 | Watercraft |
| 301.440 | Waters |
| 301.441 | Water Quality Based Effluent Limitation |
| 301.442 | Wet Weather Point Source |
| 301.443 | Whole Effluent Toxicity |
| APPENDIX A | References to Previous Rules |

AUTHORITY: Implementing Section 13 and authorized by Section 27 of the Environmental Protection Act [415 ILCS 5/13 and 27].

SOURCE: Filed with the Secretary of State January 1, 1978; amended at 3 Ill.  Reg.  25, p.  190, effective June 21, 1979; amended at 5 Ill.  Reg.  6384, effective May 28, 1981; codified at 6 Ill.  Reg.  7818; amended in R88-1 at 13 Ill.  Reg.  5984, effective April 18, 1989; amended in R88-21(A) at 14 Ill.  Reg.  2879, effective February 13, 1990; amended in R99-8 at 23 Ill. Reg. 11277, effective August 26, 1999; amended in R02-11 at           Ill. Reg.             , effective                                         .

#### Section 301.106 	Incorporations by Reference

a)	Abbreviations.  The following abbreviated names are used for materials incorporated by reference:

"ASTM" means American Society for Testing and Materials.

"GPO" means Superintendent of Documents, U.S.  Government Printing Office.

"NTIS" means National Technical Information Service.

"Standard Methods" means "Standard Methods for the Examination of Water and Wastewater", available from the American Public Health Association.

"USEPA" means United States Environmental Protection Agency.

b)	The Board incorporates the following publications by reference:

American Public Health Association et al., 1015 Fifteenth Street, N.W., Washington, D.C.  20005.

Standard Methods for the Examination of Water and Wastewater, 16th Edition, 1985

ASTM.  American Society for Testing and Materials, 100 Barr Harbor Drive, West Conshohocken, PA 19428-2959 (610) 832-9585 1976 Race Street, Philadelphia, PA 19013 (215) 299-5400

ASTM Standard E 724-80 "Standard Practice for Conducting Static Acute Toxicity Tests with Larvae of Four Species of Bivalve Molluscs", approved 1980.

ASTM Standard E 729-80 "Standard Practice for Conducting Static Acute Toxicity Tests with Fishes, Macroinvertebrates, and Amphibians", approved 1980.

ASTM Standard E 857-81 "Standard Practice for Conducting Subacute Dietary Toxicity Tests with Avian Species", approved 1981.

ASTM Standard E 1023-84 "Standard Guide for Assessing the Hazard of a Material to Aquatic Organisms and Their Uses", approved 1984.

ASTM Standard E 1103-86 "Method for Determining Subchronic Dermal Toxicity", approved 1986.

ASTM Standard E 1147-87 "Standard Test Method for Partition Coefficient (n-Octanol/Water) Estimation by Liquid Chromatography", approved February 27, 1987.

ASTM Standard E 1192-88 "Standard Guide for Conducting Acute Toxicity Tests on Aqueous Effluents with Fishes, Macroinvertebrates and Amphibians", approved 1988.

ASTM Standard E 1193-87 "Standard Guide for Conducting Renewal Life-Cycle Toxicity Tests with Daphnia Magna", approved 1987.

ASTM Standard E 1241-88 "Standard Guide for Conducting Early Life-Stage Toxicity Tests with Fishes", approved 1988.

ASTM Standard E 1242-88 "Standard Practice for Using Octanol-Water Partition Coefficients to Estimate Median Lethal Concentrations for Fish due to Narcosis", approved 1988.

ASTM Standard E 4429-84 "Standard Practice for Conducting Static Acute Toxicity Tests on Wastewaters with Daphnia", approved 1984.

NTIS.  National Technical Information Service, 5285 Port Royal Road, Springfield, VA 22161 (703) 487-4600

SIDES: STORET Input Data Editing System, January 1973, Document Number PB-227 052/8.

Water Quality Data Base Management Systems, February 1984, Document Number AD-P004 768/8.

USEPA.  United States Environmental Protection Agency, Office of Health and Environmental Assessment, Washington, D.C. 20460

Mutagenicity and Carcinogenicity Assessment for 1,3-Butadiene, September 1985, Document Number EPA/600/8-85/004A.

c)	The Board incorporates the following federal regulations by reference.  Available from the Superintendent of Documents, U.S. Government Printing Office, Washington, D.C. 20402. (202) 783-3238:

Procedure 5.b.2 of Appendix F of 40 CFR 132 (1995)

40 CFR 136 (1996)

40 CFR 141 (1988)

40 CFR 302.4 (1988)

d)	The Board incorporates the following federal regulations by reference, available from the Superintendent of Documents, U.S. Government Printing Office, Washington, D.C. 20402 (202) 783-3238:

USEPA 1996:  The Metals Translator: Guidance for Calculating a Total Recoverable Permit Limit from a Dissolved Criterion. EPA 823-B-96-007 (1996).

e)	This Section incorporates no future editions or amendments.

(Source:  Amended at __________ Ill. Reg. _______________, effective ________________)

Section 301.267	Conversion Factor

“Conversion Factor” means the fraction of the total metal found as dissolved in the toxicity tests used to derive the water quality standards of 35 Ill. Adm. Code 302.  The conversion factors are used to convert total metals water quality standards to dissolved standards.

(Source:  Added at __________ Ill. Reg. _______________, effective ________________)

Section 301.313	Metals Translator

“Metals Translator” means the fraction of total metal that is dissolved in the effluent or downstream water.  The metals translator calculates a total metal permit limit from a dissolved metal water quality standard.  In the absence of site-specific data for the effluent or receiving water, the metals translator is the reciprocal of the conversion factor.

(Source:  Added at __________ Ill. Reg. _______________, effective ________________)

Section 301.413 	Total Metal

“Total Metal” means the dissolved fraction of metal in a solution plus the suspended fraction.

(Source:  Added at __________ Ill. Reg. _______________, effective ________________)

TITLE 35:  ENVIRONMENTAL PROTECTION

SUBTITLE C:  WATER POLLUTION

CHAPTER I:  POLLUTION CONTROL BOARD

PART 302

WATER QUALITY STANDARDS

## SUBPART A:  GENERAL WATER QUALITY PROVISIONS

#### Section

302.100	Definitions

302.101	Scope and Applicability

302.102	Allowed Mixing, Mixing Zones and ZIDs

302.103	Stream Flows

302.104	Main River Temperatures

302.105	Antidegradation

## SUBPART B:  GENERAL USE WATER QUALITY STANDARDS

#### Section

302.201	Scope and Applicability

302.202	Purpose

302.203	Offensive Conditions

302.204	pH

302.205	Phosphorus

302.206	Dissolved Oxygen

302.207	Radioactivity

302.208	Numeric Standards for Chemical Constituents

302.209	Fecal Coliform

302.210	Other Toxic Substances

302.211	Temperature

302.212	Total Ammonia Nitrogen

  - Effluent Modified Waters (Ammonia)(Repealed)

## SUBPART C:  PUBLIC AND FOOD PROCESSING WATER SUPPLY STANDARDS

#### Section

302.301	Scope and Applicability

302.302	Algicide Permits

302.303	Finished Water Standards

302.304	Chemical Constituents

302.305	Other Contaminants

302.306	Fecal Coliform

## SUBPART D:  SECONDARY CONTACT AND INDIGENOUS AQUATIC LIFE STANDARDS

#### Section

302.401	Scope and Applicability

302.402	Purpose

302.403	Unnatural Sludge

302.404	pH

302.405	Dissolved Oxygen

302.406	Fecal Coliform (Repealed)

302.407	Chemical Constituents

302.408	Temperature

302.409	Cyanide

  - Substances Toxic to Aquatic Life

## SUBPART E:  LAKE MICHIGAN BASIN WATER QUALITY STANDARDS

Section

302.501	Scope, Applicability, and Definitions

302.502	Dissolved Oxygen

302.503	pH

302.504	Chemical Constituents

302.505	Fecal Coliform

302.506	Temperature

302.507	Thermal Standards for Existing Sources on January 1, 1971

302.508	Thermal Standards for Sources Under Construction But Not In Operation on January 1, 1971

302.509	Other Sources

302.510	Incorporations by Reference

302.515	Offensive Conditions

302.520	Regulation and Designation of Bioaccumulative Chemicals of Concern (BCCs)

302.521	Supplemental Antidegradation Provisions for Bioaccumulative Chemicals of Concern (BCCs)

302.525	Radioactivity

302.530	Supplemental Mixing Provisions for Bioaccumulative Chemicals of Concern (BCCs)

302.535	Ammonia Nitrogen

302.540	Other Toxic Substances

302.545	Data Requirements

302.550	Analytical Testing

302.553	Determining the Lake Michigan Aquatic Toxicity Criteria or Values - General Procedures

302.555	Determining the Tier I Lake Michigan Acute Aquatic Toxicity Criterion (LMAATC):  Independent of Water Chemistry

  - Determining the Tier I Lake Michigan Basin Acute Aquatic Life Toxicity Criterion (LMAATC):  Dependent on Water Chemistry

302.563	Determining the Tier II Lake Michigan Basin Acute Aquatic Life Toxicity Value (LMAATV)

  - Determining the Lake Michigan Basin Chronic Aquatic Life Toxicity Criterion (LMCATC) or the Lake Michigan Basin Chronic Aquatic Life Toxicity Value (LMCATV)

302.570	Procedures for Deriving Bioaccumulation Factors for the Lake Michigan Basin

  - Procedures for Deriving Tier I Water Quality Criteria and Values in the Lake Michigan Basin to Protect Wildlife

  - Procedures for Deriving Water Quality Criteria and Values in the Lake Michigan Basin to Protect Human Health – General

  - Procedures for Determining the Lake Michigan Basin Human Health Threshold Criterion (LMHHTC) and the Lake Michigan Basin Human Health Threshold Value (LMHHTV)

  - Procedures for Determining the Lake Michigan Basin Human Health Nonthreshold Criterion (LMHHNC) or the Lake Michigan Basin Human Health Nonthreshold Value (LMHHNV)

  - Listing of Bioaccumulative Chemicals of Concern, Derived Criteria and Values

## SUBPART F:  PROCEDURES FOR DETERMINING WATER QUALITY CRITERIA

#### Section

302.601	Scope and Applicability

302.603	Definitions

302.604	Mathematical Abbreviations

302.606	Data Requirements

  - Determining the Acute Aquatic Toxicity Criterion for an Individual Substance – General Procedures

302.615	Determining the Acute Aquatic Toxicity Criterion - Toxicity Independent of Water Chemistry

302.618	Determining the Acute Aquatic Toxicity Criterion - Toxicity Dependent on Water Chemistry

302.621	Determining the Acute Aquatic Toxicity Criterion - Procedure for Combinations of Substances

302.627	Determining the Chronic Aquatic Toxicity Criterion for an Individual Substance - General Procedures

302.630	Determining the Chronic Aquatic Toxicity Criterion - Procedure for Combinations of Substances

302.633	The Wild and Domestic Animal Protection Criterion

302.642	The Human Threshold Criterion

302.645	Determining the Acceptable Daily Intake

302.648	Determining the Human Threshold Criterion

302.651	The Human Nonthreshold Criterion

302.654	Determining the Risk Associated Intake

302.657	Determining the Human Nonthreshold Criterion

302.658	Stream Flow for Application of Human Nonthreshold Criterion

302.660	Bioconcentration Factor

302.663	Determination of Bioconcentration Factor

302.666	Utilizing the Bioconcentration Factor

  - Listing of Derived Criteria

APPENDIX A	References to Previous Rules

APPENDIX B	Sources of Codified Sections

APPENDIX C	Maximum total ammonia nitrogen concentrations allowable for certain combinations of pH and temperature

- TABLE A	pH-Dependent Values of the AS (Acute Standard)

TABLE B	Temperature and pH-Dependent Values of the CS (Chronic Standard) for Fish Early Life Stages Absent

- TABLE C 	Temperature and pH-Dependent Values of the CS (Chronic Standard) for Fish Early Life Stages Present

AUTHORITY:  Implementing Section 13 and authorized by Sections 11(b) and 27 of the Environmental Protection Act [415 ILCS 5/13, 11(b), and 27]

SOURCE:  Filed with the Secretary of State January 1, 1978; amended at 2 Ill. Reg. 44, p. 151, effective November 2, 1978; amended at 3 Ill. Reg. 20, p. 95, effective May 17, 1979; amended at 3 Ill. Reg. 25, p. 190, effective June 21, 1979; codified at 6 Ill. Reg. 7818; amended at 6 Ill. Reg. 11161, effective September 7, 1982; amended at 6 Ill. Reg. 13750, effective October 26, 1982; amended at 8 Ill. Reg. 1629, effective January 18, 1984; peremptory amendments at 10 Ill. Reg. 461, effective December 23, 1985; amended at R87-27 at 12 Ill. Reg. 9911, effective May 27, 1988; amended at R85-29 at 12 Ill. Reg. 12082, effective July 11, 1988; amended in R88-1 at 13 Ill. Reg. 5998, effective April 18, 1989; amended in R88-21(A) at 14 Ill. Reg. 2899, effective February 13, 1990; amended in R88-21(B) at 14 Ill. Reg. 11974, effective July 9, 1990; amended in R94-1(A) at 20 Ill. Reg. 7682, effective May 24, 1996; amended in R94-1(B) at 21 Ill. Reg. 370, effective December 23, 1996; expedited correction at 21 Ill. Reg. 6273, effective December 23, 1996; amended in R97-25 at 22 Ill. Reg. 1356, effective December 24, 1997; amended in R99-8 at 23 Ill. Reg. 11249, effective August 26, 1999; amended in R01-13 at 26 Ill. Reg. 3505, effective February 22, 2002; amended in R02-19 at 26 Ill. Reg. 16931, effective November 8, 2002; amended in R02-11 at           Ill. Reg.             , effective                                         .

## SUBPART A:  GENERAL WATER QUALITY PROVISIONS

Section 302.105	Antidegradation

The purpose of this Section is to protect existing uses of all waters of the State of Illinois, maintain the quality of waters with quality that is better than water quality standards, and prevent unnecessary deterioration of waters of the State.

a)	Existing Uses

Uses actually attained in a surface water body or water body segment on or after November 28, 1975, whether or not they are included in the water quality standards, must be maintained and protected.  Examples of degradation of existing uses of the waters of the State include:

1)	an action that would result in the deterioration of the existing aquatic community, such as a shift from a community of predominantly pollutant-sensitive species to pollutant-tolerant species or a loss of species diversity;

2)	an action that would result in a loss of a resident or indigenous species whose presence is necessary to sustain commercial or recreational activities; or

3)	an action that would preclude continued use of a surface water body or water body segment for a public water supply or for recreational or commercial fishing, swimming, paddling or boating.

- b)	Outstanding Resource Waters

1)	Waters that are designated as Outstanding Resource Waters (ORWs) pursuant to 35 Ill. Adm. Code 303.205 and listed in 35 Ill. Adm. Code 303.206 must not be lowered in quality except as provided below:

A)	Activities that result in short-term, temporary (i.e., weeks or months) lowering of water quality in an ORW; or

B)	Existing site stormwater discharges that comply with applicable federal and State stormwater management regulations and do not result in a violation of any water quality standards.

2)	Any activity in subsection (b)(1)(A) or (b)(1)(B) that requires a National Pollutant Discharge Elimination System (NPDES) or a Clean Water Act (CWA) Section 401 certification must also comply with subsection (c)(2).

3)	Any activity listed in subsection (b)(1) or any other proposed increase in pollutant loading to an ORW must also meet the following requirements:

A)	All existing uses of the water will be fully protected; and

B)	Except for activities falling under one of the exceptions provided in subsection (b)(1)(A) or (B) above:,

i)	The proposed increase in pollutant loading is necessary for an activity that will improve water quality in the ORW; and

ii)	The improvement could not be practicably achieved without the proposed increase in pollutant loading.

4)	Any proposed increase in pollutant loading requiring an NPDES permit or a CWA 401 certification for an ORW must be assessed pursuant to subsection (f) to determine compliance with this Section.

c)	High Quality Waters

1)	Except as otherwise provided in subsection (d) of this Section, waters of the State whose existing quality is better than any of the established standards of this Part must be maintained in their present high quality, unless the lowering of water quality is necessary to accommodate important economic or social development.

2)	The Agency must assess any proposed increase in pollutant loading that necessitates a new, renewed or modified NPDES permit or any activity requiring a CWA Section 401 certification to determine compliance with this Section.  The assessment to determine compliance with this Section must be made on a case-by-case basis.  In making this assessment, the Agency must:

A)	Consider the fate and effect of any parameters proposed for an increased pollutant loading.

B)	Assure the following:

i)	The applicable numeric or narrative water quality standard will not be exceeded as a result of the proposed activity;

ii)	All existing uses will be fully protected;

iii)	All technically and economically reasonable measures to avoid or minimize the extent of the proposed increase in pollutant loading have been incorporated into the proposed activity; and

iv)	The activity that results in an increased pollutant loading will benefit the community at large.

C)	Utilize the following information sources, when available:

i)	Information, data or reports available to the Agency from its own sources;

ii)	Information, data or reports supplied by the applicant;

iii)	Agency experience with factually similar permitting scenarios; and

iv)	Any other valid information available to the Agency.

d)	Activities Not Subject to a Further Antidegradation Assessment

The following activities will not be subject to a further antidegradation assessment pursuant to subsection (c) of this Section.

1)	Short-term, temporary (i.e., weeks or months) lowering of water quality;

2)	Bypasses that are not prohibited at 40 CFR 122.41(m);

3)	Response actions pursuant to the Comprehensive Environmental Response, Compensation and Liability Act (CERCLA), as amended, corrective actions, pursuant to the Resource Conservation and Recovery Act (RCRA), as amended, or similar federal or State authority, taken to alleviate a release into the environment of hazardous substances, pollutants or contaminants which may pose a danger to public health or welfare;

4)	Thermal discharges that have been approved through a CWA Section 316(a) demonstration;

5)	New or increased discharges of a non-contact cooling water:

A)	without additives, except as provided in subsection (d)(5)(B), returned to the same body of water from which it was taken, as defined by 35 Ill. Adm. Code 352.104, provided that the discharge complies with applicable Illinois thermal standards; or

B)	containing chlorine when the non-contact cooling water is treated to remove residual chlorine, and returned to the same body of water from which it was taken, as defined in 35 Ill. Adm. Code 352.104, provided that the discharge complies with applicable Illinois thermal and effluent standards at 35 Ill. Adm. Code 302, 303, and 304;

6)	Discharges permitted under a current general NPDES permit as provided by 415 ILCS 5/39(b) or a nationwide or regional CWA Section 404 permit are not subject to facility-specific antidegradation review; however, the Agency must assure that individual permits or certifications are required prior to all new pollutant loadings or hydrological modifications that necessitate a new, renewed or modified NPDES permit or CWA Section 401 certification that affects waters of particular biological significance.  Waters of particular biological significance may include streams listed in a 1991 publication by the Illinois Department of Conservation entitled “Biologically Significant Illinois Streams”; or

7)	Changes to or inclusion of a new permit limitation that does not result in an actual increase of a pollutant loading, such as those stemming from improved monitoring data, new analytical testing methods, new or revised technology or water quality based effluent limits.

e)	Lake Michigan Basin

Waters in the Lake Michigan basin as identified in 35 Ill. Adm. Code 303.443 are also subject to the requirements applicable to bioaccumulative chemicals of concern found at Section 302.521 of this Part.

f)	Antidegradation Assessments

In conducting an antidegradation assessment pursuant to this Section, the Agency must comply with the following procedures.

1)	A permit application for any proposed increase in pollutant loading that necessitates the issuance of a new, renewed, or modified NPDES permit or a CWA Section 401 certification must include, to the extent necessary for the Agency to determine that the permit application meets the requirements of this Section, the following information:

A)	Identification and characterization of the water body affected by the proposed load increase or proposed activity and the existing water body’s uses.  Characterization must address physical, biological and chemical conditions of the water body.

B)	Identification and quantification of the proposed load increases for the applicable parameters and of the potential impacts of the proposed activity on the affected waters.

C)	The purpose and anticipated benefits of the proposed activity. Such benefits may include:

i)	Providing a centralized wastewater collection and treatment system for a previously unsewered community;

ii)	Expansion to provide service for anticipated residential or industrial growth consistent with a community’s long range urban planning;

iii)	Addition of a new product line or production increase or modification at an industrial facility; or

iv)	An increase or the retention of current employment levels at a facility.

D)	Assessments of alternatives to proposed increases in pollutant loading or activities subject to Agency certification pursuant to Section 401 of the CWA that result in less of a load increase, no load increase or minimal environmental degradation.  Such alternatives may include:

i)	Additional treatment levels, including no discharge alternatives;

ii)	Discharge of waste to alternate locations, including publicly-owned treatment works and streams with greater assimilative capacity; or

iii)	Manufacturing practices that incorporate pollution prevention techniques.

E)	Any additional information the Agency may request.

F)	Proof that a copy of the application has been provided to the Illinois Department of Natural Resources.

2)	The Agency must complete an antidegradation assessment in accordance with the provisions of this Section on a case-by-case basis.

A)	The Agency must consider the criteria stated in Section 302.105(c)(2).

B)	The Agency must consider the information provided by the applicant pursuant to subsection (f)(1).

C)	After its assessment, the Agency must produce a written analysis addressing the requirements of this Section and provide a decision yielding one of the following results:

i)	If the proposed activity meets the requirements of this Section, then the Agency must proceed with public notice of the NPDES permit or CWA Section 401 certification and include the written analysis as a part of the fact sheet accompanying the public notice;

ii)	If the proposed activity does not meet the requirements of this Section, then the Agency must provide a written analysis to the applicant and must be available to discuss the deficiencies that led to the disapproval.  The Agency may suggest methods to remedy the conflicts with the requirements of this Section;

iii)	If the proposed activity does not meet the requirements of this Section, but some lowering of water quality is allowable, then the Agency will contact the applicant with the results of the review.  If the reduced loading increase is acceptable to the applicant, upon the receipt of an amended application, the Agency will proceed to public notice; or if the reduced loading increase is not acceptable to the applicant, the Agency will transmit its written review to the applicant in the context of an a NPDES permit denial or a CWA Section 401 certification denial.

3)	The Agency will conduct public notice and public participation through the public notice procedures found in 35 Ill. Adm. Code 309.109 or CWA Section 401 certifications.  The Agency must incorporate the following information into a fact sheet accompanying the public notice:

A)	A description of the activity, including identification of water quality parameters for which there will be an increased pollutant loading;

B)	Identification of the affected surface water body or water body segment, any downstream surface water body or water body segment also expected to experience a lowering of water quality, characterization of the designated and current uses of the affected surface water body or water body segment segments and identification of which uses are most sensitive to the proposed load increase;

C)	A summary of any review comments and recommendations provided by Illinois Department of Natural Resources, local or regional planning commissions, zoning boards and any other entities the Agency consults regarding the proposal;

D)	An overview of alternatives considered by the applicant and identification of any provisions or alternatives imposed to lessen the load increase associated with the proposed activity; and

E)	The name and telephone number of a contact person at the Agency who can provide additional information.

(Amended at ____ Ill. Reg. ________, effective ___________________________)

### SUBPART B:  GENERAL USE WATER QUALITY STANDARDS

#### Section 302.208	Numeric Standards for Chemical Constituents

a)	The acute standard (AS) for the chemical constituents listed in subsection (e) shall not be exceeded at any time except as provided in subsection (d).

b)	The chronic standard (CS) for the chemical constituents listed in subsection (e) shall not be exceeded by the arithmetic average of at least four consecutive samples collected over any period of at least four days, except as provided in subsection (d).  The samples used to demonstrate attainment compliance or lack of attainment compliance with a CS must be collected in a manner that which assures an average representative of the sampling period.  For the metals that have water quality based standards dependent upon hardness, the chronic water quality standard will be calculated according to subsection (e) using the hardness of the water body at the time the metals sample was collected.  To calculate attainment status of chronic metals standards, the concentration of the metal in each sample is divided by the calculated water quality standard for the sample to determine a quotient.  The water quality standard is attained if the mean of the sample quotients is less than or equal to one for the duration of the averaging period.

c)	The human health standard (HHS) for the chemical constituents listed in subsection (f) shall not be exceeded when the stream flow is at or above the harmonic mean flow pursuant to Section 302.658 nor shall an annual average, based on at least eight samples, collected in a manner representative of the sampling period, exceed the HHS except as provided in subsection (d).

d)	In waters where mixing is allowed pursuant to Section 302.102, the following apply:

1)	The AS shall not be exceeded in any waters except for those waters for which the Agency has approved a zone of initial dilutions (ZID) pursuant to Section 302.102.

2)	The CS shall not be exceeded outside of waters in which mixing is allowed pursuant to Section 302.102.

3)	The HHS shall not be exceeded outside of waters in which mixing is allowed pursuant to Section 302.102.

e)	Numeric Water Quality Standards for the Protection of Aquatic Organisms

| Constituent | STORET Storet Number | AS (µg/L) (ug/L) | CS (µg/L) (ug/L) |
| --- | --- | --- | --- |
| Arsenic (trivalent, dissolved) (total) | 22680 01002 | 360 X 1.0*=360 | 190 X 1.0*=190 |
| Cadmium (dissolved) (total) | 01025 01027 | exp[A+Bln(H)] X {1.138672-[(lnH)(0.041838)]}*, but not to exceed 50 ug/L, where A=-2.918 and B=1.128 | exp[A+Bln(H)] X {1.101672-[(lnH)(0.041838)]}*, where A=-3.490 and B=0.7852 |
| Chromium (hexavalent, total) (total hexavalent) | 01032 | 16 | 11 |
| Chromium (trivalent, dissolved) (total trivalent) | 80357 01033 | exp[A+Bln(H)] X 0.316*, where A=3.688 and B=0.8190 | exp[A+Bln(H)] X 0.860*, where A=1.561 and B=0.8190 |
| Copper (dissolved) (total) | 01040 01042 | exp[A+Bln(H)] X 0.960*, where A=-1.464 and B=0.9422 | exp[A+Bln(H)] X 0.960*. where A=-1.465 and B=0.8545 |
| Cyanide | 00718 | 22 | 5.2 |
| Lead (dissolved) (total) | 01049 01051 | exp[A+Bln(H)] X {1.46203-[(lnH)(0.145712)]}*, where A=-1.301 and B=1.273 | exp[A+Bln(H)] X {1.46203-[(lnH)(0.145712)]}*, where A=-2.863 and B=1.273 |
| Mercury (dissolved) | 71890 71900 | 2.6 X 0.85*=2.2 | 1.3 X 0.85*=1.1 |
| Nickel (dissolved) | 01065 | exp[A+Bln(H)] X 0.998*, where A=0.5173 and B=0.8460 | exp[A+Bln(H)] X 0.997*, where A=-2.286 and B=0.8460 |
| TRC | 500600 | 19 | 11 |
| Zinc (dissolved) | 01090 | exp[A+Bln(H)] X 0.978*, where A=0.9035 and B=0.8473 | Exp[A+Bln(H)] X 0.986*, where A=-0.8165 and B=0.8473 |
| Benzene | 78124 | 4200 | 860 |
| Ethylbenzene | 78113 | 150 | 14 |
| Toluene | 78131 | 2000 | 600 |
| Xylene(s) | 81551 | 920 | 360 |

where: µg/L ug/L  =  microgram per liter,

exp[x] = base natural neutral logarithms raised to the x- power, and

ln(H) = natural logarithm of Hardness (STORET 00900), and .

* = conversion factor multiplier for dissolved metals

f)	Numeric Water Quality Standard for the Protection of Human Health

| Constituent | STORET Number | (µg/L) (ug/L) |
| --- | --- | --- |
| Mercury | 71900 | 0.012 |
| Benzene | 78124 | 310 |

where: µg/L ug/L = micrograms per liter

g)	Concentrations of the following chemical constituents shall not be exceeded except in waters for which mixing is allowed pursuant to Section 302.102.

| Constituent | Unit | STORET Number | Standard |
| --- | --- | --- | --- |
| Barium (total) | mg/L | 01007 | 5.0 |
| Boron (total) | mg/L | 01022 | 1.0 |
| Chloride (total) | mg/L | 00940 | 500. |
| Fluoride | mg/L | 00951 | 1.4 |
| Iron (dissolved) | mg/L | 01046 | 1.0 |
| Manganese (total) | mg/L | 01055 | 1.0 |
| Nickel (total) | mg/L | 01067 | 1.0 |
| Phenols | mg/L | 32730 | 0.1 |
| Selenium (total) | mg/L | 01147 | 1.0 |
| Silver (total) | µg/L ug/L | 01077 | 5.0 |
| Sulfate | mg/L | 00945 | 500. |
| Total Dissolved Solids | mg/L | 70300 | 1000. |
| Zinc (total) | mg/L | 01092 | 1.0 |

where: mg/L = milligram per liter and

µg/L ug/L = microgram per liter

(Source:  Amended at __________ Ill. Reg. _______________, effective ________________)

SUBPART E:  LAKE MICHIGAN BASIN WATER QUALITY STANDARDS

#### Section 302.504	Chemical Constituents

The following concentrations of chemical constituents must not be exceeded, except as provided in Sections 302.102 and 302.530:

a)	The following standards must be met in all waters of the Lake Michigan Basin.  Acute aquatic life standards (AS) must not be exceeded at any time except for those waters for which the Agency has approved a zone of initial dilution (ZID) pursuant to Sections 302.102 and 302.530.  Chronic aquatic life standards (CS) and human health standards (HHS) must not be exceeded outside of waters in which mixing is allowed pursuant to Section 302.102 and 302.530 by the arithmetic average of at least four consecutive samples collected over a period of at least four days.  The samples used to demonstrate compliance with the CS or HHS must be collected in a manner which assures an average representation of the sampling period.

| Constituent | STORET Number | Unit | AS | CS | HHS |
| --- | --- | --- | --- | --- | --- |
| Arsenic (Trivalent, dissolved) | 22680 | g/L | 340 X 1.0*=340 | 148 X 1.0*=48 | NA |
| Cadmium (dissolved) | 01025 | g/L | exp[A +Bln(H)] X {1.138672-[(lnH)(0.041838)]}*, where A=-3.6867 and B=1.128 | exp[A +Bln(H)] X {1.101672-[(lnH)(0.041838)]}*, where A=-2.715 and B=0.7852 | NA |
| Chromium (Hexavalent, total) | 01032 | g/L | 16 | 11 | NA |
| Chromium (Trivalent, dissolved) | 80357 | g/L | exp[A +Bln(H)] X 0.316*, where A=3.7256 and B=0.819 | exp[A +Bln(H)] X 0.860*, where A=0.6848 and B=0.819 | NA |
| Copper (dissolved) | 01040 | g/L | exp[A +Bln(H)] X 0.960*, where A=-1.700 and B=0.9422 | exp[A +Bln(H)] X 0.960*, where A=-1.702 and B=0.8545 | NA |
| Cyanide (Weak acid dissociable) | 00718 | g/L | 22 | 5.2 | NA |
| Lead (dissolved) | 01049 | g/L | exp[A +Bln(H)] X {1.46203-[(lnH)(0.145712)]}*, where A=-1.055 and B=1.273 | exp[A +Bln(H)] X {1.46203-[(lnH)(0.145712)]}*, where A=-4.003 and B=1.273 | NA |
| Nickel (dissolved) | 01065 | g/L | exp[A +Bln(H)] X 0.998*, where A=2.255 and B=0.846 | exp[A +Bln(H)] X 0.997*, where A=0.0584 and B=0.846 | NA |
| Selenium (dissolved) | 01145 | g/L | NA | 5.0 | NA |
| TRC | 50060 | g/L | 19 | 11 | NA |
| Zinc (dissolved) | 01090 | g/L | exp[A +Bln(H)] X 0.978*, where A=0.884 and B=0.8473 | exp[A +Bln(H)] X 0.986*, where A=0.884 and B=0.8473 | NA |
| Benzene | 78124 34030 | g/L | 3900 NA | 800 NA | 310 |
| Chlorobenzene | 34301 | mg/L | NA | NA | 3.2 |
| 2,4-Dimethylphenol | 34606 | mg/L | NA | NA | 8.7 |
| 2,4-Dinitrophenol | 03756 | mg/L | NA | NA | 2.8 |
| Endrin | 39390 | g/L | 0.086 | 0.036 | NA |
| Ethylbenzene | 78113 | µg/L | 150 | 14 | NA |
| Hexachloroethane | 34396 | g/L | NA | NA | 6.7 |
| Methylene chloride | 34423 | mg/L | NA | NA | 2.6 |
| Parathion | 39540 | g/L | 0.065 | 0.013 | NA |
| Pentachlorophenol | 03761 | g/L | exp B ([pH] +A), where A=-4.869 and B=1.005 | exp B ([pH] +A), where A=-5.134 and B=1.005 | NA |
| Toluene | 78131 | mg/L | 2000 NA | 610 NA | 51.0 |
| Tricholroethylene | 39180 | g/L | NA | NA | 370 |
| Xylene(s) | 81551 | g/L | 1200 | 490 | NA |

Where:

NA = Not Applied

Exp[x] = base of natural logarithms

raised to the x-power

ln(H) = natural logarithm of Hardness

(STORET 00900)

* = conversion factor multiplier for dissolved metals

b)	The following water quality standards must not be exceeded at any time in any waters of the Lake Michigan Basin, unless a different standard is specified under subsection (c) of this Section.

| Constituent | STORET Number | Unit | Water Quality Standard |
| --- | --- | --- | --- |
| Barium (total) | 01007 | mg/L | 5.0 |
| Boron (total) | 01022 | mg/L | 1.0 |
| Chloride (total) | 00940 | mg/L | 500 |
| Fluoride | 00951 | mg/L | 1.4 |
| Iron (dissolved) | 01046 | mg/L | 1.0 |
| Manganese (total) | 01055 | mg/L | 1.0 |
| Phenols | 32730 | mg/L | 0.1 |
| Sulfate | 00945 | mg/L | 500 |
| Total Dissolved Solids | 70300 | mg/L | 1000 |

c)	In addition to the standards specified in subsections (a) and (b) of this Section, the following standards must not be exceeded at any time in the Open Waters of Lake Michigan as defined in Section 302.501.

| Constituent | STORET Number | Unit | Water Quality Standard |
| --- | --- | --- | --- |
| Arsenic (total) | 01002 | g/L | 50.0 |
| Barium (total) | 01007 | mg/L | 1.0 |
| Chloride | 00940 | mg/L | 12.0 |
| Iron (dissolved) | 01046 | mg/L | 0.30 |
| Lead (total) | 01051 | g/L | 50.0 |
| Manganese (total) | 01055 | mg/L | 0.15 |
| Nitrate-Nitrogen | 00620 | mg/L | 10.0 |
| Phosphorus | 00665 | g/L | 7.0 |
| Selenium (total) | 01147 | g/L | 10.0 |
| Sulfate | 00945 | mg/L | 24.0 |
| Total Dissolved Solids | 70300 | mg/L | 180.0 |
| Oil (hexane solubles or equivalent) | 00550, 00556 or 00560 | mg/L | 0.10 |
| Phenols | 32730 | g/L | 1.0 |

d)	In addition to the standards specified in subsections (a), (b) and (c) of this Section, the following human health standards (HHS) must not be exceeded in the Open Waters of Lake Michigan as defined in Section 302.501 by the arithmetic average of at least four consecutive samples collected over a period of at least four days.  The samples used to demonstrate compliance with the HHS must be collected in a manner which assures an average representation of the sampling period.

| Constituent | STORET Number | Unit | Water Quality Standard |
| --- | --- | --- | --- |
| Benzene | 34030 | g/L | 12.0 |
| Chlorobenzene | 34301 | g/L | 470.0 |
| 2,4-Dimethylphenol | 34606 | g/L | 450.0 |
| 2,4-Dinitrophenol | 03757 | g/L | 55.0 |
| Hexachloroethane (total) | 34396 | g/L | 5.30 |
| Lindane | 39782 | g/L | 0.47 |
| Methylene chloride | 34423 | g/L | 47.0 |
| Toluene | 78131 | mg/L | 5.60 |
| Trichloroethylene | 39180 | g/L | 29.0 |

e)	For the following bioaccumulative chemicals of concern (BCCs), acute aquatic life standards (AS) must not be exceeded at any time in any waters of the Lake Michigan Basin and chronic aquatic life standards (CS), human health standards (HHS), and wildlife standards (WS) must not be exceeded in any waters of the Lake Michigan Basin by the arithmetic average of at least four consecutive samples collected over a period of at least four days subject to the limitations of Sections 302.520 and 302.530.  The samples used to demonstrate compliance with the HHS and WS must be collected in a manner that assures an average representation of the sampling period.

| Constituent | STORETNumber | Unit | AS | CS | HHS | WS |
| --- | --- | --- | --- | --- | --- | --- |
| Mercury (total) | 71900 | ng/L | 1,700 | 910 | 3.1 | 1.3 |
| Chlordane | 39350 | ng/L | NA | NA | 0.25 | NA |
| DDT and metabolites | 39370 | pg/L | NA | NA | 150 | 11.0 |
| Dieldrin | 39380 | ng/L | 240 | 56 | 0.0065 | NA |
| Hexachlorobenzene | 39700 | ng/L | NA | NA | 0.45 | NA |
| Lindane | 39782 | g/L | 0.95 | NA | 0.5 | NA |
| PCBs (class) | 79819 | pg/L | NA | NA | 26 | 120 |
| 2,3,7,8-TCDD | 03556 | fg/L | NA | NA | 8.6 | 3.1 |
| Toxaphene | 39400 | pg/L | NA | NA | 68 | NA |

Where:	mg/L = milligrams per liter (10-3 grams per liter)

g/L = micrograms per liter (10-6 grams per liter)

ng/L = nanograms per liter (10-9 grams per liter)

pg/L = picograms per liter (10-12 grams per liter)

fg/L = femtograms per liter (10-15 grams per liter)

NA = Not Applied

(Source:  Amended at __________ Ill. Reg. _______________, effective ________________)

Section 302.575	Procedures for Deriving Tier I Water Quality Criteria and Values in the Lake Michigan Basin to Protect Wildlife

The Lake Michigan Basin Wildlife Criterion (LMWC) is the concentration of a substance which if not exceeded protects Illinois wild mammal and bird populations from adverse effects resulting from ingestion of surface waters of the Lake Michigan Basin and from ingestion of aquatic prey organisms taken from surface waters of the Lake Michigan Basin.  Wildlife criteria calculated under this Section protect against long-term effects and are therefore considered chronic criteria.  The methodology involves utilization of data from test animals to derive criteria to protect representative or target species: bald eagle, herring gull, belted kingfisher, mink and river otter.  The lower of the geometric mean of species specific criteria for bird species or mammal species is chosen as the LMWC to protect a broad range of species.

a)	This method shall also be used for non-BCCs when appropriately modified to consider the following factors:

1)	Selection of scientifically justified target species;

2)	Relevant routes of chemical exposure;

3)	Pertinent toxicity endpoints.

b)	Minimum data requirements:

1)	Test dose (TD).  In order to calculate a LMWC the following minimal data base is required:

A)	There must be at least one data set showing dose-response for oral, subchronic, or chronic exposure of 28 days for one bird species; and

B)	There must be at least one data set showing dose-response for oral, subchronic, or chronic exposure of 90 days for one mammal species.

2)	Bioaccumulation Factor (BAF) data requirements:

A)	For any chemical with a BAF of less than 125 the BAF may be obtained by any method; and

B)	For chemicals with a BAF of greater than 125 the BAF must come from a field measured BAF or Biota-Sediment Accumulation Factor (BSAF).

c)	Principles for development of criteria

1)	Dose standardization.  The data for the test species must be expressed as, or converted to, the form mg/kg/d  utilizing the guidelines for drinking and feeding rates and other procedures in 40 CFR 132, incorporated by reference at Section 302.510.

2)	Uncertainty factors (UF) for utilizing test dose data in the calculation of the target species value (TSV);.

A)	Correction for intermittent exposure.  If the animals used in a study were not exposed to the toxicant each day of the test period, the no observed adverse effect level (NOAEL) must be multiplied by the ratio of days of exposure to the total days in the test period.

B)	Correction from the lowest observed adverse effect level (LOAEL) to NOAEL (UFl).  For those substances for which a LOAEL has been derived, the UF1 shall not be less than one and should not exceed 10.

C)	Correction for subchronic to chronic extrapolation (UFs).  In instances where only subchronic data are available, the TD may be derived from subchronic data.  The value of the UFs shall not be less than one and should not exceed 10.

D)	Correction for interspecies extrapolations (UFa).  For the derivation of criteria, a UFa shall not be less than one and should not exceed 100.  The UFa shall be used only for extrapolating toxicity data across species within a taxonomic class.  A species specific UFa shall be selected and applied to each target species, consistent with the equation in subsection (d) below.

d)	Calculation of TSV.  The TSV, measured in milligrams per liter (mg/L), is calculated according to the equation:

TSV = { [TD x Wt] / [UFa x UFs x UFl] }  /  { W +    [FTLi x BAFWLTLi] }

Where:

TSV = target species value in milligrams of substance per liter (mg/L).

TD = test dose that is toxic to the test species, either NOAEL or LOAEL.

UFa = the uncertainty factor for extrapolating toxicity data across species (unitless).  A species-specific UFa shall be selected and applied to each target species, consistent with the equation.

UFs = the uncertainty factor for extrapolating from subchronic to chronic exposures (unitless).

UFl = the uncertainty factor for extrapolation from LOAEL to NOAEL (unitless)

Wt = average weight in kilograms (kg) of the target species.

W = average daily volume of water in liters consumed per day (L/d) by the target species.

FTLi = average daily amount of food consumed by the target species in kilograms (kg/d) for trophic level i.

BAFWLTLi = aquatic life bioaccumulation factor with units of liter per kilogram (L/kg), as derived from in Section 302.570 for trophic level i.

e)	Calculation of the Lake Michigan Basin Wildlife Criterion.  TSVs are obtained for each target species.  The geometric mean TSVs of all mammal species is calculated and also of all bird species.  The LMWC is the lower of the bird or mammal geometric mean TSV.

(Source:  Amended __________ Ill. Reg. _______________, effective ________________)

TITLE 35:  ENVIRONMENTAL PROTECTION

SUBTITLE C:  WATER POLLUTION

CHAPTER I:  POLLUTION CONTROL BOARD

TITLE 35:  ENVIRONMENTAL PROTECTION

SUBTITLE C:  WATER POLLUTION

## CHAPTER I:  POLLUTION CONTROL BOARD

### PART 304

### EFFLUENT STANDARDS

SUBPART A:  GENERAL EFFLUENT STANDARDS

Section

| 304.101 | Preamble |
| --- | --- |
| 304.102 | Dilution |
| 304.103 | Background Concentrations |
| 304.104 | Averaging |
| 304.105 | Violation of Water Quality Standards |
| 304.106 | Offensive Discharges |
| 304.120 | Deoxygenating Wastes |
| 304.121 | Bacteria |
| 304.122 | Total Ammonia Nitrogen (as N: STORET number 00610) |
| 304.123 | Phosphorus (STORET number 00665) |
| 304.124 | Additional Contaminants |
| 304.125 | pH |
| 304.126 | Mercury |
| 304.140 | Delays in Upgrading (Repealed) |
| 304.141 | NPDES Effluent Standards |
| 304.142 | New Source Performance Standards (Repealed) |

SUBPART B:  SITE SPECIFIC RULES AND EXCEPTIONS NOT OF GENERAL APPLICABILITY

Section

| 304.201 | Wastewater Treatment Plant Discharges of the Metropolitan Water Reclamation District of Greater Chicago |
| --- | --- |
| 304.202 | Chlor-alkali Mercury Discharges in St. Clair County |
| 304.203 | Copper Discharges by Olin Corporation |
| 304.204 | Schoenberger Creek: Groundwater Discharges |
| 304.205 | John Deere Foundry Discharges |
| 304.206 | Alton Water Company Treatment Plant Discharges |
| 304.207 | Galesburg Sanitary District Deoxygenating Wastes Discharges |
| 304.208 | City of Lockport Treatment Plant Discharges |
| 304.209 | Wood River Station Total Suspended Solids Discharges |
| 304.210 | Alton Wastewater Treatment Plant Discharges |
| 304.211 | Discharges From Borden Chemicals and Plastics Operating Limited Partnership Into an Unnamed Tributary of Long Point Slough |
| 304.212 | Sanitary District of Decatur Discharges |
| 304.213 | PDV Midwest Refining, L.L.C. Refinery Ammonia Discharge |
| 304.214 | Mobil Oil Refinery Ammonia Discharge |
| 304.215 | City of Tuscola Wastewater Treatment Facility Discharges |
| 304.216 | Newton Station Suspended Solids Discharges |
| 304.218 | City of Pana Phosphorus Discharge |
| 304.219 | North Shore Sanitary District Phosphorus Discharges |
| 304.220 | East St. Louis Treatment Facility, Illinois-American Water Company |
| 304.221 | Ringwood Drive Manufacturing Facility in McHenry County |
| 304.222 | Intermittent Discharge of TRC |

SUBPART C:  TEMPORARY EFFLUENT STANDARDS

Section

| 304.301 | Exception for Ammonia Nitrogen Water Quality Violations (Repealed) |
| --- | --- |
| 304.302 | City of Joliet East Side Wastewater Treatment Plant |
| 304.303 | Amerock Corporation, Rockford Facility |

| Appendix A | References to Previous Rules |
| --- | --- |

AUTHORITY:  Implementing Section 13 and authorized by Section 27 of the Environmental Protection Act [415 ILCS 5/13 and 27].

SOURCE:  Filed with the Secretary of State January 1, 1978; amended at 2 Ill. Reg. 30, p. 343, effective July 27, 1978; amended at 2 Ill. Reg. 44, p. 151, effective November 2, 1978; amended at 3 Ill. Reg. 20, p. 95, effective May 17, 1979; amended at 3 Ill. Reg. 25, p. 190, effective June 21, 1979; amended at 4 Ill. Reg. 20, p. 53 effective May 7, 1980; amended at 6 Ill. Reg. 563, effective December 24, 1981; codified at 6 Ill. Reg. 7818: amended at 6 Ill. Reg. 11161, effective September 7, 1982; amended at 6 Ill. Reg. 13750, effective October 26, 1982; amended at 7 Ill. Reg. 3020, effective March 4, 1983; amended at 7 Ill. Reg. 8111, effective June 23, 1983; amended at 7 Ill. Reg. 14515, effective October 14, 1983; amended at 7 Ill. Reg. 14910, effective November 14, 1983; amended at 8 Ill. Reg. 1600, effective January 18, 1984; amended at 8 Ill. Reg. 3687, effective March 14, 1984; amended at 8 Ill. Reg. 8237, effective June 8, 1984; amended at 9 Ill. Reg. 1379, effective January 21, 1985; amended at 9 Ill. Reg. 4510, effective March 22, 1985; peremptory amendment at 10 Ill. Reg. 456, effective December 23, 1985; amended at 11 Ill. Reg. 3117, effective January 28, 1987; amended in R84-13 at 11 Ill. Reg. 7291 effective April 3, 1987; amended in R86-17(A) at 11 Ill. Reg. 14748, effective August 24, 1987; amended in R84-16 at 12 Ill. Reg. 2445, effective January 15, 1988; amended in R83-23 at 12 Ill. Reg. 8658, effective May 10, 1988; amended in R87-27 at 12 Ill. Reg. 9905, effective May 27, 1988; amended in R82-7 at 12 Ill. Reg. 10712, effective June 9, 1988; amended in R85-29 at 12 Ill. Reg. 12064, effective July 12, 1988; amended in R87-22 at 12 Ill. Reg. 13966, effective August 23, 1988; amended in R86-3 at 12 Ill. Reg. 20126, effective November 16, 1988; amended in R84-20 at 13 Ill. Reg. 851, effective January 9, 1989; amended in R85-11 at 13 Ill. Reg. 2060, effective February 6, 1989; amended in R88-1 at 13 Ill. Reg. 5976, effective April 18, 1989; amended in R86-17(B) at 13 Ill. Reg. 7754, effective May 4, 1989; amended in R88-22 at 13 Ill. Reg. 8880, effective May 26, 1989; amended in R87-6 at 14 Ill. Reg. 6777, effective April 24, 1990; amended in R87-36 at 14 Ill. Reg. 9437, effective May 31, 1990; amended in R88-21(B) at 14 Ill. Reg. 12538, effective July 18, 1990; amended in R84-44 at 14 Ill. Reg. 20719, effective December 11, 1990; amended in R86-14 at 15 Ill. Reg. 241, effective December 18, 1990; amended in R93-8 at 18 Ill. Reg. 267, effective December 23, 1993; amended in R87-33 at 18 Ill. Reg. 11574, effective July 7, 1994; amended in R95-14 at 20 Ill. Reg. 3528, effective February 8, 1996; amended in R94-1(B) at 21 Ill. Reg. 364, effective December 23, 1996; expedited correction in R94-1(B) at 21 Ill. Reg. 6269, effective December 23, 1996; amended in R97-25 at 22 Ill. Reg. 1351, effective December 24, 1997; amended in R97-28 at 23 Ill. Reg. 3512, effective February 3, 1998; amended in R98-14 at 23 Ill. Reg.687, effective December 31, 1998; amended in R02-19 at 26 Ill. Reg. 16948, effective November 8, 2002; amended in R02-11 at           Ill. Reg.             , effective                                         .

SUBPART A:  GENERAL EFFLUENT STANDARDS

#### Section 304.120	Deoxygenating Wastes

Except as provided in 35 Ill. Adm. Code 306.SubpartC Section 306.103, all effluents containing deoxygenating wastes shall meet the following standards:

a)	No effluent shall exceed 30 mg/L mg/l of five day biochemical oxygen demand (BOD5) (STORET number 00310) or 30 mg/L mg/l of suspended solids (STORET number 00530), except that treatment works employing three stage lagoon treatment systems which are properly designed, maintained and operated, and whose effluent has a dilution ratio no less than five to one or who qualify for exceptions under subsection (c) shall not exceed 37 mg/L mg/l of suspended solids.

b)	No effluent from any source whose untreated waste load is 10,000 population equivalents or more, or from any source discharging into the Chicago River System or into the Calumet River System, shall exceed 20 mg/L mg/l of BOD5 or 25 mg/L mg/l of suspended solids.

c)	No effluent whose dilution ratio is less than five to one shall exceed 10 mg/L mg/l of BOD5 or 12 mg/L mg/l of suspended solids, except that sources employing third-stage treatment lagoons shall be exempt from this subsection (c) provided all of the following conditions are met:

1)	The waste source qualifies under one of the following categories:

A)	Any wastewater treatment works with an untreated waste load less than 2500 population equivalents, which is sufficiently isolated that combining with other sources to aggregate 2500 population equivalents or more is not practicable.

B)	Any wastewater treatment works in existence and employing third-stage treatment lagoons on January 1, 1986, whose untreated waste load is 5000 population equivalents or less and sufficiently isolated that combining to aggregate 5000 population equivalents or more is not practicable.

C)	Any wastewater treatment works with an untreated waste load of 5000 population equivalents or less, which has reached the end of its useful life by January 1, 1987, and is sufficiently isolated that combining to aggregate 5000 population equivalents or more is not practicable.

D)	Any wastewater treatment works with an untreated waste load of 5000 population equivalents or less which has reached the end of its useful life and which has received an adjusted standard determination from the Board that it qualifies for a lagoon exemption.  Such a Board determination will only be made in an adjusted standard proceeding, held in accordance with Section 28.1 of the Environmental Protection Act [415 ILCS 5/28.1] (Ill.  Rev.  Stat.  1987, ch.  111 ½, par.  1028.1) and applicable procedures set forth by 35 Ill.  Adm.  Code 104106.

i)	In an adjusted standard proceeding the Board may determine that the petitioning wastewater treatment source qualifies for a lagoon exemption if the wastewater treatment works proves that it is so situated that a land treatment system is not a suitable treatment alternative.  Factors relevant to a suitability finding may include the following: cost; influent character; geographic characteristics; climate; soil conditions; hydrologic conditions; and the availability of irrigable land.

ii)	For the purposes of this subsection (c)(1)(D), a land treatment system is a wastewater treatment system which does not directly discharge treated effluent to waters of the State but instead uses the treated effluent to irrigate terrestrial vegetation;.

2)	The lagoons are properly constructed, maintained and operated; and

3)	The deoxygenating constituents of the effluent do not, alone or in combination with other sources, cause a violation of the applicable dissolved oxygen water quality standard.

d)	No effluent discharged to the Lake Michigan basin shall exceed 4 mg/L mg/l of BOD5 or 5 mg/L mg/l of suspended solids.

e)	Compliance with the numerical standards in this Section shall be determined on the basis of the type and frequency of sampling prescribed by the NPDES permit for the discharge at the time of monitoring.

f)	For the purposes of this Section, useful life is the period of time during which it is cost effective to operate and maintain a particular wastewater treatment works under consideration.  At a minimum, the following factors relating to a wastewater treatment works shall be considered in a determination of its useful life:

1)	Structural and operational condition of components;

2)	Past operations and maintenance record;

3)	Cost for continued use; and

4)	Description and costs of for treatment alternatives.

- Compliance with the 5 day biochemical oxygen demand (BOD5) numerical standard in this Part will be determined by the analysis of 5 day carbonaceous biochemical oxygen demand (CBOD5) (STORET number 80082), unless federal regulations require treatment works treating industrial wastes to comply with more stringent requirements determined by the analysis of BOD5.  Effluent from the treatment works subject to the requirements of Section 304.120(a) shall not exceed 25 mg/L CBOD5.

(Source: Amended at __________ Ill. Reg. _______________, effective ________________).

TITLE 35:  ENVIRONMENTAL PROTECTION

SUBTITLE C:  WATER POLLUTION

CHAPTER I:  POLLUTION CONTROL BOARD

### PART 309

### PERMITS

SUBPART A:  NPDES PERMITS

Section

| 309.101 | Preamble |
| --- | --- |
| 309.102 | NPDES Permit Required |
| 309.103 | Application - General |
| 309.104 | Renewal |
| 309.105 | Authority to Deny NPDES Permits |
| 309.106 | Access to Facilities and Further Information |
| 309.107 | Distribution of Applications |
| 309.108 | Tentative Determination and Draft Permit |
| 309.109 | Public Notice |
| 309.110 | Contents of Public Notice of Application |
| 309.111 | Combined Notices |
| 309.112 | Agency Action After Comment Period |
| 309.113 | Fact Sheets |
| 309.114 | Notice to Other Governmental Agencies |
| 309.115 | Public Hearings on NPDES Permit Applications |
| 309.116 | Notice of Agency Hearing |
| 309.117 | Agency Hearing |
| 309.118 | Agency Hearing File |
| 309.119 | Agency Action After Hearing |
| 309.141 | Terms and Conditions of NPDES Permits |
| 309.142 | Water Quality Standards and Waste Load Allocation |
| 309.143 | Effluent Limitations |
| 309.144 | Federal New Source Standards of Performance |
| 309.145 | Duration of Permits |
| 309.146 | Authority to Establish Recording, Reporting, Monitoring and Sampling Requirements |
| 309.147 | Authority to Apply Entry and Inspection Requirements |
| 309.148 | Schedules of Compliance |
| 309.149 | Authority to Require Notice of Introduction of Pollutants into Publicly Owned Treatment Works |
| 309.150 | Authority to Ensure Compliance by Industrial Users with Sections 204(b), 307 and 308 of the Clean Water Act |
| 309.151 | Maintenance and Equipment |
| 309.152 | Toxic Pollutants |
| 309.153 | Deep Well Disposal of Pollutants (Repealed) |
| 309.154 | Authorization to Construct |
| 309.155 | Sewage Sludge Disposal |
| 309.156 | Total Dissolved Solids Reporting and Monitoring |
| 309.157 | Permit Limits for Total Metals |
| 309.181 | Appeal of Final Agency Action on a Permit Application |
| 309.182 | Authority to Modify, Suspend or Revoke Permits |
| 309.183 | Revision of Schedule of Compliance |
| 309.184 | Permit Modification Pursuant to Variance |
| 309.185 | Public Access to Information |
| 309.191 | Effective Date |

SUBPART B:  OTHER PERMITS

Section

| 309.201 | Preamble |
| --- | --- |
| 309.202 | Construction Permits |
| 309.203 | Operating Permits; New or Modified Sources |
| 309.204 | Operating Permits; Existing Sources |
| 309.205 | Joint Construction and Operating Permits |
| 309.206 | Experimental Permits |
| 309.207 | Former Permits (Repealed) |
| 309.208 | Permits for Sites Receiving Sludge for Land Application |
| 309.221 | Applications - Contents |
| 309.222 | Applications - Signatures and Authorizations |
| 309.223 | Applications - Registered or Certified Mail |
| 309.224 | Applications - Time to Apply |
| 309.225 | Applications - Filing and Final Action By by Agency |
| 309.241 | Standards for Issuance |
| 309.242 | Duration of Permits Issued Under Subpart B |
| 309.243 | Conditions |
| 309.244 | Appeals from Conditions in Permits |
| 309.261 | Permit No Defense |
| 309.262 | Design, Operation and Maintenance Criteria |
| 309.263 | Modification of Permits |
| 309.264 | Permit Revocation |
| 309.265 | Approval of Federal Permits |
| 309.266 | Procedures |
| 309.281 | Effective Date |
| 309.282 | Severability |

| Appendix A | References to Previous Rules |
| --- | --- |

AUTHORITY:  Implementing Sections 13 and 13.3 and authorized by Section 27 of the Environmental Protection Act [415 ILCS 5/13, 13.3 and 27].

SOURCE:  Adopted in R71-14, at 4 PCB 3, March 7, 1972; amended in R73-11, 12, at 14 PCB 661, December 5, 1974, at 16 PCB 511, April 24, 1975, and at 28 PCB 509, December 20, 1977; amended in R73-11, 12, at 29 PCB 477, at 2 Ill. Reg. 16, p. 20, effective April 20, 1978; amended in R79-13, at 39 PCB 263, at 4 Ill. Reg. 34, p. 159, effective August 7, 1980; amended in R77-12B, at 41 PCB 369, at 5 Ill. Reg. 6384, effective May 28, 1981; amended in R76-21, at 44 PCB 203, at 6 Ill. Reg. 563, effective December 24, 1981; codified at 6 Ill. Reg. 7818; amended in R82-5, 10, at 54 PCB 411, at 8 Ill. Reg. 1612, effective January 18, 1984; amended in R86-44 at 12 Ill. Reg. 2495 effective January 13, 1988; amended in R88-1 at 13 Ill. Reg. 5993, effective April 18, 1989; amended in R88-21(A) at 14 Ill. Reg. 2892, effective February 13, 1990; amended in R91-5 at 16 Ill. Reg. 7339, effective April 27, 1992; amended in R95-22 at 20 Ill. Reg. 5526, effective April 1, 1996; amended in R99-8 at 23 Ill. Reg. 11287, effective August 26, 1999; amended in R02-11 at           Ill. Reg.                      , effective                                         .

### SUBPART A: NPDES PERMITS

#### Section 309.141	Terms and Conditions of NPDES Permits

In establishing the terms and conditions of each issued NPDES Permit, the Agency shall apply and ensure compliance with all of the following, whenever applicable:

a)	Effluent limitations under Sections 301 and 302 of the CWA;

b)	Standards of performance for new sources under Section 306 of the CWA;

c)	Effluent standards, effluent prohibitions, and pretreatment standards under Section 307 of the CWA;

d)	Any more stringent limitation, including those:

1)	necessary to meet water quality standards, treatment standards, or schedules of compliance, established pursuant to any Illinois statute or regulation (under authority preserved by Section 510 of the CWA),

2)	necessary to meet any other federal law or regulation, or

3)	required to implement any applicable water quality standards,; such limitations to include any legally applicable requirements necessary to implement total maximum daily loads established pursuant to Section 303(d) of the CWA and incorporated in the continuing planning process approved under Section 303(e) of the CWA and any regulations or guidelines issued pursuant thereto;

e)	Any more stringent legally applicable requirements necessary to comply with a plan approved pursuant to Section 208(b) of the CWA;

f)	Prior to promulgation by the Administrator of the U.S.  Environmental Protection Agency of applicable effluent standards and limitations pursuant to Sections 301, 302, 306 and 307 of the CWA, such conditions as the Agency determines are necessary to carry out the provisions of the CWA;

g)	If the NPDES Permit is for the discharge of pollutants into navigable waters from a vessel or other floating craft (except that no NPDES Permit shall be issued for the discharge of pollutants from a vessel or other floating craft into Lake Michigan), any applicable regulations promulgated by the Secretary of the Department in which the Coast Guard is operating, establishing specifications for safe transportation, handling, carriage, storage and stowage of pollutants; and

h)	If the NPDES Permit is for the discharge of pollutants from other than wet weather point sources into the Lake Michigan Basin as defined at 35 Ill. Adm. Code 303.443:

1)	Total Maximum Daily Loads (TMDLs) and Waste Load Allocation (WLA) will be established through either the LaMP or a RAP for an Area of Concern. If a LaMP or RAP has not been completed and adopted, effluent limits shall be established consistent with the other provisions of this Section, including, but not limited to, Additivity, Intake Pollutants, Loading Limits, Level of Detection/Level of Quantification and Compliance Schedules. When calculation of TMDLs or a WLA Waste Load Allocation is incomplete and it is expected that limits established through other provisions will be superseded upon completion of the TMDL or WLA Waste Load Allocation process, those limits shall be identified as interim and the permit shall include a reopener clause triggered by completion of a TMDL or WLA determination. Any new limits brought about through exercise of the reopener clause shall be eligible for delayed compliance dates and compliance schedules consistent with Section 39(b) of the Act [415 ILCS 5/39(b)], 35 Ill. Adm. Code 309.148, and 35 Ill. Adm. Code 352.Subpart H.

2)	35 Ill. Adm. Code 302.590 establishes an acceptable additive risk level of one in 100,000 (105(-5)) for establishing Tier I criteria and Tier II values for combinations of substances exhibiting a carcinogenic or other nonthreshold toxic mechanism. For those discharges containing multiple nonthreshold substances application of this additive standard shall be consistent with this subsection (h).

A)	For discharges in the Lake Michigan Basin basin containing one or more 2,3,7,8-substituted chlorinated dibenzo-p-dioxins or 2,3,7,8-substituted dibenzofurans, the tetrachloro dibenzo-p-dioxin 2,3,7,8-(TCDD) toxicity equivalence concentration (TECTCDD) shall be determined as outlined in subsection (h)(2)(B).

B)	The values listed in the following Table shall be used to determine the 2,3,7,8-TCDD toxicity equivalence concentrations using the following equation:

(TEC)TCDD =  Sigma(C)x (TEF)x (BEF)x

WHERE:

(TEC)TCDD = 	2,3,7,8-TCDD toxicity equivalence concentration in effluent

(C)x =	Concentration of total chemical x in effluent

(TEF)x =	TCDD toxicity equivalency factor for x

(BEF)x =-	TCDD bioaccumulation equivalency factor for x

TABLE

| Congener | TEF | BEF |  |
| --- | --- | --- | --- |
| 2,3,7,8-TCDD | 1.0 |  | 1.0 |
| 1,2,3,7,8-PeCDD PeCdd | 0.5 |  | 0.9 |
| 1,2,3,4,7,8-HxCDD | 0.1 |  | 0.3 |
| 1,2,3,6,7,8-HxCDD | 0.1 |  | 0.1 |
| 1,2,3,7,8,9-HxCDD | 0.1 |  | 0.1 |
| 1,2,3,4,6,7,8-HpCDD 1.1,2,3,4,6,7,8-HpCDD | 0.01 |  | 0.0 |
| OCDD | 0.001 |  | 0.0 |
| 2,3,7,8-TCDF | 0.1 |  | 0.8 |
| 1,2,3,7,8-PeCDF | 0.05 |  | 0.2 |
| 2,3,4,7,8-PeCDF | 0.5 |  | 1.6 |
| 1,2,3,4,7,8-HxCDF | 0.1 |  | 0.0 |
| 1,2,3,6,7,8-HxCDF | 0.1 |  | 0.2 |
| 2,3,4,6,7,8-HxCDF | 0.1 |  | 0.7 |
| 1,2,3,7,8,9-HxCDF | 0.1 |  | 0.6 |
| 1,2,3,4,6,7,8-HpCDF | 0.01 |  | 0.0 |
| 1,2,3,4,7,8,9-HpCDF | 0.01 |  | 0.4 |
| OCDF | 0.001 |  | 0.0 |

C)	Any combination of carcinogenic or otherwise nonthreshold toxic substances shall be assessed on a case-by-case basis. The Agency shall only consider such additivity for chemicals that exhibit the same type of effect and the same mechanism of toxicity, based on available scientific information that supports a reasonable assumption of additive effects.

3)	Conversion factors for determining the dissolved concentration of metals from the total recoverable concentration.

A)	The numeric standards for certain metal parameters in 35 Ill. Adm. Code 302.504 are established as dissolved forms of the substance since the dissolved form more closely relates to the toxicology literature utilized in deriving the standard. However, most discharge monitoring data used in deriving a PEQ will be from a total recoverable analytical method and permit limits if and when established will be set at total recoverable to accommodate the total recoverable analytical method. The Agency will use a conversion factor to determine the amount of total metal corresponding to dissolved metal for each metal with a water quality standard set at dissolved concentration. In the absence of facility specific data the following default conversion factors will be used for both PEQ derivation and establishing WQBELs. The conversion factor represents the portion of the total recoverable metal presumed to be in dissolved form. The conversion values given in the following table are multiplied by the appropriate total recoverable metal concentration to obtain a corresponding dissolved concentration that then may be compared to the acute or chronic standard.  A dissolved metal concentration may be divided by the conversion factor to obtain a corresponding total metal value that will generally be the metal form regulated in NPDES permits.

| Metal | Conversion Factor Acute Standard Chronic Standard |  |
| --- | --- | --- |
| Arsenic | 1.000 | 1.000 |
| Cadmium | 0.850 | 0.850 |
| Chromium (Trivalent) | 0.316 | 0.860 |
| Chromium (Hexavalent) | 0.982 | 0.962 |
| Copper | 0.960 | 0.960 |
| Mercury | 0.850 | 0.850 |
| Nickel | 0.998 | 0.997 |
| Selenium | 0.922 | 0.922 |
| Zinc | 0.978 | 0.986 |

B)	A permittee may propose an alternate conversion factor for any particular site specific application. The request must contain sufficient site specific data, or other data that is representative of the site, to identify a representative ratio of the dissolved fraction to the total recoverable fraction of the metal in the receiving water body at the edge of the mixing zone. If a site specific conversion factor is approved, that factor will be used for PEQ derivation and establishment of a WQBEL in lieu of its default counterpart in subsection (h)(3)(A).

3 4)	Reasonable potential to exceed.

A)	The first step in determining if a reasonable potential to exceed the water quality standard exists for any particular pollutant parameter is the estimation of the maximum expected effluent concentration for that substance. That estimation will be completed for both acute and chronic exposure periods and is termed the PEQ. The PEQ shall be derived from representative facility-specific data to reflect a 95 percent confidence level for the 95th percentile value. These data will be presumed to adhere to a lognormal distribution pattern unless the actual effluent data demonstrates a different distribution pattern. If facility-specific data in excess of 10 data values is available, a coefficient of variation that is the ratio of the standard deviation to the arithmetic average shall be calculated by the Agency. The PEQ is derived as the upper bound of a 95 percent confidence bracket around the 95th percentile value through a multiplier from the following table applied to the maximum value in the data set that has its quality assured consistent with 35 Ill. Adm. Code 352.410 as appropriate for acute and chronic data sets.

PEQ = (maximum data point)(statistical multiplier)

Coefficient of Variation

| No. Samples | 0.1 | 0.2 | 0.3 | 0.4 |  | 0.5 |  | 0.6 |  | 0.7 |  | 0.8 |  | 0.9 |  | 1.0 |  | 1.1 |  | 1.2 |  | 1.3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.4 | 1.9 | 2.6 | 3.6 | 4.7 |  | 6.2 |  | 8.0 |  | 10.1 |  | 12.6 |  | 15.5 |  | 18.7 |  | 22.3 |  | 26.4 |  |
| 2 | 1.3 | 1.6 | 2.0 | 2.5 | 3.1 |  | 3.8 |  | 4.6 |  | 5.4 |  | 6.4 |  | 7.4 |  | 8.5 |  | 9.7 |  | 10.9 |  |
| 3 | 1.2 | 1.5 | 1.8 | 2.1 | 2.5 |  | 3.0 |  | 3.5 |  | 4.0 |  | 4.6 |  | 5.2 |  | 5.8 |  | 6.5 |  | 7.2 |  |
| 4 | 1.2 | 1.4 | 1.7 | 1.9 | 2.2 |  | 2.6 |  | 2.9 |  | 3.3 |  | 3.7 |  | 4.2 |  | 4.6 |  | 5.0 |  | 5.5 |  |
| 5 | 1.2 | 1.4 | 1.6 | 1.8 | 2.1 |  | 2.3 |  | 2.6 |  | 2.9 |  | 3.2 |  | 3.6 |  | 3.9 |  | 4.2 |  | 4.5 |  |
| 6 | 1.1 | 1.3 | 1.5 | 1.7 | 1.9 |  | 2.1 |  | 2.4 |  | 2.6 |  | 2.9 |  | 3.1 |  | 3.4 |  | 3.7 |  | 3.9 |  |
| 7 | 1.1 | 1.3 | 1.4 | 1.6 | 1.8 |  | 2.0 |  | 2.2 |  | 2.4 |  | 2.6 |  | 2.8 |  | 3.1 |  | 3.3 |  | 3.5 |  |
| 8 | 1.1 | 1.3 | 1.4 | 1.6 | 1.7 |  | 1.9 |  | 2.1 |  | 2.3 |  | 2.4 |  | 2.6 |  | 2.8 |  | 3.0 |  | 3.2 |  |
| 9 | 1.1 | 1.2 | 1.4 | 1.5 | 1.7 |  | 1.8 |  | 2.0 |  | 2.1 |  | 2.3 |  | 2.4 |  | 2.6 |  | 2.8 |  | 2.9 |  |
| 10 | 1.1 | 1.2 | 1.3 | 1.5 | 1.6 |  | 1.7 |  | 1.9 |  | 2.0 |  | 2.2 |  | 2.3 |  | 2.4 |  | 2.6 |  | 2.7 |  |
| 11 | 1.1 | 1.2 | 1.3 | 1.4 | 1.6 |  | 1.7 |  | 1.8 |  | 1.9 |  | 2.1 |  | 2.2 |  | 2.3 |  | 2.4 |  | 2.5 |  |
| 12 | 1.1 | 1.2 | 1.3 | 1.4 | 1.5 |  | 1.6 |  | 1.7 |  | 1.9 |  | 2.0 |  | 2.1 |  | 2.2 |  | 2.3 |  | 2.4 |  |
| 13 | 1.1 | 1.2 | 1.3 | 1.4 | 1.5 |  | 1.6 |  | 1.7 |  | 1.8 |  | 1.9 |  | 2.0 |  | 2.1 |  | 2.2 |  | 2.3 |  |
| 14 | 1.1 | 1.2 | 1.3 | 1.4 | 1.4 |  | 1.5 |  | 1.6 |  | 1.7 |  | 1.8 |  | 1.9 |  | 2.0 |  | 2.1 |  | 2.2 |  |
| 15 | 1.1 | 1.2 | 1.2 | 1.3 | 1.4 |  | 1.5 |  | 1.6 |  | 1.7 |  | 1.8 |  | 1.8 |  | 1.9 |  | 2.0 |  | 2.1 |  |
| 16 | 1.1 | 1.1 | 1.2 | 1.3 | 1.4 |  | 1.5 |  | 1.6 |  | 1.6 |  | 1.7 |  | 1.8 |  | 1.9 |  | 1.9 |  | 2.0 |  |
| 17 | 1.1 | 1.1 | 1.2 | 1.3 | 1.4 |  | 1.4 |  | 1.5 |  | 1.6 |  | 1.7 |  | 1.7 |  | 1.8 |  | 1.9 |  | 1.9 |  |
| 18 | 1.1 | 1.1 | 1.2 | 1.3 | 1.3 |  | 1.4 |  | 1.5 |  | 1.6 |  | 1.6 |  | 1.7 |  | 1.7 |  | 1.8 |  | 1.9 |  |
| 19 | 1.1 | 1.1 | 1.2 | 1.3 | 1.3 |  | 1.4 |  | 1.5 |  | 1.5 |  | 1.6 |  | 1.6 |  | 1.7 |  | 1.8 |  | 1.8 |  |
| 20 | 1.1 | 1.1 | 1.2 | 1.2 | 1.3 |  | 1.4 |  | 1.4 |  | 1.5 |  | 1.5 |  | 1.6 |  | 1.6 |  | 1.7 |  | 1.7 |  |
| 30 | 1.0 | 1.1 | 1.1 | 1.1 | 1.2 |  | 1.2 |  | 1.2 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.3 |  | 1.4 |  | 1.4 |  |
| 40 | 1.0 | 1.0 | 1.1 | 1.1 | 1.1 |  | 1.1 |  | 1.1 |  | 1.1 |  | 1.2 |  | 1.2 |  | 1.2 |  | 1.2 |  | 1.2 |  |
| 50 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |  | 1.0 |  | 1.0 |  | 1.1 |  | 1.1 |  | 1.1 |  | 1.1 |  | 1.1 |  | 1.1 |  |
| 60 or greater | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |  | 1.0 |  | 1.0 |  | 1.0 |  | 1.0 |  | 1.0 |  | 1.0 |  | 1.0 |  | 1.0 |  |

i)	If the PEQ is less than or equal to the water quality standard, there is no reasonable potential and no limit will be established in the permit.

ii)	If the PEQ is more than the water quality standard, the Agency will proceed to consideration of dilution and mixing pursuant to subsection (h)(45).

B)	If facility-specific data of 10 or less data values is available, an alternative PEQ shall be derived using the table in subsection (h)(34)(A) assuming a coefficient of variation of 0.6, applied to the maximum value in the data set that has its quality assured consistent with 35 Ill. Adm. Code 352.410.

i)	If the PEQ is less than or equal to the water quality standard, there is no reasonable potential and no limit will be established in the permit.

ii)	If the PEQ exceeds the water quality standard, an alternative PEQ will be calculated using the maximum value in the data set and a multiplier of 1.4. If the alternative PEQ also exceeds the water quality standard, the Agency will proceed to consider dilution and mixing pursuant to subsection (h)(45).

iii)	If the PEQ exceeds the water quality standard but the alternative PEQ is less than or equal to the standard, the Agency will either proceed to consider dilution and mixing pursuant to subsection (h)(45), or will incorporate a monitoring requirement and reopener clause to reassess the potential to exceed within a specified time schedule, not to exceed one year. In determining which of these options to use in any individual application, the Agency shall consider the operational and economic impacts on the permittee and the effect, if any, deferral of a final decision would have on an ultimate compliance schedule if a permit limit were subsequently determined to be necessary.

C)	The Agency shall compare monthly average effluent data values, when available, with chronic aquatic life, human health and wildlife standards to evaluate the need for monthly average water quality based effluent limitations (WQBELs). The Agency shall use daily effluent data values to determine whether a potential exists to exceed acute aquatic life water quality standards.

D)	The Agency may apply other scientifically defensible statistical methods for calculating PEQ for use in the reasonable potential analysis as provided for in Procedure 5.b.2 of Appendix F to 40 CFR 132, incorporated by reference at 35 Ill. Adm. Code 301.106.

E)	Regardless of the statistical procedure used, if the PEQ for the parameter is less than or equal to the water quality standard for that parameter, the Agency shall deem the discharge not to have a reasonable potential to exceed, and a water quality based effluent limit (WQBEL) shall not be required unless otherwise required under 35 Ill. Adm. Code 352.430.

4 5)	If the PEQ for a parameter is greater than the particular water quality standard, criteria or value for that parameter, the Agency will assess the level of treatment being provided by the discharger.  If the discharger is providing (or will be providing) a level of treatment consistent with the best degree of treatment required by 35 Ill. Adm. Code 304.102(a), the PEQ derived under subsection (h)(34) shall be compared to a preliminary effluent limitation (PEL) determined by applying an appropriate mixing zone or a default mixing zone to the discharge. Mixing opportunity and dilution credit will be considered as follows:

A)	Discharges to tributaries of the Lake Michigan Basin shall be considered to have no available dilution for either acute or chronic exposures, and the PEL will be set equivalent to the water quality standard unless dilution is documented through a mixing zone study.

B)	Bioaccumulative chemicals of concern (BCCs):

i)	No mixing shall be allowed for new discharges of BCCs commencing on or after December 24, 1997.  The PEL will be set equivalent to the water quality standard.

ii)	Mixing shall be allowed for discharges of BCCs  which existed as of December 24, 1997 in accordance with the requirements of 35 Ill. Adm. Code 302.530.

C)	Direct discharges to the Open Waters of Lake Michigan shall have a default mixing allowance of 2:1 for acute standards, criteria or values and 10:1 for chronic standards, criteria or values if the discharge configuration indicates that the effluent readily and rapidly mixes with the receiving waters. If ready and rapid mixing is in doubt the Agency shall deny any default dilution or mixing allowance and require a mixing or dispersion study to determine the proper dilution allowance. If the discharger applies for more than the default dilution or mixing allowance, it must submit a mixing or dispersion study to justify its request. Whenever a mixing or dispersion study is available, it shall be used to determine dilution or mixing allowance in lieu of the default allowance.

5 6)	Preliminary effluent limitations calculations.

(A)	The preliminary effluent limitation (PEL) is calculated in a simple mass balance approach reflecting the dilution allowance established in subsection (h)(45):

WQS = [(Qe)(PEL) + (Qd)(Cd)] / [Qe + Qd] or

PEL = [WQS(Qe + Qd) - (Qd)(Cd)] / Qe

WHERE:

WQS = applicable water quality standard, criteria or value

Qe = effluent flowrate

Qd = allowable dilution flowrate

Cd = background pollutant concentration in dilution water

B)	The representative background concentration of pollutants to develop TMDLs and WLAs calculated in the absence of a TMDL shall be established as follows:

i)	"Background" represents all pollutant loadings, specifically loadings that flow from upstream waters into the specified watershed, water body, or water body segment for which a TMDL or WLA in the absence of a TMDL is being developed and enter the specified watershed, water body, or water body segment through atmospheric deposition, chemical reaction, or sediment release or resuspension.

(ii)  	When determining what available data are acceptable for use in calculating background, the Agency shall use its best professional judgment, including consideration of the sampling location and the reliability of the data through comparison, in part, to detection and quantification levels.  When data in more than 1 of the data sets or categories described in subsection (h)(56)(B)(iii) exists, best professional judgment shall be used to select the data that most accurately reflects or estimates background concentrations.  Pollutant degradation and transport information may be considered when using pollutant loading data to estimate a water column concentration.

(iii) 	The representative background concentration for a pollutant in the specified watershed, water body, or water body segment shall be established on a case-by-case basis as the geometric mean of: acceptable water column data; water column concentrations estimated through use of acceptable caged or resident fish tissue data; or water column concentrations estimated through the use of acceptable or projected pollutant loading data.  When determining the geometric mean of the data for a pollutant that includes values both above and below the detection level, commonly accepted statistical techniques shall be used to evaluate the data.  If all of the acceptable data in a data set are below the detection level for a pollutant, then all the data for the pollutant in that data set shall be assumed to be zero.

6 7)	Water quality based effluent limitations.

A)	If the PEQ is less than or equal to the PEL, it will be concluded that there is no reasonable potential to exceed. Under such circumstances a permit limit for that contaminant will not be set unless otherwise justified under one or more provisions of 35 Ill. Adm. Code 352.430.

B)	If the PEQ is equal to or greater than the PEL, and the PEQ was calculated using a data set of more than 10 values, a water quality based effluent limitation (WQBEL) will be included in the permit. If the PEQ was calculated using a data set of less than or equal to 10 values, and the alternative PEQ calculated under subsection (h)(3)(4)(B) also exceeds the PEL, a WQBEL will be included in the permit.

C)	If the PEQ was calculated using a data set of less than or equal to 10 values, and the PEQ is greater than the PEL but the alternative PEQ is less than the PEL, the Agency will either establish a WQBEL in the permit or incorporate a monitoring requirement and reopener clause to reassess potential to exceed within a specified time schedule, not to exceed one year. In determining which of these options to use in any individual application, the Agency shall consider the operational and economic impacts on the permittee and the effect, if any, deferral of a final decision would have on an ultimate compliance schedule if a permit limit were subsequently determined to be necessary.

D)	The WQBEL will be set at the PEL, unless the PEL is appropriately modified to reflect credit for intake pollutants when the discharged water originates in the same water body to which it is being discharged. Consideration of intake credit will be limited to the provisions of 35 Ill. Adm. Code 352.425.

E)	The reasonable potential analysis shall be completed separately for acute and chronic aquatic life effects. When WQBELs are based on acute impacts, the limit will be expressed as a daily maximum. When the WQBEL is based on chronic effects, the limit will be expressed as a monthly average. Human health and wildlife based WQBELs will be expressed as monthly averages. If circumstances warrant, the Agency shall consider alternatives to daily and monthly limits.

(Source:  Amended at __________ Ill. Reg. _______________, effective ________________)

# Section 309.157	Permit Limits for Total Metals

a)	The NPDES permit limits for metals must be expressed in total metals form even though the water quality standards for metals specified in Sections 302.208(e), 302.504(a), and 304.105 are in their dissolved form.  The total metals permit limit shall be determined by multiplying the dissolved metals concentration and the appropriate metals translator.

b)	The Agency shall adopt procedures for determining site-specific metals translator in accordance with “The Metals Translator: Guidance for Calculating a Total Recoverable Permit Limit from a Dissolved Criterion,” incorporated by reference at 35 Ill. Adm. Code 301.106.

c)	Except as otherwise specified in subsection (d) of this Section, the reciprocal of the conversion factor multiplier used for obtaining the dissolved metals standards at Sections 302.208(e) and 302.504(a) becomes the metals translator and the resulting total metals value becomes the NPDES permit limit.

d)	A permittee may request the Agency, in accordance with the procedures adopted  pursuant to subsection (b) of this Section, to calculate a total metals permit limit based on a site-specific metals translator.  Upon review and approval of the information submitted by the permittee, the Agency will calculate a total metals permit limit that is protective of the dissolved metals water quality standard.

(Source:  Added at ________ Ill. Reg. ________________, effective ____________)

IT IS SO ORDERED.

Section 41(a) of the Environmental Protection Act provides that final Board orders may be appealed directly to the Illinois Appellate Court within 35 days after the Board serves the order.  415 ILCS 5/41(a) (2000); see also 35 Ill. Adm. Code 101.300(d)(2), 101.906, 102.706.  Illinois Supreme Court Rule 335 establishes filing requirements that apply when the Illinois Appellate Court, by statute, directly reviews administrative orders.  172 Ill. 2d R. 335.  The Board’s procedural rules provide that motions for the Board to reconsider or modify its final orders may be filed with the Board within 35 days after the order is received.  35 Ill. Adm. Code 101.520; see also 35 Ill. Adm. Code 101.902, 102.700, 102.702.

I, Dorothy M. Gunn, Clerk of the Illinois Pollution Control Board, certify that the Board adopted the above opinion and order on December 19, 2002, by a vote of 6-0.

Dorothy M. Gunn, Clerk

Illinois Pollution Control Board

The transcript from the January 29, 2002 hearing in Chicago will be cited as “Tr.1” and the transcript from the March 6, 2002 hearing held in Springfield will be cited as “Tr.2”.

Testimony and comments were offered on behalf of the Environmental Law and Policy Center, Prairie Rivers Network, and Sierra Club.  The Board will refer to them as “Environmental Groups” when discussing the comments and testimony.

The July 25, 2002 hearing will be cited as “Tr.3”.

The Agency original proposal used the 9.9 µg/L number; however, in a subsequent comment the Agency revised that number to 11 µg/L (see Exh. 8).