ILLINOIS POLLUTION CONTROL BOARD

January 19, 2006

| IN THE MATTER OF: PROPOSED NEW AND UPDATED RULES FOR MEASUREMENT AND NUMERICAL SOUND EMISSIONS STANDARDS AMENDMENTS TO 35 ILL. ADM CODE 901 and 910 | ) ) ) ) ) ) ) | R03-9 (Rulemaking - Noise) |
| --- | --- | --- |

Proposed Rule.  Second Notice.

OPINION AND ORDER OF THE BOARD (by G.T. Girard):

Pursuant to Sections 27 and 28 of the Illinois Environmental Protection Act (Act), (415 ILCS 5/27-5/28 (2004)) and 35 Ill. Adm. Code 102.Subpart B, on February 20, 2003, the Board opened this rulemaking to propose updated regulations governing noise found in 35 Ill. Adm. Code Subtitle H.  Many of the sound measurement definitions and techniques in the Board’s current rules do not reflect present scientific standards.  The instant proposal revises outdated numerical sound emission standards for property line noise sources found at 35 Ill. Adm. Code 901.  In addition, the proposal adds Part 910 to the Board’s rules that incorporates noise measurement techniques currently contained only in Illinois Environmental Protection Agency (Agency) rules at 35 Ill. Adm. Code 951.  The Board today proposes for second notice the amendments.

BACKGROUND

The Board opened this rulemaking by issuing a draft for public comment on February 20, 2003.  The Board elicited comments on the draft prior to first-notice publication and the Board held two hearings before Hearing Officer William F. Murphy.  The first hearing was held in Springfield on May 1, 2003, no members of the public attended that hearing.  The second hearing (Tr.2) was held in Chicago on May 15, 2003.  Joel Sternstein and Howard Chinn of the Illinois Attorney General’s Office (AGO) attended the second hearing and Mr. Chinn presented testimony.  During the time that the proposal was offered for public comment, the Board received two public comments, one from the AGO (PC 2) and one from Dr. Paul Schomer (PC 1).

On July 10, 2003, the Board proposed the rule for first notice.  On July 25, 2003, the first notice was published in the Illinois Register (27 Ill. Reg. 11908).  The Board received five public comments.  The comments were from Scot Forge (PC 3), Vaughan and Bushnell Manufacturing (PC 4), Illinois Association of Aggregate Producers (IAAP) (PC 5), Boughton Trucking and Materials Inc. (Boughton Trucking) (PC 6) and Intermet Decatur Foundry (Intermet) (PC 7).  In general, the comments requested a site-specific rule modification or requested another hearing to discuss concerns with the proposed rule.

On October 18, 2003, the Board determined that additional hearings should be held.  After holding an additional hearing, the Board determined on March 4, 2004, that administrative economy supported including site-specific changes in a new first notice.  Because of this decision, the Board withdrew the first notice (See 28 Ill. Reg. 7341, 7342 (May 21, 2004)).

The Board held two hearings before Board Hearing Officer Marie Tipsord.  The third hearing (Tr.3) was held February 10, 2004, in Chicago.  At that hearing, Mr. John P. McGillivray testified on behalf of Scot-Forge Company and Howard Chinn testified on behalf of the AGO.  The fourth hearing (Tr.4) was held November 4, 2004, in Springfield.  Mr. John Henriksen and Dane Tittman testified on behalf of the IAAP.

The Board received six additional comments during the second phase of hearings.  The comments were from Moline Forge (PC 8), Intermet (PC 9,) IAAP (PC 10), the AGO (PC 11), Village of Bridgeview (Bridgeview) (PC 12), and the Illinois Department of Natural Resources (IDNR) (PC 13).

On March 17, 2005, the Board proceeded to a second first notice with this rulemaking.  That first notice was published in the Illinois Register on April 15, 2005 (29 Ill. Reg. 5196).  The Board held an additional hearing on September 1, 2005 (Tr. 5) and at that hearing Mr. Henriksen again testified along with Mr. Willard Pierce of Evenson Explosives, LLC on behalf of IAAP.  Also Mr. Jim Hafliger of the IDNR testified.  The public comment period closed on October 17, 2005, and the Board received six additional comments during the 2005 first-notice period.  Those comments are:

PC 14	Joseph R. Angleton, Office Director, Office of Mines and Minerals, Illinois Department of Natural Resources

PC 15	John Henriksen, Executive Director, Illinois Association of Aggregate Producers

PC 16	Dr. Paul Schomer, Schomer and Associates, Inc.

PC 17	Therese M. Dorigan, Bradburne, Briller & Johnson, LLC

PC 18	Village of Bridgeview

PC 19	Dr. Paul Schomer, on behalf of Village of Bridgeview

COMMENTS AND TESTIMONY

The Board received six comments along with testimony at the September 1, 2005 hearing regarding the Board’s 2005 first-notice proposal.  The following paragraphs will summarize the comments and testimony received by the Board.

Testimony

At the September 1, 2005 hearing, Mr. John Henriksen again testified along with Mr. Willard Pierce of Evenson Explosives, LLC on behalf of IAAP.  Also Mr. Jim Hafliger of the Illinois Department of Natural Resources (IDNR), Office of Mines and Minerals testified.

Mr. John Henriksen

Mr. Henriksen manages IAAP, which is a trade association representing companies that produce aggregates (i.e. crushed sand, stone, and gravel).  Tr.5 at 6-7.  The companies have 113 members that operate in 80 Illinois counties and produce 111 million tons of materials.  Tr.5 at 7.  The aggregate industry has been regulated for blasting by IDNR since 1995 pursuant to Section 6.5 of the Surface-Mined Land Conservation and Reclamation Act (225 ILCS 715/6.5 (2004)).  Id.  IDNR’s Office of Mines and Minerals has promulgated a set of regulations that regulate blasting at coal mining and aggregate mining sites in Illinois pursuant to Section 3.13 of the Surface Coal Mining Land Conservation and Reclamation Act (225 ILCS 715/3.13 (2004)) (See 62 Ill. Adm. Code 300, 1816).  Tr.5 at 7.

Mr. Henriksen testifies that IDNR’s Office of Mines and Minerals has promulgated a comprehensive set of regulations that subjects explosives blasting at coal mining and aggregate mining operations to air blast or ground vibration monitoring, or both, as necessary to prevent property damage and protect public safety.  Tr.5 at 7-8.  Additionally, IDNR enforces the regulations using highly trained, technologically proficient inspectors and state of the art equipment.  Id.

Mr. Henriksen states that the Board is seeking to maintain a duplicative and overlapping regulatory program for coal mining and aggregate producers in Illinois.  Tr.5 at 8.  Mr. Henriksen argues that Section 25 of the Act provides that the Board “may adopt” regulations limiting noise emissions; however, the Board is not required to adopts such limitations.  Tr.5 at 9-10.  Mr. Henriksen asserts that the Board has elected not to regulate other sources of noise and can do so here as well.  Id.  Mr. Henriksen acknowledges that the Board has the authority to decide which activities should be regulated.  Tr.5 at 10.  Mr. Henriksen contends, however, that regulation of sound generated by blasting operations at aggregate or coal mines, which are already highly regulated by a sister agency, is unreasonable.  Tr.5 at 10-11.  Mr. Henriksen suggests that Part 901 not apply to explosive operations at mines.  Tr.5 at 11-12.

In response to questions from the Board, Mr. Henriksen states that only the sound produced from blasting should be exempted from the Board’s rules.  Tr.5 at 35.  Mr. Henriksen states that the Board should regulate steady noises that are a function of mining operations such as sounds generated from crushing equipment and backup alarms on equipment.  Id.  However, pursuant to Section 25 of the Act (415 ILCS 5/25 2004), Mr. Henriksen believes the Board should defer to IDNR’s regulations on blasting at mining operations.  Id.

Mr. Henriksen recommends that if the Board decides to continue to regulate the noise emissions from blasting at mining operations, the Board should adopt a regulatory program in harmony with IDNR’s program.  Tr.5 at 13.  In order for the Board’s rules to be consistent with IDNR’s program, Mr. Henriksen suggests language changes such as limiting blasting from sunrise to sunset and shifting the regulatory focus from the property line to protected structures.  Tr.5 at 13-14.  Also, Mr. Henriksen maintains that none of the monitoring methods proposed in Part 910 will monitor highly impulsive sound from blasting.  Tr.5 at 15.

Mr. Jim Hafliger

Mr. Hafliger, whose title is Chief Division Technical Explosive Specialist in the Office of Mines and Minerals at IDNR, testified to provide the Board with an overview of IDNR’s program and responsibilities related to blasting at coal mines and aggregate mines in Illinois.  Tr.5 at 17.  Mr. Hafliger attempts to point to areas of duplication between the Board’s regulations and IDNR’s program.  Id.  IDNR’s program is staffed by 12 individuals in three offices throughout the state.  Tr.5 at 17-18.  Eight of the staff members are full time inspectors regulating the use of explosives in Illinois.  Tr.5 at 18.  Part of IDNR’s statutorily mandated duties include regulating the use, handling and storage of explosives at the 172 active surface mining operations in Illinois.  Id.  IDNR’s inspectors are in the field on a daily basis.  These inspectors use 35 seismographs that register and record (for analysis by IDNR’s experts) the traces reflected as a result of air blast or air overpressure and ground vibrations at the mining operations.  Id.

IDNR’s program has been in place for many years and started with regulation of explosive use in the 1930s.  Tr.5 at 18.  In the 1980s, IDNR began regulating coal blasting operations and in the 1990s regulating blasting operations at the surface aggregate operations.  Tr.5 at 18-19.  The field inspectors are trained to use the seismographs and to analyze the data.  Tr.5 at 19.  IDNR regularly monitors the sites and takes a proactive stance to make sure the operators are fulfilling their statutory obligations.  Tr.5 at 20.  IDNR has a comprehensive training program for people who undertake blasting in Illinois and recently Illinois was recognized for using an estimated 500 million pounds of explosives over a five-year period without a reported injury.  Tr.5 at 20-21.

IDNR’s program is enforced using a hearing process within IDNR and can include fines or even suspension or revocation of actual blasting operations.  Tr.5 at 41.  The program includes requirements for recordkeeping for each blast that incorporates every minor detail of the blast.  Tr.5 at 41-42.  Mr. Hafliger testifies that there are 17 different criterion included on the blasting records which must be available for inspection by IDNR’s staff.  Tr.5 at 42-43.  Furthermore, both the mining company and the actual person performing the blasting are subject to sanctions for violation of IDNR’s rules.  Tr.5 at 43-44.  Mr. Hafliger also notes that IDNR has an 800 number to be used for any complaints relating to blasting and that number has received about 100 complaints this year.  Tr.5 at 44, 46.

Mr. Willard Pierce

Mr. Pierce is general manager of Evenson Explosives, which provides blasting services in four states with a majority of that in Illinois.  Tr. 5 at 23.  Mr. Pierce also worked for IDNR and helped establish and implement the blasting regulatory program at IDNR.  Id.  Mr. Pierce wishes to ensure that the regulatory requirements are consistent and that the requirements can be implemented.  Tr.5 at 24.  Mr. Pierce testifies that based on the regulations, blasters in Illinois need two years hand on experience, as well as class room training and examination before receiving a license.  Tr.5 at 24.

Mr. Pierce expresses concern over the inconsistency in the C-weighted numbers between those in Section 901.109 and IDNR’s rules.  Tr.5 at 25-26.  Mr. Pierce also suggests that there should only be one standard and that standard should not deviate depending on the time of day or the number of times a blast occurs during the day.  Tr.5 at 26.

Mr. Pierce also testifies that an air blast is a very specific type of sound and is a very low frequency.  Tr.5 at 26.  Mr. Pierce states that you do not actually “hear” an air blast but rather the effects of the air blast.  Id.  For example, Mr. Pierce states that you may hear surface delays or venting and you might feel the blast in your chest.  Id.

Mr. Pierce indicates that about 12 years ago the International Society of Explosive Engineers worked with manufacturers of monitoring equipment to develop standards for monitoring air blasts from blasting.  Tr.5 at 27.  The standard was published in 1999 and uses the seismograph for measuring air blast because the seismograph can measure ground vibration and air blast.  Tr.5 at 27-28.  Mr. Pierce sums up his position by stating that the industry has a standard for air blast, and a standard for monitoring which has been in place for a number of years.  Tr.5 at 29-30.  Further, the standard is based on science and is protective of structures.  Tr.5 at 30.

Mr. Pierce notes that air blast is a short-term transient noise, that lasts about three seconds.  Tr.5 at 28.  Thus, where the Board’s propose rule suggests shutting off monitoring equipment for transient noise, the rule would seem to allow shutting down monitoring equipment for air blasts.  Id.  Mr. Pierce recommends that if the Board chooses to adopt a rule requiring monitoring of air blasting, that the Board require the use of a blasting seismograph for monitoring.  Tr.5 at 29.

Mr. Pierce summarizes his testimony by stating that he has five recommendations.  The first is that the C-weighted and flat response microphones have the same number.  Tr.5 at 30.  Second, Mr. Pierce recommends that the limit for air blasts apply at a structure, not the property line.  Id.  Third, Mr. Pierce recommends limiting blasting from sunrise to sunset, except in emergency situations.  Tr.5 at 31.  Fourth, Mr. Pierce does not believe the limits should be lowered based on the time of day.  Id.  And fifth, Mr. Pierce urges the Board to require the use of monitoring procedures and equipment appropriate to blasting.  Id.

Public Comments

As indicated by the list above the Board received six comments including two from Dr. Paul Schomer including one on behalf of the Village of Bridgeview.  The Board will group the comments by the author of the comment.

Office of Mines and Minerals, IDNR

IDNR regulates blasting operations at all surface aggregate and coal mines in Illinois.  PC 14 at 1.  IDNR’s comprehensive regulations help prevent personal injury and property damage though the use of the conservative limits of one inch per second ground vibration and 133dB maximum air over pressure.  Id.  IDNR comments that these standards are nationally recognized and accepted industry practice.  Id.  IDNR determines regulatory compliance by routinely inspecting and monitoring blasting at all of the State’s 172 active aggregate and coal blasting sites.  Id.  IDNR’s staff of 13 people have over 100 years of combined explosives related experience and field inspectors trained in the technical elements of seismology regularly monitor blasting using state-of-the-art seismographs.  Id.

IDNR specifically lists several points where the Board’s proposal is inconsistent or deviates from IDNR’s standards and IDNR recommends changes for consistency.  First, IDNR notes that the amendments to Section 901.109(a) result in limits that are inconsistent with and less stringent than the IDNR limits.  PC 14 at 1.  IDNR also suggests that the limits remain constant from sunrise to sunset and not be amended based on the time of day.  PC 14 at 2.  IDNR recommends that the regulatory limits apply to the nearest protected structure rather than the property line.  Id.  IDNR also notes that the monitoring methods in Part 910 are not conducive for use in monitoring blasting and seismographs are specifically designed for monitoring blasting.  Id.

IDNR states that the proposed rules are trying to fit air over pressure within the scope of intrusive noise and sound regulations when in fact the two are separate issues.  PC 14 at 2.  IDNR suggests that the surface aggregate and coal mine blasting provisions in the Board’s rules be deferred to IDNR for regulation.  Id.

IAAP

IAAP’s public comment echoes the testimony of Mr. Henriksen in suggesting that the Board can and should defer to IDNR for the regulation of blasting at aggregate and coal mining operations.  PC 15.  IAAP points out that IDNR has regulated blasting operations at aggregate mines since 1995 and surface coal mines since 1982.  Id.  IAAP suggests that if the Board does decide to regulate blasting from mine operations, then the rules should be amended to ensure consistency between the Board and IDNR’s regulations.  Id.

Ms. Therese M. Dorigan

Ms. Dorigan filed a comment on behalf of Kropp Forge.  PC 17.  Ms. Dorigan notes that Kropp Forge is not seeking any site-specific relief in this proceeding, but wishes to preserve any right to seek site-specific relief from the Board at a future date.  Id.

Dr. Paul Schomer and Village of Bridgeview

Dr. Schomer and Bridgeview reiterate the positions taken by Bridgeview at first notice concerning both the sound monitoring personnel qualifications and sound measurement procedures for establishing nuisance noise violation.  PC 16, 18, and 19.  Bridgeview also argues that bad science and misleading evidence should not be admissible in nuisance cases any more than in numeric violation cases and challenges the use of noise measurements in nuisance noise enforcement proceedings.  PC 18 at 1.  In addition, Dr. Schomer raises concerns about the efficacy of the Board’s rules.  PC 19 at 2.  The following paragraphs will summarize the comments.

Sound Monitoring Personnel Qualifications.  Dr. Schomer comments that the lay person taking sound measurements can make errors when taking those measurements.  PC 16 at 1.  According to Dr. Schomer those errors include:

1.	A lay person does not understand or necessarily hear properly background noise;

2.	A lay person does not understand the role that wind induced noise makes on measurements;

3.	A lay person does not understand the role that reflecting surfaces have on noise measurements.  PC 16 at 2-3.

The “implications” of errors in measurement in Dr. Schomer’s opinion is that the evidence of noise emissions measured by a lay person is inherently unreliable.  PC 16 at 3.

Dr. Schomer argues that the Board has a viable option in terms of identifying qualified noise professionals.  PC 16 at 5.  Dr. Schomer urges the Board to require that measurements of noise emissions be supervised by an individual who is a “Member, Board Certified, of the Institute of Noise Control Engineering of the USA” (Institute).  Id.  Dr. Schomer states that the Institute is a non-profit technical society and certification requires education, several years experience, passing a rigorous 8-hour exam, and periodic professional activities to maintain certification.  Id.

Dr. Schomer notes that as a lessor option, the Board could require that a member of the Institute, who is also a full member of the Acoustical Society of America, supervise noise emission measurements.  PC 16 at 6.  Dr. Schomer opines that with such a requirement the individual supervising the measurements “has some working knowledge of the mathematics of acoustics and of acoustical measurement.”  Id.

In Bridgeview’s final comment, Dr. Schomer states that there is one way to correctly perform measurements, but a multitude of ways to mess up measurements.  PC 19 at 1.  Dr. Schomer argues that ANSI procedures must be followed using ANSI approved methods and instrumentation.  Id.  Dr. Schomer opines that many measurement devices sold at stores are worthless and best used for indoor measurements.  Id.

Sound Measurement Procedures for Establishing Nuisance Noise Violation.  Dr. Schomer states that standards exist so that within a reasonable degree of scientific certainty, two people can perform the same measurements under identical conditions and get the similar results.  PC 16 at 4.  Dr. Schomer asserts that unless standards are followed, the Board has no valid scientific method by which to assess the accuracy of data submitted to the Board.  PC 16 at 4-5.  Dr. Schomer opines that the Board should use this rulemaking to make the standards for accurate measurements clear for both numeric and nuisance cases.  PC 16 at 5.

Efficacy of the Rules.  Dr. Schomer argues that allowing a lay person to make hand held A-weighted measurements does a disservice to the rules and to the noise receiver.  PC 16 at 3.  Dr. Schomer states that there is a trade-off between the complexity of the rules and the efficacy of the regulations.  Id.  Dr. Schomer notes that the Illinois rules are more complex than in many states, but as a result, Illinois’ rules are more useful and protective.  Id.  Dr. Schomer asserts that because measurements made with simple, hand-held A-weighted instruments fail to address the spectrum of octaves covered under Illinois regulations, the measurements give a misleading picture of the noise emissions.  PC 16 at 4.  Dr. Schomer opines that to the extent the Board allows such evidence to be admitted in a nuisance noise case, the entire decision making process may be tainted by misleading information.  Id.

Dr. Schomer, in his comment filed for Bridgeview, asserts that poor instruments may not detect many valid nuisance noise instances.  PC 19 at 2.  The instruments that do not measure individual octave bands may result in missing noise at a low frequency, according to Dr. Schomer.  Id.  Dr. Schomer argues that the octave band nature of the Board’s rules is a positive factor and should not be “squandered” by substituting A-weighted measurements of questionable validity.  Id.

Nuisance Noise Cases.  Bridgeview argues the Board should take the opportunity in these rules to establish that in a nuisance complaint valid noise measurements must be used.  PC 18 at 1.  Bridgeview states that if noise measurements are taken in a nuisance case, the measurements should be taken as directed in Part 910.  Id.  Bridgeview maintains that to do otherwise is misleading to the complainant, prejudicial to the Board’s decisions, and fundamentally unfair and costly to respondents.  Id.

Bridgeview argues that admitting noise emissions measurements taken by a lay person other than pursuant to the requirements of Part 910 is contrary to the Board’s responsibility for admitting scientific evidence based on Daubert v. Merrell Dow Pharmaceuticals, Inc., 501 U.S. 579 (1993).  PC 18 at 2.  Bridgeview asserts that the Board recognizes the legal principal articulated in Daubert but retreats froM the sound science approach in nuisance cases.  Id.  Bridgeview opines that allowing “quick and dirty” sound measurements to be used in nuisance cases, the Board is encouraging frivolous lawsuits.  Id.  Bridgeview further argues that the Board appears to be shifting the burden of proof by requiring the respondent to expose problems with the measurements.  PC 18 at 3.

DISCUSSION

During the first-notice period, the comments and testimony received by the Board reiterated issues raised prior to the Board proceeding to first notice in March 2005.  The issues are:  (1) whether blasting operations by aggregate producers and surface coal mines should be governed by rules adopted by the Board or should the Board defer to IDNR’s regulations; and (2) whether the Board should require that noise measurements taken for presentation in a nuisance noise enforcement case should be taken pursuant to 35 Ill. Adm. Code 910.  The Board will address these issues below.

Applicability of Noise Rules to Aggregate Producers and Surface Coal Mines

At first notice, the Board did make changes to the rule in response to IAAP’s comments.  After consideration of the testimony and comments from IAAP and IDNR during the first-notice comment period, the Board is convinced that additional changes to the rule are warranted.

The testimony of IAAP and IDNR clearly demonstrates that IDNR has an extensive regulatory program in place that regulates the noise from blasting at facilities operated by aggregate producers and surface coal mines.  Further, IDNR has an active staff that regularly oversees blasting and monitoring of the blasting at sites in Illinois.  IDNR also has stringent enforcement tools for any violation of the regulations when blasting.  Therefore, the Board finds that the Board can appropriately defer the regulation of noise emissions from blasting at aggregate and surface coal mines to IDNR.  The Board will make changes to the rule under Sections 901.107 and 901.109 to reflect that deference.

The Board notes that IAAP’s testimony indicates that IAAP only asks the Board to defer to IDNR for blasting at the aggregate producers sites in Illinois.  All other operations, such as back up beepers, ingress and egress, and daily production noise emission should be subject to the Board rules according to IAAP.  In light of this, the Board limits the scope of the proposed exemption to only to impulsive sound produced by explosive blasting activities, which are regulated by IDNR in accordance with Section 6.5 and Section 3.13 of the Surface Coal Mining Land Conservation and Reclamation Act (225 ILCS 715/6.5 and 3.13 (2004)).

Applicability of Part 910 to Nuisance Noise Enforcement Cases

The Board has long held in that noise measurements may be used to substantiate or refute a nuisance noise claim.  See Discovery South Group v. PCB, 275 Ill. App. 3d 547, 549, 656 N.E.2d 51, 59 (1st Dist. 1995); Charter Hall Home Owners Association v. Overland Transportation System PCB 98-81 (Oct. 1, 1998); Pawlowski v. Benchwarmers Pub, PCB 99-82 (Apr. 6, 2000); Gladys Knox and David Knox v. Turris Coal Company and AEI Resources, Inc., PCB 00-140 (Jan. 9, 2003).  In the Board’s first-notice opinion and order, the Board stated:

Section 910.104 clarifies that sound pressure level measurements are not required to establish a violation of the Board’s nuisance noise standard at Section 900.102, although such measurements may be used as corroborating evidence.  In light of this, the Board finds that compliance with the detailed sound measurement procedures set forth in Part 951 or the proposed Part 910 are not required for sound measurements used as corroborating evidence in a nuisance noise proceeding.  The Board will assign appropriate weight to sound measurement data submitted in any nuisance noise complaint based on the information in the hearing record and in accordance with the provisions of the Act.  As noted in the first first-notice opinion (Proposed New and Updated Rules for Measurement and Numerical Sound Emissions Standards; Amendments to 35 Ill. Adm. Code 901 and 910, R03-9 (July 10, 1003)), the Board believes that this approach is consistent with the Board’s precedence and case law concerning nuisance noise complaints.  The Board declines to make the changes suggested by IAAP.  Proposed New and Updated Rules for Measurement and Numerical Sound Emissions Standards; Amendments to 35 Ill. Adm. Code 901 and 910, R03-9, (Mar. 17, 2005).

The Board also discussed, at first notice, the request that noise measurements be supervised by an individual who is certified.  The Board stated:

The Board finds that the present regulatory structure, where the Board determines the validity of sound data on a case-by-case basis in noise enforcement cases, is a more prudent approach than prescribing vague qualification requirements.  The Board believes this is especially true absent a State approved licensing or certification process.  Proposed New and Updated Rules for Measurement and Numerical Sound Emissions Standards; Amendments to 35 Ill. Adm. Code 901 and 910, R03-9, (Mar. 17, 2005).

The Board has carefully reviewed the arguments and concerns expressed by Bridgeview and Dr. Schomer.  However, the Board is unconvinced that a change to the rule is necessary.  First, as to the suggestion that the Board require compliance with Part 910 for measurement of noise emissions in a nuisance noise enforcement, the Board does not believe that change is necessary.  If a complainant complies with the requirements of Part 910 when alleging a violation, then such a violation would be of the numeric standards and not a nuisance violation.  The Board has long heard testimony in nuisance noise enforcement cases concerning the use of noise measurements.  The Board examines that testimony and the measurements in light of the entirety of the evidence and renders a decision as to whether the noise unreasonablY interferes with the enjoyment of life.  See e.g., Zivoli v. Somebody’s Bar and Restaurant, PCB 90-200 (May 21, 1992).  Therefore, the Board declines to make a change to the rule.

regarding the qualifications of individual supervising the measurement of emissions, the Board is not persuaded that requiring certification by the Institute is appropriate.  The Board will continue to review the noise emission measurements on a case-by-case basis and make determination based on the evidence provided.

The Board finds that the extensive record in this proceeding supports proceeding to second notice with these noise regulations.  The Board further finds that the rules are technically feasible and economically reasonable.  Therefore the Board will proceed to second notice.

CONCLUSION

The Board today adopts for second notice regulations concerning measurement of and standards for noise emissions in Illinois.  The Board defers regulation of blasting operations at aggregate and surface coal mines to IDNR, who has an extensive regulatory scheme in place.  The Board declines to alter the Board’s longstanding position on noise emission measurements in nuisance noise enforcement cases and to require measurements be supervised by a certified individual.  The Board sends this proceeding to the Joint Committee on Administrative Review for second notice consideration.

## ORDER

The Board directs the Clerk to cause the filing of the following with the Joint Committee on Administrative Review for second notice consideration.

# TITLE 35:  ENVIRONMENTAL PROTECTION

# SUBTITLE H:  NOISE

## CHAPTER I:  POLLUTION CONTROL BOARD

### PART 901

### SOUND EMISSION STANDARDS AND LIMITATIONS FOR PROPERTY LINE-NOISE-SOURCES

Section

| 901.101 | Classification of Land According to Use |
| --- | --- |
| 901.102 | Sound Emitted to Class A Land |
| 901.103 | Sound Emitted to Class B Land |
| 901.104 | Highly - Impulsive Sound |
| 901.105 | Impact Forging Operations |
| 901.106 | Prominent Discrete Tones |
| 901.107 | Exceptions |
| 901.108 | Compliance Dates for Part 901 |
| 901.109 | Highly - Impulsive Sound from Explosive Blasting |
| 901.110 | Amforge Operational Level |
| 901.111 | Modern Drop Forge Operational Level |
| 901.112 | Wyman-Gordon Operational Level |
| 901.113 | Wagner Casting Site-Specific Operational Level (Repealed) |
| 901.114 | Moline Forge Operational Level |
| 901.115 | Cornell Forge Hampshire Division Site-Specific Operational Level |
| 901.116 | Forgings and Stampings, Inc. Operational Level |
| 901.117 | Rockford Drop Forge Company Operational Level |
| 901.118 | Atlas Forgings Division of Scot Forge Company – Franklin Park Division Operational Level |
| 901.119 | Clifford-Jacobs Operational Level |
| 901.120 | C.S. Norcross Operational Level |
| 901.121 | Vaughan & Bushnell Operational Level |
| APPENDIX A | Old Rule Numbers Referenced |
| APPENDIX B | Land-Based Classification Standards Standard Land Use Coding System |

AUTHORITY:  Implementing Section 25 and authorized by Section 27 of the Environmental Protection Act [415 ILCS 5/25 and 27].

SOURCE:  Originally filed as Part 2 of Chapter 8: Noise Pollution, effective August 10, 1973; amended at 2 Ill.  Reg.  27, p.  223, effective June 26, 1978; amended at 5 Ill.  Reg.  6371, effective June 1, 1981; amended at 5 Ill.  Reg.  8533, effective August 10, 1981; amended at 6 Ill.  Reg.  10960, effective September 1, 1982; codified at 7 Ill.  Reg.  13646; amended at 7 Ill.  Reg.  14519, effective October 17, 1983; amended in R83-35 at 8 Ill.  Reg.  18893, effective September 25, 1984; amended in R83-33, 26, 29, 30 and R83-34 at 9 Ill.  Reg.  1405, effective January 17, 1985; Section 901.105(f)(1), (2) and (3) recodified to Sections 901.110, 901.111 and 901.112 at 9 Ill.  Reg.  7147; amended in R83-25, 31 and 32 at 9 Ill.  Reg.  7149, effective May 7, 1985; amended in R83-7 at 11 Ill.  Reg.  3136, effective January 28, 1987; amended in R04-11, at 28 Ill. Reg. 11910, effective July 30, 2004; amended in R03-9 at 30 Ill. Reg._______, effective_________.

#### Section 901.101	Classification of Land According to Use

a)	The land use classification system used for the purposes of applying numeric sound standards for this Part is based on the Land-Based Classification Standards (LBCS) (Jeer, Sanjay. 2001. Land-Based Classification Standards .  Online, http://www.planning.org/LBCS.  American Planning Association:  Chicago, Illinois).  The LBCS applicable to this Part is set forth in Appendix B.

b) a)	Class A land includes shall include all land used as specified by LBCS Codes 1000 through 1340, 2410 through 2455, 5200 through 5230, 5500, 6100 through 6145, 6222, 6510 through 6530, 6568 through 6600.SLUCM Codes 110 through 190 inclusive, 651, 674, 681 through 683 inclusive, 691, 711, 762, 7121, 7122, 7123 and 921.

c) b)	Class B land includes shall include all land used as specified by LBCS Codes 2100 through 2336, 2500 through 2720, 3500 through 3600, 4220 through 4243, 5100 through 5160, 5300 through 5390, 5400, 6147, 6210 through 6221, 6300 through 6320, 6400 through 6430, 6560 through 6567, 6700 through 6830, 7100 through 7380.  SLUCM Codes 397, 471 through 479 inclusive, 511 through 599 inclusive, 611 through 649 inclusive, 652 through 673 inclusive, 675, 692, 699, 7124, 7129, 719, 721, 722 except 7223, 723 through 761 inclusive except 7311, 769 through 790 inclusive, and 922.

d) c)	Class C land includes shall include all land used as specified by LBCS Codes 3100 through 3440, 4120 through 4180, 4210 through 4212, 4300 through 4347, 7400 through 7450, 8000 through 8500, and 9100 through 9520.  SLUCM Codes 211 through 299 inclusive, 311 through 396 inclusive, 399, 411 except 4111, 412 except 4121, 421, 422, 429, 441, 449, 460, 481 through 499 inclusive, 7223 and 7311 used for automobile and motorcycle racing, and 811 through 890 inclusive.

e) d)	A parcel or tract of land used as specified by LBCS SLUCM Code 9100, 9400, or 5500 81, 83, 91, or 922, when adjacent to Class B or C land may be classified similarly by action of a municipal government having zoning jurisdiction over such land.  Notwithstanding any subsequent changes in actual land use, land so classified retains shall retain such B or C classification until the municipal government removes the classification adopted by it.

(Source:  Amended at 30 Ill. Reg. ____________, effective _____________)

Section 901.102	Sound Emitted to Class A Land

a)	Except as elsewhere provided in this Part provided, no person shall cause or allow the emission of sound during daytime hours from any property-line-noise-source located on any Class A, B or C land to any receiving Class A land which exceeds any allowable octave band sound pressure level specified in the following table, when measured at any point within such receiving Class A land, provided, however, that no measurement of sound pressure levels shall be made less than 25 feet from such property-line-noise-source.

| Octave Band Center Frequency (Hertz) | Allowable Octave Band Sound Pressure Levels (dB) of Sound Emitted to any Receiving Class A Land from |
| --- | --- |

|  | Class C Land | Class B Land | Class A Land |
| --- | --- | --- | --- |
| 31.5 | 75 | 72 | 72 |
| 63 | 74 | 71 | 71 |
| 125 | 69 | 65 | 65 |
| 250 | 64 | 57 | 57 |
| 500 | 58 | 51 | 51 |
| 1000 | 52 | 45 | 45 |
| 2000 | 47 | 39 | 39 |
| 4000 | 43 | 34 | 34 |
| 8000 | 40 | 32 | 32 |

b)	Except as provided elsewhere in this Part provided, no person shall cause or allow the emission of sound during nighttime hours from any property-line-noise-source located on any Class A, B or C land to any receiving Class A land which exceeds any allowable octave band sound pressure level specified in the following table, when measured at any point within such receiving Class A land, provided, however, that no measurement of sound pressure levels shall be made less than 25 feet from such property-line-noise-source.

| Octave Band Center Frequency (Hertz) | Allowable Octave Band Sound Pressure Levels (dB) of Sound Emitted to any Receiving Class A Land from |
| --- | --- |

|  | Class C Land | Class B Land | Class A Land |
| --- | --- | --- | --- |
| 31.5 | 69 | 63 | 63 |
| 63 | 67 | 61 | 61 |
| 125 | 62 | 55 | 55 |
| 250 | 54 | 47 | 47 |
| 500 | 47 | 40 | 40 |
| 1000 | 41 | 35 | 35 |
| 2000 | 36 | 30 | 30 |
| 4000 | 32 | 25 | 25 |
| 8000 | 32 | 25 | 25 |

(Source:  Amended at 30 Ill.  Reg.  __________, effective ___________)

#### Section 901.103	Sound Emitted to Class B Land

Except as provided elsewhere in this Part provided, no person shall cause or allow the emission of sound from any property-line-noise-source located on any Class A, B or C land to any receiving Class B land which exceeds any allowable octave band sound pressure level specified in the following table, when measured at any point within such receiving Class B land, provided, however, that no measurement of sound pressure levels shall be made less than 25 feet from such property-line-noise-source.

| Octave Band Center Frequency (Hertz) | Allowable Octave Band Sound Pressure Levels (dB) of Sound Emitted to any Receiving Class B Land from |
| --- | --- |

|  | Class C Land | Class B Land | Class A Land |
| --- | --- | --- | --- |
| 31.5 | 80 | 79 | 72 |
| 63 | 79 | 78 | 71 |
| 125 | 74 | 72 | 65 |
| 250 | 69 | 64 | 57 |
| 500 | 63 | 58 | 51 |
| 1000 | 57 | 52 | 45 |
| 2000 | 52 | 46 | 39 |
| 4000 | 48 | 41 | 34 |
| 8000 | 45 | 39 | 32 |

(Source:  Amended at 30 Ill.  Reg.  __________, effective ___________)

#### Section 901.104	Highly-Impulsive Sound

Except as provided elsewhere in this Part provided, no person shall cause or allow the emission of highly-impulsive sound from any property-line-noise-source located on any Class A, B, or C land to any receiving Class A or B land which exceeds the allowable A-weighted sound levels, measured with fast dynamic characteristic, specified in the following table when measured in accordance with the procedure of 35 Ill. Adm. Code 900.103 at any point within such receiving Class A or B land, provided, however, that no measurement of sound levels shall be made less than 25 feet from such property-line-noise-source.

| Classification of Land on which Property-Line Noise-Source: is Located | Allowable A-weighted Sound Levels in Decibels of Highly-Impulsive Sound Emitted to Receiving Class A or B Land |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Class B Land | Class A Land |  |
|  |  |  | Daytime | Nighttime |
| Class A Land |  | 50 47 | 50 47 | 45 37 |
| Class B Land |  | 57 54 | 50 47 | 45 37 |
| Class C Land |  | 61 58 | 56 53 | 46 43 |

(Source:  Amended at 30 Ill. Reg. ___________, effective __________)

#### Section 901.105	Impact Forging Operations

a)	For purposes of this Section, rule only the following are applicable:

1)	Daytime hours means shall mean any continuous 16-hour period between 6:00 a.m. and 11:00 p.m. local time; and

2)	Nighttime hours means shall mean those 8 hours between 10:00 p.m. and 7:00 a.m. which are not part of the 16 continuous daytime hours.

3)	The reference time for Leq, as defined in 35 Ill.  Adm. Code 900.101 is one hour.

4)	New Impacting Forging Operation is that property-line-noise-source comprised of impact forging operation on which construction began after September 1, 1982.

5)	Existing Impact Forging Operation is that property-line-noise-source comprised of impact forging operations which are in existence on September 1, 1982,

b)	Emission Limitations for New Impact Forging Operation.

No impact forging operation shall cause or allow the emission of impulsive sound to any receiving Class A or B land which exceeds the allowable sound levels specified in the following table when measured at any point within such receiving land, provided however, that no measurement of sound levels shall be made less than 25 feet from such new impact forging operation's property-line.

- Allowable Highly- Impulsive Sound Levels (Leq) in Decibels Emitted

- To Class A or B Land from New

- Impact Forging Operation

| - Class B Land | - Class A Land |
| --- | --- |

|  | - Daytime | - Nighttime |
| --- | --- | --- |
| - 59.5 Leq | - 53.5 Leq | - 48.5 Leq |

- Limitations for Existing Impact Forging Operation

No existing impact forging operation shall cause or allow the emission of highly-impulsive sound to any receiving Class A or B land which exceeds the allowable sound levels specified in the following table, when measured at any point within such receiving land, provided however, that no measurement of sound levels shall be made less than 25 feet from such existing impact forging operation's property-line, unless such forging operation is granted a permanent site specific allowable operational level pursuant to subsection subparagraph (d).

- Allowable Highly- Impulsive Sound Levels (Leq) in Decibels Emitted

- To Class A or B Land from Existing

- Impact Forging Operation

| - Class B Land | - Class A Land |
| --- | --- |

|  | - Daytime | - Nighttime |
| --- | --- | --- |
| - 64.5 Leq | - 58.5 Leq | - 53.5 Leq |

d)	Site Specific Allowable Operational Level for Existing Impact Forging Operation

1)	An existing impact forging operation which does not comply with subsection subparagraph (c) may seek a permanent site specific allowable operational level from the Board.  A permanent site specific level is that level of operation allowed petitioner after review and approval by the Board and after implementation of abatement abated measures, if any, approved by the Board.

2)	Any existing impact forging operation seeking a permanent site specific operational level must submit as its petition the following:

A)	The location of the petitioner, a description of the surrounding community, and a map locating the petitioner within the community;

B)	A description of the petitioner's operations, the number and size of the petitioner's forging hammers, the current hours of hammer operation, the approximate number of forgings manufactured during each of the three prior calendar years and the approximate number of hammer blows used to manufacture the forgings.

C)	A description of any existing sound abatement measure.

D)	The sound levels in excess of those permitted by subsection subparagraph (c) emitted by the petitioner into the community, in 5 decibel increments measured in Leq Leq, shown on the map of the community.

E)	The number of residences exposed to sound levels in excess of those permitted by subsection subparagraph (c);

F)	A description of other significant sources of noise (mobile and stationary) and their location shown on the map of the community;

G)	A description of the proposed operational level and proposed physical abatement measures, if any, a schedule for their implementation and their costs;

H)	The predicted improvement in community sound levels as a result of implementation of the proposed abatement measures; and

I)	A description of the economic and technical considerations which justify the permanent site specific allowable operational level sought by petitioner.

3)	An existing impact forging operation seeking a permanent site specific operational level shall prepare and file its petition with the Board and Agency as applicable no later than December 1, 1983.

4)	The Agency shall prepare a written evaluation regarding each petition seeking a permanent site specific operational level and shall file said evaluation within ninety (90) days following receipt of the petition with both the Board and the petitioner.

e)	Land Use Classifications Preserved

The land use classifications in effect within a one-mile radius of an existing impact forging operation on September 1, 1982 remains shall remain the applicable land use classification for enforcement of these rules against an existing forging operation and any future modification thereof, regardless of actual subsequent changes in land use unless such actual changes would impose less restrictive limitations on the impact forging operations.

f)	Site-Specific Operational Levels

Each individual existing forging operation identified in Sections 901.110, 901.111 and 901.112 must comply with the site-specific operational level defined, or is otherwise subject to Section 901.105(c).

(Source:  Amended at 30 Ill.  Reg.  __________, effective ___________)

#### Section 901.106	Prominent Discrete Tones

a)	No person shall cause or allow the emission of any prominent discrete tone from any property-line-noise-source located on any Class A, B or C land to any receiving Class A, B or C land, provided, however, that no measurement of one-third octave band sound pressure levels shall be made less than 25 feet from such property-line source.

b)	This rule shall not apply to prominent discrete tones having a one-third octave band sound pressure level 10 or more dB below the allowable octave band sound pressure level specified in the applicable tables in Sections 901.102 through 901.104 for the octave band which contains such one-third octave band.  In the application of this sub-section, the applicable numeric standard table for sound emitted from any existing property-line-noise-source to receiving Class A land, for both daytime and nighttime operations, is shall be found in Section 901.102(a).

(Source:  Amended at 30 Ill. Reg. ________, effective __________)

#### Section 901.107	Exceptions

a)	Sections 901.102 through 901.106 inclusive does shall not apply to sound emitted from land used as specified by LBCS Codes 1100, 6600 and 5500 SLUCM Codes 110, 140, 190, 691, and 742 except 7424 and 7425.

b)	Sections 901.102 through 901.106 inclusive does shall not apply to sound emitted from emergency warning devices and unregulated safety relief valves.

c)	Sections 901.102 through 901.106 inclusive does shall not apply to sound emitted from lawn care maintenance equipment and agricultural field machinery used during daytime hours.  For the purposes of this sub-section, grain dryers operated off the farm are shall not be considered agricultural field machinery.

d)	Sections 901.102 through 901.106 inclusive do shall not apply to sound emitted from equipment being used for construction.

e)	Section 901.102(b) do shall not apply to sound emitted from existing property-line-noise-sources during nighttime hours, provided, however, that sound emitted from such existing property-line-noise-sources are shall be governed during nighttime hours by the limits specified in Section 901.102.

f)	Sections 901.102 through 901.106 inclusive do shall not apply to the operation of any vehicle registered for highway use while such vehicle is being operated within any land used as specified by Section 901.101 in the course of ingress to or egress from a highway.

g)	Sections 901.102 through 901.106 inclusive do shall not apply to sound emitted from land used as specified by LBCS Codes 5130 and 5140 SLUCM Codes 7223 and 7311 when used for automobile and motorcycle racing; and, any land used for contests, rallies, time trials, test runs or similar operations of any self-propelled device, and upon or by which any person is or may be transported or drawn, when such self-propelled device is actually being used for sport or recreation and is actually participating in an activity or event organized, regulated, and supervised under the sponsorship and sanction of a club, organization or corporation having national or statewide recognition; provided, however, that the exceptions granted in this subsection subparagraph do shall not apply to any automobile and motorcycle race, contest, rally, time trial, test run or similar operation of any self-propelled device if such event is started between the hours of 10:30 p.m. to 7:00 a.m., local time weekdays, or between the hours of 11:00 p.m. and 7:00 a.m., local time, weekend days.

h)	Section 901.104 shall not apply to impulsive sound produced by explosive blasting activities conducted on any Class C land other than the land used as specified by LBCS Codes 8300 and 8500 SLUCN codes 852 and 854, but such operations shall be governed by Section 901.109.

i)	Part 901 shall not apply to impulsive sound produced by explosive blasting activities, which are:

1)	Conducted on any Class C land used as specified by LBCS Codes 8300 and 8500; and

2)	Regulated by the Department of Natural Resources in accordance with Section 6.5 and Section 3.13 of the Surface Coal Mining Land Conservation and Reclamation Act (225 ILCS 715/6.5 and 3.13 (2004)).

ji)	Sections 901.102 through 901.106 inclusive, do shall not apply to sound emitted from snowmobiles.

(Source:  Amended at 30 Ill.  Reg.  __________, effective ___________)

#### Section 901.108	Compliance Dates for Part 901

a)	Except as provided in subsections subparagraphs (g), (i), and (j), every owner or operator of a new property-line-noise-source must shall comply with the standards and limitations of this Part on and after August 10, 1973.

b)	Except as otherwise provided in this rule, every owner or operator of an existing property-line-noise-source must shall comply with the standards and limitations of this Part on and August 10, 1974.

c)	Every owner or operator of an existing property-line-noise-source who emits sound which exceeds any allowable octave band sound pressure level of Section 901.102 or 901.103 by 10 dB or more in any octave band with a center frequency of 31.5 Hertz, 63 Hertz or 125 Hertz must shall comply with the standards and limitations of this Part on and after February 10, 1975.

d)	Except as provided in subsections subparagraphs (g) and (h), every owner or operator of an existing property-line-noise-source required to comply with Section 901.104 must shall comply with the standards and limitations of this Part on and after February 10, 1975.

e)	Every owner or operator of an existing property-line-noise-source required to comply with Section 901.106 must shall comply with the standards and limitations of this Part on and after February 10, 1975.

f)	Repealed

f)g)	Every owner or operator of Class C land now and hereafter used as specified by LBCS Code 4120 SLUCM Code 4112  shall will have until August 10, 1976 to bring the sound from railroad car coupling in compliance with Section 901.104.

g)h)	Existing impact forging operations as defined in Section 901.105 which do not seek permanent site specific allowable operational levels must shall comply with Section 901.105 by December 1, 1983.  Those seeking permanent site specific allowable operational levels pursuant to Section 901.105(d) must shall comply as of the effective date of the site specific rule granted or denied.

h)i)	Every owner or operator of Class C land now or hereafter used as specified by LBCS Code 3310 SLUCM Code 291 must shall comply with the standards and limitations of this Part on August 10, 1975.

i)j)	Every owner or operator of Class C land now or hereafter used as specified by LBCS Code 5130 and 5140 SLUCM Code 7223 and 7311 when used for automobile and motorcycle racing shall must comply with the standards and limitations of this Part on February 10, 1976.

(Source:  Amended at 30 Ill.  Reg. ____________, effective __________)

#### Section 901.109	Highly-Impulsive Sound From Explosive Blasting

a)	During the daytime hours that cover the period,after sunrise and before sunset, no person shall cause or allow any explosive blasting conducted on any Class C land other than land used as specified by LBCS Codes 8300 and 85200 SLUCM Codes 852 and 854  so as to allow the emission of sound to any receiving Class A or B land which exceeds the allowable outdoor C-weighted sound levels, measured with the slow dynamic characteristic, specified in the following table, when measured at any point, of reasonable interference with the use of such receiving Class A or B land.

Allowable Outdoor C-Weighted

Sound Exposure Levels in Decibels of Explosive

Blasting Sounds Emitted to Receiving

Class A or B Land from Any Class C Land other than Land

Used as Specified by LBCS Code 8300 or 8500 SLUCM Code 852 or 854

| Receiving Class A Land | Receiving Class B Land |
| --- | --- |
| 109 107 | 114 112 |

The allowable sound exposure level limits in the above table must be lowered by three decibels (3 dB) for each doubling of the number of blasts during the day or night.

b)	Compliance with outdoor peak sound pressure level limits in the following table shall constitute prima facie level limits of this rule when measured on such receiving Class A or B land.

Equivalent Maximum Sound Pressure Level (Peak) Limits in Decibels

| Lower Frequency Limit of Measuring System for Flat Response, a Variation from Linear Response of + or - 3dB (Hz) | Receiving Class A Land (dB) | Receiving Class B Land (dB) |
| --- | --- | --- |
| < o 0.1 | 135 | 140 |
| < 2.0 but > 0.1 | 1332 | 1337 |
| < 6.0 but greater than 2.0 | 130 | 135 |

The allowable equivalent maximun sound pressure level limits in the above table

shall be lowered by two decibels (2 dB) for each doubling of the number of blasts

during the day or night.

c)	During the period defined by both the beginning of the nighttime hours (10:00 pm) or sunset, whichever occurs earlier and the ending of the nighttime hours (7:00 am) or , after sunset and before sunrise, whichever occurs later, the allowable sound level limits in subsections subparagraphs (a) and (b) must shall be reduced by 10 decibels decibles except in emergency situations where rain, lightning, other atmospheric conditions, or operator or public safety requires unscheduled nighttime hour explosive blasting.

d)	Persons causing or allowing explosive blasting to be conducted on any Class C land other than land used as specified by LBCS Code 8300 or 8500 SLUCM code 852 or 854  must shall notify the local public of such blasting prior to its occurrence, except when emergency situations require unscheduled blasting, by publication of a blasting schedule, identifying the work days or dates and time periods when explosives are expected to be detonated, at least every three months in a newspaper of general circulation in the locality of the blast site.

(Source:  Amended at 30 Ill.  Reg.  __________, effective ___________)

#### Section 901.110	Amforge Operational Level

Amforge Division of Rockwell International located at 119th Street, Chicago, Illinois must shall:

a)	Operate only ten forging hammers at any one time; and

b)	Operation of its forging hammers is limited to the hours of 7:00 a.m. through 11:00 p.m., with occasional operations beginning at 6:00 a.m. and ending at midnight, Monday through Saturdays; and

c)	Install sound Sound absorptive materials shall be installed on each of the forging hammer structures as each is routinely overhauled, but no later than January 1, 1987.

(Source:  Amended at 30 Ill.  Reg.  __________, effective ___________)

#### Section 901.111	Modern Drop Forge Operational Level

Modern Drop Forge Company located at 139th Street and Western Avenue in Blue

Island, Illinois must shall:

a)	Operate only twenty-one forging hammers at any one time; and

b)	Operate its forging hammers only during the hours of 6:00 a.m. through midnight, Mondays through Fridays, and 6:30 a.m. until 7:30 p.m. on Saturdays.

(Source:  Amended at 30 Ill.  Reg.  __________, effective ___________)

#### Section 901.113	Wagner Casting Site-Specific Operational Level (Repealed)

Wagner Casting Company and future owners of the forging facility located at the southeast corner of Sangamon and Jasper Streets in Decatur, Illinois, must comply with the following site-specific operational level or are otherwise subject to Section 901.105(c):

a)	Shall operate no more than nine forging hammers at any one time; and

b)	Shall operate its forging hammers only between the hours of 5:00 a.m. Monday through 9:00 p.m. Saturday.

(Source:  Repealed at 30 Ill.  Reg.  __________, effective ___________)

#### Section 901.118	Atlas Forgings Division of Scot Forge Company – Franklin Park Division Operational Level

Atlas Forge Division of Scot Forge and future owners of the forging facility located at 9394 W. Belmont Avenue, Franklin Park 1501 South 55th Court, Cicero, Illinois, must shall comply with the following site-specific operational level:

a)	Operate no more than seven three forging hammers at any one time; and

b)	Operate its forging hammers only between the hours of 6:00 a.m. and 6:00 p.m. Monday through Saturday.

(Source:  Amended at 30 Ill.  Reg.  __________, effective ___________)

#### Section 901.121	Vaughan & Bushnell Operational Level

Vaughan & Bushnell Manufacturing Company and the future owners of the forging facility located at the intersection of Davis and Main Streets, Bushnell, Illinois, must shall comply with the following site-specific operational level:

a)	Operate no more than ten hammers at any one time; and

b)	Operate its forging hammers only between the hours of 6:00 a.m. and 1:30 a.m. p.m. Monday through Saturday.

(Source:  Amended at 30 Ill.  Reg.  __________, effective ___________)

Section 901.APPENDIX B	Land-Based Classification Standards and Corresponding 35 Ill. Adm. Code 901 Land Classes Standard Land Use Coding System

| LBCS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Main Category | Function Code |  | Description | 35 IAC 901 Land Class |
| Residence or accommodation functions | 1000 |  | Residence or accommodation functions | # A |
|  | 1100 |  | Private household |  |
|  | 1200 |  | Housing services for the elderly |  |
|  | 1210 |  | Retirement housing services |  |
|  | 1220 |  | Congregate living services |  |
|  | 1230 |  | Assisted-living services |  |
|  | 1240 |  | Life care or continuing care services |  |
|  | 1250 |  | Skilled-nursing services |  |
|  | 1300 |  | Hotels, motels, or other accommodation services |  |
|  | 1310 |  | Bed and breakfast inn |  |
|  | 1320 |  | Rooming and boarding |  |
|  | 1330 |  | Hotel, motel, or tourist court |  |
|  | 1340 |  | Casino hotel | # A |
| General sales or services | 2000 |  | General sales or services |  |
|  | 2100 |  | Retail sales or service | B |
|  | 2110 |  | Automobile sales or service establishment |  |
|  | 2111 |  | Car dealer |  |
|  | 2112 |  | Bus, truck, mobile homes, or large vehicles |  |
|  | 2113 |  | Bicycle, motorcycle, ATV, etc. |  |
|  | 2114 |  | Boat or marine craft dealer |  |
|  | 2115 |  | Parts, accessories, or tires |  |
|  | 2116 |  | Gasoline service |  |
|  | 2120 |  | Heavy consumer goods sales or service |  |
|  | 2121 |  | Furniture or home furnishings |  |
|  | 2122 |  | Hardware, home centers, etc. |  |
|  | 2123 |  | Lawn and garden supplies |  |
|  | 2124 |  | Department store, warehouse club or superstore |  |
|  | 2125 |  | Electronics and Appliances |  |
|  | 2126 |  | Lumber yard and building materials |  |
|  | 2127 |  | Heating and plumbing equipment |  |
|  | 2130 |  | Durable consumer goods sales and service |  |
|  | 2131 |  | Computer and software |  |
|  | 2132 |  | Camera and photographic supplies |  |
|  | 2133 |  | Clothing, jewelry, luggage, shoes, etc. |  |
|  | 2134 |  | Sporting goods, toy and hobby, and musical instruments |  |
|  | 2135 |  | Books, magazines, music, stationery |  |
|  | 2140 |  | Consumer goods, other |  |
|  | 2141 |  | Florist |  |
|  | 2142 |  | Art dealers, supplies, sales and service |  |
|  | 2143 |  | Tobacco or tobacconist establishment |  |
|  | 2144 |  | Mail order or direct selling establishment |  |
|  | 2145 |  | Antique shops, flea markets, etc. |  |
|  | 2150 |  | Grocery, food, beverage, dairy, etc. |  |
|  | 2151 |  | Grocery store, supermarket, or bakery |  |
|  | 2152 |  | Convenience store |  |
|  | 2153 |  | Specialty food store |  |
|  | 2154 |  | Fruit and vegetable store |  |
|  | 2155 |  | Beer, wine, and liquor store |  |
|  | 2160 |  | Health and personal care |  |
|  | 2161 |  | Pharmacy or drug store |  |
|  | 2162 |  | Cosmetic and beauty supplies |  |
|  | 2163 |  | Optical |  |
|  | 2200 |  | Finance and Insurance |  |
|  | 2210 |  | Bank, credit union, or savings institution |  |
|  | 2220 |  | Credit and finance establishment |  |
|  | 2230 |  | Investment banking, securities, and brokerages |  |
|  | 2240 |  | Insurance-related establishment |  |
|  | 2250 |  | Fund, trust, or other financial establishment |  |
|  | 2300 |  | Real estate, and rental and leasing |  |
|  | 2310 |  | Real estate services |  |
|  | 2320 |  | Property management services |  |
|  | 2321 |  | Commercial property-related |  |
|  | 2322 |  | Rental housing-related |  |
|  | 2330 |  | Rental and leasing |  |
|  | 2331 |  | Cars |  |
|  | 2332 |  | Leasing trucks, trailers, RVs, etc. |  |
|  | 2333 |  | Recreational goods rental |  |
|  | 2334 |  | Leasing commercial, industrial machinery, and equipment |  |
|  | 2335 |  | Consumer goods rental |  |
|  | 2336 |  | Intellectual property rental (video, music, software, etc.) | ### B |
|  | 2400 |  | Business, professional, scientific, and technical services |  |
|  | 2410 |  | Professional services | # A |
|  | 2411 |  | Legal services |  |
|  | 2412 |  | Accounting, tax, bookkeeping, payroll services |  |
|  | 2413 |  | Architectural, engineering, and related services |  |
|  | 2414 |  | Graphic, industrial, interior design services |  |
|  | 2415 |  | Consulting services (management, environmental, etc.) |  |
|  | 2416 |  | Research and development services (scientific, etc.) |  |
|  | 2417 |  | Advertising, media, and photography services |  |
|  | 2418 |  | Veterinary services |  |
|  | 2420 |  | Administrative services |  |
|  | 2421 |  | Office and administrative services |  |
|  | 2422 |  | Facilities support services |  |
|  | 2423 |  | Employment agency |  |
|  | 2424 |  | Business support services |  |
|  | 2425 |  | Collection agency |  |
|  | 2430 |  | Travel arrangement and reservation services |  |
|  | 2440 |  | Investigation and security services |  |
|  | 2450 |  | Services to buildings and dwellings |  |
|  | 2451 |  | Extermination and pest control |  |
|  | 2452 |  | Janitorial |  |
|  | 2453 |  | Landscaping |  |
|  | 2454 |  | Carpet and upholstery cleaning |  |
|  | 2455 |  | Packing, crating, and convention and trade show services | # A |
|  | 2500 |  | Food services | B |
|  | 2510 |  | Full-service restaurant |  |
|  | 2520 |  | Cafeteria or limited service restaurant |  |
|  | 2530 |  | Snack or nonalcoholic bar |  |
|  | 2540 |  | Bar or drinking place |  |
|  | 2550 |  | Mobile food services |  |
|  | 2560 |  | Caterer |  |
|  | 2570 |  | Food service contractor |  |
|  | 2580 |  | Vending machine operator |  |
|  | 2600 |  | Personal services |  |
|  | 2700 |  | Pet and animal sales or service (except veterinary) |  |
|  | 2710 |  | Pet or pet supply store |  |
|  | 2720 |  | Animal and pet services | # B |
| Manufacturing and wholesale trade | 3000 |  | Manufacturing and wholesale trade |  |
|  | 3100 |  | Food, textiles, and related products | C |
|  | 3110 |  | Food and beverages |  |
|  | 3120 |  | Tobacco manufacturing establishment |  |
|  | 3130 |  | Textiles |  |
|  | 3140 |  | Leather and allied products |  |
|  | 3200 |  | Wood, paper, and printing products |  |
|  | 3210 |  | Wood products establishment |  |
|  | 3220 |  | Paper and printing materials |  |
|  | 3230 |  | Furniture and related products |  |
|  | 3300 |  | Chemicals, and metals, machinery, and electronics manufacturing |   |
|  | 3310 |  | Petroleum and coal products |  |
|  | 3320 |  | Chemicals, plastics, and rubber products |  |
|  | 3330 |  | Nonmetallic mineral products |  |
|  | 3340 |  | Primary metal manufacturing |  |
|  | 3350 |  | Machinery manufacturing |  |
|  | 3360 |  | Electrical equipment, appliance, and components manufacturing |   |
|  | 3370 |  | Transportation equipment, automobiles, etc. |  |
|  | 3400 |  | Miscellaneous manufacturing |  |
|  | 3410 |  | Jewelry and silverware |  |
|  | 3420 |  | Dolls, toys, games, and musical instruments |  |
|  | 3430 |  | Office supplies, inks, etc. |  |
|  | 3440 |  | Signs | # C |
|  | 3500 |  | Wholesale trade establishment | B |
|  | 3510 |  | Durable goods |  |
|  | 3520 |  | Nondurable goods |  |
|  | 3600 |  | Warehouse and storage services | # B |
| Transportation, communication, information, and utilities | 4000 |  | Transportation, communication, information, and utilities |  |
|  | 4100 |  | Transportation services |  |
|  | 4110 |  | #### Air transportation | U |
|  | 4111 |  | Air passenger transportation |  |
|  | 4112 |  | Air freight transportation |  |
|  | 4113 |  | Airport and support establishment |  |
|  | 4114 |  | Aircraft and accessories |  |
|  | 4115 |  | Other air transportation (including scenic, balloon, etc.) | # U |
|  | 4120 |  | Rail transportation | C |
|  | 4121 |  | Rail passenger transportation |  |
|  | 4122 |  | Rail freight transportation |  |
|  | 4123 |  | Rail transportation support establishment |  |
|  | 4130 |  | Road, ground passenger, and transit transportation |  |
|  | 4131 |  | Local transit systems-mixed mode |  |
|  | 4132 |  | Local transit systems-commuter rail |  |
|  | 4133 |  | Local transit systems-bus, special needs, and other motor vehicles |   |
|  | 4134 |  | Interurban, charter bus, and other similar establishments |  |
|  | 4135 |  | School and employee bus transportation |  |
|  | 4136 |  | Special purpose transit transportation (including scenic, sightseeing, etc.) |   |
|  | 4137 |  | Taxi and limousine service |  |
|  | 4138 |  | Towing and other road and ground services |  |
|  | 4140 |  | Truck and freight transportation services |  |
|  | 4141 |  | General freight trucking, local |  |
|  | 4142 |  | General freight trucking, long-distance |  |
|  | 4143 |  | Freight trucking, specialized (used household and office goods) |   |
|  | 4144 |  | Freight trucking, specialized (except used goods) |  |
|  | 4150 |  | Marine and water transportation |  |
|  | 4151 |  | Marine passenger transportation |  |
|  | 4152 |  | Marine freight transportation |  |
|  | 4153 |  | Marine port and harbor operations |  |
|  | 4154 |  | Marine cargo handling and dry dock services |  |
|  | 4155 |  | Marine navigational and other services |  |
|  | 4160 |  | Courier and messenger services |  |
|  | 4170 |  | Postal services |  |
|  | 4180 |  | Pipeline transportation | # C |
|  | 4200 |  | Communications and information |  |
|  | 4210 |  | Publishing | C |
|  | 4211 |  | Newspapers, books, periodicals, etc. | C |
|  | 4212 |  | Software publisher | # C |
|  | 4220 |  | Motion pictures and sound recording | B |
|  | 4221 |  | Motion picture and video production, publishing, and distribution |  |
|  | 4222 |  | Motion picture viewing and exhibition services |  |
|  | 4223 |  | Sound recording, production, publishing, and distribution |  |
|  | 4230 |  | Telecommunications and broadcasting |  |
|  | 4231 |  | Radio and television broadcasting |  |
|  | 4232 |  | Cable networks and distribution |  |
|  | 4233 |  | Wireless telecommunications |  |
|  | 4234 |  | Telephone and other wired telecommunications |  |
|  | 4240 |  | Information services and data processing industries |  |
|  | 4241 |  | Online information services |  |
|  | 4242 |  | Libraries and archives |  |
|  | 4243 |  | News syndicate | # B |
|  | 4300 |  | Utilities and utility services | # C |
|  | 4310 |  | Electric power |  |
|  | 4311 |  | Hydroelectric |  |
|  | 4312 |  | Fossil |  |
|  | 4313 |  | Nuclear |  |
|  | 4314 |  | Alternative energy sources |  |
|  | 4320 |  | Natural gas, petroleum, fuels, etc. |  |
|  | 4330 |  | Water, steam, air conditioning supply |  |
|  | 4331 |  | Drinking water |  |
|  | 4332 |  | Irrigation and industrial water supply |  |
|  | 4333 |  | Air conditioning and steam supply |  |
|  | 4340 |  | Sewer, solid waste, and related services |  |
|  | 4341 |  | Hazardous waste collection |  |
|  | 4342 |  | Hazardous waste treatment and disposal |  |
|  | 4343 |  | Solid waste collection |  |
|  | 4344 |  | Solid waste combustor or incinerator |  |
|  | 4345 |  | Solid waste landfill |  |
|  | 4346 |  | Waste treatment and disposal |  |
|  | 4347 |  | Septic tank and related services | # C |
| Arts, entertainment, and recreation | 5000 |  | Arts, entertainment, and recreation |  |
|  | 5100 |  | Performing arts or supporting establishment | B |
|  | 5110 |  | Theater, dance, or music establishment |  |
|  | 5120 |  | Sports team or club |  |
|  | 5130 |  | Racetrack establishment |  |
|  | 5140 |  | Promoter of performing arts, sports, and similar events |  |
|  | 5150 |  | Agent for management services |  |
|  | 5160 |  | Independent artist, writer, or performer | # B |
|  | 5200 |  | Museums and other special purpose recreational institutions | A |
|  | 5210 |  | Museum |  |
|  | 5220 |  | Historical or archeological institution |  |
|  | 5230 |  | Zoos, botanical gardens, arboreta, etc. | # A |
|  | 5300 |  | Amusement, sports, or recreation establishment | B |
|  | 5310 |  | Amusement or theme park establishment |  |
|  | 5320 |  | Games arcade establishment |  |
|  | 5330 |  | Casino or gambling establishment |  |
|  | 5340 |  | Miniature golf establishment |  |
|  | 5350 |  | Skiing |  |
|  | 5360 |  | Marina or yachting club facility operators |  |
|  | 5370 |  | Fitness, recreational sports, gym, or athletic club, |  |
|  | 5380 |  | Bowling, billiards, pool, etc. |  |
|  | 5390 |  | Skating rinks, roller skates, etc. |  |
|  | 5400 |  | Camps, camping, and related establishments | # B |
|  | 5500 |  | Natural and other recreational parks | A |
| Education, public admin., health care, and other inst. | 6000 |  | Education, public admin., health care, and other inst. |  |
|  | 6100 |  | Educational services |  |
|  | 6110 |  | Nursery and preschool | A |
|  | 6120 |  | Grade schools |  |
|  | 6121 |  | Elementary |  |
|  | 6122 |  | Middle |  |
|  | 6123 |  | Senior |  |
|  | 6124 |  | Continuance |  |
|  | 6125 |  | Alternate education services |  |
|  | 6126 |  | Adult education services |  |
|  | 6130 |  | Colleges and universities |  |
|  | 6140 |  | Technical, trade, and other specialty schools |  |
|  | 6141 |  | Beauty schools |  |
|  | 6142 |  | Business management |  |
|  | 6143 |  | Computer training |  |
|  | 6144 |  | Driving education |  |
|  | 6145 |  | Fine and performing arts education | # A |
|  | 6146 |  | Flight training | # U |
|  | 6147 |  | Sports and recreation education | # B |
|  | 6200 |  | Public administration |  |
|  | 6210 |  | Legislative and executive functions | B |
|  | 6220 |  | Judicial functions | B |
|  | 6221 |  | Courts | # B |
|  | 6222 |  | Correctional institutions | A |
|  | 6300 |  | Other government functions | B |
|  | 6310 |  | Military and national security |  |
|  | 6320 |  | Space research and technology |  |
|  | 6400 |  | Public Safety |  |
|  | 6410 |  | Fire and rescue |  |
|  | 6420 |  | Police |  |
|  | 6430 |  | Emergency response | # B |
|  | 6500 |  | Health and human services |  |
|  | 6510 |  | Ambulatory or outpatient care services | A |
|  | 6511 |  | Clinics |  |
|  | 6512 |  | Family planning and outpatient care centers |  |
|  | 6513 |  | Medical and diagnostic laboratories |  |
|  | 6514 |  | Blood and organ banks |  |
|  | 6520 |  | Nursing, supervision, and other rehabilitative services |  |
|  | 6530 |  | Hospital | # A |
|  | 6560 |  | Social assistance, welfare, and charitable services | B |
|  | 6561 |  | Child and youth services |  |
|  | 6562 |  | Child day care |  |
|  | 6563 |  | Community food services |  |
|  | 6564 |  | Emergency and relief services |  |
|  | 6565 |  | Other family services |  |
|  | 6566 |  | Services for elderly and disabled |  |
|  | 6567 |  | Veterans affairs | # B |
|  | 6568 |  | Vocational rehabilitation | A |
|  | 6600 |  | Religious institutions | # A |
|  | 6700 |  | Death care services | B |
|  | 6710 |  | Funeral homes and services |  |
|  | 6720 |  | Cremation services and cemeteries |  |
|  | 6800 |  | Associations, nonprofit organizations, etc. |  |
|  | 6810 |  | Labor and political organizations |  |
|  | 6820 |  | Business associations and professional membership organizations |  |
|  | 6830 |  | Civic, social, and fraternal organizations | # B |
| Construction-related businesses | 7000 |  | Construction-related businesses |  |
|  | 7100 |  | Building, developing, and general contracting | ### B |
|  | 7110 |  | Residential construction |  |
|  | 7120 |  | Land development and subdivision |  |
|  | 7130 |  | Industrial, commercial and institutional building construction |  |
|  | 7200 |  | Machinery related |  |
|  | 7210 |  | Building equipment and machinery installation contractors |  |
|  | 7220 |  | Excavation contractor |  |
|  | 7230 |  | Water well drilling contractor |  |
|  | 7240 |  | Wrecking and demolition establishment |  |
|  | 7250 |  | Structural steel erection contractor |  |
|  | 7300 |  | #### Special trade contractor |  |
|  | 7310 |  | Carpentry, floor, and tile contractor |  |
|  | 7320 |  | Concrete contractor |  |
|  | 7330 |  | Electrical contractor |  |
|  | 7340 |  | Glass and glazing contractor |  |
|  | 7350 |  | Masonry and drywall contractors |  |
|  | 7360 |  | Painting and wall covering |  |
|  | 7370 |  | Plumbing, heating, and air-conditioning |  |
|  | 7380 |  | Roofing, siding, and sheet metal contractors | # B |
|  | 7400 |  | Heavy construction | # C |
|  | 7410 |  | Highway and street construction; |  |
|  | 7420 |  | Bridge and tunnel construction |  |
|  | 7430 |  | Water, sewer, and pipeline construction |  |
|  | 7440 |  | Power lines, communication and transmission lines |  |
|  | 7450 |  | Industrial and other nonbuilding construction | # C |
| Mining and extraction establishments | 8000 |  | Mining and extraction establishments | # C |
|  | 8100 |  | Oil and natural gas |  |
|  | 8200 |  | Metals (iron, copper, etc.) |  |
|  | 8300 |  | Coal |  |
|  | 8400 |  | Nonmetallic mining |  |
|  | 8500 |  | Quarrying and stone cutting establishment | # C |
| Agriculture, forestry, fishing and hunting | 9000 |  | Agriculture, forestry, fishing and hunting |  |
|  | 9100 |  | Crop production | C |
|  | 9110 |  | Grain and oilseed |  |
|  | 9111 |  | Wheat |  |
|  | 9112 |  | Corn |  |
|  | 9113 |  | Rice |  |
|  | 9114 |  | Soybean and oilseed |  |
|  | 9115 |  | Dry pea and bean |  |
|  | 9120 |  | Vegetable farming or growing services |  |
|  | 9130 |  | Fruits and trees |  |
|  | 9140 |  | Greenhouse, nursery, and floriculture |  |
|  | 9141 |  | Food crops grown under cover |  |
|  | 9142 |  | Nursery and tree production |  |
|  | 9143 |  | Floriculture production |  |
|  | 9150 |  | All other crops |  |
|  | 9151 |  | Tobacco crop |  |
|  | 9152 |  | Cotton crop |  |
|  | 9153 |  | Sugarcane crop |  |
|  | 9154 |  | Hay |  |
|  | 9155 |  | Peanut crop |  |
|  | 9200 |  | Support functions for agriculture |  |
|  | 9210 |  | Farm and farm labor management services |  |
|  | 9220 |  | Spraying, dusting, and other related services |  |
|  | 9230 |  | Crop harvesting and post harvest crop activities (including drying, siloing, etc.) |   |
|  | 9240 |  | Cotton ginning, grist milling, etc. |  |
|  | 9300 |  | Animal production including slaughter |  |
|  | 9310 |  | Cattle ranch and crops |  |
|  | 9311 |  | Beef cattle ranch establishments |  |
|  | 9312 |  | Cattle feedlot establishment |  |
|  | 9320 |  | Dairy cattle and milk production |  |
|  | 9330 |  | Hog and pig farm |  |
|  | 9340 |  | Poultry and egg production and hatcheries |  |
|  | 9350 |  | Sheep and goat farming establishments |  |
|  | 9360 |  | Fish hatcheries, fisheries, and aquaculture |  |
|  | 9370 |  | All other animal production |  |
|  | 9371 |  | Apiculture (bees, wax, and related operations) |  |
|  | 9372 |  | Horse and equine production |  |
|  | 9373 |  | Fur-bearing animal production |  |
|  | 9380 |  | Support functions for animal production |  |
|  | 9400 |  | Forestry and Logging |  |
|  | 9410 |  | Logging |  |
|  | 9420 |  | Forest nurseries |  |
|  | 9430 |  | Support functions for forestry |  |
|  | 9500 |  | Fishing, hunting and trapping, game preserves |  |
|  | 9510 |  | Fishing |  |
|  | 9520 |  | Hunting and trapping, game retreats, game and fishing preserves | # C |
|  | 9900 |  | Unclassifiable function | # U |
|  | 9910 |  | Not applicable to this dimension |  |
|  | 9990 |  | To be determined |  |
|  | 9999 |  | To be determined | # U |

# APPENDIX B

STANDARD LAND USE CODING SYSTEM

U.S. Department of Transportation

Federal Highway Administration

Reprinted 1969

B.	A STANDARD SYSTEM FOR IDENTIFYING AND CODING LAND USE ACTIVITIES -- TWO-, THREE-, AND FOUR-DIGIT

LEVELS

SIC	Land*

Code	Category	Code	Category	Code	Category	Reference1	Class

11	Household units.	110	Household units.	1100	Household units.2	----	A

12	Group quarters.	121	Rooming and boarding houses.	1210	Rooming and boarding houses.3	         7021

122	Membership lodgings.	1221	Fraternity and sorority houses.	4Incl. 7041

1229	Other membership lodgings, NEC.5	 Incl. 7041

123	Residence halls or dormitories.	1231	Nurses’ homes.	----

1232	College dormitories.	----

1239	Other residence halls or dormitories,	----

NEC.

124	Retirement homes and orphan-	1241	Retirement homes.	----

ages.	1242	Orphanages.	----

125	Religious quarters.	1251	Convents.	----

1252	Monasteries.	----

1253	Rectories.	----

1259	Other religious quarters, NEC.	----

129	Other group quarters, NEC.	1290	Other group quarters, NEC.	----

13	Residential hotels.	130	Residential hotels.	1300	Residential hotels.6	Incl. 7011

14	Mobile home parks or courts.	140	Mobile home parks or courts.	1400	Mobile home parks or courts.	        7031

15	Transient lodgings.	151	Hotels, tourist courts, and motels.	1510	Hotels, tourist courts, and motels.	        7011

159	Other transient lodgings, NEC.	1590	Other transient lodgings, NEC.7

19	Other residential, NEC.	190	Other residential, NEC.	1900	Other residential, NEC.	----	A

FOOTNOTES

1The SIC codes are listed for purposes of reference.  They are the	3Code 1210----”Rooming and boarding houses” are those that have

codes in the SIC system that most nearly correspond to the 4-digit	5 or more persons renting rooms, with or without board, and not re-

land use activity indicated.  (See ch. III, Sec. A3, “The Use of Stand-	lated to the had or person in charge.  Where there are less than 5

and Industrial Classification Nomenclature.”)  A dash indicates that	roomers, the total quarters are considered as one “Household unit”

there is no corresponding SIC code.			and is coded 1100.

2Code 1100--”Household units” are defined as a house, and apart-	   4"Incl. 7041" is an abbreviation which indicates that the 4-digit

ment, or other group of rooms, or a single room that is intended for	land use activity category is only one of several categories of establish-

occupancy as separate living quarters.  Occupants of “Household	ments that are included under SIC code 7041.

units” do not live and eat with other persons in the structure (such as	  5"NEC” is an abbreviation for “not elsewhere coded.”

in a boarding house), and there is either (1) direct access from the 	  6Code 1300--”Residential hotels” are those that have 75 percent or

outside or through a common hall, or (2) there is a kitchen or cooking	more of the available accommodations occupied by permanent guests

equipment for the exclusive use of the occupants of the unit.  The	(i.e., persons who reside more than 30 days).  Hotels with less than

occupants may be a family, a group of unrelated persons, or a person	75 percent are included under code 15, “Transient lodgings.”

living alone.  Mobile homes not in “Mobile home parks or courts”	  7Code 1590--”Other transient lodgings, NEC” includes such estab-

(code 14), but resting on a permanent type of foundation (e.g., a brick	lishments as the YMCA, YWCA, and YMHA when 50 percent or

or concrete block foundation) are included as well as units that are	more of the floor area is devoted to lodging and associated activities

vacant or that are used on a seasonal basis.  Farm homes are also in-	and when less than 75 percent of the accommodations are occupied by

cluded under “household units” and should be identified separately	permanent guests.  If 75 percent or more of the guests are permanent,

from the remainder of the farm which is coded under “Agriculture,”	use code 1300.  If 50 percent or more of the floor area is devoted to

code 81.				recreational activity, use code 7424, “Recreation centers (general).”

*”Land Class” refers to Rule 201 Classifications.  “A” denotes a

Class A Land, “B” denotes a Class B Land and “C” denotes a

Class C Land.  “U” denotes a Land unclassified in rule 201.

(Added by IEPA.)

B-1

B.	A STANDARD SYSTEM FOR IDENTIFYING AND CODING LAND USE ACTIVITIES -- TWO-, THREE-, AND FOUR-DIGIT

LEVELS -- Continued

SIC	Land*

Code	Category	Code	Category	Code	Category	Reference1	Class

21	Food and kindred prod-	211	Meat products -- manufacturing.	2111	Meat packing -- manufacturing.	2011	C

ucts--manufacturing.			2112	Sausages and other prepared meat 	2013

products--manufacturing.

2113	Poultry and small game dressing and	2015

packing.

212	Dairy products--manufacturing.	2121	Creamery butter--manufacturing.	2021

2122	Cheese, natural and processed.	2022

2123	Condensed and evaporated milk--	2023

manufacturing.

2124	Ice cream and frozen desserts--	2024

manufacturing.

2125	Fluid milk processing.	2026

213	Canning and preserving of fruits,	2131	Canning and curing seafoods.	2031

vegetables, and seafoods.	2132	Canning specialty foods.	2032

2133	Canning fruits, vegetables, preserves,	2033

jams, and jellies.

2134	Drying and dehydrating fruits and 	2034

vegetables.

2135	Pickling  fruits and vegetables; vege-	2035

table sauces and seasonings; salad

dressings--manufacturing.

2136	Fresh or frozen packaged fish and 	2036

seafoods.

2137	Frozen fruits, fruit juices, vegetables,	2037

and specialities.

214	Grain mill products--manufactur-	2141	Flour and other grain mill products.	2041

ing.	2142	Preparing feeds for animals and fowls.	2042

2143	Cereal preparations.	2043

2144	Rice milling.	2044

2145	Blending and preparing flour.	2045

2146	Wet corn milling.	2046

215	Bakery products--manufacturing.	2150	Bakery products--manufacturing.	205

216	Sugar--manufacturing.	2160	Sugar--manufacturing.	206

217	Confectionery and related prod-	2171	Candy and other confectionery prod-	2071

ucts--manufacturing.		  ucts--manufacturing.

2172	Chocolate and cocoa products--	2072

manufacturing.

2173	Chewing gum--manufacturing.	2073

218	Beverage--manufacturing.	2181	Malt liquors--manufacturing.	2082

2182	Malt--manufacturing.	2083

2183	Wine, brandy, and brandy spirits--	2084

manufacturing.

2184	Distilling, rectifying, and blending	2085

liquors.

2185	Bottling and canning soft drinks and	2086

carbonated waters.

2186	Flavor extracts and flavoring sirups	2087

manufacturing, NEC.

219	Other food preparations and kin-	2191	Cottonseed oil milling.	2091

dred products manufacturing,	2192	Soybean oil milling.	2092

NEC.	2193	Vegetable oil milling (except cotton-	2093

seed and soybean).

2194	Animal and marine fats and oils (in-	2094

cluding grease and tallow)--manu-

facturing.

2195	Roasting coffee and coffee products--	2095

manufacturing.

2196	Shortening, table oils, margarine,	2096

and other edible fats and oils--

manufacturing.

2197	Ice--manufacturing	2097

2198	Macaroni, spaghetti, vermicelli, and 	2098

noodles--manufacturing.

2199	Other food preparations and kindred	2099	C

products manufacturing, NEC.

B-2

B.	A STANDARD SYSTEM FOR IDENTIFYING AND CODING LAND USE ACTIVITIES -- TWO-, THREE-, AND FOUR-DIGIT

LEVELS -- Continued

SIC	Land*

Code	Category	Code	Category	Code	Category	Reference1	Class

22	Textile mill products--manu-	221	Broad and narrow woven fabrics	2210	Broad and narrow woven fabrics and	221,222,2239	C

facturing.		  and other small wares (cotton,		  other small wares (cotton, manmade	  and 224

manmade fibers, silk, and wool)--		  fibers, silk, and wool)--manufacturing.

manufacturing.

222	Knit goods--manufacturing.	2220	Knit goods--manufacturing.	225

223	Dyeing and finishing of textiles	2230	Dyeing and finishing of textiles (ex-	226

(except wool fabrics and knit		  cept wool fabrics and knit goods).

goods.

224	Floor coverings (rugs and carpets)	2240	Floor coverings (rugs and carpets)--	227

manufacturing.		  manufacturing.

225	Yarns and threads--manufacturing.	2250	Yarns and threads--manufacturing.	228

229	Other textile goods manufacturing,	2291	Felt goods (except woven felts and	2291

NEC.		  hats)--manufacturing.

2292	Lace Goods--manufacturing.	2292

2293	Padding and upholstery filling--	2293

manufacturing.

2294	Processing waste and recovering fibers	2294

and flock.

2295	Artificial leather and oil cloth manu-	2295

facturing and other impregnating and

coating fabrics (except rubberizing).

2296	Tire cording and fabric--manufacturing.	2296

2297	Wool scouring, worsted combing, and 	2297

towing to top.

2299	Other textile goods manufacturing,	2299

NEC.

23	Apparel and other finished	231	Men’s, youths’ and boys’ suits,	2310	Men’s, youth’s, and boys’ suits, coats,	231

products made from fab-	  	 coats, and overcoats--manufac-		  and overcoats--manufacturing.

rics, leather, and similar 		 turing.

materials-- manufacturing.

232	Men’s, youths’, and boys’ furnish-	2320	Men’s, youths’, and boys’ furnishings,	232

ings, work clothing, and allied		   work clothing, and allied garments--

garments--manufacturing.		  manufacturing.

233	Women’s, misses’, juniors’, girls’,	2330	Women’s, misses’, juniors’, girls’,	233 and

children’s, and infants’ outerwear--		  children’s, and infants’ outerwear--	236

manufacturing.		  manufacturing.

234	Women’s, misses’, children’s, and	2340	Women’s, misses’, children’s, and	234

infants’ undergarments--manufact-		  infants’ undergarments--manufacturing.

uring.

235	Hats, caps, and millinery-manu-	2350	Hats, caps, and millinery--manufac-	235

facturing.		  turing.

236	Leather and leather products--	2361	Leather tanning and finishing.	3111

manufacturing.	2362	Industrial leather belting and packing--	3121

manufacturing.

2363	Boot and shoe cut stock and findings--	3131

manufacturing.

2364	Footwear (except rubber)--manufacturing.	314

2365	Leather gloves and mittens-manufacturing.	315

2366	Luggage--manufacturing.	316

2367	Handbags and other personal leather	317

goods--manufacturing.

2369	Other leather products manufacturing,	319

NEC.

237		2370	Fur goods--manufacturing.	237	C

B-3

B.	A STANDARD SYSTEM FOR IDENTIFYING AND CODING LAND USE ACTIVITIES -- TWO-, THREE-, AND FOUR-DIGIT

LEVELS -- Continued

SIC	Land*

Code	Category	Code	Category	Code	Category	Reference1	Class

23	Apparel and other finished	238	Miscellaneous apparel and acces-	2381	Dress and work gloves (except knit	2381	C

products made from fab-		  sories--manufacturing.		  and all leather)--manufacturing.

rics, leather, and similar			2382	Robes and dressing gowns-manu-	2384

materials--manufacturing--				facturing.

Continued			2383	Raincoats and other waterproof outer	2385

garments--manufacturing.

2384	Leather and sheep lined clothing--	2386

manufacturing.

2385	Apparel belts--manufacturing.	2387

2389	Other miscellaneous apparel and ac-	2389

cessory manufacturing, NEC.

239	Other fabricated textile products	2391	Curtains and draperies--manufac-	2391

manufacturing, NEC.		  turing.

2392	House furnishings (except curtains	2392

and draperies)--manufacturing.

2393	Textile bags--manufacturing.	2393

2394	Canvas products--manufacturing.	2394

2395	Pleating, decorative and novelty	2395

stitching and tucking for the trade.

2396	Apparel findings and related prod-	2396

ucts--manufacturing.

2399	Other fabricated textile products	2397 and

manufacturing, NEC.	2399

24	Lumber and wood prod-	241	Logging camps and logging con-	2410	Logging camps and logging contrac-	241

ucts (except furniture)--		  tractors.		  tors.

manufacturing.

242	Sawmills and planing mills.	2421	Sawmills and planing mills, general.	2421

2422	Hardwood dimension and flooring--	2426

manufacturing.

2429	Special sawmill products manufac-	2429

turing, NEC.

243	Millwork, veneer, plywood, and	2431	Millwork.	2431

prefabricated structural wood	2432	Veneer and plywood--manufactur-	2432

products--manufacturing.		  ing.

2433	Prefabricating wooden buildings and	2433

structural members--manufacturing.

244	Wooden containers--manufactur-	2440	Wooden containers--manufacturing.	244

ing.

249	Other lumber and wood products.	2491	Wood preserving.	2491

(except furniture) manufactur-	2499	Other lumber and wood products	2499

ing, NEC.		  (except furniture) manufacturing,

NEC.

25	Furniture and fixtures--	251	Household furniture--manufactur-	2510	Household furniture--manufacturing.	251

manufacturing.		  ing.

252	Office furniture--manufacturing.	2520	Office furniture--manufacturing.	252

253	Public building and related fur-	2530	Public building and related furni-	2531

niture--manufacturing.		  ture--manufacturing.

254	Partitions, shelving, lockers, and	2540	Partitions, shelving, lockers, and	254

office and store fixtures--manu-		  office and store fixtures--manu-

facturing.		  facturing.

259	Other furniture and fixtures manu-	2591	Venetian blinds and shades--manu-	2591

facturing, NEC.		  facturing.

2599	Other furniture and fixtures manu-	2599

facturing, NEC.

26	Paper and allied products--	261	Pulp--manufacturing.	2610	Pulp--manufacturing.	261

manufacturing.

262	Paper (except building paper)--	2620	Paper (except building paper)--	262	C

manufacturing.		  manufacturing.

B-4

B.	A STANDARD SYSTEM FOR IDENTIFYING AND CODING LAND USE ACTIVITIES -- TWO-, THREE-, AND FOUR-DIGIT

LEVELS -- Continued

SIC	Land*

Code	Category	Code	Category	Code	Category	Reference1	Class

26	Paper and allied products--	263	Paperboard--manufacturing.	2630	Paperboard--manufacturing.	263	C

manufacturing--Continued

264	Converted paper and paperboard	2641	Paper coating and glazing.	2641

products (except containers and 	2642	Envelope--manufacturing.	2642

boxes)--manufacturing.	2643	Bags (except textile bags)--manu-	2643

facturing.

2644	Wallpaper--manufacturing.	2644

2645	Die cut paper and paperboard; and	2645

cardboard--manufacturing.

2646	Pressed and molded pulp goods--	2646

manufacturing.

2647	Sanitary paper products--manufac-	2647

turing.

2649	Other converted paper and paper-	2649

board products (except containers

and boxes) manufacturing, NEC.

265	Paperboard containers and boxes--	2650	Paperboard containers and boxes--	265

manufacturing.		  manufacturing.

266	Building paper and building 	2660	Building paper and building board--	266

board--manufacturing.		  manufacturing.

27	Printing, publishing, and	271	Newspapers:  publishing, publish-	2710	Newspapers:  publishing, publishing	271

allied industries.		  ing and printing.		  and printing.

272	Periodicals:  publishing, publishing	2720	Periodicals:  publishing, publishing	272

and printing.		  and printing.

273	Books:  publishing, publishing and	2730	Books:  publishing, publishing and 	273

printing.		  printing.

274	Commercial printing.	2740	Commercial printing.	275

275	Manifold business forms--manu-	2750	Manifold business forms--manufac-	276

facturing.		  turing.

276	Greeting card--manufacturing.	2760	Greeting card--manufacturing.	277

277	Bookbinding and related indus-	2771	Blankbooks, looseleaf binders, and	2782

tries--manufacturing.		  devices--manufacturing.

2772	Bookbinding and miscellaneous re-	2789

lated work--manufacturing.

278	Printing trade service industries.	2781	Typesetting.	2791

2782	Photoengraving.	2793

2783	Electrotyping and stereotyping.	2794

2789	Other printing trade service indus-	2799

tries, NEC.

279	Other printing and publishing, NEC.	2790	Other printing and publishing, NEC.	274

28	Chemicals and allied prod-	281	Industrial inorganic and organic	2810	Industrial inorganic and organic	281

ucts--manufacturing.		  chemicals--manufacturing.		  chemicals--manufacturing.

282	Plastics materials and synthetic	2820	Plastics materials and synthetic	282

resins, synthetic rubber, syn-		  resins, synthetic rubber, synthetic

thetic and other manmade fibers		  and other manmade fibers (except

(except glass)--manufacturing.		  glass)--manufacturing.

283	Drug--manufacturing.	2831	Biological products--manufacturing.	2831

2832	Medicinal chemicals and botanical	2833

products--manufacturing.

2833	Pharmaceutical preparations--man-	2834

ufacturing.

284	Soap, detergents, and cleaning	2841	Soap and detergents (except spe-	2841

preparations, perfumes, cos-		  cialty cleaners)--manufacturing.

metics, and other toilet prepa-	2842	Specialty cleaning, polishing, and	2842

rations--manufacturing.		  sanitation preparations (except

soap and detergents)--manufac-

turing.

2843	Surface active agents, finishing	2843

agents, sulfonated oils, and as-

sistants--manufacturing.

2844	Perfumes, cosmetics, and other toilet	2844	C

preparations-manufacturing.

B-5

B.	A STANDARD SYSTEM FOR IDENTIFYING AND CODING LAND USE ACTIVITIES -- TWO-, THREE-, AND FOUR-DIGIT

LEVELS -- Continued

SIC	Land*

Code	Category	Code	Category	Code	Category	Reference1	Class

28	Chemicals and allied prod-	285	Paints, varnishes, lacquers, en-	2850	Paints, varnishes, lacquers, enamels,	285	C

ucts--manufacturing--		  amels, and allied products--		  and allied products--manufacturing.

Continued.		  manufacturing.

286	Gum and wood chemicals-manu-	2860	Gum and wood chemicals--manufac-	286

facturing.		  turing.

287	Agricultural chemicals--manufac-	2870	Agricultural chemicals--manufac-	287

turing.		  turing.

289	Other chemicals and allied prod-	2891	Glue and gelatin--manufacturing.	2891

ucts--manufacturing, NEC.	2892	Explosives--manufacturing.	2892

2893	Printing ink--manufacturing.	2893

2894	Carbon black--manufacturing.	2895

2899	Other chemicals and allied products	2899

manufacturing, NEC.

29	Petroleum refining and re-	291	Petroleum refining.	2910	Petroleum refining.	291

lated  industries.

292	Paving and roofing  materials--	2921	Paving mixtures and blocks--manu-	2951

manufacturing.		  facturing.

2922	Asphalt felts and coatings--manu-	2952

facturing.

299	Other petroleum refining and re-	2991	Lubricating oils and greases--manu-	2992

lated industries, NEC.		  facturing.

2999	Other petroleum and coal products	2999

manufacturing, NEC.

31	Rubber and miscellaneous	311	Tires and inner tubes--manufac-	3110	Tires and inner tubes--manufactur-	301

plastic products--manu-		  turing.		  ing.

facturing.

312	Rubber footwear--manufacturing.	3120	Rubber footwear--manufacturing.	302

313	Reclaiming rubber.	3130	Reclaiming rubber.	303

314	Miscellaneous plastic products--	3140	Miscellaneous plastic products--	307

manufacturing.		  manufacturing.

319	Other fabricated rubber products	3190	Other fabricated rubber products	306

manufacturing, NEC.		  manufacturing, NEC.

32	Stone, clay, and glass prod-	321	Flat glass--manufacturing.	3210	Flat glass--manufacturing.	3211 and

ucts--manufacturing.					incl. 3231

322	Glass and glassware (pressed or	3221	Glass containers--manufacturing.	3221 and

blown)--manufacturing.			incl. 3231

3229	Other glass and glassware (pressed	3229 and

or blown) manufacturing, NEC.	incl. 3231

323	Cement (hydraulic)--manufactur-	3230	Cement (hydraulic)--manufacturing.	324

ing.

324	Structural clay products--manu-	3241	Brick and structural clay tile--manu-	3251

facturing.		  facturing.

3242	Ceramic wall and floor tile--manu-	3253

facturing.

3243	Clay refractories--manufacturing.	3255

3249	Other structural clay products manu-	3259

facturing, NEC.

325	Pottery and related products--	3251	Vitreous china plumbing fixtures,	3261

manufacturing.		  china, earthenware fittings, and

bathroom accessories--manufacturing.

3252	Vitreous china table and kitchen	3262

articles--manufacturing.

3253	Fine earthenware (whiteware) table	3263

and kitchen articles--manufacturing.

3254	Porcelain electrical supplies--manu-	3264

facturing.

3259	Other pottery and related products	3269	C

manufacturing, NEC.

B-6

B.	A STANDARD SYSTEM FOR IDENTIFYING AND CODING LAND USE ACTIVITIES -- TWO-, THREE-, AND FOUR-DIGIT

LEVELS -- Continued

SIC	Land*

Code	Category	Code	Category	Code	Category	Reference1	Class

32	Stone, clay, and glass prod-	326	Concrete, gypsum, and plaster	3261	Concrete brick and block--manu-	3271	C

ucts--manufacturing--		  products--manufacturing.		  facturing.

Continued.			3262	Concrete products (excluding brick	3272

and block)--manufacturing.

3263	Concrete (ready mixed)--manufac-	3273

turing.

3264	Lime products--manufacturing.	3274

3265	Gypsum products--manufacturing.	3275

327	Cut stone and stone products--	3270	Cut stone and stone products--manu-	328

manufacturing.		  facturing.

328	Abrasive, asbestos, and miscella-	3280	Abrasive, asbestos, and miscella-	329

neous nonmetallic mineral prod-		  neous nonmetallic mineral prod-

ucts--manufacturing.		  ucts--manufacturing.

33	Primary metal industries.	331	Blast furnaces, steel works, and	3311	Blast furnaces (including coke ovens),	3312

the rolling and finishing of fer-		  steel works, and the rolling of fer-

rous metals.		  rous metals.

3312	Electrometallurgical products--man-	3313

ufacturing.

3313	Steel wire drawing and steel nails	3315

and spikes--manufacturing.

3314	Cold rolled sheet, strip, and bars--	3316

manufacturing.

3315	Steel pipe and tubes--manufacturing.	3317

332	Iron and steel foundries.	3320	Iron and steel foundries.	332

333	Primary smelting and refining of	3331	Primary smelting and refining of	3331

nonferrous metals.		  copper.

3332	Primary smelting and refining of	3332

lead.

3333	Primary smelting and refining of	3333

zinc.

3334	Primary production of aluminum.	3334

3339	Other primary smelting and refining	3339

of nonferrous metals, NEC.

334	Secondary smelting and refining of	3340	Secondary smelting and refining of	334

nonferrous metals and alloys.		  nonferrous metals and alloys.

335	Rolling, drawing, and extruding of	3351	Rolling drawing, and extruding of 	3351

nonferrous metals.		  copper.

3352	Rolling, drawing, and extruding of	3352

aluminum.

3353	Rolling, drawing, and extruding of	3356

nonferrous metals (except copper

and aluminum).

3354	Drawing and insulating of nonferrous	3357

wire.

336	Nonferrous foundries.	3360	Nonferrous foundries.	336

339	Other primary metal industries,	3390	Other primary metal industries,	339

NEC.		  NEC.

34	Fabricated metal prod-	341	Ordinance and accessories.	3411	Guns, howitzers, mortars, and re-	191

ucts--manufacturing.				  lated equipment--manufacturing.

3412	Ammunition (except small arms)	192

manufacturing and complete as-

sembling of guided missiles and

space vehicles.

3413	Tanks and tank components--manu-	193

facturing.

3414	Sighting and fire control equipment--	194

manufacturing.

3415	Small arms--manufacturing.	195

3416	Small arms ammunition--manufac-	196

turing.

3419	Other ordinance and accessories	199	C

manufacturing, NEC.

B-7

B.	A STANDARD SYSTEM FOR IDENTIFYING AND CODING LAND USE ACTIVITIES -- TWO-, THREE-, AND FOUR-DIGIT

LEVELS -- Continued

SIC	Land*

Code	Category	Code	Category	Code	Category	Reference1	Class

34	Fabricated metal prod-	342	Machinery (except electrical)--	3421	Engines and turbines--manufactur-	351	C

ucts--manufacturing--		  manufacturing.		  ing.

Continued.			3422	Farm machinery and equipment--	352

manufacturing.

3423	Construction, mining, and materials	353

handling machinery and equip-

ment--manufacturing.

3424	Metalworking machinery and equip-	354

ment--manufacturing.

3425	Special industry machinery (except	355

metalworking machinery)--manu-

facturing.

3426	General industrial machinery and 	356

equipment--manufacturing.

3427	Office, computing, and accounting	357

machines--manufacturing.

3428	Service industry machines--manu-	358

facturing.

3429	Other machinery --manufacturing	359

(except electrical), NEC.

343	Electrical machinery, equipment,	3431	Electrical transmission and distribu-	361

and supplies--manufacturing.		  tion equipment--manufacturing.

3432	Electrical industrial apparatus--	362

manufacturing.

3433	Household appliances--manufactur-	363

ing.

3434	Electric lighting and wiring equip-	364

ment--manufacturing.

3435	Radio and television receiving sets	365

(except communication types)--

manufacturing.

3436	Communication equipment--manu-	366

facturing.

3437	Electronic components and acces-	367

sories--manufacturing.

3439	Other electrical machinery, equip-	369

ment, and supplies manufactur-

ing, NEC.

344	Transportation equipment--man-	3441	Motor vehicles and motor vehicle	371

ufacturing.		  equipment--manufacturing.

3442	Aircraft and parts--manufacturing.	372

3443	Ship and boat building and repairing.	373

3444	Railroad equipment--manufacturing.	374

3445	Motorcycles, bicycles, and parts--	375

manufacturing.

3449	Other transportation equipment	379

manufacturing, NEC.

349	Other fabricated metal products	3491	Metal cans--manufacturing.	341

manufacturing, NEC.	3492	Cutlery, hand tools, and general	342

hardware--manufacturing.

3493	Heating apparatus (except electri-	343

cal) and plumbing fixtures--man-

ufacturing.

3494	Fabricated structural metal prod-	344

ucts--manufacturing.

3495	Screw machine products and bolts,	345

nuts, screws, rivets, and washers--

manufacturing.

3496	Metal stamping--manufacturing.	346

3497	Coating, engraving, and allied serv-	347

ices.

3498	Fabricated wire products (miscella-	348

neous products)--manufacturing.

3499	Other fabricated metal products	349	C

manufacturing, NEC.

B-8

B.	A STANDARD SYSTEM FOR IDENTIFYING AND CODING LAND USE ACTIVITIES -- TWO-, THREE-, AND FOUR-DIGIT

LEVELS -- Continued

SIC	Land*

Code	Category	Code	Category	Code	Category	Reference1	Class

35	Professional, scientific, and	351	Engineering, laboratory, and sci-	3510	Engineering, laboratory, and scien-	381                    C

controlling instruments:		  entific and research instruments		  tific and research instruments and

photographic and optical		  and associated equipment--		  associated equipment--manufac-

goods; watches and clocks--		  manufacturing.		  turing.

manufacturing.

352	Instruments for measuring, con-	3521	Mechanical measuring and control-	3821

trolling, and indicating physical		  ling instruments (except automatic

characteristics--manufacturing.		  temperature controls)--manufac-

turing.

3522	Automatic temperature controls--	3822

manufacturing.

353	Optical instruments and lenses--	3530	Optical instruments and lenses--	383

manufacturing.		  manufacturing.

354	Surgical, medical, and dental in-	3541	Surgical and medical instruments	3841

struments and supplies--manu-		  and apparatus--manufacturing.

facturing.

3542	Orthopedic, prosthetic, and surgical	3842

appliances and supplies--manufac-

turing.

3543	Dental equipment and supplies--	3843

manufacturing.

355	Ophthalmic goods--manufactur-	3550	Ophthalmic goods--manufacturing.	385

ing.

356	Photographic equipment and sup-	3560	Photographic equipment and sup-	386

plies--manufacturing.		  plies--manufacturing.

357	Watches, clocks, clockwork oper-	3570	Watches, clocks, clockwork operated	387

ated devices, and parts--manu-		  devices, and parts--manufacturing.

facturing.

39	Miscellaneous manufactur-	391	Jewelry, silverware, and plated	3911	Jewelry and precious metals-manu-	3911

ing, NEC.		  ware--manufacturing.		  facturing.

3912	Jewelers’ findings and materials--	3912

manufacturing.

3913	Lapidary work.	3913

3914	Silverware and plated ware--manu-	3914

facturing.

392	Musical instruments and parts--	3920	Musical instruments and parts--	393

manufacturing.		  manufacturing.

393	Toys, amusement, sporting, and	3930	Toys, amusement, sporting, and	394

athletic goods--manufacturing.		  athletic goods--manufacturing.

394	Pens, pencils, and other office	3940	Pens, pencils, and other office and	395

and artists’ materials--manu-		  artists’ materials--manufacturing.

facturing.

395	Costume jewelry, costume novel-	3950	Costume jewelry, costume novelties,	396

ties, buttons, and miscellaneous		  buttons, and miscellaneous notions

notions (except precious metals)		  (except precious metals)--manu-

--manufacturing.		  facturing.

396	Tobacco--manufacturing.	3961	Cigarettes--manufacturing.	211

3962	Cigars--manufacturing.	212

3963	Tobacco (chewing and smoking) and	213

snuff--manufacturing.

3964	Tobacco stemming and redrying.	214	C

397	Motion picture production.	3970	Motion picture production.	7811                   B

B-9

B.	A STANDARD SYSTEM FOR IDENTIFYING AND CODING LAND USE ACTIVITIES -- TWO-, THREE-, AND FOUR-DIGIT

LEVELS -- Continued

SIC	Land*

Code	Category	Code	Category	Code	Category	Reference1	Class

39	Miscellaneous manufactur-	399	Other miscellaneous manufactur-	3991	Brooms and brushes--manufactur-	3981	C

ing, NEC--Continued.		   ing, NEC.		  ing.

3992	Linoleum, asphalted-felt base, and	3982

other hard surface floor cover

manufacturing, NEC.

3993	Matches--manufacturing.	3983

3994	Lamp shades--manufacturing.	3987

3995	Mortician’s goods-manufacturing.	3988

3996	Fur dressing and dyeing	3992

3997	Signs and advertising displays--	3993

manufacturing.

3998	Umbrellas, parasols, and canes--	3995

manufacturing.

3999	Other miscellaneous manufacturing,	3984 and

NEC.	3999	C

41	Railroad, rapid rail transit,	411	Railroad transportation.	4111	Railroad right-of-way (excluding	----	U

and street railway trans-				  switching and marshaling yards).

portation.			4112	Railroad switching and marshaling	----	C

yards.

4113	Railroad terminals (passenger).	----

4114	Railroad terminals (freight).	----

4115	Railroad terminals (passenger and	----

freight).

4116	Railroad equipment and mainte-	----

nance.

4119	Other railroad transportation, NEC.	----	C

412	Rapid rail transit and street rail-	4121	Rapid rail transit and street railway	----	U

way transportation.		  right-of-way.2

4122	Rapid rail transit and street railway	----	C

passenger terminals.3

4123	Rapid rail transit and street railway	----

equipment maintenance.

4129	Other rapid rail transit and street	----

railway transportation, NEC.

42	Motor  vehicle transporta-	421	Bus transportation.	4211	Bus passenger terminals (intercity).4	----

tion.			4212	Bus passenger terminals (local).4	----

4213	Bus passenger terminals (intercity	----

and local).4

4214	Bus garaging and equipment mainte-	----

nance.

4219	Other bus transportation, NEC.	----

422	Motor freight transportation.	4221	Motor freight terminals.	----

4222	Motor freight garaging and equip-	----

ment maintenance.

4229	Other motor freight transportation,	----	C

NEC.

FOOTNOTES

1The SIC codes are listed for purposes of reference.  The are the

codes in the SIC system that most nearly correspond to the 4-digit land

use activity indicated.  (See ch. III, Sec. A3, “The Use of Standard

Industrial Classification Nomenclature.”)  A dash indicates that there

is no corresponding SIC code.

B-10

B.	A STANDARD SYSTEM FOR IDENTIFYING AND CODING LAND USE ACTIVITIES -- TWO-, THREE-, AND FOUR-DIGIT

LEVELS -- Continued

SIC	Land*

Code	Category	Code	Category	Code	Category	Reference1	Class

42	Motor vehicle transporta-	429	Other motor vehicle transporta-	4291	Taxicab transportation.	4121	C

tion--Continued		  tion, NEC.	4299	Other motor vehicle transportation,	----	C

NEC.

43	Aircraft transportation.	431	Airports and flying fields.	4311	Airport and flying field landing/	----	U

takeoff fields.

4312	Airport and flying field terminals	----

(passenger).

4313	Airport and flying field terminals	----

(freight).

4314	Airport and flying field terminals	----

(passenger and freight).

4315	Aircraft storage and equipment	----

maintenance.

4319	Other airports and flying fields,	----

NEC.

439	Other aircraft  transportation,	4391	Heliport landing/takeoff pads.5	----	U

NEC.	4399	Other aircraft transportation, NEC.	----

44	Marine craft transporta-	441	Marine terminals.7	4411	Marine terminals (passenger).8	----	C

tion.8			4412	Marine terminals (freight).	----

4413	Marine terminals (passenger and 	----

freight).

4414	Marine terminals (commercial fish-	----

ing).

4419	Other marine terminals, NEC.	----

449	Other marine craft transportation,	4490	Other marine craft transportation,	----	C

NEC.		  NEC.

45	Highway and street right-of-way.	451	Freeways.	4510	Freeways.9	----	U

452	Expressways.	4520	Expressways.10	----

453	Parkways.	4530	Parkways.11	----

454	Arterial streets.	4540	Arterial streets.12	----

455	Collector/distributor streets.	4550	Collector/distributor streets.13	----

456	Local access streets.	4560	Local access streets.14	----

457	Alleys.	4570	Alleys.15	----

459	Other highway and street right-of-	4590	Other highway and street right-of-	----	U

way, NEC.		   way, NEC.

46	Automobile parking.	460	Automobile parking.	4600	Automobile parking.16	----	C

47	Communication.	471	Telephone communication.	4711	Telephone exchange stations.	----	B

4712	Telephone relay towers (microwave).	----

4719	Other telephone communication,	----

NEC.

472	Telegraph communication.	4721	Telegraph message centers.	----

4722	Telegraph transmitting and receiv-	----

ing stations (only).

4729	Other telegraph communication,	----

NEC.

473	Radio communication.	4731	Radio broadcasting studios (only).	----

4732	Radio transmitting stations and	----

towers.

4739	Other radio communication, NEC.	----

474	Television communication.	4741	Television broadcasting studios (only).	----

4742	Television transmitting stations and	----

relay towers.

4749	Other television communication,	----

NEC.

475	Radio and television communica-	4751	Radio and television broadcasting	----

tion (combined systems).		  studios, only (combined systems).

4759	Other combined radio and television	----

communication, NEC.

479	Other communication, NEC.	4790	Other communication. NEC.	----	B

B-11

B.	A STANDARD SYSTEM FOR IDENTIFYING AND CODING LAND USE ACTIVITIES -- TWO-, THREE-, AND FOUR-DIGIT

LEVELS -- Continued

SIC	Land*

Code	Category	Code	Category	Code	Category	Reference1	Class

48	Utilities	481	Electric utility.	4811	Electric transmission right-of-way.17	----	C

4812	Electric generation plants.	----

4813	Electricity regulating substations.	----

4819	Other electric utility, NEC.	----

482	Gas utility.	4821	Gas pipeline right-of-way.17	----

4822	Gas production plants.	----

4823	Natural or manufactured gas storage	----

and distribution points.

4824	Gas pressure control stations.	----

4829	Other gas utilities, NEC.	----

483	Water utilities and irrigation.	4831	Water pipeline right-of-way.17	----

4832	Water treatment plants (purifica-	----

tion).

4833	Water storage.18	----

4834	Irrigation distribution channels.	----

4835	Water pressure control stations.	----

4839	Other water utilities and irrigation,	----

NEC.

484	Sewage disposal.	4841	Sewage treatment plants.	----

4842	Sewage sludge drying beds.	----

4843	Sewage pressure control stations.	----

4849	Other sewage disposal, NEC.	----

485	Solid waste disposal.	4851	Refuse incineration.	----

4852	Central garbage grinding stations.	----

4853	Compositing plants.	----

4854	Sanitary landfills.	----

4855	Refuse disposals.	----

4856	Industrial waste disposals.	----

4857	Active slag dumps and mineral waste	----

disposals.

4859	Other solid waste disposal, NEC.	----

489	Other utilities, NEC.	4890	Other utilities, NEC.	----

49	Other transportation, com-	491	Other pipeline right-of-way and	4911	Petroleum pipeline right-of-way.17	----

munication, and utilities,		  pressure control stations, NEC.	4912	Petroleum pressure control stations.	----

NEC.			4919	Other pipeline right-of-way and pres-	----

sure control stations, NEC.

492	Transportation services and ar-	4921	Freight forwarding services.	4041 and

rangements.			4712

4922	Packing and crating services.	4783

4923	Travel arranging services.	472

4924	Transportation ticket services. 18	----

4929	Other transportation services and 	----

arrangements, NEC.

499	Other transportation, communica-	4900	Other transportation, communica-	----	C

tion, and utilities, NEC.		  tion, and utilities, NEC.

FOOTNOTES				5Code 4391--”Heliport landing/takeoff pads” are identified only

1The SIC codes are listed for purposes of reference.  They are the 	when they are separate activities and not a part of code 431, “Airports

codes in the SIC system that most nearly correspond to the 4-digit	and flying fields.”

land use activity indicated.  (See ch. III, Sec. A3, “The Use of Stand-		6Code 44--”Marine craft transportation.” The maintenance and

ard Industrial Classification Nomenclature.”)  A dash indicates that	repair of marine craft should be coded 3443, “Ship and boat building

there is no corresponding SIC code.			and repairing.”

2Code 4121--”Rapid rail transit and street railway right-of-way”		7Code 441--”Marine terminals” include all docking facilities (e.g.,

includes only that land which is not within public right-of-way (e.g.,	wharves, piers, and docks) and associated areas that are used by com-

within a public street right-of-way).  Railroad right-of-way used by	mercial passenger, freight, and fishing craft.  Docking facilities that

rapid rail transit is considered to be railroad right-of-way and is coded	primarily serve recreational type of marine craft are identified under

4111.				code 744, “Marinas,”

3Code 4122--”Rapid rail transit and street railway passenger ter-		8Code 4411--”Marine terminals (passenger)” include ferry terminals.

minals” identifies only those terminals that are not located within		9Code 4510--”Freeways” are divided highways for through traffic

public right-of-way (e.g., within a public street right-of-way).	with full control of access and no cross traffic at grade.

4Codes 4211, 4212, 4213--”Bus passenger terminals” identifies only		10Code 4520--”Expressways” are divided highways for through

those terminals that are not located on the public right-of-way (e.g.,	traffic with full or partial control of access with grade separations at

within a public street right-of-way).			major crossroads.

Footnotes continued on following page.

B-12

B.	A STANDARD SYSTEM FOR IDENTIFYING AND CODING LAND USE ACTIVITIES -- TWO-, THREE-, AND FOUR-DIGIT

LEVELS -- Continued

11Code 4530--”Parkways” are highways for noncommercial traffic,           17 parking spaces), and that serves no other single type of activity.

with full or partial control of access.  They are located within a park                        This code includes parking on open lots, parking within parking struc-

or within a ribbon of parklike development.	                                     tures, parking within structures that also house other activities (e.g.,

12Code 4540--”Arterial streets” are those streets which serve move-                   parking area in an office building), and the parking area at shopping

ments of traffic and are not freeways, expressways, and parkways.                           centers.  Parking areas of less than 5,000 square feet are not identified

Generally there is no control of access.		                                     as a separate activity.

13Code 4550--”Collector/distributor streets” are those streets that		17Codes 4811, 4821, 4831, 4911--These codes identify those areas

collect traffic from the local streets and channel it into the arterial	where the surface is devoted exclusively to the right-of-way of the respec-

system.  These streets also provide necessary cross-connections be-	tive activities and is used for no other purposes.

tween arterials.  The collector/distributor street does not handle long		18Code 4833--”Water storage” includes impounded surface water

through trips, and it is not continuous for any great length.	areas or water tanks used for storage.  Lakes or other natural water

14Code 4560--”Local access streets” are those used primarily for 	formations are identified under code 93, “Water areas.”

access to residences, businesses, or other abutting activities.		19Code 4924--”Transportation ticket services” include the ticket

15Code 4570--”Alleys” are minor narrow streets usually without	offices of any of the transportation systems.  The ticket offices are

sidewalks and on which building adjoin from the rear.	identified only when they are a separate and distinct activity, not

16Code 4600--”Automobile parking” includes nonresidential off-	located within one of the transportation terminals.

street parking that is 5,000 square feet or greater (or approximately

SIC	Land*

Code	Category	Code	Category	Code	Category	Reference1	Class

51	Wholesale trade.2	511	Motor vehicles and automotive	5111	Automobiles and other motor vehi-	5012	B

equipment--wholesale.		  cles--wholesale.

5112	Automotive equipment--wholesale.	5013

5113	Tires and tubes--wholesale.	5014

512	Drugs, chemicals, and allied	5121	Drugs, drug proprietaries and drug-	5022

products--wholesale.		  gists’ sundries--wholesale.

5122	Paints and varnishes--wholesale.	5028

5129	Other drugs, chemicals, and allied	5029

products, wholesale, NEC.

513	Dry goods and apparel--whole-	5131	Dry goods, piece goods, and notions--	5032

sale.		  wholesale.

5132	Apparel and accessories, hosiery, and	5035

lingerie--wholesale.

5133	Footwear--wholesale.	5039

514	Groceries and related products--	5141	Groceries (general line)--wholesale.	5042

wholesale.	5142	Dairy products--wholesale.	5043

5143	Poultry and poultry products--	5044

wholesale.

5144	Confectionery--wholesale.	5045

5145	Fish and seafoods--wholesale.	5046

5146	Meats and meat products--wholesale.	5047

5147	Fruits and vegetables (fresh)--	5048

wholesale.

5149	Other groceries and related products	5049

wholesale, NEC.

515	Farm products (raw materials)--	5151	Cotton--wholesale.	Incl. 5051

wholesale.	5152	Grain--wholesale.	Incl. 5051

5153	Hides, skins, and raw furs-whole-	Incl. 5051

sale.

5154	Leaf tobacco--wholesale.	Incl. 5051

5155	Wool and mohair--wholesale.	Incl. 5051

5156	Livestock--wholesale.	Incl. 5051

5157	Horses and mules--wholesale.	Incl. 5051

5159	Other farm products wholesale,	Incl. 5051

NEC.

516	Electrical goods--wholesale.	5161	Electrical apparatus and equipment,	5063

wiring supplies, and construction

materials--wholesale.

5162	Electrical appliances, television, and	5064

radio sets--wholesale.

5163	Electronic parts and equipment--	5065

wholesale.

517	Hardware, plumbing, heating	5171	Hardware--wholesale.	5072

equipment, and supplies--	5172	Plumbing and heating equipment	5074

wholesale.		  and supplies--wholesale.

5173	Air conditioning, refrigerated equip-	5077	B

ment, and supplies--wholesale.

B-13

B.	A STANDARD SYSTEM FOR IDENTIFYING AND CODING LAND USE ACTIVITIES -- TWO-, THREE-, AND FOUR-DIGIT

LEVELS -- Continued

SIC	Land*

Code	Category	Code	Category	Code	Category	Reference1	Class

51	Wholesale trade 2--	518	Machinery, equipment, and	5181	Commercial and industrial machin-	5082	B

Continued		  supplies--wholesale.		  ery, equipment, and supplies--

wholesale.

5182	Farm machinery and equipment--	5083

wholesale.

5183	Professional equipment and sup-	5086

plies--wholesale.

5184	Equipment and supplies for service	5087

establishments--wholesale.

5185	Transportation equipment and sup-	5088

lies (except motor vehicles)--

wholesale.

5189	Other machinery, equipment, and	5089

supplies wholesale, NEC.

519	Other wholesale trade, NEC.	5191	Metals and minerals (except petro-	5091

leum products and scrap)--whole-

sale.

5192	Petroleum bulk stations and termi-	5092

nals--wholesale.

5193	Scrap and waste materials--whole-	5093

sale.

5194	Tobacco and tobacco products--	5094

wholesale.

5195	Beer, wine, and distilled alcoholic	5095

beverages--wholesale.

5196	Paper and paper products--whole-	5096

sale.

5197	Furniture and home furnishings--	5097

wholesale.

5198	Lumber and construction materials--	5098

wholesale.

5199	Other wholesale trade, NEC.	5099

52	Retail trade--building ma-	521	Lumber and other building ma-	5211	Lumber years-retail	5211

terials, hardware, and		  terials--retail.	5212	Building materials--retail.	5212

farm equipment.

522	Heating and plumbing equip-	5220	Heating and plumbing equipment--	522

ment-retail.		  retail.

523	Paint, glass, and wallpaper--	5230	Paint, glass, and wallpaper-retail.	523

retail.

524	Electrical supplies--retail.	5240	Electrical supplies--retail.	524

525	Hardware and farm equip-	5251	Hardware--retail	5251

ment--retail.	5252	Farm equipment retail.	5252

53	Retail trade--general  mer-	531	Department stores--retail.	5310	Department stores--retail.	531

chandise.

532	Mail order houses--retail.	5320	Mail order houses--retail.	532

533	Limited price variety stores--	5330	Limited price variety stores--retail.	533

retail.

534	Merchandise vending machine	5340	Merchandise vending machine op-	534

operators--retail.		  erators--retail.

535	Direct selling organizations--	5350	Direct selling organizations--retail.	535

retail.

539	Other retail trade--general	5391	Dry goods and general merchan-	5392

merchandise, NEC.		  dise--retail.

5392	General stores--retail.	5393

54	Retail trade--food.	541	Groceries (with or without	5410	Groceries (with or without meat)--	541

meat)--retail		  retail.

542	Meats and fish--retail.	5421	Meats--retail.	5422

5422	Fish and seafoods--retail.	5423	B

B-14

B.	A STANDARD SYSTEM FOR IDENTIFYING AND CODING LAND USE ACTIVITIES -- TWO-, THREE-, AND FOUR-DIGIT

LEVELS -- Continued

SIC	Land*

Code	Category	Code	Category	Code	Category	Reference1	Class

54	Retail trade--food--	543	Fruits and vegetables--retail.	5430	Fruits and vegetables--retail.	543	B

Continued

544	Candy, nut, and confectionery--	5440	Candy, nut, and confectionery--	544

retail.		  retail.

545	Dairy products-retail.	5450	Dairy products--retail.	545

546	Bakeries--retail.	5461	Bakeries (manufacturing)--retail.3	5462

5462	Bakeries (nonmanufacturing)--re-	5463

tail.4

549	Other retail trade--food, NEC.	5491	Egg and poultry--retail.	5491

5499	Other retail trade--food, NEC.	5499

55	Retail trade--automotive,	551	Motor vehicles--retail.	5511	Motor  vehicles (new and used cars)--	551

marine craft, aircraft,				  retail.

and accessories.			5512	Motor vehicles (used cars only)--	552

retail.

552	Tires, batteries, and acces-	5520	Tires, batteries, and accessories--	553

sories--retail.		  retail.

553	Gasoline service stations.	5530	Gasoline service stations.	554

559	Other retail trade--automotive,	5591	Marine craft and accessories--retail	Incl. 5599

marine craft, aircraft, and	5592	Aircraft and accessories--retail.	Incl. 5599

accessories, NEC.	5599	Other retail trade--automotive, 	Incl. 5599

marine craft, aircraft, and acces-

sories, NEC.

56	Retail trade--apparel and	561	Men’s and boys’ clothing and	5610	Men’s and boys’ clothing and fur-	561

accessories.		  furnishings--retail.		  nishings--retail.

562	Women’s read-to-wear-retail.	5620	Women’s ready-to-wear-retail.	562

563	Women’s accessories and spe-	5630	Women’s accessories and special-	563

cialties--retail.		  ties--retail.

564	Children’s and infants’ wear--	5640	Children’s and infants’ wear--retail.	564

retail.

565	Family clothing--retail.	5650	Family clothing--retail.	565

566	Shoes--retail.	5660	Shoes--retail.	566

567	Custom tailoring.	5670	Custom tailoring.	567

568	Furriers and fur apparel--retail.	5680	Furriers and fur apparel--retail.	568

569	Other retail trade--apparel and	5690	Other retail trade--apparel and ac-	569

accessories, NEC.		  cessories, NEC.

57	Retail trade--furniture, 	571	Furniture, home furnishings, and	5711	Furniture--retail.	5712

home furnishings, and		  equipment--retail.	5712	Floor coverings--retail.	5713

equipment.			5713	Draperies, curtains, and upholstery--	5714

retail.

5714	China, glassware, and metalware---	5715

retail.

5719	Other furniture, home furnishings,	5719

and equipment retail, NEC.

572	Household appliances--retail.	5720	Household appliances--retail.	572

573	Radios, televisions, and music	5731	Radios and televisions--retail.	5732

supplies--retail.	5732	Music supplies--retail.	5733

58.	Retail trade--eating and	581	Eating places.	5810	Eating places.5	5812

drinking.

582	Drinking places (alcoholic bev-	5820	Drinking places (alcoholic beverages).	5813	B

erages).

B-15

B.	A STANDARD SYSTEM FOR IDENTIFYING AND CODING LAND USE ACTIVITIES -- TWO-, THREE-, AND FOUR-DIGIT

LEVELS -- Continued

SIC	Land*

Code	Category	Code	Category	Code	Category	Reference1	Class

59	Other retail trade, NEC.	591	Drug and proprietary--retail.	5910	Drug and proprietary--retail.	591	B

592	Liquor--retail.	5920	Liquor--retail.	592

593	Antiques and secondhand mer-	5931	Antiques--retail.	5932

chandise--retail.	5932	Secondhand merchandise retail.	5933

594	Book and stationery--retail.	5941	Books-retail.	5942

5942	Stationery--retail.	5943

595	Sporting goods and bicycles--	5951	Sporting goods--retail.	5952

retail.	5952	Bicycles--retail.	5953

596	Farm and garden supplies--re-	5961	Hay, grains, and feeds-retail.	5962

tail.	5969	Other farm and garden supplies	5969

retail, NEC.

597	Jewelry--retail.	5970	Jewelry--retail.	597

598	Fuel and ice--retail.	5981	Fuel and ice dealers (except fuel oil	5982

and bottled gas dealers)--retail.

5982	Fuel oil--retail.	5983

5983	Bottled gas--retail.	5984

599	Other retail trade, NEC.	5991	Florists--retail.	5992

5992	Cigars and cigarettes--retail.	5993

5993	Newspapers and magazines--retail.	5994

5994	Cameras and photographic supplies--	5996

retail.

5995	Gifts, novelties, and souvenirs--re-	5997

tail.

5996	Optical goods--retail.	5998

5999	Other retail trade, NEC.	5999

61	Finance, insurance, and	611	Banking and bank-related func-	6111	Banking services.	601, 602, 603,

real estate services.		  tions.			and 604

6112	Bank-related functions.	605

612	Credit services (other than	6121	Savings and loan associations.	612

banks).	6122	Agricultural, business, and personal	613, 614 and 615

credit services (including credit

unions).

6129	Other credit services (other than	611 and 616

banks), NEC.

613	Security and commodity brok-	6131	Security brokers, dealers, and flota-	6211

ers, dealers, exchanges, and		  tion services.

services.	6132	Commodity contracts brokers and	6221

dealers services.

6133	Security and commodity exchanges.	623

6134	Security and commodity allied serv-	628

ices.

614	Insurance carriers, agents, brok-	6141	Insurance carriers.	63

ers, and services.	6142	Insurance agents, brokers, and serv-	64	B

ices.

FOOTNOTES			the premises.  For  example, 5122-0 is a wholesaler of paints and var-

nishes who has a definite storage area set aside for  his merchandise.

1The SIC codes are listed for purposes of reference.  They are the	3Code 5461--”bakeries (manufacturing)--retail” include only those

codes in the SIC system that most nearly correspond to the 4-digit	bakeries that produce on the premises some or all of the products sold.

land use activity indicated.  (See ch. 111. sec. A3, “The Use of Standard	    4Code 5462--”Bakeries (nonmanufacturing)--retail” include only

Industrial Classification Nomenclature.”)  A dash indicates that there	those bakeries that do not produce on the premises the products that

is no corresponding SIC code.			are sold.

2Code 51--”Wholesale trade.”  A code of “0" is used in the auxiliary	    5Code 5810--”Eating places” include both establishments serving

position for those wholesalers who maintain a definite storage area on	only food as well as those serving both food and alcoholic beverages.

B-16

B.	A STANDARD SYSTEM FOR IDENTIFYING AND CODING LAND USE ACTIVITIES -- TWO-, THREE-, AND FOUR-DIGIT

LEVELS -- Continued

SIC	Land*

Code	Category	Code	Category	Code	Category	Reference1	Class

61	Finance, insurance, and	615	Real estate and related services.	6151	Real estate operators (except devel-	651	B

real estate services--				  opers) and lessors.

Continued.			6152	Real estate agents, brokers, and	653

management services.

6153	Title abstracting services.	654

6154	Real estate subdividing and develop-	655

ing services.

6155	Real estate operative builders.	656

6156	Combinations of real estate, insur-	661

ance, loan, and law services.

6159	Other real estate and related services,	----

NEC.

616	Holding and investment serv-	6160	Holding and investment services.	67

ices.

619	Other finance, insurance, and 	6190	Other finance, insurance, and real	----

real estate services, NEC.		  estate services, NEC.

62	Personal services.	621	Laundering, dry cleaning, and	6211	Laundering, dry cleaning, and dye-	7211, 7212, and

dyeing services.		  ing services (except rugs).	7216

6212	Linen supply and industrial laundry	7213

services.

6213	Diaper services.	7214

6214	Laundering and dry cleaning (self-	7215

service).

6215	Rug cleaning and repair services.	7217

622	Photographic services (includ-	6220	Photographic services (including	722

ing commercial).		  commercial).

623	Beauty and barber services.	6231	Beauty services.	723

6232	Barber services.	724

624	Funeral and crematory services;	6241	Funeral and crematory services.	726

cemeteries.	6242	Cemeteries.	----

625	Apparel repair, alteration and 	6251	Pressing, alteration, and garment re-	7271

cleaning pickup services; shoe		  pair; laundry and dry cleaning

repair services.		  pickup services (only).

6252	Fur repair and storage services.	7272

6253	Shoe repair, shoe shining, and hat	725

cleaning services.

629	Other personal services, NEC.	6290	Other personal services, NEC.	729

63	Business services.	631	Advertising services.	6311	Advertising services (general).	7311

6312	Outdoor advertising services.	7312

6319	Other advertising services, NEC.	7319

632	Consumer and mercantile credit	6320	Consumer and mercantile credit re-	7321

reporting services, adjustment		  porting services; adjustment and

and collection services.		  collection services.

633	Duplicating mailing and steno-	6331	Direct mail advertising services.	7331

graphic services.	6332	Blueprinting and photocopying serv-	7332

ice.

6339	Stenographic services and other 	7339

duplicating and mailing services, NEC.

634	Dwelling and other building	6341	Window cleaning services.	7341

services.	6342	Disinfecting and exterminating serv-	7342

vices.

6349	Other dwelling and building services,	7349

NEC.

635	News syndicate services.	6350	News syndicate services.	735

636	Employment services.	6360	Employment services.	736	B

B-17

B.	A STANDARD SYSTEM FOR IDENTIFYING AND CODING LAND USE ACTIVITIES -- TWO-, THREE-, AND FOUR-DIGIT

LEVELS -- Continued

SIC	Land*

Code	Category	Code	Category	Code	Category	Reference1	Class

63	Business services--	637	Warehousing and storage serv-	6371	Farm products warehousing and	4221	B

Continued		  ices.2		  storage (excluding stockyards).

6372	Stockyards.	4731

6373	Refrigerated warehousing (except	4222

food lockers).

6374	Food lockers (with or without food	4223

preparation facilities).

6375	Household goods warehousing and 	4224

storage.

6376	General warehousing and storage.	4225

6379	Other warehousing and storage,	4226

NEC.

639	Other business services, NEC.	6391	Research, development, and testing	7391

services.

6392	Business and management consulting 	7392

services.

6393	Detective and protective services.	7393

6394	Equipment rental and leasing serv-	7394

ices.

6395	Photofinishing services.	7395

6396	Trading stamp services.	7396

6397	Automobile and truck rental serv-	7511

ices.

6398	Motion picture distribution and 	7812 and 782

services.

6399	Other business services, NEC.	7399

64	Repair services.	641	Automobile repair and services.	6411	Automobile repair services.	753

6412	Automobile wash services.	Incl. 7541

6419	Other automobile services (except re-	Incl. 7541

pair and wash), NEC.

649	Other repair services, NEC.	6491	Electrical repair services (except	Incl. 7621

radio and television).

6492	Radio and television repair services.	7622

6493	Watch, clock, and jewelry repair	763

services.

6494	Reupholstery and furniture repair	764

services.

6495	Armature rewinding services.	7694

6499	Other repair services, NEC.	7699	B

65	Professional services.	651	Medical and other health serv-	6511	Physicians’ services.	801, 803,	A

ices.			and 804

6512	Dental services.	802

6513	Hospital services.	806

6514	Medical laboratory services.	8071

6515	Dental laboratory services.	8072

6516	Sanitariums, convalescent, and rest	8092

home services.

6517	Medical clinics--out-patient services.	----

6519	Other medical and health services,	8099	A

NEC.

652	Legal services.	6520	Legal services.	811	B

659	Other professional services,	6591	Engineering and architectural serv-	891

NEC.		  ices.

6592	Educational and scientific research	892

services.

6593	Accounting, auditing, and book-	893

keeping services.

6594	Urban planning services.	Incl. 899

6599	Other professional services, NEC.	Incl. 899

66	Contract construction	661	General contract construction	6611	Building construction--general con-	151

services.		  services.		  tractor services.

6619	Other general construction contrac-	16	B

tor  services, NEC.

B-18

B.	A STANDARD SYSTEM FOR IDENTIFYING AND CODING LAND USE ACTIVITIES -- TWO-, THREE-, AND FOUR-DIGIT

LEVELS -- Continued

SIC	Land*

Code	Category	Code	Category	Code	Category	Reference1	Class

66	Contract construction	662	Special construction trade serv-	6621	Plumbing, heating, and air condi-	171	B

services--Continued		  ices.		  tioning services.

6622	Painting, paper hanging, and decor-	172

ating services.

6623	Electrical services.	173

6624	Masonry, stonework, tile setting,	174

and plastering service.

6625	Carpentering and wood flooring.	175

6626	Roofing and sheet metal services.	176

6627	Concrete services.	177

6628	Water well drilling services.	178

6629	Other special construction trade	179

services, NEC.

67	Governmental services.	671	Executive, legislative, and ju-	6710	Executive, legislative, and judicial	Incl. 919,

dicial functions.		  functions.3	929, and 939

672	Protective functions and their	6721	Police protection and related activi-	Incl. 919,

related activities.		  ties.	929, and 939

6722	Fire protection and related activities.	Incl. 919,

929, and 939

6723	Civil defense and related activities.	Incl. 919,

929, and 939

673	Postal services.	6730	Postal services.	Incl. 919	B

674	Correctional institutions.	6741	Prisons.	Incl. 919,	A

929, and 939

6749	Other correctional institutions, NEC.	Incl. 919,

929, and 939	A

675	Military bases and reservations.4	6751	Military training bases.	Incl. 919	B

and 929

6752	Military defense installations.	Incl. 919

and 929

6753	Military storage depots and trans-	Incl. 919

portation centers.	 and 929

6754	Military maintenance centers.	Incl. 919

and 929

6755	Military administration or command	Incl. 919

centers.	 and 929

6756	Military communication centers.	Incl. 919

and 929

6759	Other military bases and reserva-	Incl. 919

tions, NEC.	 and 929	B

68	Educational services.	681	Nursery, primary, and secondary	6811	Nursery schools.	Incl. 821	A

education.	6812	Primary (elementary) schools.5	Incl. 821

6813	Secondary schools.6	Incl. 821

682	University, college, junior college,	6821	Universities and colleges.	Incl. 8221

and professional school edu-	6822	Junior colleges.	Incl. 8222

cation.	6823	Professional schools.	Incl. 8221

683	Special training and schooling.	6831	Vocational or trade schools.	8242

6832	Business and stenographic schools.	Incl. 8299

6833	Barber and beauty schools.	  Incl. 723

and 724

6834	Art and music schools.	Incl. 8299

6835	Dancing schools.	Incl. 7911

6836	Driving schools.	Incl. 8299

6837	Correspondence schools.	        8241

6839	Other special training and schooling,	Incl. 8299	A

NEC.

B-19

B.	A STANDARD SYSTEM FOR IDENTIFYING AND CODING LAND USE ACTIVITIES -- TWO-, THREE-, AND FOUR-DIGIT

LEVELS -- Continued

SIC	Land*

Code	Category	Code	Category	Code	Category	Reference1	Class

69	Miscellaneous services.	691	Religious activities.7	6911	Churches, synagogues, and temples.	Incl. 866	A

6919	Other religious activities, NEC.	Incl. 866	A

692	Welfare and charitable services.	6920	Welfare and charitable services.	867	B

699	Other miscellaneous services,	6991	Business associations.	861

NEC.	6992	Professional membership organiza-	862

tions.

6993	Labor unions and similar labor	863

organizations.

6994	Civic, social, and fraternal associa-	864

tions.

6999	Other miscellaneous services, NEC.	----	B

FOOTNOTES

1The SIC codes are listed for purposes of reference.  They are the	legislature and courts.  All operational activities (e.g., shipbuilding,

codes in the SIC system that most nearly correspond to the 4-digit land	schools, or hospitals) should be identified separately under the respective

use activity indicated.  (See ch. III, sec. A3, “The Use of Standard	activity codes.

Industrial Classification Nomenclature.”)  A dash indicates that there	  4Code 675--”Military bases and reservations” include the installa-

is no corresponding SIC code.			tions used by both the active military as well as the Reserves and the

2Code 637--”Warehousing and storage services” include only those	National Guard.

facilities that are used by or are open to the public.  When ware-	  5Code 6812--”Primary (elementary) schools” may or may not include

housing and storage is functionally and organizationally linked to 	a kindergarten, but they do include grades 1 through 6.

another activity (e.g., a general contractor or an apparel manufacturer),	  6Code 6813--”Secondary schools” are schools that include grades 7

the facilities are identified and coded the same as the parent activity	through 12, popularly known as junior and senior high schools.

and with a code of 4 (warehousing and storage) in the auxiliary position.	  7Code 691--”Religious activities” include only those places operated

For example, 2310-4 is a warehouse and storage area of a manufacturer	for  worship or for the promotion of religious activities.  Activities

of  men’s, youths’, and boys’ suits, coats, and overcoats.	maintained by the religious organizations (e.g., schools, hospitals, pub-

3Code 6710--”Executive, legislative, and judicial functions” include	lishing houses, etc.) should be identified separately under the respective

only the central and administrative office activities of the agencies or	activity codes.

special authorities involved in government functions, including the

71	Cultural activities and 	711	Cultural activities.	7111	Libraries	823	A

nature exhibitions.			7112	Museums.	Incl. 841

7113	Art galleries.2	Incl. 841

7119	Other cultural activities, NEC.	        ----

712	Nature exhibitions.	7121	Planetaria.	        ----

7122	Aquariums.	Incl. 8421

7123	Botanical gardens and arboretums.	Incl. 842	A

7124	Zoos.	        ----	B

7129	Other nature exhibitions, NEC.	        ----

719	Other cultural activities and	7191	Historic and monument sites.3	        ----

nature exhibitions, NEC.	7199	Other cultural activities and nature	        ----

exhibitions, NEC.

72	Public assembly.	721	Entertainment assembly.	7211	Amphitheaters.	        ----

7212	Motion picture theaters.	Incl. 7831

7213	Drive-in  movies.	Incl. 7831

7214	Legitimate theaters.	        ----

7219	Other entertainment assembly, NEC.	        ----

722	Sports assembly.4	7221	Stadiums.5	Incl. 7941

7222	Arenas and field houses.	        ----	B

7223	Race tracks.6	Incl. 7948	C

7229	Other sports assembly, NEC.	        ----	B

723	Public assembly, miscellaneous	7231	Auditoriums.	        ----

purposes.	7232	Exhibition halls.	        ----

7239	Other miscellaneous assembly, NEC.	        ----

729	Other public assembly, NEC.	7290	Other public assembly, NEC.	        ----	B

73	Amusements.	731	Fairgrounds and amusement	7311	Fairgrounds.	Incl. 7949	*

parks.	7312	Amusement parks.	Incl. 7949	B

*Fairgrounds are Class B Land; when used for automobile and

motorcycle racing, Fairgrounds are Class C Land.

B-20

B.	A STANDARD SYSTEM FOR IDENTIFYING AND CODING LAND USE ACTIVITIES -- TWO-, THREE-, AND FOUR-DIGIT

LEVELS -- Continued

SIC	Land*

Code	Category	Code	Category	Code	Category	Reference1	Class

73	Amusements--Continued	739	Other amusements, NEC.	7391	Penny arcades.	Incl. 7949	B

7392	Miniature golf.	Incl. 7949

7393	Golf driving ranges.	Incl. 7949

7394	Go-cart tracks.	Incl. 7949

7399	Other amusements, NEC	Incl. 7949

74	Recreational activities.	741	Sports activities.	7411	Golf courses (without country club).              7942

7412	Golf courses (with country club).	        7947

7413	Tennis courts.	        ----

7414	Ice skating	Incl. 7945

7415	Roller  skating	Incl. 7945

7416	Riding stables.	          ----

7417	Bowling	Incl. 7931

7418	Skiing and tobogganing.	          ----

7419	Other sports activities, NEC.	          ----

742	Playgrounds and athletic areas.	7421	Play lots or tot lots.7	          ----

7422	Playgrounds.8	          ----

7423	Playfields or athletic fields.9	          ----

7424	Recreation centers (general).10	          ----

7425	Gymnasiums and athletic clubs.	Incl. 7949

7429	Other playground and athletic areas, NEC.      ----

743	Swimming areas.	7431	Swimming beaches.11	          ----

7432	Swimming pools.12	        7944

744	Marinas.13	7441	Yachting clubs.	Incl. 7949

7442	Boat rentals and boat access sites.	          ----

7449	Other marinas, NEC.	          ----

749	Other recreation, NEC.	7491	Camping and picnicking areas.14	          ----

7499	Other recreation, NEC.	          ----

75	Resorts and group camps.	751	Resorts.	7511	General resorts.15	          ----

7512	Dude ranches.	Incl. 7032

7513	Health resorts.	Incl. 8092

7514	Ski resorts.	          ----

7515	Hunting and fishing clubs.18	Incl. 7032

7519	Other resorts, NEC.	          ----

752	Group or organized camps.	7520	Group or organized camps.17	Incl. 7032

76	Parks.	761	Parks--general recreation.	7610	Parks--general recreation.18	          ----	B

762	Parks--leisure and ornamental.	7620	Parks--leisure and ornamental.19	          ----	A

769	Other parks, NEC.	7690	Other parks, NEC.	          ----	B

79	Other cultural, entertain-	790	Other cultural, entertainment,	7900	Other cultural, entertainment, and 	          ----	B

ment, and recreational		  and recreational activities, NEC.		  recreational activities, NEC.

activities, NEC.

FOOTNOTES			contain such facilities as sand boxes, slides, teeters, swings, climbing

1The SIC codes are listed for purposes of reference.  They are the 	apparatus, etc.  They are identified and coded only when found as a

codes in the SIC system that most nearly correspond to the 4-digit	separate activity and not subsidiary to or serving another activity (e.g.

land use activity indicated.  (See ch. III. sec. A3, “The Use of Standard	apartment house or playground).

Industrial Classification Nomenclature,”)  A dash indicates that there	  8Code 7422--”Playgrounds” are areas that have been developed for

is no corresponding SIC code.			active play and recreation.  They are identified and coded only  when

2Code 7113--”Art galleries” do not include those galleries that sell	found as a separate activity and not subsidiary to or serving another

art objects commercially.  Commercial sales are coded 5999, “Other	activity (e.g., a school).

retail trade, NEC.”			  9Code 7423--”Playfields or athletic fields” contain a playground as

3Code 7191--”Historic and monument sites” include those locations	well as a field(s) or court(s) for competitive sports (e.g., baseball, foot-

set aside for no other purpose than to commemorate an historical event,	ball, or tennis).  Bleachers or grandstands may be provided.  They

activity, or person.			are identified and coded only when  found as a separate activity and not

4Code 722--”Sports assembly” includes only the public assembly	subsidiary to or serving another activity (e.g., a school).

areas used for nonparticipating sports.			  10Code 7414'’”Recreation centers (general)” include diversified

5Code 7221--”Stadiums” include those used for individual sports,	recreation for a wide variety of activities for all ages and interests.

e.g., baseball or football, as well as those used for several sports activi-	The recreation centers may contain, but are not limited to a gymnasium,

ties.					social or play rooms, game rooms, arts and craft shops, etc.

6Code 7223--”Race tracks” include those used for individual racing	  11Code 7431--”Swimming beaches” are beach areas that have been

activities, e.g., horse racing and automobile racing, as well as those used	set aside specifically for the purpose of swimming.  They are identified

for several racing activities.			only when they are not a part of a larger activity (e.g., a park).

7Code 7421--”Play lots or tot lots” are small areas developed espe-	  12Code 7432--”Swimming Pools” are separately identified if they are

cially for preschool or elementary school aged children.  They may	Footnotes continued on following page.

B-21

B.	A STANDARD SYSTEM FOR IDENTIFYING AND CODING LAND USE ACTIVITIES -- TWO-, THREE-, AND FOUR-DIGIT

LEVELS -- Continued

independent of other functions (e.g., code 72, “Public assembly” or	 16Code 7515--”Hunting and fishing clubs” include areas on which

code 11, “Household units”).  They may be indoor or outdoor pools.	artificially propagated game or fish are released for purposes of hunting

13Code 744--”Marinas” include marine terminals and associated	or  fishing.  If there are other uses made of the property (e.g., agricul-

areas that are primarily for recreational marine craft.  The sale and 	tural use), these “other” uses should take priority in identifying the

repair of recreational marine craft is coded 5591, “Marine craft and	activity of the parcel.

accessories--retail” and code 3443, “Ship and boat building and re-	  17Code 7520--”Group or organized camps” include general camps for

pairing,” respectively.			children, as well as Boy Scout and Girl Scout camps.

14Code 7491--”Camping and picnicking areas” are separately identified	  18Code 7610--”Parks--general recreation” may include, but are not

if they are not a part of a larger activity (e.g., a park).		limited to, picnic areas, bathing beaches, playfields, hiking trails,

15Code 7511--”General resorts” have rooms for 20 or more persons	camping grounds, and other manmade recreation facilities.

and have provision for at least 2 types of recreational activities, exclud-	  19Code 7620--”Parks--leisure and ornamental”are largely for scenic

ing lawn games, children’s playgrounds, and swimming pools.	or  leisure purposes.  They may contain beaches, children’s play facilit-

ties, monuments, or statues.

SIC	Land*

Code	Category	Code	Category	Code	Category	Reference1	Class

81	Agriculture.2	811	Farms (predominant crop,	8111	Farms (predominant crop, cotton).	----	C

fibers).3	8119	Farms (other type fiber crops).	----

812	Farms (predominant crop, cash	8120	Farms (predominant crop, cash	----

grains).3		  grains).

813	Farms (field crops other than	8130	Farms (field crops other than fiber	----

fiber or cash grain crops).3		  or cash grain crops).

814	Farms (predominant crop,	8141	Farms (predominant crop, fruits).	----

fruits, tree nuts, or vegeta-	8142	Farms (predominant crop, tree nuts).	----

bles).3	8143	Farms (predominant crop, vege-	----

tables).

815	Farms (predominantly dairy	8150	Farms (predominantly dairy prod-	----

products).3		  ucts).

816	Farms and ranches (livestock	8161	Farms and ranches (predominantly	----

other than dairy).3		  cattle).

8162	Farms and ranches (predominantly	----

hog).

8163	Farms and ranches (predominantly	----

sheep).

8164	Farms and ranches (predominantly	----

goat).

8169	Farms and ranches (other livestock),	----

NEC.

817	Farms (predominantly poultry).3	8170	Farms (predominantly poultry).	----

818	Farms (general--no predomi-	8180	Farms (general--no predominance).	----

nance).3

819	Other agriculture and related	8191	Range and grassland pastures (not	----

activities, NEC.		  farm or  ranch).4

8192	Horticultural specialties.	0192

8193	Apiary farms.	Incl. 0193

8194	Farms and ranches (predomi-	Incl. 0193

nantly horse raising).

8199	Other agriculture and related activi-         ----

ties, NEC.

82	Agricultural related activi-	821	Agricultural processing.3	8211	Cotton ginning and compressing.	        0712

ties.			8212	Grist milling services.	        0713

8213	Corn shelling, hay baling, and thresh-   0714

ing services.

8214	Contract sorting, grading, and pack-      0715

aging services (fruits and vegeta-

bles).

8219	Other agricultural processing serv-         0719

ices, NEC.

822	Animal husbandry services.	8221	Veterinarian services.	 Incl. 0722

8222	Animal hospital services.	 Incl. 0722

8223	Poultry hatchery services.	         0723

8229	Other animal husbandry services,	         0729	C

NEC.

B-22

B.	A STANDARD SYSTEM FOR IDENTIFYING AND CODING LAND USE ACTIVITIES -- TWO-, THREE-, AND FOUR-DIGIT

LEVELS -- Continued

SIC	Land*

Code	Category	Code	Category	Code	Category	Reference1	Class

82	Agricultural related activi	829	Other agricultural related activi-	8291	Horticultural services.	0731	C

ties--Continued		  ties, NEC.	8299	Other agricultural related activities,	----

NEC.

83	Forestry activities and re-	831	Commercial forestry produc-	8311	Timber production--predominantly	----

lated services.6		  tion.7		  for pulp wood.

8312	Timber production--predominantly	----

for saw logs.

8313	Timber production--predominantly	----

for veneer logs.

8314	Timber production--mixed uses.	----

8315	Tree products production--predomi-	0842

nantly gum extracting (except pine

gum) and bark.

8316	Tree products production--predomi-	0843

nantly pine gum extraction.

8317	Timber and tree products produc-	----

tion--mixed uses.

8319	Other commercial forestry produc-	----

tion, NEC.

832	Forestry services.	8321	Forest nurseries.	0822

8329	Other forestry services, NEC.	0851

839	Other forestry activities and re-	8390	Other forestry activities and related	----

lated services, NEC.		  services, NEC.

84	Fishing activities and re-	841	Fisheries and marine products.8	8411	Finfish fisheries.	0912

lated services.			8412	Shellfish fisheries.	0913

8419	Other fisheries and marine products,	0914 and 0919

NEC.

842	Fishery services.	8421	Fish hatcheries.	Incl. 0989

8429	Other fishery services, NEC.	Incl. 0989

849	Other fishery activities and re-	8490	Other fishery activities and related	           ----

lated services, NEC.		  services, NEC.

85	Mining activities and re-	851	Metal ore mining.	8511	Iron ore-mining.	101

lated services.			8512	Copper ore mining.	102

8513	Lead and zinc ore mining.	103

8514	Gold and silver ore mining.	104

8515	Bauxite and other aluminum ore	105

mining.

8516	Ferroalloy ore (except vanadium)	106

mining.

8519	Other metal ore mining, NEC.	109

852	Coal mining.	8521	Anthracite coal mining.	111

8522	Bituminous coal mining.	1211

8523	Lignite coal mining.	1212

853	Crude petroleum and natural gas.	8530	Crude petroleum and natural gas.	131

854	Mining and quarrying of non-	8541	Dimension stone.	141

metallic minerals (except	8542	Crushed and broken stone (including	142

fuels).		  riprap)--quarrying.

8543	Sand and gravel--quarrying.	144

8544	Clay, ceramic, and refractory miner-	145

als--mining.

8545	Chemical and fertilizers (mineral)--	147

mining.

8549	Other mining and quarrying of non-	148

metallic minerals (except fuels), NEC.

855	Mining services.	8551	Metal mining services.	108

8552	Coal mining services.	 1112 and 1213

8553	Crude, petroleum and gas field	138

services.

8554	Nonmetallic mining (except fuel)--	148

services.

8559	Other mining services, NEC.	----

89	Other reSource:  production	890	Other reSource:  production and	8900	Other reSource:  production and extrac-	----	C

and extraction, NEC.		  extraction, NEC.		  tion, NEC.

B-23

B.	A STANDARD SYSTEM FOR IDENTIFYING AND CODING LAND USE ACTIVITIES -- TWO-, THREE-, AND FOUR-DIGIT

LEVELS -- Continued

FOOTNOTES			farm or ranch.  These areas are usually part of the public domain in

which grazing has been permitted.

1The SIC codes are listed for purposes of reference.  They are the 	  5Code 821--”Agricultural processing” includes only preliminary

codes in the SIC system that most nearly correspond to the 4-digit land	processing of agricultural products.  Any extensive processing, packing,

use activity indicated.  (See ch. III, Se. A3, “The Use of Standard	canning, or manufacturing is coded 21, “Food and kindred products--

Industrial Classification Nomenclature.”)  A dash indicates that there	manufacturing.”

is no corresponding SIC code.			  6Code 83--”Forestry activities and related services.”  The categories

2Code 81--”Agriculture.”  A parcel of land is considered to be in	in this classification are based upon primary use of the land.  It is

agricultural use if 10 or more acres are under cultivation, in tree or 	recognized that other activities, e.g., recreation or the grazing of live-

bush crops, or are used for livestock or poultry purposes.  The 10 acres	stock may also be taking place within these forested areas.  However,

may include the area of the residence, if there is one, and the immediate	these types of activities are considered secondary in nature and not

associated area surrounding the residence.		coded.  Activities such as mining (code 85), permanent camping areas

3A farm may consist of several ownership or rented parcels of land	(code 7491), and logging camps (code 2410), located within the

that are noncontiguous.  However, each parcel should be linked to-	forested  areas should be separately identified.

gether as one “farm management unit,” and identified by only one	  7Code 831--”Commercial forestry production” includes those

3- or 4-digit category, e.g., code 8120, “Farms (predominant crops, cash	forested areas not on the farms or ranches that are being managed or

grains).”				have been set aside to grow tree crops for “industrial wood” or to obtain

Using the “Farm use” categories in app. 2 of this report, farm (i.e.,	tree products such as sap, bark, or seeds.  “Industrial wood” includes

the farm management units) are to be classified by the most predominant	commercial roundwood products, such as saw logs and pulpwood, but

use made of the land (the farm use taking up the greatest percentage of	excludes fuel wood and fence posts.  “Forest reserves,” i.e., areas

farm area) excluding the areas used by farm residences and associated	withdrawn from any commercial use of the trees, are coded 921.

buildings, nonresident farm buildings, areas in feed crops, and grazed	“Parks,” e.g., national or State parks, are also a type of forest reserve,

and nongrazed forested areas.  Exceptions to this rule are:	but because of their designation for recreational activity, they should

a.  If 50 percent or more of the value of the farm products sold in the	be identified as “Parks” under code 76.  Forested areas not on farms,

previous year came from the sale of dairy products, or the sale of cows	ranches, or estates with no commercial use made of the trees are coded

and calves, the farm is identified as “Farm (predominantly dairy	922, “nonreserve forests (undeveloped).”

products),” code 8150.			 8Code 841--”Fisheries and marine products” include those establish-

b.  If 50 percent or more of the value of the farm products sold in	ments primarily engaged in commercial fishing, the catching or taking

the previous year came from the sale of livestock, wool, or mohair,	of shellfish, or the gathering of seaweed, sponges, turtles, frogs, etc.

the farm is identified as “Farms and ranches (livestock, other than	These activities may include some preliminary processing, e.g., salting.

the farm is identified as “Farms and ranches (livestock, other than	However, any extensive processing, packing, canning, or manufacturing

dairy),” code 816.			of these products should be coded under “Food and kindred products--

c.  If 50 percent or more of the value of the farm products sold in	manufacturing,” code 21.  These categories also include the docking

the previous year came from the sale of poultry and eggs, the farm	facilities and their associated areas when these facilities are an integral

is identified as “Farms (predominantly poultry),” code 8170.	part of a single fishery operation.  When docking facilities serve several

d.  If the percentage of dairy, livestock, and poultry products,	separate fishery establishments, they are coded 4414, “Marine terminals

each came to less than 50 percent of the value of the farm products	(predominantly fishing vessels).”

sold in the previous years, and if there are 3 or more “Farm uses”	  9Code 85--”Mining activities and related services” include those

(e.g., corn crop, cotton crop, etc.) within a farm, none of which takes	surface areas being used for mining or drilling purposes.  the process

up 25 percent of the total farm area (excluding those areas referred	may be tunnel excavation, strip mining, quarrying, or by drilling,

to above), the farm is identified as “Farms (general--no predomi-	These categories also include those areas where preliminary processing

nance),” code 8180.			of raw materials (e.g., washing, crushing, screening, etc.) are taking

4Code 8191--”Range and grass land pastures (not form or ranch)”	place if these processes are an integral part of the mining operation.

include grasslands used for grazing purposes that are not a part of a

SIC	Land*

Code	Category	Code	Category	Code	Category	Reference1	Class

91	Undeveloped and unused	910	Undeveloped and unused land	9100	Undeveloped and unused land area	----	U

land area (excluding non-		  area (excluding noncommer-		  (excluding noncommercial forest

commercial forest devel-		  cial forest development).		  development).2

opment).

92	Noncommercial forest de-	921	Forest reserves.3	9211	Forest reserves (wilderness areas).	----	A

velopment.			9212	Forest reserves (wildlife refuges).	----

9219	Other forest reserves, NEC.	----	A

922	Nonreserve forests (undeveloped).	9220	Nonreserve forests (undeveloped).4	----	B

93	Water areas.	931	Rivers, streams, or creeks.	9310	Rivers, streams, or creeks.	----	U

932	Lakes.	9320	Lakes.5	----

933	Bays or lagoons.	9330	Bays or lagoons.	----

934	Oceans and seas.	9340	Oceans and seas.	----

939	Other water areas, NEC.	9390	Other water areas, NEC.	----

94	Vacant floor area.	940	Vacant floor area.	9400	Vacant floor area.	----

95	Under construction.	951	Under construction (residential).	9510	Under construction (residential).6	----

952	Under construction (nonresidential).	9520	Under construction (nonresidential).7	----

99	Other undeveloped land	990	Other undeveloped land and 	9900	Other undeveloped land and water	----	U

and water areas, NEC.		  water areas, NEC.		  areas, NEC.

See footnotes on following page.	B-24

B.	A STANDARD SYSTEM FOR IDENTIFYING AND CODING LAND USE ACTIVITIES -- TWO-, THREE-, AND FOUR-DIGIT

LEVELS -- Continued

FOOTNOTES			  4Code 9220--”Nonreserve forests (undeveloped)” are major forested

1The SIC codes are listed for purposes of reference.  They are the 	areas not on a farm, ranch, or large estate with no commercial use made

codes in the SIC system that most nearly correspond to the 4-digit	of the trees.  It is recognized that other activities such as recreation or

land use activity indicated.  (See ch. III, Se. A3, “The Use of Standard	the grazing of livestock may also be taking place within these forested

Industrial Classification Nomenclature.”)  A dash indicates that there	areas.  However, these types of activities are considered secondary in

is no corresponding SIC code.			nature and not coded.  Activities such as mining (code 85), permanent

2Code 9100--”Undeveloped and unused land area (excluding non-	camping areas (code 7491), and logging camps (code 2410), located

commercial forest development)” identifies those parcels of land that 	within these forested areas should be separately identified.

appear to be undeveloped or if previously developed, are presently	  5Code 9320--”Lakes” include permanent lakes (natural or manmade)

vacant and unused.  This category includes such areas as vacant lands	with a minimum size of 1 acre.  Impounded surface water areas used

that once were farms, as well as vacant parcels where structures have	for  storage should be identified as “Water storage,” code 4833.

been demolished.  Vacant nonresidential buildings are coded 9400,	  6Code 9510--Residential facilities under construction are considered

“Vacant floor area.”			to be completed when all exterior windows and doors are installed and

3Code 921--”Forest reserves” are forested areas withdrawn from 	the usable floors are in place.  If construction has not reached this

commercial utilization, and which are reserved through statute or admin-	point, the parcel should be identified as “Under construction (resi

istrative regulation for specific conservation purposes.  Forested areas	dential),” code 9510

designated as park sites may also be restricted from commercial forestry	  7Code 9520--”Under construction (nonresidential)” is used only if

production, but because of their designation for recreational activity	there is no means of identifying the activity or activities that will

they should be identified and coded as “Parks” (code 76).	occupy the structure when it is completed.

(Source: Amended at 30 Ill. Reg. _____________, effective _______________)

TITLE 35:  ENVIRONMENTAL PROTECTION

SUBTITLE H:  NOISE

# CHAPTER I:  ILLINOIS POLLUTION CONTROL BOARD

PART 910

MEASUREMENT PROCEDURES FOR THE ENFORCEMENT

OF 35 ILL. ADM. CODE 900 & 901

## Section

910.100	General

910.102	Instrumentation

910.103	Definitions

910.104	Measurement Techniques for 35 Ill. Adm. Code 900

910.105	Measurement Techniques for 35 Ill. Adm. Code 901

  - Protocols for Determination of Sound Levels

  - Measurement Techniques for Highly-Impulsive Sound Under 35 Ill. Adm. Code 104

910.APPENDIX A	Tables of Long-Term Background Ambient Noise

910. TABLE A	Daytime long-term background ambient Leq levels in decibels by land use categories and 1/3 octave-band level

910. TABLE B	Nighttime long-term background ambient Leq levels in decibels by land use categories and 1/3- octave-band level

910. TABLE C	Daytime long-term background ambient Leq levels in decibels by land use categories and octave-band level

910. TABLE D	Nighttime long-term background ambient Leq levels in decibels by land use categories and octave-band level

AUTHORITY:  Implementing and authorized by Sections 25 and 27 of the Environmental Protection Act [415 ILCS 5/25 and 27]

SOURCE:  Adopted at 30 Ill. Reg. __________, effective________________.

Section 910.100	General

This Part specifies the instrumentation to be used when conducting acoustical noise measurements and sets forth the specific acoustical measurement techniques to be employed when conducting time-averaged sound level (Leq) measurements.  The instrumentation requirements and measurement techniques as more specifically set forth in this Part must be used in determining whether a noise source is in compliance with 35 Ill. Adm. Code 900 and 901.

### Section 910.102	Instrumentation

a)	Sound Measuring Equipment

1)	An integrating sound level meter used alone or used in conjunction with an octave-band or 1/3 octave-band filter set or a real-time sound analyzer (octave-band or 1/3 octave-band) must conform with the following standards incorporated by reference at 35 Ill. Adm. Code 900.106:

A)	ANSI S1.4 – 1983 (R2001) “American National Standard Specification for Sound Level Meters”, and ANSI S1.4 A-1985 “Amendment to ANSI S1.4-1983.”

B)	ANSI S1.11 - 1986 (R1998) “American National Standard Specifications for Octave-Band and Fractional-Octave-Band Analog and Digital Filters.”

C)	ANSI S1.6 – 1984 (R2001) “American National Standard Preferred Frequencies, Frequency Levels, and Band Numbers for Acoustical Measurements.”

D)	ANSI S1.8 - 1989 “American National Standard Reference Quantities for Acoustical Levels.”

- International Electrotechnical Commission, IEC 804-2000 Integrating/Averaging Sound level meters.

- A magnetic tape recorder, graphic level recorder or other indicating device used must meet the requirements of the Society of Automotive Engineers (SAE) Recommended Practice J184 “Qualifying a Sound Data Acquisition System,” November 1998, incorporated by reference at 35 Ill. Adm. Code 900.106

    - The laboratory calibration of instrumentation used for acoustic measurement must be traceable to the National Bureau of Standards, and must be performed no less often than once every 12 months.

    - For outdoor measurement, a windscreen must be attached to the microphone.

b)	Weather Measuring Equipment

1)	An anemometer and compass or other devices must be used to measure wind speed and direction in accordance with the manufacturer's recommended procedures.

2)	A thermometer, designed to measure ambient temperature, must be used in accordance with the manufacturer's recommended procedures.

3)	A hygrometer must be used in accordance with the manufacturer's recommended procedures to measure the relative humidity.

4)	A barometer must be used in accordance with the manufacturer's recommended procedures to measure the barometric pressure.

Section 910.103	Definitions

The definitions contained in 35 Ill. Adm. Code 900.101 apply to this Part.

Section 910.104	Measurement Techniques for 35 Ill. Adm. Code 900

Sound pressure level measurements are not required to establish a violation of 35 Ill. Adm. Code 900.102 (nuisance noise).  However, sound pressure level measurements may be introduced as corroborating evidence when alleging a violation of 35 Ill. Adm. Code 900.102.  If sound pressure level measurements are collected, manufacturer’s instructions must be followed for the equipment used and 35 Ill. Adm. Code 910.105 may be used as guidance in gathering data.

Section 910.105	Measurement Techniques for 35 Ill. Adm. Code 901

Sound pressure level measurements must be obtained in accordance with the following measurement techniques to determine whether a noise source is in compliance with 35 Ill. Adm. Code 901:

a)	Site Selection

1)	Measurements may be taken at one or more microphone positions within the appropriate receiving land.  Measurement instruments must be set up outdoors within the boundaries of the receiving land for the purpose of determining whether a noise source is in compliance with 35 Ill. Adm. Code 901.

2)	Measurement instruments must be set up not less than 25 feet (7.6 meters (m)) from the property-line-noise-source.  The 25-foot (7.6 m) setback requirement is from the noise source and not the property line unless the noise source is contiguous to the property line.

3)	Other measurement locations may be used for investigatory purposes such as, but not limited to, the following:

A)	Determining the extent of noise pollution caused by the source of sound;

B)	Determining the ambient; and

C)	Analyzing those acoustical parameters that describe the sound source.

4)	For measurements of sound sources with no audible discrete tones, microphones should not be set up less than 25 feet (7.6 m) from any reflective surface that may affect data.  If measurements must be taken within 25 feet (7.6 m), the effect, if any, of the reflective surface on the measured data must be determined.

5)	For measurements of sound sources with audible discrete tones, microphones must not be set up less than 50 feet (15.2 m) from any reflective surface that may affect data.  If measurements must be taken within 50 feet (15.2 m), the effect, if any, of the reflective surface on the measured data must be determined.

6)	Objects with small dimensions (trees, posts, bushes, etc.) must not be within 5 feet (1.5 m) of the microphone position.  If measurements must be taken within 5 feet (1.5 m) of such objects, the effect, if any, on the measured data must be determined.

b)	Instrumentation Set Up

1)	A tripod must be set at the chosen site.  The tripod must be extended to a height between 3 feet 8 inches (1.12 m) and 4 feet 10 inches (1.47 m) above ground.

2)	A microphone must be attached to the appropriate end of a 5-foot (1.5 m) or longer cable and must be affixed to the top of the tripod.  The other end of the cable must be connected to the measuring instrument.

3)	The angle of incidence of the microphone must be adjusted to yield the flattest frequency response in accordance with the manufacturer’s specifications.

4)	The measuring instrument must be separated from the microphone so as to minimize any influence on the measurements.  The cable movement must be minimized during the measurement period.

c)	Measurement Site Operation and Instrument Calibration

1)	Before taking sound pressure level measurements, measure and record (near the measurement site):

A)	Wind speed and direction;

B)	Ambient temperature;

C)	Relative humidity; and

D)	Barometric pressure.

2)	Turn the measuring instrument on and allow the instrument to stabilize.  Monitor and record the battery condition of the calibrator and all measuring instruments.

3)	Turn the calibrator on at its appropriate frequency.  Allow the calibrator to stabilize and calibrate the measuring system according to the manufacturer's specifications.  After the measuring system has been calibrated, remove the calibrator and attach a windscreen to the microphone.

4)	Adjust the microphone to the angle of incidence that will yield the frequency response in accordance with the manufacturer's specifications.

5)	Measure the sound pressure level data within the limitations of subsection (d) and according to the manufacturer's recommended procedures.  Other sound pressure levels may be used for investigatory purposes such as, but not limited to, the following:

A)	Determining the extent of noise pollution caused by the source of sound;

B)	Determining the ambient; and

C)	Analyzing those acoustical parameters that describe the sound source.

6)	While sound measurements are being taken, the operator must be separated from the microphone so as to minimize any influence on the measurements.

7)	While measurements are being taken, visual and aural surveillance of extraneous sound sources and varying wind conditions must be made to insure that the conditions of measurement are accurately known.  Record any variations in these parameters that may affect data.  The number and basis for affected data block must be recorded.  When using a tape recorder, voice commentary concerning conditions will be recorded on the cue track.

8)	To minimize wind effects on the microphone, sound measurements must not be taken when the wind velocity is greater than 12 miles per hour (5.4 m/second) at the microphone position.

9)	For the purposes of data correction, the ambient sound at the measurement site must be determined by means of measurement or analysis.

10)	After taking sound pressure level measurements, remove the windscreen and attach the calibrator to the microphone.  Turn the calibrator on at its appropriate frequency.  After allowing the calibrator to stabilize, monitor and record the measuring system response.  When the measuring system response varies by more than + 0.5 dB from the most recent field calibration, the sound pressure level measurements obtained since such most recent field calibration cannot be used for enforcement purposes.

11)	Before removing the calibrator from the microphone, turn the calibrator off.  If the ambient sound has not been determined by means of measurement, determine the noise floor of the measuring system.  If the noise floor is within 10 dB of the measured sound pressure level data, such noise floor measurements must be recorded.

12)	At the end of the sound survey, monitor and record the battery condition of the calibrator and all measuring instruments.  Near the measurement site, measure and record:

A)	Windspeed and direction;

B)	Ambient temperature;

C)	Relative humidity; and

D)	Barometric pressure.

13)	Record the physical and topographical description of the ground surface within the vicinity of the measurement site, survey site location, a description of the sound source, a diagram of the area, the location of reflective surfaces near the microphone, and the approximate location of the noise source relative to the microphone position.

14)	A magnetic tape recorder may be used to preserve the raw data. Calibration signals must be recorded at the beginning and end of each tape as well as at intermediate times such as when relocating to a new measurement site.  Voice commentary concerning local conditions and affected data blocks must be recorded on the cue track.  The original tape recording must be preserved for subsequent evaluation.  Laboratory analyses may be performed on magnetic tape recorded field data.  A description of the laboratory instrumentation and procedures must be recorded.  Analyses used in the laboratory must be correlated to field measurement techniques.

d)	Limiting Procedures for Specific Types of Data Acquisition

1)	For measurements of non-impulsive sound with audible discrete tones, 1/3 octave-band sound pressure levels must be obtained in determining whether a noise source is in compliance with 35 Ill. Adm. Code 901.106.

2)	For measurements of non-impulsive sound with no audible discrete tones, octave-band sound pressure levels must be obtained in determining whether a noise source is in compliance with 35 Ill. Adm. Code 901.102 and 901.103.

e)	Correction Factors

If necessary, correction factors rounded to the nearest 1/2 decibel must be applied to sound pressure level measurements.  The correction factors applicable to the measurement system may include, but are not limited to, corrections for windscreen interference and the sound pressure level difference between consecutive field calibrations.  Such calibration correction factors must only be used to make negative corrections (subtraction from the field data).  In no case must such calibration correction factors be added to the measured sound pressure levels so as to raise the sound pressure level field data.  The correction factors applicable to the measurement site may include, but are not limited to, corrections for reflective surfaces and ambient sound.

#### Section 910.106	Protocols for Determination of Sound Levels

a)	The raw data collection procedures for the determination of equivalent continuous sound pressure level (Leq) are described in this Section using as an example the determination of a 1-hour Leq corrected for ambient.  The following procedures must be used:

1)	Using small blocks:

A)	The 1-hour interval is divided into many small blocks of time so that corruption of the data from short-term background transient sound and loss of data can be limited to the corrupted or bad blocks.  The block duration in seconds must remain fixed for any measurement hour.  The duration must be neither less than 10 seconds nor greater than 100 seconds.  For example, if the block duration is chosen to be 60 seconds (1 minute), then the data collection proceeds for 60, 1-minute periods of measurement.

B)	The collected data for each block represents a block duration Leq (or sound exposure level (SEL)) in octave-bands (or 1/3 octave-bands if prominent discrete tones may be present).

C)	Data for any block corrupted by one or more short-term background transient sounds must be deleted.

D)	After deleting corrupted data blocks, there will be a fixed number of “good” data blocks remaining.  This number is designated as NPLNS, where PLNS stands for Property-Line-Noise-Source.  These remaining “good” blocks must be numbered consecutively.  The subscript i is used to denote the numbering of the blocks in time order after corrupted data blocks have been deleted.

  - The data for the NPLNS remaining blocks are time averaged on an energy basis by octave (or 1/3 octave-band) using Equation 1 below.  In this equation, two subscripts are used, i to designate time and j to designate the specific frequency, either an octave-band or 1/3 octave-band.  The raw, 1-hour Leq in the jth frequency band is given by:

Leqj=10log1NPLNSi=1NPLNS10Leqij10             [Equation 1]

where Leq is the Leq in the jth frequency band for the ith non-deleted data block.

F)	In terms of SEL, the raw SEL in the jth frequency band is given by:

SELj=10logi=1NPLNS10SELij10                    [Equation 2]

G)	The raw, 1-hour Leq in the jth frequency band is given in terms of the corresponding SELj by:

Leqj=SELj+10log3600NPLNSΔT              [Equation 3]

Where T is the block duration in seconds, NPLNS is the number of non-discarded data blocks, and 3600 is the number of seconds in an hour.

2)	Continuous Data Collection:

A)	The measuring instrument must be adjusted to continuously measure sound pressure and accumulate Leq for each block of time.  For convenience, the hour may be split into several smaller blocks such as 10, 6-minute blocks or 4, 15-minute blocks, etc.

B)	A switch on the measuring instrument must be available to inhibit data collection whenever a short-term background transient sound occurs.  This switch shall be used to prevent short-term background ambient sounds from corrupting the data.

C)	Data collection must proceed for one hour. The energy average of the several measured Leqij each weighted by the number of seconds actually accumulated during the ith block results in the raw, 1-hour Leq in each frequency band given by:

Leqj=10log1TPLNSi=1NPLNSTi10Leqij10           [Equation 4]

Where Leqij is the Leq in the jth frequency band for the ith large block. Ti is the actual number of seconds of “good” data accumulated in the ith block of time (e.g., 6 to 15 minutes); and

TPLNS=i=1NPLNSTi                                   [Equation 5]

3)	Minimum data collection requirements:

A)	Initial Measurement Duration.  The property-line-noise-source measurements must proceed initially for one hour.  Because of correction for short-term background transient sounds, actual reported data collection time T, in seconds, may be less than 3600 seconds (one hour).

i)	If small blocks of data are used for data collection, then the total measurement duration in seconds, TPLNS, is given by NPLNS T, where T is the length of each block in seconds and NPLNS is the number of non-discarded blocks.  If data inhibition is used for data collection, then TPLNS is the number of non-inhibited seconds during the measurement hour.  In either case, TPLNS must be no less than 900 seconds.

ii)	 If very few blocks were used for data collection, then the   duration of each block, T, may be too long and should be reduced.

iii)	For either data collection method, sounds considered to be short-term transient may actually be part of the long-term background ambient and should be so redefined.

B)	Extended Measurement Duration. If TPLNS is less than 900 seconds during the first hour of measurements, the raw data collection procedures must be appropriately modified and new measurements must proceed for an additional hour.  If TPLNS after combining the first and the second hour of measurements is also less than 900 seconds, then the raw data collection must continue using the data inhibition method or method employed during the second hour until TPLNS is greater than or equal to 900 seconds.

4)	Correction for Long-Term Background Ambient Sound:

A)	The raw 1-hour Leq must be corrected for long-term background ambient sound.  The subsection below describes methods to obtain the long-term background ambient sound level in the jth frequency band.  The correction is dependent on the difference (in decibels) between the raw, 1-hour, jth band property-line-noise-source:  Leqj and corresponding jth band long-term background ambient sound level.  The correction to be applied is as follows:

i)	If the difference between the raw 1-hour Leq and the long-term background ambient sound is larger than 10 decibels, then the correction must be set to 0.

ii)	If the difference between the raw 1-hour Leq and the long-term background ambient sound difference is less than 3 decibels, then the jth frequency-band level, Leqj, must be set equal to 0.

iii)	If the difference between the raw 1-hour Leq and the long-term background ambient sound is between 3 and 10 decibels, then the correction given in Table 1 below must be subtracted from the raw, 1-hour property-line-noise-source Leqj

###### Table 1

Corrections in dB for long-term

###### background ambient sound

|  | Difference | Correction |
| --- | --- | --- |
|  | (dB) | (dB) |
|  | 3 | 3 |
|  | 4 | 2.3 |
|  | 5 | 1.7 |
|  | 6 | 1.3 |
|  | 7 | 1.0 |
|  | 8 | 0.7 |
|  | 9 | 0.6 |
|  | 10 | 0.5 |

B)	The long-term background ambient corrected level must be the property-line-noise-source Leqj reported for the jth frequency band.

b)	Obtaining the background ambient sound level:

1)	The background ambient must be measured for the purposes of this Section during a 10-minute interval.

2)	Long-term background ambient measurement procedures are similar to procedures to measure the property-line-noise-source itself.  Eliminating short-term background ambient transient sounds from the measurement of average long-term background ambient sound proceeds in a manner similar to the measurement of the property-line-noise-source emissions themselves.  The two methods for measurement are:  to divide the 10-minute measurement into short blocks of data, or inhibit data collection when short-term background transient sounds occur.  The same method must be used for gathering both the property-line-noise-source data and the corresponding long-term background ambient data.  The measurement procedures for each method are given in subsections (b)(3), (b)(4) and (b)(5) of this Section:

3)	Using Small Blocks of Data

A)	The 10-minute measurement of long-term background ambient must be divided into short measurement blocks.  The duration of these blocks must remain constant during the entire measurement, both when measuring the long-term background ambient and when measuring the property-line-noise-source.  The duration of this measurement block in seconds, T, must divide exactly (without remainder) into 600 and must be neither greater than 100 seconds nor less than 10 seconds.

B)	All data for any measurement block corrupted by one or more short-term ambient transient sounds must be discarded.  The number of remaining, non-discarded measurement blocks is designated NBA, where BA stands for background ambient.

C)	The Leq for each octave-(or 1/3 octave-) band are time-averaged on an energy basis over the NBA remaining measurement blocks to obtain average long-term background ambient Leq per band.  Equation 1 (see subsection (a) (1) (E) of this Section) is used for this calculation with NBA replacing NPLNS as the number of elemental blocks to be summed.  The total duration of the measurement in seconds, TBA, is given by NBA multiplied by T.

4)	Continuous Data Collection

A)	The measuring instrument must be adjusted according to manufacturer’s instructions to continuously measure sound pressure and accumulate (i.e. record) Leq.  A switch must be available to inhibit data collection whenever a short-term background transient sound occurs, (and on some instruments, a button may be available to delete the most recent, previous data).

B)	The switches or buttons must be used to prevent short-term background ambient sounds from corrupting the data.

C)	Data collection must proceed for 10 minutes.  The result is the 10-minute, long-term background ambient Leq in each band.

D)	TBA is the number of non-inhibited measurement seconds during the 10-minute measurement period.

5)	The minimum duration, for either method, TBA  must be no less than 150 seconds.  If TBA is less than 150 seconds, then the measurement of the long-term background ambient must continue beyond the original 10 minutes and until TBA for the total long-term background ambient measurement is greater than or equal to 150 seconds.

6)	Measurement Alternatives.  The long-term background ambient noise should ideally be measured at the potential violation site just before measurement of the property-line-noise-source emissions.  However, turning off the property-line-noise-source may not always be possible.  The following are a hierarchical order of five procedures for obtaining the long-term background ambient noise.  The first four procedures involve direct measurement; the fifth procedure provides for use of tables of values obtained from extensive measurements.  These are not equivalent procedures but are ordered from what is considered to be the most accurate to what is considered to be the least accurate procedure.

A)	Direct Measurement Procedure –1:  With the property-line-noise-source (PLNS) turned off, measure the long-term background ambient noise within the hour before or within the hour after measurement of the PLNS emissions at the location where the PLNS measurements are being taken and with the measurement equipment used for the PLNS measurements.

B)	Direct Measurement Procedure-2:  With the PLNS turned off, measure the long-term background ambient during a similar time period in terms of background ambient sound level, within one to 24 hours before, or within one to 24 hours after measurement of the PLNS emissions at the location where the PLNS measurements are being taken and with the measurement equipment used for the PLNS.

C)	Direct Measurement Procedure- 3:  With the PLNS turned off, measure the long-term background ambient during some other acoustically similar period within one to 30 days before, or within one to 30 days after measurement of the PLNS emissions.  This alternate long-term background ambient measurement time might be a Saturday night or anytime during a Sunday or holiday.  The measurements would be made at the location where the PLNS measurements are being taken and with the measurement equipment (or like equipment) used for the PLNS measurement.

D)	Direct Measurement Procedure-4:  With the PLNS turned off, measure the long-term background ambient noise during some other acoustically similar period within 30 to 90 days before, or within 30 to 90 days after measurement of the PLNS emissions.  These measurements would be made at the location where the PLNS measurements are being taken and with the measurement equipment (or like equipment) used for the property-line-noise-source measurements.

E)	Tables of Long-Term Background Ambient Noise.  Where none of the alternatives can be used, use the applicable long-term background ambient data taken from Tables A through D in Appendix A of this Part.  These tables are organized by predominant land use and time of day (daytime or nighttime).  There are separate tables for octave- and 1/3- octave-bands.  The background environments presented in the table are based on extensive measurements conducted in the Chicago area and are divided into the five categories given below in accordance with G.L. Bonvallet, “Levels and Spectra of Traffic, Industrial, and Residential Area Noise,” Journal of the Acoustical Society of America, 23 (4), pp 435-439, July 1951; and Dwight E. Bishop and Paul D. Schomer, Handbook of Acoustical Measurements and Noise Control, Chapter 50, Community Noise Measurements, 3rd Edition, Cyril M Harris, Editor, McGraw-Hill Book Co., New York (1991).

i)	Category 1:  Noisy Commercial and Industrial Areas.  Very heavy traffic conditions, such as in busy downtown commercial areas, at intersections of mass transportation and other vehicles, including the Chicago Transit Authority trains, heavy motor trucks and other heavy traffic, and street corners where motor buses and heavy trucks accelerate.

ii)	Category 2:  Moderate Commercial and Industrial Areas, and Noisy Residential Areas.  Heavy traffic areas with conditions similar to subsection (b)(6)(E)(i) of this Section but with somewhat less traffic, routes of relatively heavy or fast automobile traffic but where heavy truck traffic is not extremely dense, and motor bus routes.

iii)	Category 3:  Quiet Commercial and Industrial Areas, and Moderate Residential Areas.  Light traffic conditions where no mass transportation vehicles and relatively few automobiles and trucks pass, and where these vehicles generally travel at low speeds.  Residential areas and commercial streets and intersections with little traffic comprise this category.

iv)	Category 4:  Quiet Residential Areas.  These areas are similar to Category 3 in subsection (b)(6)(E)(iii) of this Section but, for this group, the background is either distant traffic or is unidentifiable.

v)	Category 5:  Very Quiet, Sparse Suburban or Rural Areas.  These areas are similar to Category 4 subsection (b)(6)(E)(iv) of this Section but are usually in unincorporated areas and, for this group, there are few if any near neighbors.

Section 910.107	Measurement Techniques for Highly-Impulsive Sound Under 35 Ill. Adm. Code 901.104.

a)	Measurement of highly-impulsive sound under 35 Ill. Adm. Code 901.104 can be made in two distinct and equally valid ways, namely the general method and the controlled test method.

b)	General Method:  The general method is to measure the 1-hour, A-weighted Leq (not the octave- or 1/3 octave-band levels) using essentially one of the two procedures described in Sections 910.105 and 910.106.

1)	The procedure using small blocks of time to collect data is as follows:

A)	The hour must be divided into small blocks and the A-weighted Leq must be measured for each of these small blocks of time.  Leq must be measured for the entire hour but data collection must be inhibited whenever a short-term background transient sound occurs.

B)	The duration of each block must be held constant during the hour.  This duration in seconds must divide exactly into 900 and must be neither greater than 100 seconds nor less than 10 seconds.

C)	The data for any block corrupted by one or more short-term background ambient sounds must be discarded.

2)	The continuous data collection procedure is as follows:

A)	Leq must be measured for the entire hour.

B)	Data collection must be inhibited whenever a short-term background transient sound occurs.

3)	Correction for the long-term background ambient must be accomplished using all of the other procedures and requirements enumerated in Sections 910.105 and 910.106.  These requirements must be complied with to determine an A-weighted, 1-hour, background-ambient-corrected Leq for the highly impulsive property-line-noise-source under study.

c)	Controlled Test Method:  For this method, the following procedures must be used:

1)	General Measurement Description

A)	The sound exposure per impulse from each separate individual impulsive source is measured.

B)	The total sound exposure per hour from each source is the sound exposure per event multiplied by the number of events per hour.

C)	The grand total sound exposure (SE) per hour is the sum of the sound exposures per hour from each of the separate individual sources.

D)	The reported SEL is obtained from the grand total sound exposure (SE) per hour using the following:

SEL = 10 log (SE) + 94			[Equation  7]

E)	The equivalent level, Leq corresponding to a SEL measured or predicted for one hour (3600 seconds) is given by:

Leq = SEL - 10 log (3600)			[Equation  8]

2)	Determination of sound exposure per event must be as follows:

A)	The sound exposure per event from each, separate, individual source must be determined by measuring the total A-weighted sound exposure for about 10 repetitions of this source.  This set of about 10 measurements may be performed continuously over a short period of time, or this set of measurements may be performed over a discontinuous set of measurement periods.  In either case, the total measurement duration must be less than 100 seconds.

B)	These separate, individual property-line-noise- source controlled measurements must be free of any short-term ambient sounds.  If any short-term background transient sounds occur during these measurements, then the measurement must be repeated until measurement data, free of any corrupting short-term background ambient sounds, are obtained.

C)	The total measured A-weighted sound exposure for this group of about 10 repetitions must be corrected for long-term background ambient by subtracting the A-weighted long-term background ambient sound exposure.  The sound exposure value subtracted must be the long-term A-weighted background ambient sound exposure per second multiplied by the number of seconds used to measure the several source repetitions.

D)	The reported Source:  A-weighted sound exposure per event must be the total corrected sound exposure divided by the number of source repetitions measured.

E)	The background ambient must be measured for a short time, at least 30 seconds as near in time to the source measurements as possible, but within ½ hour.  The total A-weighted long-term background ambient sound exposure per second is the total measured long-term background ambient sound exposure divided by the number of seconds of background ambient measurement.

F)	There must be no short-term background ambient sounds present during the measurement of the long-term background ambient.  If any short-term background transient sounds occur during these measurements, then the measurements must be repeated until long-term background ambient measurement data free of any corrupting short-term background ambient sound are obtained.

910.APPENDIX A Tables of Long-Term Background Ambient Noise

#### 910.TABLE A.  Daytime long-term background ambient Leq levels in decibels by land use categories and 1/3 octave-band level

|  | Background Category |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octave-Band Center Frequency (Hz) |  | 1 | 2 | 3 | 4 | 5 |  |
| 20 |  | 63 | 56 | 48 | 42 | 36 |  |
| 25 |  | 64 | 57 | 49 | 43 | 37 |  |
| 31 |  | 65 | 58 | 50 | 44 | 38 |  |
| 40 |  | 65 | 58 | 51 | 44 | 38 |  |
| 50 |  | 66 | 59 | 51 | 45 | 39 |  |
| 63 |  | 66 | 59 | 52 | 46 | 40 |  |
| 80 |  | 67 | 60 | 52 | 46 | 40 |  |
| 100 |  | 68 | 60 | 53 | 47 | 41 |  |
| 125 |  | 67 | 59 | 52 | 46 | 40 |  |
| 160 |  | 66 | 59 | 52 | 46 | 40 |  |
| 200 |  | 66 | 58 | 51 | 45 | 39 |  |
| 250 |  | 65 | 58 | 50 | 44 | 38 |  |
| 315 |  | 64 | 57 | 49 | 43 | 37 |  |
| 400 |  | 63 | 55 | 48 | 42 | 36 |  |
| 500 |  | 62 | 54 | 46 | 40 | 34 |  |
| 630 |  | 61 | 53 | 44 | 38 | 32 |  |
| 800 |  | 60 | 51 | 42 | 36 | 30 |  |
| 1000 |  | 58 | 49 | 40 | 34 | 28 |  |
| 1250 |  | 56 | 47 | 38 | 32 | 26 |  |
| 1600 |  | 54 | 45 | 36 | 30 | 24 |  |
| 2000 |  | 52 | 43 | 33 | 28 | 21 |  |
| 2500 |  | 50 | 41 | 30 | 25 | 19 |  |
| 3150 |  | 49 | 39 | 28 | 23 | 17 |  |
| 4000 |  | 48 | 37 | 25 | 20 | 15 |  |
| 5000 |  | 46 | 35 | 23 | 18 | 13 |  |
| 6300 |  | 44 | 33 | 21 | 16 | 10 |  |
| 8000 |  | 43 | 31 | 19 | 14 | 8 |  |
| 10,000 |  | 41 | 29 | 17 | 12 | 6 |  |
| 12,500 |  | 39 | 27 | 15 | 10 | 4 |  |

910. APPENDIX A Tables of Long-Term Background Ambient Noise

910.TABLE B.  Nighttime long-term background ambient Leq levels in decibels by land use categories and 1/3 octave-band level

|  | Background Category |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octave-Band Center Frequency (Hz) |  | 1 | 2 | 3 | 4 | 5 |  |
| 20 |  | 53 | 48 | 43 | 37 | 31 |  |
| 25 |  | 54 | 49 | 44 | 38 | 32 |  |
| 31 |  | 55 | 50 | 45 | 39 | 33 |  |
| 40 |  | 55 | 50 | 46 | 39 | 33 |  |
| 50 |  | 56 | 51 | 46 | 40 | 34 |  |
| 63 |  | 56 | 51 | 47 | 41 | 35 |  |
| 80 |  | 57 | 52 | 47 | 41 | 35 |  |
| 100 |  | 58 | 52 | 48 | 42 | 36 |  |
| 125 |  | 57 | 51 | 47 | 41 | 35 |  |
| 160 |  | 56 | 51 | 47 | 41 | 35 |  |
| 200 |  | 56 | 50 | 46 | 40 | 34 |  |
| 250 |  | 55 | 50 | 45 | 39 | 33 |  |
| 315 |  | 54 | 49 | 44 | 38 | 32 |  |
| 400 |  | 53 | 47 | 43 | 37 | 31 |  |
| 500 |  | 52 | 46 | 41 | 35 | 29 |  |
| 630 |  | 51 | 45 | 39 | 33 | 27 |  |
| 800 |  | 50 | 43 | 37 | 31 | 25 |  |
| 1000 |  | 48 | 41 | 35 | 29 | 23 |  |
| 1250 |  | 46 | 39 | 33 | 27 | 21 |  |
| 1600 |  | 44 | 37 | 31 | 25 | 19 |  |
| 2000 |  | 42 | 35 | 28 | 23 | 16 |  |
| 2500 |  | 40 | 33 | 25 | 20 | 14 |  |
| 3150 |  | 39 | 31 | 23 | 18 | 12 |  |
| 4000 |  | 38 | 29 | 20 | 15 | 10 |  |
| 5000 |  | 36 | 27 | 18 | 13 | 8 |  |
| 6300 |  | 34 | 25 | 16 | 11 | 5 |  |
| 8000 |  | 33 | 23 | 14 | 9 | 3 |  |
| 10,000 |  | 31 | 21 | 12 | 7 | 1 |  |
| 12,500 |  | 29 | 19 | 10 | 2 |  |  |

910. APPENDIX A Tables of Long-Term Background Ambient Noise

910.TABLE C.  Daytime long-term background ambient Leq levels in decibels by land use categories and octave-band level

|  | Background Category |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octave-Band Center Frequency (Hz) |  | 1 | 2 | 3 | 4 | 5 |  |
| 31 |  | 70 | 63 | 55 | 49 | 43 |  |
| 63 |  | 71 | 64 | 57 | 51 | 45 |  |
| 125 |  | 72 | 64 | 57 | 51 | 45 |  |
| 250 |  | 70 | 63 | 55 | 49 | 43 |  |
| 500 |  | 67 | 59 | 51 | 45 | 39 |  |
| 1000 |  | 63 | 54 | 45 | 39 | 33 |  |
| 2000 |  | 57 | 48 | 38 | 33 | 26 |  |
| 4000 |  | 53 | 42 | 30 | 25 | 20 |  |
| 8000 |  | 48 | 36 | 24 | 19 | 13 |  |

910. APPENDIX A Tables of Long-term Background Ambient Noise

910.TABLE D.  Nighttime long-term background ambient Leq levels in decibels by land use categories and octave-band level

|  | Background Category |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octave-Band Center Frequency (Hz) |  | 1 | 2 | 3 | 4 | 5 |  |
| 31 |  | 60 | 55 | 50 | 44 | 38 |  |
| 63 |  | 61 | 56 | 52 | 46 | 40 |  |
| 125 |  | 62 | 56 | 52 | 46 | 40 |  |
| 250 |  | 60 | 55 | 50 | 44 | 38 |  |
| 500 |  | 57 | 51 | 46 | 40 | 34 |  |
| 1000 |  | 53 | 46 | 40 | 34 | 28 |  |
| 2000 |  | 47 | 40 | 33 | 28 | 21 |  |
| 4000 |  | 43 | 34 | 25 | 20 | 15 |  |
| 8000 |  | 38 | 28 | 19 | 14 | 8 |  |

IT IS SO ORDERED.

I, Dorothy M. Gunn, Clerk of the Illinois Pollution Control Board, certify that the Board adopted the above opinion and order on January 19, 2006, by a vote of 4-0.

Dorothy M. Gunn, Clerk

Illinois Pollution Control Board