TITLE 35:  ENVIRONMENTAL PROTECTION

SUBTITLE C:  WATER POLLUTION

CHAPTER I:  POLLUTION CONTROL BOARD

PART 302

WATER QUALITY STANDARDS

SUBPART A:  GENERAL WATER QUALITY PROVISIONS

Section

| 302.100 | Definitions |
| --- | --- |
| 302.101 | Scope and Applicability |
| 302.102 | Allowed Mixing, Mixing Zones and ZIDS |
| 302.103 | Stream Flows |
| 302.104 | Main River Temperatures |
| 302.105 | Nondegradation |

SUBPART B:  GENERAL USE WATER QUALITY STANDARDS

Section

| 302.201 | Scope and Applicability |
| --- | --- |
| 302.202 | Purpose |
| 302.203 | Offensive Conditions |
| 302.204 | pH |
| 302.205 | Phosphorus |
| 302.206 | Dissolved Oxygen |
| 302.207 | Radioactivity |
| 302.208 | Numeric Standards for Chemical Constituents |
| 302.209 | Fecal Coliform |
| 302.210 | Other Toxic Substances |
| 302.211 | Temperature |
| 302.212 | Ammonia Nitrogen and Un-ionized Ammonia |
| 302.213 | Effluent Modified Waters (Ammonia) |

SUBPART C:  PUBLIC AND FOOD PROCESSING WATER SUPPLY STANDARDS

Section

| 302.301 | Scope and Applicability |
| --- | --- |
| 302.302 | Algicide Permits |
| 302.303 | Finished Water Standards |
| 302.304 | Chemical Constituents |
| 302.305 | Other Contaminants |
| 302.306 | Fecal Coliform |

SUBPART D:  SECONDARY CONTACT AND INDIGENOUS AQUATIC LIFE STANDARDS

Section

| 302.401 | Scope and Applicability |
| --- | --- |
| 302.402 | Purpose |
| 302.403 | Unnatural Sludge |
| 302.404 | pH |
| 302.405 | Dissolved Oxygen |
| 302.406 | Fecal Coliform (Repealed) |
| 302.407 | Chemical Constituents |
| 302.408 | Temperature |
| 302.409 | Cyanide |
| 302.410 | Substances Toxic to Aquatic Life |

SUBPART E:  LAKE MICHIGAN BASIN WATER QUALITY STANDARDS

Section

| 302.501 | Scope, Applicability, and Definitions |
| --- | --- |
| 302.502 | Dissolved Oxygen |
| 302.503 | pH |
| 302.504 | Chemical Constituents |
| 302.505 | Fecal Coliform |
| 302.506 | Temperature |
| 302.507 | Thermal Standards for Existing Sources on January 1, 1971 |
| 302.508 | Thermal Standards for Sources Under Construction But Not in Operation on January 1, 1971 |
| 302.509 | Other Sources |
| 302.510 | Incorporations by Reference |
| 302.515 | Offensive Conditions |
| 302.520 | Regulation and Designation of Bioaccumulative Chemicals of Concern (BCCs) |
| 302.521 | Supplemental Antidegradation Provisions for BCCs |
| 302.525 | Radioactivity |
| 302.530 | Supplemental Mixing Provisions for Bioaccumulative Chemicals of Concern (BCCs) |
| 302.535 | Ammonia Nitrogen |
| 302.540 | Other Toxic Substances |
| 302.545 | Data Requirements |
| 302.550 | Analytical Testing |
| 302.553 | Determining the Lake Michigan Aquatic Toxicity Criteria or Values - General Procedures |
| 302.555 | Determining the Tier I Lake Michigan Acute Aquatic Toxicity Criterion (LMAATC): Independent of Water Chemistry |
| 302.560 | Determining the Tier I Lake Michigan Basin Acute Aquatic Life Toxicity Criterion (LMAATC): Dependent on Water Chemistry |
| 302.563 | Determining the Tier II Lake Michigan Basin Acute Aquatic Life Toxicity Value (LMAATV) |
| 302.565 | Determining the Lake Michigan Basin Chronic Aquatic Life Toxicity Criterion (LMCATC) or the Lake Michigan Basin Chronic Aquatic Life Toxicity Value (LMCATV) |
| 302.570 | Procedures for Deriving Bioaccumulation Factors for the Lake Michigan Basin |
| 302.575 | Procedures for Deriving Tier I Water Quality Criteria in the Lake Michigan Basin to Protect Wildlife |
| 302.580 | Procedures for Deriving Water Quality Criteria and Values in the Lake Michigan Basin to Protect Human Health - General |
| 302.585 | Procedures for Determining the Lake Michigan Basin Human Health Threshold Criterion (LMHHTC) and the Lake Michigan Basin Human Health Threshold Value (LMHHTV) |
| 302.590 | Procedures for Determining the Lake Michigan Basin Human Health Nonthreshold Criterion (LMHHNC) or the Lake Michigan Basin Human Health Nonthreshold Value (LMHHNV) |
| 302.595 | Listing of Bioaccumulative Chemicals of Concern, Derived Criteria and Values |

SUBPART F:  PROCEDURES FOR DETERMINING WATER QUALITY CRITERIA

Section

| 302.601 | Scope and Applicability |
| --- | --- |
| 302.603 | Definitions |
| 302.604 | Mathematical Abbreviations |
| 302.606 | Data Requirements |
| 302.612 | Determining the Acute Aquatic Toxicity Criterion for an Individual Substance - General Procedures |
| 302.615 | Determining the Acute Aquatic Toxicity Criterion - Toxicity Independent of Water Chemistry |
| 302.618 | Determining the Acute Aquatic Toxicity Criterion - Toxicity Dependent on Water Chemistry |
| 302.621 | Determining the Acute Aquatic Toxicity Criterion - Procedures for Combinations of Substances |
| 302.627 | Determining the Chronic Aquatic Toxicity Criterion for an Individual Substance - General Procedures |
| 302.630 | Determining the Chronic Aquatic Toxicity Criterion - Procedure for Combination of Substances |
| 302.633 | The Wild and Domestic Animal Protection Criterion |
| 302.642 | The Human Threshold Criterion |
| 302.645 | Determining the Acceptable Daily Intake |
| 302.648 | Determining the Human Threshold Criterion |
| 302.651 | The Human Nonthreshold Criterion |
| 302.654 | Determining the Risk Associated Intake |
| 302.657 | Determining the Human Nonthreshold Criterion |
| 302.658 | Stream Flow for Application of Human Nonthreshold Criterion |
| 302.660 | Bioconcentration Factor |
| 302.663 | Determination of Bioconcentration Factor |
| 302.666 | Utilizing the Bioconcentration Factor |
| 302.669 | Listing of Derived Criteria |

| APPENDIX A | References to Previous Rules |
| --- | --- |
| APPENDIX B | Sources of Codified Sections |

AUTHORITY:  Implementing Section 13 and authorized by Sections 11(b) and 27 of the Environmental Protection Act [415 ILCS 5/13, 11(b), and 27]

SOURCE:  Filed with the Secretary of State January 1, 1978; amended at 2 Ill. Reg. 44, p. 151, effective November 2, 1978; amended at 3 Ill. Reg. 20, p. 95, effective May 17, 1979; amended at 3 Ill. Reg. 25, p. 190, effective June 21, 1979; codified at 6 Ill. Reg. 7818; amended at 6 Ill. Reg. 11161, effective September 7, 1982; amended at 6 Ill. Reg. 13750, effective October 26, 1982; amended at 8 Ill. Reg. 1629, effective January 18, 1984; peremptory amendments at 10 Ill. Reg. 461, effective December 23, 1985; amended at R87-27 at 12 Ill. Reg. 9911, effective May 27, 1988; amended at R85-29 at 12 Ill. Reg. 12082, effective July 11, 1988; amended in R88-1 at 13 Ill. Reg. 5998, effective April 18, 1989; amended in R88-21(A) at 14 Ill. Reg. 2899, effective February 13, 1990; amended in R88-21(B) at 14 Ill. Reg. 11974, effective July 9, 1990; amended in R94-1(A) at 20 Ill. Reg. 7682, effective May 24, 1996; amended in R94-1(B) at 20 Ill. Reg. 370, effective December 23, 1996; expedited correction at 20 Ill. Reg. 6273, effective December 23, 1996; amended in R97-25 at 21 Ill. Reg. 1356, effective December 24, 1997; amended in R99-8 at 23 Ill. Reg. 11249, effective August 26, 1999; amended in __________ at __________ Ill. Reg. _______________, effective ________________, 2002.

### SUBPART B:  GENERAL USE WATER QUALITY STANDARDS

#### Section 302.208	Numeric Standards for Chemical Constituents

a)	The acute standard (AS) for the chemical constituents listed in subsection (e) shall not be exceeded at any time except as provided in subsection (d).

b)	The chronic standard (CS) for the chemical constituents listed in subsection (e) shall not be exceeded by the arithmetic average of at least four consecutive samples collected over any period of at least four days, except as provided in subsection (d).  The samples used to demonstrate compliance or lack of compliance with a CS must be collected in a manner which assures an average representative of the sampling period.

c)	The human health standard (HHS) for the chemical constituents listed in subsection (f) shall not be exceeded when the stream flow is at or above the harmonic mean flow pursuant to Section 302.658 nor shall an annual average, based on at least eight samples, collected in a manner representative of the sampling period, exceed the HHS except as provided in subsection (d).

d)	In waters where mixing is allowed pursuant to Section 302.102, the following apply:

1)	The AS shall not be exceeded in any waters except for those waters for which the Agency has approved a ZID pursuant to Section 302.102.

2)	The CS shall not be exceeded outside of waters in which mixing is allowed pursuant to Section 302.102.

3)	The HHS shall not be exceeded outside of waters in which mixing is allowed pursuant to Section 302.102.

e)	Numeric Water Quality Standards for the Protection of Aquatic Organisms

| Constituent | Storet Number | AS (µg/L) (ug/L) | CS (µg/L) (ug/L) |
| --- | --- | --- | --- |
| Arsenic (trivalent, dissolved) (total) | 22680 01002 | 360 X 1.0* = 360 | 190 X 1.0* = 190 |
| Cadmium (dissolved) (total) | 01025 01027 | exp[A+Bln(H)] X {1.138672-[(lnH)(0.041838)]}*, but not to exceed 50 ug/L, where A=-2.918 and B=1.128 | exp[A+Bln(H)] X {1.101672-[(lnH)(0.041838)]}*, where A=-3.490 and B=0.7852 |
| Chromium (hexavalent, total) (total hexavalent) | 01032 | 16 | 11 |
| Chromium (trivalent, dissolved) (total trivalent) | 80357 01033 | exp[A+Bln(H)] X 0.316* where A=3.688, and B=0.8190 | exp[A+Bln(H)] X 0.860* where A=1.561, and B=0.8190 |
| Copper (dissolved) (total) | 01040 01042 | exp[A+Bln(H)] X 0.960* where A=-1.464, and B=0.9422 | exp[A+Bln(H)] X 0.960* where A=-1.465, and B=0.8545 |
| Cyanide (weak acid dissociable) | 00718 | 49 22 | 9.9 5.2 |
| Lead (dissolved) (total) | 01049 01051 | exp[A+Bln(H)] X {1.46203-[(lnH)(0.145712)]}* where A=-1.301, and B=1.273 | exp[A+Bln(H)] X {1.46203-[(lnH)(0.145712)]}* where A=-2.863, and B=1.273 |
| Mercury (dissolved) | 71890 71900 | 2.6 X 0.85* = 2.2 | 1.3 X 0.85* = 1.1 |
| Nickel (dissolved) | 01065 | exp[A+Bln(H)] X 0.998* where A=0.5173, and B=0.8460 | exp[A+Bln(H)] X 0.997* where A=-2.286, and B=0.8460 |
| TRC | 500600 | 19 | 11 |
| Zinc (dissolved) | 01090 | exp[A+Bln(H)] X 0.978* where A=0.8875, and B=0.8473 | exp[A+Bln(H)] X 0.986* where A=-0.8227, and B=0.8473 |
| Benzene | 78124 | 4200 | 860 |
| Ethylbenzene | 78113 | 150 | 14 |
| Toluene | 78131 | 2000 | 600 |
| Xylene(s) | 81551 | 920 | 360 |

where: µg/L ug/L  =  microgram per liter,

exp[x] = base natural neutral logarithms raised to the x- power, and

ln(H) = natural logarithm of Hardness (STORET 00900).

* = conversion factor multiplier for dissolved metals

f)	Numeric Water Quality Standard for the Protection of Human Health

| Constituent | STORET Number | (µg/L) (ug/L) |
| --- | --- | --- |
| Mercury | 71900 | 0.012 |
| Benzene | 78124 | 310 |

Where µg/L ug/L = micrograms per liter

g)	Concentrations of the following chemical constituents shall not be exceeded except in waters for which mixing is allowed pursuant to Section 302.102.

| Constituent | Unit | STORET Number | Standard |
| --- | --- | --- | --- |
| Barium (total) | mg/L | 01007 | 5.0 |
| Boron (total) | mg/L | 01022 | 1.0 |
| Chloride (total) | mg/L | 00940 | 500. |
| Fluoride | mg/L | 00951 | 1.4 |
| Iron (dissolved) | mg/L | 01046 | 1.0 |
| Manganese (total) | mg/L | 01055 | 1.0 |
| Nickel (total) | mg/L | 01067 | 1.0 |
| Phenols | mg/L | 32730 | 0.1 |
| Selenium (total) | mg/L | 01147 | 1.0 |
| Silver (total) | µg/L ug/L | 01077 | 5.0 |
| Sulfate | mg/L | 00945 | 500. |
| Total Dissolved Solids | mg/L | 70300 | 1000. |
| Zinc (total) | mg/L | 01092 | 1.0 |

where: mg/L = milligram per liter and

µg/L ug/L = microgram per liter

(Source:  Amended at 20 Ill. Reg.7682, effective May 24, 1996; amended in __________ at __________ Ill. Reg. _______________, effective ________________, 2002)

SUBPART E:  LAKE MICHIGAN BASIN WATER QUALITY STANDARDS

#### Section 302.504	Chemical Constituents

The following concentrations of chemical constituents must not be exceeded, except as provided in Sections 302.102 and 302.530:

a)	The following standards must be met in all waters of the Lake Michigan Basin.  Acute aquatic life standards (AS) must not be exceeded at any time except for those waters for which the Agency has approved a zone of initial dilution (ZID) pursuant to Sections 302.102 and 302.530.  Chronic aquatic life standards (CS) and human health standards (HHS) must not be exceeded outside of waters in which mixing is allowed pursuant to Section 302.102 and 302.530 by the arithmetic average of at least four consecutive samples collected over a period of at least four days.  The samples used to demonstrate compliance with the CS or HHS must be collected in a manner which assures an average representation of the sampling period.

| Constituent | STORET Number | Unit | AS | CS | HHS |
| --- | --- | --- | --- | --- | --- |
| Arsenic (Trivalent, dissolved) | 22680 | mg/L | 340 X 1.0* = 340 | 148 X 1.0* = 148 | NA |
| Cadmium (dissolved) | 01025 | mg/L | exp[A +Bln(H)] X {1.38672-[(lnH)(0.041838)]}* A=-3.6867, and B=1.128 | exp[A +Bln(H)] X {1.101672-[(lnH)(0.041838)]}* A = -2.715, and B = 0.7852 | NA |
| Chromium (Hexavalent, total) | 01032 | mg/L | 16 | 11 | NA |
| Chromium (Trivalent, dissolved) | 80357 | mg/L | exp[A +Bln(H)] X 0.316* where A = 3.7256, and B =0.819 | exp[A +Bln(H)] X 0.860* where A = 0.6848, and B = 0.819 | NA |
| Copper (dissolved) | 01040 | mg/L | exp[A +Bln(H)] X 0.960* where A = -1.700, and B = 0.9422 | exp[A +Bln(H)] X 0.960* where A = -1.702, and B = 0.8545 | NA |
| Cyanide (Weak acid dissociable) | 00718 | mg/L | 22 | 5.2 | NA |
| Lead (dissolved) | 01049 | mg/L | exp[A +Bln(H)] X {1.46203-[(lnH)(0.145712)]}* where A = -1.055, and B = 1.273 | exp[A +Bln(H)] X {1.46203-[(lnH)(0.145712)]}* where A = -4.003, and B = 1.273 | NA |
| Nickel (dissolved) | 01065 | mg/L | exp[A +Bln(H)] X 0.998* where A = 2.255, and B = 0.846 | exp[A +Bln(H)] X 0.997* where A = 0.0584, and B = 0.846 | NA |
| Selenium (dissolved) | 01145 | mg/L | NA | 5.0 | NA |
| TRC | 50060 | mg/L | 19 | 11 | NA |
| Zinc (dissolved) | 01090 | mg/L | exp[A +Bln(H)] X 0.978* where A = 0.884, and B = 0.8473 | exp[A +Bln(H)] X 0.986* where A = 0.884, and B = 0.8473 | NA |
| Benzene | 78124 34030 | mg/L | 3900 NA | 800 NA | 310 |
| Chlorobenzene | 34301 | mg/L | NA | NA | 3.2 |
| 2,4-Dimethylphenol | 34606 | mg/L | NA | NA | 8.7 |
| 2,4-Dinitrophenol | 03756 | mg/L | NA | NA | 2.8 |
| Endrin | 39390 | mg/L | 0.086 | 0.036 | NA |
| Ethylbenzene | 78113 | µg/L | 150 | 14 | NA |
| Hexachloroethane | 34396 | mg/L | NA | NA | 6.7 |
| Methylene chloride | 34423 | mg/L | NA | NA | 2.6 |
| Parathion | 39540 | mg/L | 0.065 | 0.013 | NA |
| Pentachlorophenol | 03761 | mg/L | exp B ([pH] +A) where A = -4.869, and B = 1.005 | exp B ([pH] +A) where A = -5.134, and B = 1.005 | NA |
| Toluene | 78131 | mg/L | 2000 NA | 610 NA | 51.0 |
| Tricholroethylene | 39180 | mg/L | NA | NA | 370 |
| Xylene(s) | 81551 | mg/L | 1200 | 490 | NA |

Where:

NA = Not Applied

Exp[x] = base of natural logarithms

raised to the x-power

ln(H) = natural logarithm of Hardness

(STORET 00900)

* = conversion factor multiplier for dissolved metals

(Source:  Amended at 21 Ill. Reg. 1356, effective December 24, 1997, amended in __________ at __________ Ill. Reg. _______________, effective ________________, 2002)

#### Section 302.575	Procedures for Deriving Tier I Water Quality Criteria and Values in the Lake Michigan Basin to Protect Wildlife

The Lake Michigan Basin Wildlife Criterion (LMWC) is the concentration of a substance which if not exceeded protects Illinois wild mammal and bird populations from adverse effects resulting from ingestion of surface waters of the Lake Michigan Basin and from ingestion of aquatic prey organisms taken from surface waters of the Lake Michigan Basin.  Wildlife criteria calculated under this Section protect against long-term effects and are therefore considered chronic criteria.  The methodology involves utilization of data from test animals to derive criteria to protect representative or target species: bald eagle, herring gull, belted kingfisher, mink and river otter.  The lower of the geometric mean of species specific criteria for bird species or mammal species is chosen as the LMWC to protect a broad range of species.

d)	Calculation of TSV.  The TSV, measured in milligrams per liter (mg/L), is calculated according to the equation:

TSV = { [TD x Wt] / [UFa x UFs x UFl] }  /  { W +    S[FTLi x BAFWLTLi] }

Where:

TSV = target species value in milligrams of substance per liter (mg/L).

TD = test dose that is toxic to the test species, either NOAEL or LOAEL.

UFa = the uncertainty factor for extrapolating toxicity data across species (unitless).  A species-specific UFa shall be selected and applied to each target species, consistent with the equation

UFs = the uncertainty factor for extrapolating from subchronic to chronic exposures (unitless)

UFl = the uncertainty factor for extrapolation from LOAEL to NOAEL (unitless)

Wt = average weight in kilograms (kg) of the target species

W = average daily volume of water in liters consumed per day (L/d) by the target species

FTLi = average daily amount of food consumed by the target species in kilograms (kg/d) for trophic level i

BAFWLTLi = aquatic life bioaccumulation factor with units of liter per kilogram (L/kg), as derived in Section 302.570 for trophic level

e)	Calculation of the Lake Michigan Basin Wildlife Criterion.  TSVs are obtained for each target species.  The geometric mean TSVs of all mammal species is calculated and also of all bird species.  The LMWC is the lower of the bird or mammal geometric mean TSV.

(Source:  Added at 21 Ill. Reg. 1356, effective December 24, 1997, amended in __________ at __________ Ill. Reg. _______________, effective ________________, 2002)