- ILLINOIS POLLUTION CONTROL BOARD

March 1, 2007

| IN THE MATTER OF: ORGANIC MATERIAL EMISSION STANDARDS AND LIMITATIONS FOR THE CHICAGO AND METRO-EAST AREAS: PROPOSED AMENDMENTS TO 35 ILL. CODE 218 AND 219 | ) ) ) ) ) ) ) | R06-21 (Rulemaking – Air) |
| --- | --- | --- |

Proposed Rule.  Second Notice.

OPINION AND ORDER OF THE BOARD (by T.E. Johnson):

Today the Board adopts the proposed rule for second notice pursuant to the Illinois Administrative Procedure Act (5 ILCS 100/1-1 et seq. (2004)).  This rulemaking was filed by the Illinois Environmental Protection Agency (Agency) on December 22, 2005.  The Agency proposes amendments to Parts 218 and 219 of the volatile organic material (VOM) rules (415 ILCS 5/218, 219 (2004)) to allow for the use of add-on controls as a compliance option for operations using cold cleaning solvent degreasing.  The proposed amendments affect cold cleaning degreasing operations located in the Chicago and Metro-East ozone nonattainment areas.

The Board adopted first notice rules on September 21, 2006.  The proposed amendments were published in the Illinois Register on October 6, 2006.  30 Ill. Reg. 40, at 15867, 15892  (Oct. 6, 2006).  The Board received no public comments during the 45-day first-notice public comment period, which ended November 26, 2006, and the proposal adopted here is substantively unchanged from that adopted in the Board’s first-notice opinion and order.  In this opinion, the Board provides the procedural history of this rulemaking, an overview of the proposed standards and limitations, and a discussion of the second-notice proposal.

Today the Board adopts this proposal for second-notice review by the Joint Committee on Administrative Rules (JCAR).

BACKGROUND

The Agency filed the proposal on December 22, 2005.  If adopted, the proposed amendments will amend two subsections of the requirements found in Parts 218.182 and 219.182 of the Environmental Protection Act (Act).  415 ILCS 5/218.182, 219.182 (2004).  The amendments will also amend Appendix H of both Parts.

Solvent cleaning, or degreasing as it is commonly called, is a process using aqueous liquids or non-aqueous organic solvents to clean and remove soils from surfaces.  Proposal (Prop.) at 2.  Solvent cleaning is divided into three major types:  cold cleaning; open-top vapor degreasing; and conveyorized degreasing.  Id.  Evaporation of the solvent used during degreasing results in VOM emissions that react with other pollutants such as nitrogen oxides and carbon monoxide to form ozone.  Id.

In 1997, as a part of the 9% Rate of Progress (ROP) plan, the Board adopted control measures proposed by the Agency to reduce VOM emissions for cold cleaning degreaser operations in the Chicago and Metro-East ozone nonattainment areas.  Prop. at 3.  These measures, which are codified at 35 Ill. Adm. Code 218.182 and 219.182, lowered the allowed solvent vapor pressure for operations in the Chicago and Metro-East areas to 2.0 millimeters of mercury (mmHg) by 1999 and 1.0 mmHg by 2001.  Id.  The Agency estimated that the implementation of the lower vapor pressure would result in a VOM emission reduction of 11.35 tons per day by 1999 and an additional 11.68 tons per day by 2001.  The 1997 ROP regulations do not allow the use of add-on controls for cold cleaning degreasers as an alternative to complying with the solvent vapor limit.

Diversapack, a printing company that recycles its solvents in a totally enclosed parts washer and uses add-on controls for the abatement of process emissions, contacted the Agency in May 2003, regarding a possible variance from the vapor pressure limits at 35 Ill. Adm. Code 218.182(c).  Prop. at 2.  Diversapack uses solvents compatible with its printing operations, but which do not meet the current vapor pressure requirements.  Id.  The Agency identified three additional point sources in the Chicago area that use add-on controls plus solvents that do not meet the lower vapor pressure limits.  Id.

In lieu of the site-specific rulemakings for each of the identified facilities, the Agency is proposing revisions to 35 Ill. Adm. Code 218 and 219.  Prop. at 2.  Specifically, the Agency proposes revising Sections 218.182 and 219.182 to allow for add-on controls or equivalent alternative control plans as compliance options, as well as additional revisions concerning solvent sale, solvent use, add-on control testing, and recordkeeping requirements.  Id. at 3.

The Agency notes that currently an 8-hour ozone National Ambient Air Quality Standard (NAAQS) is in effect for the Chicago and the Metro-East areas, designating each as a moderate ozone nonattainment area.  Prop. at 3.  This 8-hour ozone NAAQS replaced the previous 1-hour standard on June 15, 2005.  However, given that the United States Environmental Protection Agency (USEPA) has a policy against regression, the control measures adopted to meet the 1-hour standard must still be maintained.  Id. at 4.  Further, under the federal Clean Air Act and the USEPA State Implementation Plan (SIP), failure to obtain equivalent emissions reductions from the cold cleaning sources would require the Agency to make up the deficiency through the implementation of contingency control measures.  Id. at 3-4.  According to the Agency, USEPA has indicated that the 95% control level proposed here is sufficient for this emissions source category.  Id. at 4.

Two hearings were held before Board Hearing Officer John Knittle.  The first hearing was held on April 19, 2006, in Chicago (Tr.1).  The second hearing was held on May 17, 2006, in Edwardsville (Tr.2).  During those hearings, the Board heard the testimony of Gary E. Beckstead from the Agency’s Bureau of Air.  In addition, the Board has received two public comments in this proceeding; the first from the Illinois Environmental Regulatory Group (IERG), filed in support of the proposal (PC 1) on June 14, 2006; the second from the Agency, also filed in support of the proposal (PC 2) on June 15, 2006.

Beckstead described the proposal generally, and testified that the proposed provisions for add-on controls would result in less VOM emissions than using solvents meeting the currently required vapor pressure limits.  Further, Beckstead stated that costs and inefficiencies would occur if the impacted sources were required to convert to the low-vapor pressure solvents.

In its public comment, IERG asserted that this proposed rulemaking rightly restores the option of add-on controls and adds the equivalent alternative control plan option, while remaining consistent with the intent of the 1997 revisions to reduce VOM emissions in the nonattainment areas.  PC 1 at 3.  IERG also opined that the compliance options provided by this rulemaking are justified historically, environmentally, technically, and economically.  Id. at 5.  The Agency stated in its public comment that, per the Board’s request during the first hearing, the Agency conducted an additional outreach effort towards retailers of solvent.  The effort resulted in one association requesting to be added to the Agency’s mailing list for future related proposed rulemakings.  PC 2 at 1.

As required by Section 27(b) of the Act (415 ILCS 5/27(b) (2004)), the Board requested that the Department of Commerce and Economic Opportunity (DCEO) conduct an economic impact study (EcIS) on this rulemaking.  The Board’s EcIS request, dated February 22, 2006, was placed in this rulemaking’s docket.  DCEO did not respond to the Board’s request.  At the second hearing, the hearing officer noted the Board’s EcIS request to DCEO and DCEO’s lack of response, affording anyone the opportunity to testify.  No one testified about DCEO’s lack of response.  Tr.2 at 5-6.

During the first-notice public comment period, JCAR submitted no corrections to be included in the Board’s second-notice proposal.

# SUMMARY OF PROPOSAL FOR SECOND NOTICE

The Agency’s proposal is unchanged since first notice.  The proposal, as amended by Agency errata sheets 1 (ES1) and 2 (ES2), revises the Board’s cold cleaning degreaser rules applicable to facilities in the Chicago and Metro-East ozone nonattainment areas.  The proposed revisions allow the use of add-on controls as an alternative to using solvents with vapor pressure of 1.0 mmHg or less.  Likewise, the proposal also allows the use of an equivalent alternative control plan to comply with the control measure requirements.  In addition, the proposal provides the testing procedures and recordkeeping requirements for add-on controls and equivalent alternative controls.

While the proposal discusses only the changes to Section 218.182 below, the proposal makes parallel changes to 35 Ill. Adm. Code 218.182(c) and (d) and 219.182(c) and (d).  Prop. at 5.  The Agency’s proposal seeks to add a new subsection at Section 218.182(c)(3)(A) that would allow the sale of solvents with vapor pressure greater than 1.0 mmHg in units greater than five gallons, but only if the purchaser provides a copy of a valid state or federal construction or operating permit or a copy of a Federal Register notice demonstrating that the source is in compliance with the add-on control requirements, or is exempt.  Id.  The proposed provision at Section 218.182(c)(3)(B) allows the operation of a cold cleaning degreaser using a solvent with a vapor pressure greater than 1.0 mmHg, but only if the source is in compliance with the add-on control requirements or is exempt.

The proposal sets forth control requirements for add-on controls at Section 218.182(c)(4).  Prop. at 5.  Subsection (c)(4)(A) requires sources using solvents with vapor pressures greater than 1.0 mmHg but less than 56 mmHg to control their emissions to an overall capture and control efficiency of no less than 95%.  Id. at 6; ES2.  The requirements for equivalent alternative emissions control plans will be set forth at Section 218.182(c)(4)(B).  These requirements stipulate that emissions from a solvent with a vapor pressure of 1.0 mmHg must be the basis for assessment of equivalent emissions for any proposed control plan.  Further, if used as an equivalent alternative control plan, an add-on control must have at least 95% reduction in VOM emissions.  Prop. at 7; ES2.

Provisions will also be added mandating the testing of add-on controls at Section 218.182(c)(4)(C).  Prop. at 7.  Under the proposal, currently operating add-on controls will have to be tested by March 1, 2007, and newly constructed add-on controls will have to be tested within 90 days after startup.  Id.  The testing procedures, monitoring, and recordkeeping provisions for add-on controls and equivalent alternative control plans will be consistent with the provisions in Section 218.105 and Section 219.105, which require that the control devices be operated and maintained at the manufacturer’s specifications and continuously monitored to assure that they are operating at the required compliance levels.  Finally, recordkeeping and reporting requirements applicable to the operation of add-on controls will be set forth under Section 218.182(d).  All records must be kept for a minimum of three years.  Id.

The Agency’s proposal, including its statement of reasons, is available through the Clerk’s Office in Chicago (312-814-3620) and on the Board’s Web site (www.ipcb.state.il.us) using the Clerk’s Office On-Line or “COOL.”

# DISCUSSION OF SECOND-NOTICE PROPOSAL

The Board has received no public comments during the first-notice public comment period. At second notice, no changes were received from JCAR and, therefore, discussion in addition to that contained in the first notice opinion and order is not required.

The Board agrees with the Agency’s assertion that the technical feasibility of add-on controls for point source cold cleaning can result in waste being minimized through solvent being reused.  The Board also agrees with the Agency that the proposal is economically reasonable and offers cost benefits comparable to converting to low vapor pressure solvents for some cold cleaning operations.  The Board agrees with the Agency that additional costs and inefficiencies would occur if the existing sources are required to use the low vapor pressure solvents.  The Board also maintains its proposed amendments to the “paper coating” note at Appendix H in Parts 218 and 219, which were inadvertently not made in consolidated rulemaking docket R04-12/20.  These changes will maintain wording consistency in conjunction with the changes proposed to Parts 218 and 219.

Additionally, the Board has made changes in the effective date of the rule.  The rule, as proposed by the Agency, provided that the requirements of Sections 218.182(c)(3) and 219.182(c)(3) would be effective after November 30, 2006.  In order to avoid a retroactive application of this rule, the Board has changed that date to May 30, 2007.  For consistency, a corresponding change was made to the added provisions mandating the testing of add-on controls at Section 218.182(c)(4)(C) and 219.182(c)(4)(C), and currently operating add-on controls will have to be tested by August 31, 2007, with newly constructed add-on controls will have to be tested within 90 days after startup.

##### CONCLUSION

The Board finds that the Agency’s proposed amendments are technically feasible and economically reasonable.  The Board adopts the proposal for second-notice review by JCAR. The 45-day second-notice public comment period will begin on the date written notice is received by JCAR and the Board will accept comments only from JCAR during the second-notice period.  See 35 Ill. Adm. Code 102.606.

### ORDER

The Board directs the Clerk to cause the filing of the following rule with the Joint Committee on Administrative Rules for its second-notice review.  Proposed additions to Parts 218 and 219 are underlined; proposed deletions appear stricken.

TITLE 35:  ENVIRONMENTAL PROTECTION

SUBTITLE B:  AIR POLLUTION

CHAPTER I:  POLLUTION CONTROL BOARD

SUBCHAPTER c:  EMISSIONS STANDARDS AND

LIMITATIONS FOR STATIONARY SOURCES

PART 218

ORGANIC MATERIAL EMISSION STANDARDS AND

LIMITATIONS FOR THE CHICAGO AREA

SUBPART A:  GENERAL PROVISIONS

Section

218.100	Introduction

218.101	Savings Clause

218.102	Abbreviations and Conversion Factors

218.103	Applicability

218.104	Definitions

218.105	Test Methods and Procedures

218.106	Compliance Dates

218.107	Operation of Afterburners

218.108	Exemptions, Variations, and Alternative Means of Control or Compliance Determinations

218.109	Vapor Pressure of Volatile Organic Liquids

218.110	Vapor Pressure of Organic Material or Solvent

218.111	Vapor Pressure of Volatile Organic Material

218.112	Incorporations by Reference

218.113	Monitoring for Negligibly-Reactive Compounds

218.114	Compliance with Permit Conditions

SUBPART B:  ORGANIC EMISSIONS FROM STORAGE AND LOADING OPERATIONS

Section

218.119	Applicability for VOL

218.120	Control Requirements for Storage Containers of VOL

218.121	Storage Containers of VPL

218.122	Loading Operations

218.123	Petroleum Liquid Storage Tanks

218.124	External Floating Roofs

218.125	Compliance Dates

218.126	Compliance Plan (Repealed)

218.127	Testing VOL Operations

218.128	Monitoring VOL Operations

218.129	Recordkeeping and Reporting for VOL Operations

SUBPART C:  ORGANIC EMISSIONS FROM MISCELLANEOUS EQUIPMENT

Section

218.141	Separation Operations

218.142	Pumps and Compressors

218.143	Vapor Blowdown

218.144	Safety Relief Valves

SUBPART E:  SOLVENT CLEANING

Section

218.181	Solvent Cleaning in General

218.182	Cold Cleaning

218.183	Open Top Vapor Degreasing

218.184	Conveyorized Degreasing

218.185	Compliance Schedule (Repealed)

218.186	Test Methods

SUBPART F:  COATING OPERATIONS

Section

218.204	Emission Limitations

218.205	Daily-Weighted Average Limitations

218.206	Solids Basis Calculation

218.207	Alternative Emission Limitations

218.208	Exemptions from Emission Limitations

218.209	Exemption from General Rule on Use of Organic Material

218.210	Compliance Schedule

218.211	Recordkeeping and Reporting

218.212	Cross-Line Averaging to Establish Compliance for Coating Lines

218.213	Recordkeeping and Reporting for Cross-Line Averaging Participating Coating Lines

218.214	Changing Compliance Methods

218.215	Wood Furniture Coating Averaging Approach

218.216	Wood Furniture Coating Add-On Control Use

218.217	Wood Furniture Coating Work Practice Standards

SUBPART G:  USE OF ORGANIC MATERIAL

Section

218.301	Use of Organic Material

218.302	Alternative Standard

218.303	Fuel Combustion Emission Units

218.304	Operations with Compliance Program

SUBPART H:  PRINTING AND PUBLISHING

Section

218.401	Flexographic and Rotogravure Printing

218.402	Applicability

218.403	Compliance Schedule

218.404	Recordkeeping and Reporting

218.405	Lithographic Printing:  Applicability

218.406	Provisions Applying to Heatset Web Offset Lithographic Printing Prior to March 15, 1996

218.407	Emission Limitations and Control Requirements for Lithographic Printing Lines On and After March 15, 1996

218.408	Compliance Schedule for Lithographic Printing On and After March 15, 1996

218.409	Testing for Lithographic Printing On and After March 15, 1996

218.410	Monitoring Requirements for Lithographic Printing

218.411	Recordkeeping and Reporting for Lithographic Printing

SUBPART Q:  SYNTHETIC ORGANIC CHEMICAL AND POLYMER MANUFACTURING PLANT

Section

218.421	General Requirements

218.422	Inspection Program Plan for Leaks

218.423	Inspection Program for Leaks

218.424	Repairing Leaks

218.425	Recordkeeping for Leaks

218.426	Report for Leaks

218.427	Alternative Program for Leaks

218.428	Open-Ended Valves

218.429	Standards for Control Devices

218.430	Compliance Date (Repealed)

218.431	Applicability

218.432	Control Requirements

218.433	Performance and Testing Requirements

218.434	Monitoring Requirements

218.435	Recordkeeping and Reporting Requirements

218.436	Compliance Date

SUBPART R:  PETROLEUM REFINING AND RELATED INDUSTRIES; ASPHALT MATERIALS

Section

218.441	Petroleum Refinery Waste Gas Disposal

218.442	Vacuum Producing Systems

218.443	Wastewater (Oil/Water) Separator

218.444	Process Unit Turnarounds

218.445	Leaks: General Requirements

218.446	Monitoring Program Plan for Leaks

218.447	Monitoring Program for Leaks

218.448	Recordkeeping for Leaks

218.449	Reporting for Leaks

218.450	Alternative Program for Leaks

218.451	Sealing Device Requirements

218.452	Compliance Schedule for Leaks

218.453	Compliance Dates (Repealed)

SUBPART S:  RUBBER AND MISCELLANEOUS PLASTIC PRODUCTS

Section

218.461	Manufacture of Pneumatic Rubber Tires

218.462	Green Tire Spraying Operations

218.463	Alternative Emission Reduction Systems

218.464	Emission Testing

218.465	Compliance Dates (Repealed)

218.466	Compliance Plan (Repealed)

SUBPART T:  PHARMACEUTICAL MANUFACTURING

Section

218.480	Applicability

218.481	Control of Reactors, Distillation Units, Crystallizers, Centrifuges and Vacuum Dryers

218.482	Control of Air Dryers, Production Equipment Exhaust Systems and Filters

218.483	Material Storage and Transfer

218.484	In-Process Tanks

218.485	Leaks

218.486	Other Emission Units

218.487	Testing

218.488	Monitoring for Air Pollution Control Equipment

218.489	Recordkeeping for Air Pollution Control Equipment

SUBPART V:  BATCH OPERATIONS AND AIR OXIDATION PROCESSES

Section

218.500	Applicability for Batch Operations

218.501	Control Requirements for Batch Operations

218.502	Determination of Uncontrolled Total Annual Mass Emissions and Average Flow Rate Values for Batch Operations

218.503	Performance and Testing Requirements for Batch Operations

218.504	Monitoring Requirements for Batch Operations

218.505	Reporting and Recordkeeping for Batch Operations

218.506	Compliance Date

218.520	Emission Limitations for Air Oxidation Processes

218.521	Definitions (Repealed)

218.522	Savings Clause

218.523	Compliance

218.524	Determination of Applicability

218.525	Emission Limitations for Air Oxidation Processes

218.526	Testing and Monitoring

218.527	Compliance Date (Repealed)

SUBPART W:  AGRICULTURE

Section

218.541	Pesticide Exception

SUBPART X:  CONSTRUCTION

Section

218.561	Architectural Coatings

218.562	Paving Operations

218.563	Cutback Asphalt

SUBPART Y:  GASOLINE DISTRIBUTION

Section

218.581	Bulk Gasoline Plants

218.582	Bulk Gasoline Terminals

218.583	Gasoline Dispensing Operations - Storage Tank Filling Operations

218.584	Gasoline Delivery Vessels

218.585	Gasoline Volatility Standards

218.586	Gasoline Dispensing Operations - Motor Vehicle Fueling Operations

SUBPART Z:  DRY CLEANERS

Section

218.601	Perchloroethylene Dry Cleaners (Repealed)

218.602	Applicability (Repealed)

218.603	Leaks (Repealed)

218.604	Compliance Dates (Repealed)

218.605	Compliance Plan (Repealed)

218.606	Exception to Compliance Plan (Repealed)

218.607	Standards for Petroleum Solvent Dry Cleaners

218.608	Operating Practices for Petroleum Solvent Dry Cleaners

218.609	Program for Inspection and Repair of Leaks

218.610	Testing and Monitoring

218.611	Applicability for Petroleum Solvent Dry Cleaners

218.612	Compliance Dates (Repealed)

218.613	Compliance Plan (Repealed)

SUBPART AA:  PAINT AND INK MANUFACTURING

Section

218.620	Applicability

218.621	Exemption for Waterbase Material and Heatset Offset Ink

218.623	Permit Conditions (Repealed)

218.624	Open Top Mills, Tanks, Vats or Vessels

218.625	Grinding Mills

218.626	Storage Tanks

218.628	Leaks

218.630	Clean Up

218.636	Compliance Schedule

218.637	Recordkeeping and Reporting

SUBPART BB:  POLYSTYRENE PLANTS

Section

218.640	Applicability

218.642	Emissions Limitation at Polystyrene Plants

218.644	Emissions Testing

SUBPART CC:  POLYESTER RESIN PRODUCT MANUFACTURING PROCESS

Section

218.660	Applicability

218.666	Control Requirements

218.667	Compliance Schedule

218.668	Testing

218.670	Recordkeeping and Reporting for Exempt Emission Units

218.672	Recordkeeping and Reporting for Subject Emission Units

SUBPART DD:  AEROSOL CAN FILLING

Section

218.680	Applicability

218.686	Control Requirements

218.688	Testing

218.690	Recordkeeping and Reporting for Exempt Emission Units

218.692	Recordkeeping and Reporting for Subject Emission Units

SUBPART FF:  BAKERY OVENS (REPEALED)

Section

218.720	Applicability (Repealed)

218.722	Control Requirements (Repealed)

218.726	Testing (Repealed)

218.727	Monitoring (Repealed)

218.728	Recordkeeping and Reporting (Repealed)

218.729	Compliance Date (Repealed)

218.730	Certification (Repealed)

SUBPART GG:  MARINE TERMINALS

Section

218.760	Applicability

218.762	Control Requirements

218.764	Compliance Certification

218.766	Leaks

218.768	Testing and Monitoring

218.770	Recordkeeping and Reporting

SUBPART HH:  MOTOR VEHICLE REFINISHING

Section

218.780	Emission Limitations

218.782	Alternative Control Requirements

218.784	Equipment Specifications

218.786	Surface Preparation Materials

218.787	Work Practices

218.788	Testing

218.789	Monitoring and Recordkeeping for Control Devices

218.790	General Recordkeeping and Reporting (Repealed)

218.791	Compliance Date

218.792	Registration

218.875	Applicability of Subpart BB (Renumbered)

218.877	Emissions Limitation at Polystyrene Plants (Renumbered)

218.879	Compliance Date (Repealed)

218.881	Compliance Plan (Repealed)

218.883	Special Requirements for Compliance Plan (Repealed)

218.886	Emissions Testing (Renumbered)

SUBPART PP:  MISCELLANEOUS FABRICATED PRODUCT MANUFACTURING PROCESSES

Section

218.920	Applicability

218.923	Permit Conditions (Repealed)

218.926	Control Requirements

218.927	Compliance Schedule

  - Testing

218.929	Cementable and Dress or Performance Shoe Leather

SUBPART QQ:  MISCELLANEOUS FORMULATION MANUFACTURING PROCESSES

Section

218.940	Applicability

218.943	Permit Conditions (Repealed)

218.946	Control Requirements

218.947	Compliance Schedule

218.948	Testing

SUBPART RR:  MISCELLANEOUS ORGANIC CHEMICAL MANUFACTURING PROCESSES

Section

218.960	Applicability

218.963	Permit Conditions (Repealed)

218.966	Control Requirements

218.967	Compliance Schedule

218.968	Testing

SUBPART TT:  OTHER EMISSION UNITS

Section

218.980	Applicability

218.983	Permit Conditions (Repealed)

218.986	Control Requirements

218.987	Compliance Schedule

218.988	Testing

SUBPART UU:  RECORDKEEPING AND REPORTING

Section

218.990	Exempt Emission Units

218.991	Subject Emission Units

218.APPENDIX A:	List of Chemicals Defining Synthetic Organic Chemical and Polymer Manufacturing

218.APPENDIX B:	VOM Measurement Techniques for Capture Efficiency (Repealed)

218.APPENDIX C:		Reference Methods and Procedures

218.APPENDIX D:	Coefficients for the Total Resource Effectiveness Index (TRE) Equation

218.APPENDIX E:		List of Affected Marine Terminals

218.APPENDIX G:	TRE Index Measurements for SOCMI Reactors and Distillation Units

218.APPENDIX H:	Baseline VOM Content Limitations for Subpart F, Section 218.212 Cross-Line Averaging

AUTHORITY:  Implementing Section 10 and authorized by Sections 27, 28, 28.5 of the Environmental Protection Act [415 ILCS 5/10 and 28.5].

SOURCE:  Adopted at R91-7 at 15 Ill. Reg. 12231, effective August 16, 1991; amended in R91-24 at 16 Ill. Reg. 13564, effective August 24, 1992; amended in R91-28 and R91-30 at 16 Ill. Reg. 13864, effective August 24, 1992; amended in R93-9 at 17 Ill. Reg. 16636, effective September 27, 1993; amended in R93-14 at 18 Ill. Reg. at 1945, effective January 24, 1994; amended in R94-12 at 18 Ill. Reg. 14973, effective September 21, 1994; amended in R94-15 at 18 Ill. Reg. 16392, effective October 25, 1994; amended in R94-16 at 18 Ill. Reg. 16950, effective November 15, 1994; amended in R94-21, R94-31 and R94-32 at 19 Ill. Reg. 6848, effective May 9, 1995; amended in R94-33 at 19 Ill. Reg. 7359, effective May 22, 1995; amended in R96-13 at 20 Ill. Reg. 14428, effective October 17, 1996; amended in R97-24 at 21 Ill. Reg. 7708, effective June 9, 1997;  amended in R97-31 at 22 Ill. Reg. 3556, effective February 2, 1998; amended in R98-16 at 22 Ill. Reg. 14282, effective July 16, 1998; amended in R02-20 at 27 Ill. Reg 7283, effective April 8, 2003; amended in R04-12/20 at 30 Ill. Reg. 9684, effective May 15, 2006; amended in R06-21 at 31 Ill. Reg. _________, effective ___________.

SUBPART E:  SOLVENT CLEANING

#### Section 218.182	Cold Cleaning

a)	Operating Procedures:  No person shall operate a cold cleaning degreaser unless:

1)	Waste solvent is stored in covered containers only and not disposed of in such a manner that more than 20% of the waste solvent (by weight) is allowed to evaporate into the atmosphere;

2)	The cover of the degreaser is closed when parts are not being handled; and

3)	Parts are drained until dripping ceases.

b)	Equipment Requirements:  No person shall operate a cold cleaning degreaser unless:

1)	The degreaser is equipped with a cover which is closed whenever parts are not being handled in the cleaner.  The cover shall be designed to be easily operated with one hand or with the mechanical assistance of springs, counter-weights or a powered system if:

A)	The solvent vapor pressure is greater than 2 kPa (15 mmHg or 0.3 psi) measured at 38oC (100oF);

B)	The solvent is agitated; or

C)	The solvent is heated above ambient room temperature.

2)	The degreaser is equipped with a device for draining cleaned parts. The drainage device shall be constructed so that parts are enclosed under the cover while draining unless:

A)	The solvent vapor pressure is less than 4.3 kPa (32 mmHg or 0.6 psi) measured at 38oC (100oF); or

B)	An internal drainage device cannot be fitted into the cleaning system, in which case the drainage device may be external.

3)	The degreaser is equipped with one of the following control devices if the vapor pressure of the solvent is greater than 4.3 kPa (32 mmHg or 0.6 psi) measured at 38oC (100oF) or if the solvent is heated above 50oC (120oF) or its boiling point:

A)	A freeboard height of 7/10 of the inside width of the tank or 91 cm (36 in), whichever is less; or

B)	Any other equipment or system of equivalent emission control as approved by the Agency and further processed consistent with Section 218.108 of this Part.  Such a system may include a water cover, refrigerated chiller or carbon adsorber.

4)	A permanent conspicuous label summarizing the operating procedure is affixed to the degreaser; and

5)	If a solvent spray is used, the degreaser is equipped with a solid fluid stream spray, rather than a fine, atomized or shower spray.

c)	Material and Control Requirements:

1)	On and after March 15, 1999, no person shall:

A)	Cause or allow the sale of solvent with a vapor pressure which exceeds 2.0 mmHg (0.038 psi) measured at 20o C (68o F) in units greater than five gallons, for use in cold cleaning degreasing operations located in the area covered by Section 218.103 of this Part.

B)	Operate a cold cleaning degreaser with a solvent vapor pressure which exceeds 2.0 mmHg (0.038 psi) measured at 20o C (68o F).

2)	On and after March 15, 2001, no person shall:

A)	Cause or allow the sale of solvent with a vapor pressure which exceeds 1.0 mmHg (0.019 psi) measured at 20o C (68o F) in units greater than five gallons, for use in cold cleaning degreasing operations located in the area covered by Section 218.103 of this Part.

B)	Operate a cold cleaning degreaser with a solvent vapor pressure which exceeds 1.0 mmHg (0.019 psi) measured at 20o C (68o F).

3)	On and after May 30, 2007, no person shall:

A)	Cause or allow the sale of solvent with a vapor pressure which exceeds 1.0 mmHg (0.019 psi) measured at 20° C (68° F) in units greater than five gallons, for use in cold cleaning degreasing operations located in the area covered by Section 218.103 of this Part, unless the purchaser provides a copy of a valid state or federal construction or operating permit or a copy of the Federal Register demonstrating that the purchaser is in compliance with the control requirements of subsection (c)(4) of this Section or is exempt under subsection (f) or (g) of this Section.

B)	Operate a cold cleaning degreaser with a solvent vapor pressure which exceeds 1.0 mmHg (0.019 psi) measured at 20° C (68° F), unless the person is in compliance with the control requirements of subsection (c)(4) of this Section or is exempt under subsection (f) or (g) of this Section.

4)	Control Requirements:

A)	A person may operate a cold cleaning degreaser using solvent with a vapor pressure greater than 1.0 mmHg (0.019 psi) but less than 56 mmHg (1.064 psi) measured at 20° C (68° F) provided add-on control devices demonstrating at least 95 percent overall capture and control of emissions are used.  The add-on controls may include, but are not limited to, carbon adsorbers or afterburners.

B)	An equivalent alternative control plan may be used to meet the control requirements of this Section pursuant to Section 218.108 of this Part.  Pursuant to the material requirements of subsection (c)(3)(B) of this Section, a solvent with a vapor pressure of 1.0 mmHg (0.019 psi) measured at 20° C (68° F) shall be the basis for assessment of equivalent emissions from any equivalent alternative control plan.  If used as an equivalent alternative control plan, an add-on control must demonstrate at least a 95 percent overall capture and control efficiency.  A control plan approved by the Agency shall be effective only when included in a federally enforceable permit or approved by the USEPA as a SIP revision pursuant to Section 218.108 of this Part.

C)	Add-on controls operating at a source prior to May 30, 2007, must be tested by August 31, 2007.  Add-on controls constructed on or after May 30, 2007, must be tested within 90 days of initial startup.  Testing procedures and recordkeeping for add-on controls and equivalent alternative controls subject to subsections (c)(4)(A) and (B) of this Section are to be performed pursuant to Section 218.105 (c), (d), (e) and (f) of this Part.

d)	Recordkeeping and Reporting Requirements: On and after March 15, 1999:

1)	All persons subject to the requirements of subsections (c)(1)(A), and (c)(2)(A), and (c)(3)(A) of this Section must maintain records which include for each sale:

A)	The name and address of the solvent purchaser;

B)	The date of sale;

C)	The type of solvent;

D)	The unit volume of solvent;

E)	The total volume of solvent; and

F)	The vapor pressure of the solvent measured in mmHg at 20o C (68o F).

2)	All persons subject to the requirements of subsections (c)(1)(B), and (c)(2)(B), and (c)(3)(B) of this Section must maintain records which include for each purchase:

A)	The name and address of the solvent supplier;

B)	The date of purchase;

C)	The type of solvent; and

D)	The vapor pressure of the solvent measured in mmHg at 20o C (68o F).; and

E)	For any mixture of solvents, the vapor pressure of the mixture, as used, measured in mmHg at 20° C (68° F).

3)	All persons subject to the requirements of subsection (c)(4) of this Section must maintain records, which include for each purchase:

A)	The name and address of the solvent supplier;

B)	The date of purchase;

C)	The type of solvent;

D)	The unit volume of solvent;

E)	The total volume of solvent;

F)	The vapor pressure of the solvent measured in mmHg at 20° C (68° F); and

G)	For any mixture of solvents, the vapor pressure of the mixture, as used, measured in mmHg at 20° C (68° F).

4)	All persons subject to the requirements of subsection (c)(4) of this Section shall maintain records documenting the use of good operating practices consistent with the equipment manufacturer’s specifications for the cold cleaning degreasers and add-on control equipment.  At a minimum these records shall include:

A)	Records for periodic inspection of the cold cleaning degreasers and add-on control equipment with date of inspection, individual performing the inspection, and nature of inspection;

B)	Records for repair of malfunctions and breakdowns with identification and description of incident, date identified, date repaired, nature of repair, and the amount of VOM that escaped into the atmosphere as a result of the incident;

C)	Control device monitoring and recording data; and

D)	A daily log of operating time for the control device, monitoring equipment, and all associated degreasers.

5)	All persons subject to the requirements of subsection (c) of this Section shall notify the Agency at least 30 days before changing the method of compliance between subsection (c)(3) and (c)(4) of this Section.  Such notification shall include a demonstration of compliance with the newly applicable subsection.

6)	All persons subject to the requirements of subsection (b) or (c) of this Section shall notify the Agency of any violation of subsection (b) or (c) of this Section by sending a description of the violation and copies of records documenting such violations to the Agency within 30 days following the occurrence of the violation.

e)	All records required by subsection (d) of this Section shall be retained for three years and shall be made available to the Agency upon request.

f)	The cleaning of electronic components as defined in 35 Ill. Adm. Code Section 211.1885 is exempt from the requirements of subsection(c) of this Section.

g)	Any cold cleaning taking place in a Detrex cold batch degreaser Model #2D-CC-SPL Size 24-4-10, or substantial equivalent, including automated loading of parts, totally enclosed operation (excluding loading or unloading) and permitted by the Agency, is exempt from the requirements of subsection (c) of this Section.

(Source:  Amended at 30 Ill. Reg. __________, effective _____________)

#### Section 218.APPENDIX H 	Baseline VOM Content Limitations for Subpart F, Section 218.212 Cross-Line Averaging

This Appendix contains limitations for purposes of determining compliance with the requirements in Section 218.212 of this Part.  A source must establish that, at very least, each participating coating line used for purposes of cross-line averaging meets the Federal Implementation Plan level of VOM content, as listed below.  The emission limitations for participating coating lines that must not be exceeded are as follows:

a)	Automobile or Light-Duty Truck Coating	kg/l	lb/gal

1)		Prime coat	0.14	(1.2)

2)		Primer surface coat	1.81	(15.1)

(Note: The primer surface coat limitation is in units of kg (lbs) of VOM per l (gal) of coating solids deposited.  Compliance with the limitation shall be based on the daily-weighted average from an entire primer surface operation.  Compliance shall be demonstrated in accordance with the topcoat protocol referenced in Section 218.105(b) and the recordkeeping and reporting requirements specified in Section 218.211(f).  Testing to demonstrate compliance shall be performed in accordance with the topcoat protocol and a detailed testing proposal approved by the Agency and USEPA specifying the method of demonstrating compliance with the protocol.  Section 218.205 does not apply to the primer surface limitation.)

kg/l	lb/gal

3)		Topcoat	1.81	(15.1)

(Note: The topcoat limitation is in units of kg (lbs) of VOM per l (gal) of coating solids deposited.  Compliance with the limitation shall be based on the daily-weighted average from an entire topcoat operation.  Compliance shall be demonstrated in accordance with the topcoat protocol referenced in Section 218.105(b) of this Part and the recordkeeping and reporting requirements specified in Section 218.211(f).  Testing to demonstrate compliance shall be performed in accordance with the topcoat protocol and a detailed testing proposal approved by the Agency and USEPA specifying the method of demonstrating compliance with the protocol.  Section 218.205 of this Part does not apply to the topcoat limitation.)

kg/l	lb/gal

4)		Final repair coat	0.58	(4.8)

b)	Can Coating	kg/l	lb/gal

1)	Sheet basecoat and overvarnish	0.34	(2.8)

2)	Exterior basecoat and overvarnish	0.34	(2.8)

3)	Interior body spray coat	0.51	(4.2)

4)	Exterior end coat	0.51	(4.2)

5)	Side seam spray coat	0.66	(5.5)

6)	End sealing compound coat	0.44	(3.7)

kg/l	lb/gal

c)	Paper Coating	0.35	(2.9)

(Note: The paper coating limitation shall not apply to any owner or operator of any paper coating line on which flexographic or rotogravure printing is performed if the paper coating line complies with the emissions limitations in Subpart H: Printing and Publishing, Section 218.401 of this Part.  In addition, screen printing on paper is not regulated as paper coating, but is regulated under Subpart TT of this Part.)

kg/l	lb/gal

d)	Coil Coating	0.31	(2.6)

e)	Fabric Coating	0.35	(2.9)

f)	Vinyl Coating	0.45	(3.8)

g)	Metal Furniture Coating

1)	Air Dried	0.36	(3.0)

2)	Baked	0.36	(3.0)

h)	Large Appliance Coating

1)	Air Dried	0.34	(2.8)

2)	Baked	0.34	(2.8)

(Note: The limitation shall not apply to the use of quick-drying lacquers for repair of scratches and nicks that occur during assembly, provided that the volume of coating does not exceed 0.95 l (1 quart) in any one rolling eight-hour period.)

kg/l	lb/gal

i)	Magnet Wire Coating	0.20	(1.7)

j)	Miscellaneous Metal Parts and

Products Coating

1)	Clear coating	0.52	(4.3)

2)	Extreme performance coating

A)	Air Dried	0.42	(3.5)

B)	Baked	0.42	(3.5)

3)	Steel pail and drum interior coating	0.52	(4.3)

4)	All other coatings

A)	Air Dried	0.42	(3.5)

B)	Baked	0.36	(3.0)

k)	Heavy Off-Highway Vehicle Products Coating	kg/l	lb/gal

1)	Extreme performance prime coat	0.42	(3.5)

2)	Extreme performance top- coat (air dried)	0.42	(3.5)

3)	Final repair coat (air dried)	0.42	(3.5)

4)	All other coatings are subject to the emission limitations for miscellaneous metal parts and products coatings in subsection (j) above.

l)	Wood Furniture Coating	kg/l	lb/gal

1)	Clear topcoat	0.67	(5.6)

2)	Opaque stain	0.56	(4.7)

3)	Pigmented coat	0.60	(5.0)

4)	Repair coat	0.67	(5.6)

5)	Sealer	0.67	(5.6)

6)	Semi-transparent stain	0.79	(6.6)

7)	Wash coat	0.73	(6.1)

(Note: An owner or operator of a wood furniture coating operation subject to this Section shall apply all coatings, with the exception of no more than 37.8 l (10 gal) of coating per day used for touch-up and repair operations, using one or more of the following application systems: airless spray application system, air-assisted airless spray application system, electrostatic spray application system, electrostatic bell or disc spray application system, heated airless spray application system, roller coating, brush or wipe coating application system, dip coating application system or high volume low pressure (HVLP) application system.)

m)	Existing Diesel-Electric Locomotive Coating Lines

in Cook County	kg/l	lb/gal

1)	Extreme performance prime coat	0.42	(3.5)

2)	Extreme performance topcoat

(air dried)	0.42	(3.5)

3)	Final repair coat (air dried)	0.42	(3.5)

4)	High-temperature aluminum coating	0.72	(6.0)

5)	All other coatings	0.36	(3.0)

n)	Plastic Parts Coating:

Automotive/Transportation	kg/l	lb/gal

1)	Interiors

A)	Baked

i)	Color Coat	0.49	(4.1)

ii)	Primer	0.46	(3.8)

B)	Air Dried

i)	Color Coat	0.38	(3.2)

ii)	Primer	0.42	(3.5)

2)	Exteriors (flexible and non-flexible)

A)	Baked

i)	Primer	0.60	(5.0)

ii)	Primer non-flexible	0.54	(4.5)

iii)	Clear Coat	0.52	(4.3)

iv)	Color Coat	0.55	(4.6)

B)	Air Dried

i)	Primer	0.66	(5.5)

ii)	Clear coat	0.54	(4.5)

iii)	Color coat (red &

black)	0.67	(5.6)

iv)	Color coat (others)	0.61	(5.1)

3)	Specialty

A)	Vacuum metallizing basecoats, texture

basecoats	0.66	(5.5)

B)	Black coatings, reflective argent coatings,

air bag cover coatings, and soft

coatings	0.71	(5.9)

C)	Gloss reducers, vacuum metallizing

topcoats, and texture

topcoats	0.77	(6.4)

D)	Stencil coatings, adhesion primers,

ink pad coatings, electrostatic prep

coatings, and resist

coatings	0.82	(6.8)

E)	Head lamp lens coatings	0.89	(7.4)

o)	Plastic Parts Coating:  Business Machine	kg/l	lb/gal

1)	Primer	0.14	(1.2)

2)	Color coat (non-texture coat)	0.28	(2.3)

3)	Color coat (texture coat)	0.28	(2.3)

4)	Electromagnetic interference/radio

frequency interference (EMI/RFI)

shielding coatings	0.48	(4.0)

5)	Specialty Coatings

A)	Soft coat	0.52	(4.3)

B)	Plating resist	0.71	(5.9)

C)	Plating sensitizer	0.85	(7.1)*

(Source:  Amended at 30 Ill.  Reg. _____, effective _________)

TITLE 35:  ENVIRONMENTAL PROTECTION

SUBTITLE B:  AIR POLLUTION

CHAPTER I:  POLLUTION CONTROL BOARD

SUBCHAPTER c:  EMISSIONS STANDARDS AND

LIMITATIONS FOR STATIONARY SOURCES

PART 219

ORGANIC MATERIAL EMISSION STANDARDS AND LIMITATIONS FOR THE METRO EAST AREA

SUBPART A:  GENERAL PROVISIONS

Section

219.100	Introduction

219.101	Savings Clause

219.102	Abbreviations and Conversion Factors

219.103	Applicability

219.104	Definitions

219.105	Test Methods and Procedures

219.106	Compliance Dates

219.107	Operation of Afterburners

219.108	Exemptions, Variations, and Alternative Means of Control or Compliance Determinations

219.109	Vapor Pressure of Volatile Organic Liquids

219.110	Vapor Pressure of Organic Material or Solvent

219.111	Vapor Pressure of Volatile Organic Material

219.112	Incorporations by Reference

219.113	Monitoring for Negligibly-Reactive Compounds

SUBPART B:  ORGANIC EMISSIONS FROM STORAGE AND LOADING OPERATIONS

Section

219.119	Applicability for VOL

219.120	Control Requirements for Storage Containers of VOL

219.121	Storage Containers of VPL

219.122	Loading Operations

219.123	Petroleum Liquid Storage Tanks

219.124	External Floating Roofs

219.125	Compliance Dates

219.126	Compliance Plan (Repealed)

219.127	Testing VOL Operations

219.128	Monitoring VOL Operations

219.129	Recordkeeping and Reporting for VOL Operations

SUBPART C:  ORGANIC EMISSIONS FROM MISCELLANEOUS EQUIPMENT

Section

219.141	Separation Operations

219.142	Pumps and Compressors

219.143	Vapor Blowdown

219.144	Safety Relief Valves

SUBPART E:  SOLVENT CLEANING

Section

219.181	Solvent Cleaning in General

219.182	Cold Cleaning

219.183	Open Top Vapor Degreasing

219.184	Conveyorized Degreasing

219.185	Compliance Schedule (Repealed)

219.186	Test Methods

SUBPART F:  COATING OPERATIONS

Section

219.204	Emission Limitations

219.205	Daily-Weighted Average Limitations

219.206	Solids Basis Calculation

219.207	Alternative Emission Limitations

219.208	Exemptions From Emission Limitations

219.209	Exemption From General Rule on Use of Organic Material

219.210	Compliance Schedule

219.211	Recordkeeping and Reporting

219.212	Cross-Line Averaging to Establish Compliance for Coating Lines

219.213	Recordkeeping and Reporting for Cross-Line Averaging Participating Coating Lines

219.214	Changing Compliance Methods

219.215	Wood Furniture Coating Averaging Approach

219.216	Wood Furniture Coating Add-On Control Use

219.217	Wood Furniture Coating Work Practice Standards

SUBPART G:  USE OF ORGANIC MATERIAL

Section

219.301	Use of Organic Material

219.302	Alternative Standard

219.303	Fuel Combustion Emission Units

219.304	Operations with Compliance Program

SUBPART H:  PRINTING AND PUBLISHING

Section

219.401	Flexographic and Rotogravure Printing

219.402	Applicability

219.403	Compliance Schedule

219.404	Recordkeeping and Reporting

219.405	Lithographic Printing:  Applicability

219.406	Provisions Applying to Heatset Web Offset Lithographic Printing Prior to March 15, 1996

219.407	Emission Limitations and Control Requirements for Lithographic Printing Lines On and After March 15, 1996

219.408	Compliance Schedule for Lithographic Printing On and After March 15, 1996

219.409	Testing for Lithographic Printing On and After March 15, 1996

219.410	Monitoring Requirements for Lithographic Printing

219.411	Recordkeeping and Reporting for Lithographic Printing

SUBPART Q:  SYNTHETIC ORGANIC CHEMICAL AND POLYMER MANUFACTURING PLANT

Section

219.421	General Requirements

219.422	Inspection Program Plan for Leaks

219.423	Inspection Program for Leaks

219.424	Repairing Leaks

219.425	Recordkeeping for Leaks

219.426	Report for Leaks

219.427	Alternative Program for Leaks

219.428	Open-Ended Valves

219.429	Standards for Control Devices

219.430	Compliance Date (Repealed)

219.431	Applicability

219.432	Control Requirements

219.433	Performance and Testing Requirements

219.434	Monitoring Requirements

219.435	Recordkeeping and Reporting Requirements

219.436	Compliance Date

SUBPART R:  PETROLEUM REFINING AND RELATED INDUSTRIES; ASPHALT MATERIALS

Section

219.441	Petroleum Refinery Waste Gas Disposal

219.442	Vacuum Producing Systems

219.443	Wastewater (Oil/Water) Separator

219.444	Process Unit Turnarounds

219.445	Leaks:  General Requirements

219.446	Monitoring Program Plan for Leaks

219.447	Monitoring Program for Leaks

219.448	Recordkeeping for Leaks

219.449	Reporting for Leaks

219.450	Alternative Program for Leaks

219.451	Sealing Device Requirements

219.452	Compliance Schedule for Leaks

219.453	Compliance Dates (Repealed)

SUBPART S:  RUBBER AND MISCELLANEOUS PLASTIC PRODUCTS

Section

219.461	Manufacture of Pneumatic Rubber Tires

219.462	Green Tire Spraying Operations

219.463	Alternative Emission Reduction Systems

219.464	Emission Testing

219.465	Compliance Dates (Repealed)

219.466	Compliance Plan (Repealed)

SUBPART T:  PHARMACEUTICAL MANUFACTURING

Section

219.480	Applicability

219.481	Control of Reactors, Distillation Units, Crystallizers, Centrifuges and Vacuum Dryers

219.482	Control of Air Dryers, Production Equipment Exhaust Systems and Filters

219.483	Material Storage and Transfer

219.484	In-Process Tanks

219.485	Leaks

219.486	Other Emission Units

219.487	Testing

219.488	Monitoring for Air Pollution Control Equipment

219.489	Recordkeeping for Air Pollution Control Equipment

SUBPART V:  BATCH OPERATIONS AND AIR OXIDATION PROCESSES

Section

219.500	Applicability for Batch Operations

219.501	Control Requirements for Batch Operations

219.502	Determination of Uncontrolled Total Annual Mass Emissions and Actual Weighted Average Flow Rate Values for Batch Operations

219.503	Performance and Testing Requirements for Batch Operations

219.504	Monitoring Requirements for Batch Operations

219.505	Reporting and Recordkeeping for Batch Operations

219.506	Compliance Date

219.520	Emission Limitations for Air Oxidation Processes

219.521	Definitions (Repealed)

219.522	Savings Clause

219.523	Compliance

219.524	Determination of Applicability

219.525	Emission Limitations for Air Oxidation Processes (Renumbered)

219.526	Testing and Monitoring

219.527	Compliance Date (Repealed)

SUBPART W:  AGRICULTURE

Section

219.541	Pesticide Exception

SUBPART X:  CONSTRUCTION

Section

219.561	Architectural Coatings

219.562	Paving Operations

219.563	Cutback Asphalt

SUBPART Y:  GASOLINE DISTRIBUTION

Section

219.581	Bulk Gasoline Plants

219.582	Bulk Gasoline Terminals

219.583	Gasoline Dispensing Operations - Storage Tank Filling Operations

219.584	Gasoline Delivery Vessels

219.585	Gasoline Volatility Standards

219.586	Gasoline Dispensing Operations - Motor Vehicle Fueling Operations (Repealed)

SUBPART Z:  DRY CLEANERS

Section

219.601	Perchloroethylene Dry Cleaners (Repealed)

219.602	Exemptions (Repealed)

219.603	Leaks (Repealed)

219.604	Compliance Dates (Repealed)

219.605	Compliance Plan (Repealed)

219.606	Exception to Compliance Plan (Repealed)

219.607	Standards for Petroleum Solvent Dry Cleaners

219.608	Operating Practices for Petroleum Solvent Dry Cleaners

219.609	Program for Inspection and Repair of Leaks

219.610	Testing and Monitoring

219.611	Exemption for Petroleum Solvent Dry Cleaners

219.612	Compliance Dates (Repealed)

219.613	Compliance Plan (Repealed)

SUBPART AA:  PAINT AND INK MANUFACTURING

Section

219.620	Applicability

219.621	Exemption for Waterbase Material and Heatset-Offset Ink

219.623	Permit Conditions

219.624	Open-Top Mills, Tanks, Vats or Vessels

219.625	Grinding Mills

219.626	Storage Tanks

219.628	Leaks

219.630	Clean Up

219.636	Compliance Schedule

219.637	Recordkeeping and Reporting

SUBPART BB:  POLYSTYRENE PLANTS

Section

219.640	Applicability

219.642	Emissions Limitation at Polystyrene Plants

219.644	Emissions Testing

SUBPART FF:  BAKERY OVENS (REPEALED)

Section

219.720	Applicability (Repealed)

219.722	Control Requirements (Repealed)

219.726	Testing (Repealed)

219.727	Monitoring (Repealed)

219.728	Recordkeeping and Reporting (Repealed)

219.729	Compliance Date (Repealed)

219.730	Certification (Repealed)

SUBPART GG:  MARINE TERMINALS

Section

219.760	Applicability

219.762	Control Requirements

219.764	Compliance Certification

219.766	Leaks

219.768	Testing and Monitoring

219.770	Recordkeeping and Reporting

SUBPART HH:  MOTOR VEHICLE REFINISHING

Section

219.780	Emission Limitations

219.782	Alternative Control Requirements

219.784	Equipment Specifications

219.786	Surface Preparation Materials

219.787	Work Practices

219.788	Testing

219.789	Monitoring and Recordkeeping for Control Devices

219.790	General Recordkeeping and Reporting (Repealed)

219.791	Compliance Date

219.792	Registration

219.875	Applicability of Subpart BB (Renumbered)

219.877	Emissions Limitation at Polystyrene Plants (Renumbered)

219.879	Compliance Date (Repealed)

219.881	Compliance Plan (Repealed)

219.883	Special Requirements for Compliance Plan (Repealed)

219.886	Emissions Testing (Renumbered)

SUBPART PP:  MISCELLANEOUS FABRICATED PRODUCT MANUFACTURING PROCESSES

Section

219.920	Applicability

219.923	Permit Conditions

219.926	Control Requirements

219.927	Compliance Schedule

219.928	Testing

SUBPART QQ:  MISCELLANEOUS FORMULATION MANUFACTURING PROCESSES

Section

219.940	Applicability

219.943	Permit Conditions

219.946	Control Requirements

219.947	Compliance Schedule

219.948	Testing

SUBPART RR:  MISCELLANEOUS ORGANIC CHEMICAL MANUFACTURING PROCESSES

Section

219.960	Applicability

219.963	Permit Conditions

219.966	Control Requirements

219.967	Compliance Schedule

219.968	Testing

SUBPART TT:  OTHER EMISSION UNITS

Section

219.980	Applicability

219.983	Permit Conditions

219.986	Control Requirements

219.987	Compliance Schedule

219.988	Testing

SUBPART UU:  RECORDKEEPING AND REPORTING

Section

219.990	Exempt Emission Units

219.991	Subject Emission Units

APPENDIX A:	List of Chemicals Defining Synthetic Organic Chemical and Polymer Manufacturing

APPENDIX B:	VOM Measurement Techniques for Capture Efficiency (Repealed)

APPENDIX C:	Reference Methods And Procedures

APPENDIX D:	Coefficients for the Total Resource Effectiveness Index (TRE) Equation

APPENDIX E:	List of Affected Marine Terminals

APPENDIX G:	TRE Index Measurements for SOCMI Reactors and Distillation Units

APPENDIX H:	Baseline VOM Content Limitations for Subpart F, Section 219.212 Cross-Line Averaging

AUTHORITY:  Implementing Section 10 and authorized by Section 27, 28 and 28.5 of the Environmental Protection Act [415 ILCS 5/10, 27, 28 and 28.5].

SOURCE:  Adopted at R91-8 at 15 Ill. Reg. 12491, effective August 16, 1991; amended in R91-24 at 16 Ill. Reg. 13597, effective August 24, 1992; amended in R91-30 at 16 Ill. Reg. 13883, effective August 24, 1992; emergency amendment in R93-12 at 17 Ill. Reg. 8295, effective May 24, 1993, for a maximum of 150 days, amended in R93-9 at 17 Ill. Reg. 16918, effective September 27, 1993 and October 21, 1993; amended in R93-28 at 18 Ill. Reg. 4242, effective March 3, 1994; amended in R94-12 at 18 Ill. Reg. 14987, effective September 21, 1994; amended in R94-15 at 18 Ill. Reg. 16415, effective October 25, 1994; amended in R94-16 at 18 Ill. Reg. 16980, effective November 15, 1994; emergency amendment in R95-10 at 19 Ill. Reg. 3059, effective February 28, 1995, for a maximum of 150 days; amended in R94-21, R94-31 and R94-32 at 19 Ill. Reg. 6958, effective May 9, 1995; amended in R94-33 at 19 Ill. Reg. 7385, effective May 22, 1995; amended in R96-2 at 20 Ill. Reg. 3848, effective February 15, 1996; amended in R96-13 at 20 Ill. Reg. 14462, effective October 28, 1996; amended in R97-24 at 21 Ill. Reg. 7721, effective June 9, 1997; amended in R97-31 at 22 Ill. Reg. 3517, effective February 2, 1998; amended in R04-12/20 at 30 Ill. Reg. 9799, effective May 15, 2006; amended in R06-21 at 30 Ill. Reg. _____________, effective _________________.

SUBPART E:  SOLVENT CLEANING

Section 219.182	Cold Cleaning

a)	Operating Procedures:  No person shall operate a cold cleaning degreaser unless:

1)	Waste solvent is stored in covered containers only and not disposed of in such a manner that more than 20% of the waste solvent (by weight) is allowed to evaporate into the atmosphere;

2)	The cover of the degreaser is closed when parts are not being handled; and

3)	Parts are drained until dripping ceases.

b)	Equipment Requirements:  No person shall operate a cold cleaning degreaser unless:

1)	The degreaser is equipped with a cover which is closed whenever parts are not being handled in the cleaner.  The cover shall be designed to be easily operated with one hand or with the mechanical assistance of springs, counter-weights or a powered system if:

A)	The solvent vapor pressure is greater than 2 kPa (15 mmHg or 0.3 psi) measured at 38o C (100o F);

B)	The solvent is agitated; or

C)	The solvent is heated above ambient room temperature.

2)	The degreaser is equipped with a device for draining cleaned parts.  The drainage device shall be constructed so that parts are enclosed under the cover while draining unless:

A)	The solvent vapor pressure is less than 4.3 kPa (32 mmHg or 0.6 psi) measured at 38o C (100o F); or

B)	An internal drainage device cannot be fitted into the cleaning system, in which case the drainage device may be external.

- The degreaser is equipped with one of the following control devices if the vapor pressure of the solvent is greater than 4.3 kPa (32 mmHg or 0.6 psi) measured at 38o C (100° F) or if the solvent is heated above 50o C (120o F) or its boiling point:

A)	A freeboard height of 7/10 of the inside width of the tank or 91 cm (36 in), whichever is less; or

B)	Any other equipment or system of equivalent emission control as approved by the Agency and further processed consistent with Section 219.108 of this Part.  Such a system may include a water cover, refrigerated chiller or carbon adsorber.

4)	A permanent conspicuous label summarizing the operating procedure is affixed to the degreaser; and

5)	If a solvent spray is used, the degreaser is equipped with a solid fluid stream spray, rather than a fine, atomized or shower spray.

c)	Material and Control Requirements:

1)	On and after March 15, 1999, no person shall:

A)	Cause or allow the sale of solvent with a vapor pressure which exceeds 2.0 mmHg (0.038 psi) measured at 20o C (68o F) in units greater than five (5) gallons, for use in cold cleaning degreasing operations located in the area covered by Section 219.103 of this Part.

B)	Operate a cold cleaning degreaser with a solvent vapor pressure which exceeds 2.0 mmHg (0.038 psi) measured at 20o C (68o F).

2)	On and after March 15, 2001, no person shall:

A)	Cause or allow the sale of solvent with a vapor pressure which exceeds 1.0 mmHg (0.019 psi) measured at 20o C (68o F) in units greater than five (5) gallons, for use in cold cleaning degreasing operations located in the area covered by Section 219.103 of this Part.

B)	Operate a cold cleaning degreaser with a solvent vapor pressure which exceeds 1.0 mmHg (0.019 psi) measured at 20o C (68o F).

3)	On and after May 30, 2007, no person may:

A)	Cause or allow the sale of solvent with a vapor pressure which exceeds 1.0 mmHg (0.019 psi) measured at 20° C (68° F) in units greater than five gallons, for use in cold cleaning degreasing operations located in the area covered by Section 219.103 of this Part, unless the purchaser provides a copy of a valid state or federal construction or operating permit or a copy of the Federal Register demonstrating that the purchaser is in compliance with the control requirements of subsection (c)(4) of this Section or is exempt under subsection (f) or (g) of this Section.

B)	Operate a cold cleaning degreaser with a solvent vapor pressure which exceeds 1.0 mmHg (0.019 psi) measured at 20° C (68° F), unless the person is in compliance with the control requirements of subsection (c)(4) of this Section or is exempt under subsection (f) or (g) of this Section.

4)	Control Requirements:

A)	A person may operate a cold cleaning degreaser using solvent with a vapor pressure greater than 1.0 mmHg (0.019 psi) but less than 56 mmHg (1.064 psi) measured at 20° C (68° F) provided add-on control devices demonstrating at least 95 percent overall capture and control of emissions are used.  The add-on controls may include, but are not limited to, carbon adsorbers or afterburners.

B)	An equivalent alternative control plan may be used to meet the control requirements of this Section pursuant to Section 219.108 of this Part.  Pursuant to the material requirements of subsection (c)(3)(B) of this Section, a solvent with a vapor pressure of 1.0 mmHg (0.019 psi) measured at 20° C (68° F) shall be the basis for assessment of equivalent emissions from any equivalent alternative control plan.  If used as an equivalent alternative control plan, an add-on control must demonstrate at least a 95 percent overall capture and control efficiency.  A control plan approved by the Agency shall be effective only when included in a federally enforceable permit or approved by the USEPA as a SIP revision pursuant to Section 219.108 of this Part.

C)	Add-on controls operating at a source prior to May 30, 2007, must be tested by August 31, 2007.  Add-on controls constructed on or after May 30, 2007 must be tested within 90 days of initial startup.  Testing procedures and recordkeeping for add-on controls and equivalent alternative controls subject to subsections (c)(4)(A) and (B) of this Section are to be performed pursuant to Section 219.105 (c), (d), (e) and (f) of this Part.

d)	Recordkeeping and Reporting Requirements: On and after March 15, 1999:

1)	All persons subject to the requirements of subsections (c)(1)(A), and (c)(2)(A), and (c)(3)(A) of this Section must maintain records which include for each sale:

A)	The name and address of the solvent purchaser;

B)	The date of sale;

C)	The type of solvent;

D)	The unit volume of solvent;

E)	The total volume of solvent; and

F)	The vapor pressure of the solvent measured in mmHg at 20o C (68o F).

2)	All persons subject to the requirements of subsections (c)(1)(B), and (c)(2)(B), and (c)(3)(B) of this Section must maintain records which include for each purchase:

A)	The name and address of the solvent supplier;

B)	The date of purchase;

C)	The type of solvent; and

D)	The vapor pressure of the solvent measured in mmHg at 20o C (68o F).; and

E)	For any mixture of solvents, the vapor pressure of the mixture, as used, measured in mmHg at 20° C (68° F).

3)	All persons subject to the requirements of subsection (c)(4) of this Section must maintain records, which include for each purchase:

A)	The name and address of the solvent supplier;

B)	The date of purchase;

C)	The type of solvent;

D)	The unit volume of solvent;

E)	The total volume of solvent;

F)	The vapor pressure of the solvent measured in mmHg at 20° C (68° F); and

G)	For any mixture of solvents, the vapor pressure of the mixture, as used, measured in mmHg at 20° C (68° F).

4)	All persons subject to the requirements of subsection (c)(4) of this Section shall maintain records documenting the use of good operating practices consistent with the equipment manufacturer’s specifications for the cold cleaning degreasers and add-on control equipment.  At a minimum these records shall include:

A)	Records for periodic inspection of the cold cleaning degreasers and add-on control equipment with date of inspection, individual performing the inspection, and nature of inspection;

B)	Records for repair of malfunctions and breakdowns with identification and description of incident, date identified, date repaired, nature of repair, and the amount of VOM that escaped into the atmosphere as a result of the incident;

C)	Control device monitoring and recording data; and

D)	A daily log of operating time for the control device, monitoring equipment, and all associated degreasers.

5)	All persons subject to the requirements of subsection (c) of this Section shall notify the Agency at least 30 days before changing the method of compliance between subsection (c)(3) and (c)(4) of this Section.  Such notification shall include a demonstration of compliance with the newly applicable subsection.

6)	All persons subject to the requirements of subsection (b) or (c) of this Section shall notify the Agency of any violation of subsection (b) or (c) of this Section by sending a description of the violation and copies of records documenting such violations to the Agency within 30 days following the occurrence of the violation.

e)	All records required by subsection (d) of this Section shall be retained for three years and shall be made available to the Agency upon request.

f)	The cleaning of electronic components as defined in 35 Ill. Adm. Code Section 211.1885 is exempt from the requirements of subsection (c) of this Section.

g)	Any cold cleaning taking place in a Detrex cold batch degreaser Model #2D-CC-SPL Size 24-4-10, or substantial equivalent, including automated loading of parts, totally enclosed operation (excluding loading and unloading) and permitted by the Agency, is exempt from the requirements of subsection (c) of this Section.

(Source:  Amended at 30 Ill. Reg. __________, effective ______________)

#### Section 219.Appendix H	Baseline VOM Content Limitations for Subpart F, Section 219.212 Cross-Line Averaging

This Appendix contains limitations for purposes of determining compliance with the requirements in Section 219.212 of this Part.  A source must establish that, at very least, each participating coating line used for purposes of cross-line averaging meets the Federal Implementation Plan level of VOM content, as listed below.  The emission limitations for participating coating lines that must not be exceeded are as follows:

kg/l	lb/gal

a)	Automobile or Light-Duty
Truck Coating

1)	Prime coat	0.14	(1.2)

2)	Primer surface coat	1.81	(15.1)

(Note: The primer surface coat limitation is in units of kg (lbs) of VOM per l (gal) of coating solids deposited.  Compliance with the limitation shall be based on the daily-weighted average from an entire primer surface operation.  Compliance shall be demonstrated in accordance with the topcoat protocol referenced in Section 219.105(b) and the recordkeeping and reporting requirements specified in Section 219.211(f).  Testing to demonstrate compliance shall be performed in accordance with the topcoat protocol and a detailed testing proposal approved by the Agency and USEPA specifying the method of demonstrating compliance with the protocol.  Section 219.205 does not apply to the primer surface limitation.)

kg/l	lb/gal

3)	Topcoat	1.81	(15.1)

(Note: The topcoat limitation is in units of kg (lbs) of VOM per l (gal) of coating solids deposited.  Compliance with the limitation shall be based on the daily-weighted average from an entire topcoat operation.  Compliance shall be demonstrated in accordance with the topcoat protocol referenced in Section 219.105(b) of this Part and the recordkeeping and reporting requirements specified in Section 219.211(f).  Testing to demonstrate compliance shall be performed in accordance with the topcoat protocol and a detailed testing proposal approved by the Agency and USEPA specifying the method of demonstrating compliance with the protocol.  Section 219.205 of this Part does not apply to the topcoat limitation.)

kg/l	lb/gal

4)	Final repair coat	0.58	(4.8)

b)	Can Coating	kg/l	lb/gal

1)	Sheet basecoat and	0.34	(2.8)

overvarnish

2)	Exterior basecoat and	0.34	(2.8)

overvarnish

3)	Interior body spray coat	0.51	(4.2)

4)	Exterior end coat	0.51	(4.2)

5)	Side seam spray coat	0.66	(5.5)

6)	End sealing	0.44	(3.7)

compound coat

kg/l	lb/gal

c)	Paper Coating	0.35	(2.9)

(Note: The paper coating limitation shall not apply to any owner or operator of any paper coating line on which flexographic or rotogravure printing is performed if the paper coating line complies with the emissions limitations in Subpart H: Printing and Publishing, Section 219.401 of this Part.  In addition, screen printing on paper is not regulated as paper coating, but is regulated under Subpart TT of this Part.)

kg/l	lb/gal

d)	Coil Coating	0.31	(2.6)

e)	Fabric Coating	0.35	(2.9)

f)	Vinyl Coating	0.45	(3.8)

g)	Metal Furniture Coating

1)	Air Dried	0.36	(3.0)

2)	Baked	0.36	(3.0)

h)	Large Appliance Coating

1)	Air Dried	0.34	(2.8)

2)	Baked	0.34	(2.8)

(Note: The limitation shall not apply to the use of quick-drying lacquers for repair of scratches and nicks that occur during assembly, provided that the volume of coating does not exceed 0.95 l (1 quart) in any one rolling eight-hour period.)

kg/l	lb/gal

i)	Magnet Wire Coating	0.20	(1.7)

j)	Miscellaneous Metal Parts and

Products Coating

1)	Clear coating	0.52	(4.3)

2)	Extreme performance

coating

A)	Air Dried	0.42	(3.5)

B)	Baked	0.42	(3.5)

3)	Steel pail and drum	0.52	(4.3)

interior coating

4)	All other coatings

A)	Air Dried	0.42	(3.5)

B)	Baked	0.36	(3.0)

k)	Heavy Off-Highway Vehicle	kg/l	lb/gal

Products Coating

1)	Extreme performance	0.42	(3.5)

prime coat

2)	Extreme performance top-	0.42	(3.5)

coat (air dried)

3)	Final repair coat	0.42	(3.5)

(air dried)

4)	All other coatings are subject to the emission limitations for miscellaneous metal parts and products coatings in subsection (j) above.

l)	Wood Furniture Coating	kg/l	lb/gal

1)	Clear topcoat	0.67	(5.6)

2)	Opaque stain	0.56	(4.7)

3)	Pigmented coat	0.60	(5.0)

4)	Repair coat	0.67	(5.6)

5)	Sealer	0.67	(5.6)

6)	Semi-transparent stain	0.79	(6.6)

7)	Wash coat	0.73	(6.1)

(Note: An owner or operator of a wood furniture coating operation subject to this Section shall apply all coatings, with the exception of no more than 37.8 l (10 gal) of coating per day used for touch-up and repair operations, using one or more of the following application systems: airless spray application system, air-assisted airless spray application system, electrostatic spray application system, electrostatic bell or disc spray application system, heated airless spray application system, roller coating, brush or wipe coating application system, dip coating application system or high volume low pressure (HVLP) application system.)

m)	Plastic Parts Coating:	Automotive/Transportation

kg/l	lb/gal

1)	Interiors

A)	Baked

i)	Color coat	0.49*	(4.1)*

ii)	Primer	0.46*	(3.8)*

B)	Air Dried

i)	Color coat 	0.38*	(3.2)*

ii)	Primer	0.42*	(3.5)*

2)	Exteriors (flexible
and non-flexible)

A)	Baked

i)	Primer	0.60*	(5.0)*

ii)	Primer non-	0.54*	(4.5)*

flexible

iii)	Clear coat	0.52*	(4.3)*

iv)	Color coat	0.55*	(4.6)*

B)	Air Dried

i)	Primer	0.66*	(5.5)*

ii)	Clear coat	0.54*	(4.5)*

iii)	Color coat	0.67*	(5.6)*

(red & black)

iv)	Color coat	0.61*	(5.1)*

- (others)

3)	Specialty

A)	Vacuum metallizing	0.66*	(5.5)*

basecoats, texture

basecoats

B)	Black coatings,	0.71*	(5.9)*

reflective argent

coatings, air

bag cover coatings,

and soft coatings

C)	Gloss reducers,	0.77*	(6.4)*

vacuum metallizing

topcoats, and texture topcoats

D)	Stencil coatings,	0.82*	(6.8)*

adhesion primers,

ink pad coatings,

electrostatic prep

coatings, and resist

coatings

E)	Head lamp lens	0.89*	(7.4)*

- coatings

n)	Plastic Parts Coating:	Business Machine

kg/l	lb/gal

1)	Primer	0.14*	(1.2)*

2)	Color coat (non-	0.28*	(2.3)*

texture coat)

3)	Color coat (texture	0.28*	(2.3)*

coat)

4)	Electromagnetic	0.48*	(4.0)*

interference/radio

frequency interference

(EMI/RFI) shielding coatings

5)	Specialty Coatings

A)	Soft coat	0.52*	(4.3)*

B)	Plating resist	0.71*	(5.9)*

C)	Plating sensitizer	0.85*	(7.1)*

(Source:  Amended at 30 Ill. Reg. _________, effective ________)

IT IS SO ORDERED.

I, Dorothy M. Gunn, Clerk of the Illinois Pollution control Board, certify that the Board adopted the above opinion and order on March 1, 2007, by a vote of 4-0.

Dorothy M. Gunn, Clerk

Illinois Pollution Control Board