TITLE 35:  ENVIRONMENTAL PROTECTION

SUBTITLE C:  WATER POLLUTION

CHAPTER II:  ENVIRONMENTAL PROTECTION AGENCY

PART 372

ILLINOIS DESIGN STANDARDS FOR SLOW RATE LAND

APPLICATION OF TREATED WASTEWATER

SUBPART A:  INTRODUCTION

Section

372.100   Purpose

372.110   Scope and Applicability

SUBPART B:  SITE SELECTION CONSIDERATIONS

Section

372.200   General

372.210   Site Location

372.220   Hydrogeology and Soils

372.230   Buffer Zone

372.240   Loading Factors on Application Fields

372.250   Project Layout

SUBPART C:  APPLICATION SYSTEM

Section

372.300   Equipment Design

372.310   Runoff Control

372.320   Application Area Access Control

SUBPART D:  PREAPPLICATION TREATMENT AND STORAGE

Section

372.400   Degree of Treatment Required Relative to Application Area

372.410   Preapplication Treatment Plant Design

372.420   Storage

372.430   Treatment and Storage Area Access Control

SUBPART E:  MONITORING REQUIREMENTS

Section

372.500   Groundwater Monitoring

372.510   System Flow and Stored Volume Measurement

AUTHORITY:  Implementing and authorized by Sections 4(h) and 39(a) of the

Environmental Protection Act [415 ILCS 5/4(h) and 39(a)].

SOURCE:  Adopted at 19 Ill. Reg. 1297, effective January 31, 1995.

Section 372.100  Purpose

The purpose of this Part is to establish design standards and permit

application requirements for wastewater treatment systems involving the

non-discharging low-rate application of wastewater to land.

Section 372.110  Scope and Applicability

a)  These design standards apply to non-discharging low-rate land

application of secondary and tertiary treated domestic wastewater

to land upon which crops, turf or trees are grown.  These design

standards are to be used in the preparation of all engineering

documents.

b)  A preliminary engineering report must be submitted to the Agency

for review and approval prior to the preparation and submission of

plans and specifications or permit applications.

c)  Detailed design requirements for treatment, transport and storage

facilities are contained in 35 Ill. Adm. Code 370, Illinois

Recommended Standards for Sewage Works.

SUBPART B:  SITE SELECTION CONSIDERATIONS

Section 372.200  General

The preliminary engineering report required under subsection 372.110(b)

shall address all of the elements contained in this Subpart B, as well as

any of the planning elements of Subparts C through E that are applicable to

the feasibility of the project for which the preliminary engineering report

is submitted.

Section 372.210  Site Location

a)  General

The following factors shall be considered in the selection of the

site:

1)  Present and proposed land use regarding residences,

buildings, developments, public access areas, for the site

and adjoining properties;

2)  Present and proposed water supply wells;

3)  Surface waters and waterways, wetlands and 10-year

floodplains;

4)  Subsurface drainage tiles and storm sewers;

5)  Abandoned wells and shafts;

6)  Buffer zones;

7)  Area for expansion of the wastewater treatment and land

application systems;

8)  Depth to groundwater;

9)  Depth to bedrock; and

10) Topography.

b)  Well Setback Requirements

The location of the treatment facilities and land application area

with respect to wells shall be in conformance with the setback

zone requirements of the Illinois Groundwater Protection Act [415

ILCS 55].

c)  Topography

1)  Cultivated Fields

Slopes on cultivated fields shall be limited to 4% unless

runoff control measures such as berms, collection ditches or

check dams are provided.

2)  Sodded Fields and Forested Areas

Slopes on sodded fields and forested areas shall be limited

to 8% unless runoff control measures such as berms,

collection ditches or check dams are provided.

3)  Steep Slopes

For slopes steeper than those covered under subsections

(c)(1) and (2) above, the engineering documents shall include

the runoff control measure recommendations of the Soil

Conservation Service (SCS) or county soil conservation

district.

d)  Location to Surface Water

Treated wastewater shall not be applied or discharged to wetlands,

streams, waterways or other surface waters.  Floodplains which

have a flood return frequency of less than 10 years shall not be

used for land application unless alternate application sites or

additional lagoon storage is provided.  Storage shall be designed

in accordance with Section 372.420.

e)  Depth to Groundwater

The system design shall be based on rational calculations that

take into account that treated wastewater is not to be applied

when groundwater is within 4 feet of the ground surface.  The

design shall include storage capacity for such high groundwater

periods and such periods shall be accounted for in the computation

of the available treated wastewater application days.  The design

shall also take into account the effect that the application of

additional water will have on groundwater levels.

f)  Depth to Bedrock

For land application of treated wastewater the proposed site shall

have a minimum of 10 feet of earth cover over bedrock, unless the

preapplication treatment system complies with Section 372.400(b).

Section 372.220  Hydrogeology and Soils

a)  General

The engineering documents shall contain information on location,

geology, groundwater, soil characteristics, ground slopes, area

for expansion, and any other factors that will affect the

feasibility and acceptability of the proposed land application

system.  Data shall be obtained from available and identified

sources or onsite investigations.

b)  Geology

Geological conditions present at the land treatment site and their

potential effects, including depth to bedrock, thickness of

surficial deposits, and the presence of any special conditions

must be described.  The major geological factors which shall be

considered are:

1)  Structure, weathering and fracturing of bedrock;

2)  Lithology;

3)  Texture; and

4)  Soil profile.

c)  Groundwater

The following data and information for the proposed site shall be

provided:

1)  Depth to the seasonal high groundwater table and the duration

of seasonal variations;

2)  Identification and depth to each aquifer based on Illinois

State Water Survey data or other available well drilling logs

or boring records for the area;

3)  Direction of groundwater movement and the location of any

points of groundwater resurfacing;

4)  A chemical analysis of the existing groundwater quality for

those parameters set out in Section 372.500(d) which may be

affected by the application of treated wastewater; and

5)  An evaluation of the effects of the applied treated

wastewater on groundwater movement and quality.

d)  Soil Characteristics

The soil at the proposed site must be evaluated based on on-site

surveys and the most up-to-date published soil survey for:

1)  Types and texture classifications;

2)  Mantle thickness;

3)  Chemical characteristics;

4)  pH;

5)  Nutrient levels including nitrogen and phosphorus;

6)  Cation exchange capacity;

7)  Subsurface soil characteristics;

8)  Soil borings to a minimum depth of 10 feet; and

9)  Permeability of the most impermeable layer of the soil mantle

at each soil boring location.

Section 372.230  Buffer Zone

a)  Non-Spray Surface Application

For non-spray surface application of treated wastewater using

gated pipes, subsurface irrigation, or equivalent methods, the

area wetted by the treated wastewater shall not encroach on any

wetlands, streams, waterways, surface waters, public road rights

of way or residential lot lines.

b)  Spray Irrigation

For spray irrigation of treated wastewater, the outer edge of the

area wetted by the spray mist shall not be closer than the

following distances from any residential lot line including

application under design maximum wind conditions with peripheral

spray units operating (refer to Section 370.300 (c)(1)(C)):

1)  200 feet, unless the requirements in (b)(2) or (b)(3) of this

Section are met;

2)  25 feet, if the application area is surrounded by a fence

with a minimum height of 40 inches; or

3)  No distance restriction when the application area is:

A)  a golf course, if the application occurs only during the

hours between dusk and dawn; or

B)  a restricted access area (an area to which public access

is controlled), if the application and its associated

drying time occur during a period when the area is

closed to the public.

The outer edge of the area wetted by spray mist shall not encroach

on any wetlands, streams, waterways or other surface waters or

public road rights of way under design maximum wind conditions.

The engineering document shall provide engineering data for the

spray equipment specified on design pressure, wind velocity,

height of the spray, spray mist drift distances at design

operating pressures and wind velocities with peripheral spray

units operating.

Section 372.240  Loading Factors on Application Fields

a)  General

The size of the application area shall be based on the limiting

characteristic of the treated wastewater and the site.  Balances

shall be calculated and submitted for water, nitrogen, phosphorus,

and 5 Day Biochemical Oxygen Demand (BOD[5]).  Loading rates must

be established for each parameter.  The critical loading rate will

determine the application area required.

b)  Water Balance

1)  The water balance for the application site shall include the

applied treated wastewater, runon from adjacent areas,

precipitation, evapotranspiration, permeability, groundwater

recharge rate and effect on water table depth and subsurface

tile drainage.  The system shall be designed so that runoff

does not occur as a result of treated wastewater application.

Treated wastewater shall not be applied during precipitation

events; however, runoff in response to precipitation is

acceptable.  The water balance shall be calculated on a

rational basis for each month and exclude the days on which

application cannot occur due to rainfall, frozen or wet

ground conditions, cropping practices, high groundwater

conditions or wind in excess of design conditions.

2)  The design shall provide for alternate wetting and drying

periods in order to maintain aerobic conditions in the

topsoil, as well as to maintain a viable cover crop.

3)  The design daily percolation rate shall not exceed 10% of the

minimum soil permeability at the site.  The percolation rate

calculations shall exclude the days on which application

cannot occur due to rainfall, frozen or wet ground

conditions, cropping practices, high groundwater conditions

or wind in excess of design conditions.

c)  Nitrogen Balance

1)  On an annual basis, the total nitrogen applied in the treated

wastewater shall be within agronomic rates and shall be

accounted for by crop uptake, volatilization,

denitrification, adsorption, mineralization and metal

precipitation.  Any application of chemical fertilizer must

be accounted for in the design.

2)  Land application areas shall be managed according to normal

agricultural and horticultural practices, including but not

limited to cultivated farmland with harvest, fallow land,

set-aside programs, pasture land, golf courses, sod farms,

urban parks, or forest preserves.

Section 372.250  Project Layout

A single topographic map of the proposed project and the area within 2,500

ft. of the project shall be submitted as a part of the engineering design.

Segmented maps may be provided for large projects.  The topographic map

shall show the following information:

a)  Application area boundaries including buffer zones;

b)  Treatment and storage facilities;

c)  Piping and layout of the irrigation system;

d)  Present and proposed land use regarding residences, buildings,

developments, public access areas, etc;

e)  Present and proposed water supply wells and abandoned wells and

shafts;

f)  Direction of groundwater movement and any points of groundwater

resurfacing;

g)  Surface waters and waterways, wetlands and 10-year floodplains;

h)  Subsurface drainage tiles and storm sewers; and

i)  Slopes of the application areas.

SUBPART C:  APPLICATION SYSTEM

Section 372.300  Equipment Design

a)  General Requirements

1)  The design of all application systems shall provide

facilities to assure uniform distribution of the treated

wastewater over the disposal area.

2)  Sufficient spare equipment and parts shall be available to

assure continuity of application during application periods.

3)  A single irrigation pumping unit may be utilized if a spare

pump in working condition is available in dry storage.

4)  The design shall provide for draining all pipes and equipment

to prevent freeze damage.

b)  Non-Spray Surface Application Systems

Non-spray surface application systems which provide for even

distribution of wastewater effluent on the land site may be

utilized, including ridge and furrow, gated pipe, or equivalent

systems.  These systems may be mobile or fixed on the site.  They

shall be designed to minimize clogging and to allow for ease of

maintenance.

c)  Spray Irrigation Systems

1)  Spray Equipment

A)  Fixed head systems and center pivot rigs may be

utilized.  A permanent connection point must be provided

for each setting of moveable spray irrigation equipment.

The design shall include provisions for shutting off the

peripheral spray heads to prevent drift of spray beyond

the application area under design wind conditions.

B)  The irrigation system controls shall be simple and be

protected from lightning damage.

C)  The design maximum wind velocities shall be 15 MPH in

urban and residential areas and 25 MPH in agricultural

areas.

2)  Nozzle Pressure Regulation

Regulation of nozzle pressure to compensate for field

elevations and line losses shall be provided.

Section 372.310  Runoff Control

The design shall provide for control of the application intensity to

prevent runoff in response to treated wastewater application on all parts

of the application field.  In order to minimize runoff during precipitation

events, the exclusion of runon from adjacent areas shall be considered.

Section 372.320  Application Area Access Control

The entire application area and buffer area shall be posted at 100 yard

intervals around the perimeter identifying the area as a "Treated

Wastewater Application Area".  The application area shall be fenced to

prevent access by children and unauthorized personnel unless the

pretreatment provided meets the urban area pretreatment requirements of

Section 372.400(b) or the land application system is located in an

agricultural area or a forested area that does not have general public

access.

SUBPART D:  PREAPPLICATION TREATMENT AND STORAGE

Section 372.400  Degree of Treatment Required Relative to Application Area

a)  Agricultural Areas

Agricultural or forested areas that do not have public access

shall provide at a minimum a two cell lagoon system or a

mechanical secondary treatment facility.

b)  Urban Areas

Urban parks, forest preserves and golf courses and other areas

with public access shall utilize as a minimum a two cell lagoon

system with tertiary sand filtration and disinfection or a

mechanical secondary treatment facility with disinfection.

Section 372.410  Preapplication Treatment Plant Design

a)  Design and Construction Requirements

All preapplication treatment systems shall be designed and

constructed in accordance with 35 Ill. Adm. Code 370, Illinois

Recommended Standards for Sewage Works.

b)  Screening

All treatment systems except those requiring tertiary filtration

must have a screening device sized to minimize plugging of the

spray nozzles following the storage lagoon prior to the irrigation

distribution system.

Section 372.420  Storage

a)  Storage Volume

All land application systems must provide adequate storage, with

adequacy being determined using either of the following methods,

except that those facilities that do not generate wastewater year

round must use Method I:

1)  Method I

Adequate storage shall be based on a rational design that

must include capacity for the wettest year with a 20-year

return frequency.  The volume provided shall be sufficient to

hold flows received during the following periods:

A)  When the soil is frozen, including subsoil frost layers;

B)  When there is an ice or snow cover on the ground;

C)  When the soil temperature at 4" depth is less than 40	F

or the mean air temperature is less than 35	F;

D)  When the ground is saturated or there is standing water

(as from late winter snowmelt or spring rains);

E)  When the groundwater table is within 4 feet of the

surface;

F)  During days when precipitation exceeds 0.1 inch;

G)  During agricultural and horticultural practices;

H)  During days set aside for equipment maintenance;

I)  During days when the design maximum wind velocity is

exceeded; and

J)  When the soil is barren, except for seeded areas, areas

with growing crops, or areas with a trashy cover to

prevent erosion.

2)  Method II

The minimum storage capacity, by volume, shall be capable of

storing at least 150 days production of wastewater, at design

average flow, except that in southern Illinois areas (defined

as all areas south of Interstate 70) a minimum of 120 days

storage capacity shall be provided.

b)  Design and Construction Requirements

The storage lagoon must be designed and constructed in accordance

with 35 Ill. Adm. Code 370, Illinois Recommended Standards for

Sewage Works.

Section 372.430  Treatment and Storage Area Access Control

The entire treatment and storage lagoon area shall be enclosed with fencing

to preclude livestock and prevent access by the general public.  Warning

signs shall be posted at 100-yard intervals on exterior fences designating

the area as a "Wastewater Treatment Facility".  A vehicle access gate of

sufficient width to accommodate mowing equipment and maintenance vehicles

shall be provided.  All access gates shall be provided with locks.

SUBPART E:  MONITORING REQUIREMENTS

Section 372.500  Groundwater Monitoring

a)  General

1)  Saturated zone groundwater monitoring shall be provided for

all land application systems.  A minimum of three groundwater

monitoring wells must be provided, one upgradient for

determining background concentrations and two downgradient in

the dominant direction of groundwater movement from the land

application system.

2)  Provision shall be made for sampling of the discharge from

any drainage tiles underlying the application area.

b)  Potable Water Supply Wells

Where the project site is within 1,000 feet of existing potable

water wells but the wells are outside the minimum setback zone of

the Illinois Groundwater Protection Act [415 ILCS 55], groundwater

monitoring wells must be provided between each potable water well

and the land application system.

c)  Monitoring Well Design

The monitoring wells shall be constructed with provisions for

sampling at the surface of the water table and at 5 feet below the

water table at each monitoring site.

d)  Testing and Monitoring Equipment

Provision shall be made for testing groundwater for nitrates,

ammonia nitrogen, chlorides, sulfates, pH, total dissolved solids,

phosphate, and coliform bacteria as specified by the Agency permit

for the project to measure compliance with groundwater standards.

Testing shall be performed in accordance with 40 CFR 136 (1992)

(no later amendments or editions), and may be done either at an

onsite laboratory or through a contractual arrangement with an

offsite laboratory.

Section 372.510  System Flow and Stored Volume Measurement

a)  System Flows

Plant influent and effluent irrigation flow measurement shall be

provided.  Flow measurement shall not be less than elapsed time

meters used in conjunction with pumping rate tests or calibrated

weirs.  All flow measurement equipment shall be sized to function

effectively in the full range of flows expected and shall be

protected against freezing.

b)  Stored Volume Measurement

A staff gauge shall be provided in the storage lagoon located near

the draw-off structure and must be easily read from the lagoon

dike.

c)  Monitoring Systems

Monitoring equipment for wastewater application sites shall

include equipment for measuring air temperature, soil temperature,

precipitation, wind speed and direction, and depth to groundwater.