TITLE 35:  ENVIRONMENTAL PROTECTION

SUBTITLE C:  WATER POLLUTION

CHAPTER I:  POLLUTION CONTROL BOARD

PART 307

SEWER DISCHARGE CRITERIA

SUBPART A:  GENERAL PROVISIONS

Section

307.101	Preamble (Renumbered)

307.102	General Requirements (Renumbered)

307.103	Mercury (Renumbered)

307.104	Cyanide (STORET number 00720) (Renumbered)

307.105	Pretreatment Requirements (Repealed)

307.1001	Preamble

307.1002	Definitions

307.1003	Test Procedures for Measurement

307.1005	Toxic Pollutants

307.1006	Electronic Reporting

SUBPART B:  GENERAL AND SPECIFIC PRETREATMENT REQUIREMENTS

Section

307.1101	General and Specific Requirements

307.1102	Mercury

307.1103	Cyanide

SUBPART F:  DAIRY PRODUCTS PROCESSING

Section

307.1501	Receiving Stations

307.1502	Fluid Products

307.1503	Cultured Products

307.1504	Butter

307.1505	Cottage Cheese and Cultured Cream Cheese

307.1506	Natural and Processed Cheese

307.1507	Fluid Mix for Ice Cream and Other Frozen Desserts

307.1508	Ice Cream, Frozen Desserts, Novelties, and Other Dairy Desserts

307.1509	Condensed Milk

307.1510	Dry Milk

307.1511	Condensed Whey

307.1512	Dry Whey

SUBPART G:  GRAIN MILLS

Section

307.1601	Corn Wet Milling

307.1602	Corn Dry Milling

307.1603	Normal Wheat Flour Milling

307.1604	Bulgur Wheat Flour Milling

307.1605	Normal Rice Milling

307.1606	Parboiled Rice Milling

307.1607	Animal Feed

307.1608	Hot Cereal

307.1609	Ready-to-Eat Cereal

307.1610	Wheat Starch and Gluten

SUBPART H:  CANNED AND PRESERVED FRUITS AND VEGETABLES

Section

307.1700	General Provisions

307.1701	Apple Juice

307.1702	Apple Products

307.1703	Citrus Products

307.1704	Frozen Potato Products

307.1705	Dehydrated Potato Products

307.1706	Canned and Preserved Fruits

307.1707	Canned and Preserved Vegetables

307.1708	Canned and Miscellaneous Specialties

SUBPART I:  CANNED AND PRESERVED SEAFOOD

Section

307.1801	Farm-Raised Catfish

307.1815	Fish Meal Processing Subcategory (Repealed)

SUBPART J:  SUGAR PROCESSING

Section

307.1901	Beet Sugar Processing

307.1902	Crystalline Cane Sugar Refining

307.1903	Liquid Cane Sugar Refining

SUBPART K:  TEXTILE MILLS

Section

307.2000	General Provisions

307.2001	Wool Scouring

307.2002	Wool Finishing

307.2003	Low Water Use Processing

307.2004	Woven Fabric Finishing

307.2005	Knit Fabric Finishing

307.2006	Carpet Finishing

307.2007	Stock and Yarn Finishing

307.2008	Nonwoven Manufacturing

307.2009	Felted Fabric Processing

SUBPART L:  CEMENT MANUFACTURING

Section

307.2101	Nonleaching

307.2102	Leaching

307.2103	Materials Storage Piles Runoff

SUBPART M:  CONCENTRATED ANIMAL FEEDING OPERATIONS

Section

307.2201	General

307.2202	Ducks

SUBPART N:  ELECTROPLATING

Section

307.2300	General Provisions

307.2301	Electroplating of Common Metals

307.2302	Electroplating of Precious Metals

307.2304	Anodizing

307.2305	Coatings

307.2306	Chemical Etching and Milling

307.2307	Electroless Plating

307.2308	Printed Circuit Boards

SUBPART O:  ORGANIC CHEMICALS, PLASTICS, AND SYNTHETIC FIBERS

Section

307.2400	General Provisions

307.2401	Rayon Fibers

307.2402	Other Fibers

307.2403	Thermoplastic Resins

307.2404	Thermosetting Resins

307.2405	Commodity Organic Chemicals

307.2406	Bulk Organic Chemicals

307.2407	Specialty Organic Chemicals

307.2410	Indirect Discharge Point Sources

307.2490	Non-Complexed Metal-Bearing and Cyanide-Bearing Waste Streams

307.2491	Complexed Metal-Bearing Waste Streams

SUBPART P:  INORGANIC CHEMICALS MANUFACTURING

Section

307.2500	General Provisions

307.2501	Aluminum Chloride Production

307.2502	Aluminum Sulfate Production

307.2503	Calcium Carbide Production

307.2504	Calcium Chloride Production

307.2505	Calcium Oxide Production

307.2506	Chlor-Alkali Process (Chlorine and Sodium or Potassium Hydroxide Production)

307.2508	Hydrofluoric Acid Production

307.2509	Hydrogen Peroxide Production

307.2511	Potassium Metal Production

307.2512	Potassium Dichromate Production

307.2513	Potassium Sulfate Production

307.2514	Sodium Bicarbonate Production

307.2516	Sodium Chloride Production

307.2517	Sodium Dichromate and Sodium Sulfate Production

307.2520	Sodium Sulfite Production

307.2522	Titanium Dioxide Production

307.2523	Aluminum Fluoride Production

307.2524	Ammonium Chloride Production

307.2527	Borax Production

307.2528	Boric Acid Production

307.2529	Bromine Production

307.2530	Calcium Carbonate Production

307.2531	Calcium Hydroxide Production

307.2533	Carbon Monoxide and Byproduct Hydrogen Production

307.2534	Chrome Pigments Production

307.2535	Chromic Acid Production

307.2536	Copper Salts Production

307.2538	Ferric Chloride Production

307.2540	Fluorine Production

307.2541	Hydrogen Production

307.2542	Hydrogen Cyanide Production

307.2543	Iodine Production

307.2544	Lead Monoxide Production

307.2545	Lithium Carbonate Production

307.2547	Nickel Salts Production

307.2549	Oxygen and Nitrogen Production

307.2550	Potassium Chloride Production

307.2551	Potassium Iodide Production

307.2553	Silver Nitrate Production

307.2554	Sodium Bisulfite Production

307.2555	Sodium Fluoride Production

307.2560	Stannic Oxide Production

307.2563	Zinc Sulfate Production

307.2564	Cadmium Pigments and Salts Production

307.2565	Cobalt Salts Production

307.2566	Sodium Chlorate Production

307.2567	Zinc Chloride Production

SUBPART R:  SOAP AND DETERGENTS

Section

307.2701	Soap Manufacturing by Batch Kettle

307.2702	Fatty Acid Manufacturing by Fat Splitting

307.2703	Soap Manufacturing by Fatty Acid Neutralization

307.2704	Glycerine Concentration

307.2705	Glycerine Distillation

307.2706	Manufacture of Soap Flakes and Powders

307.2707	Manufacture of Bar Soaps

307.2708	Manufacture of Liquid Soaps

307.2709	Oleum Sulfonation and Sulfation

307.2710	Air-Sulfur Trioxide Sulfation and Sulfonation

307.2711	Sulfur Trioxide Solvent and Vacuum Sulfonation

307.2712	Sulfamic Acid Sulfation

307.2713	Chlorosulfonic Acid Sulfation

307.2714	Neutralization of Sulfuric Acid Esters and Sulfonic Acids

307.2715	Manufacture of Spray-Dried Detergents

307.2716	Manufacture of Liquid Detergents

307.2717	Manufacturing of Detergents by Dry Blending

307.2718	Manufacture of Drum-Dried Detergents

307.2719	Manufacture of Detergent Bars and Cakes

SUBPART S:  FERTILIZER MANUFACTURING

Section

307.2801	Phosphate

307.2802	Ammonia

307.2803	Urea

307.2804	Ammonium Nitrate

307.2805	Nitric Acid

307.2806	Ammonium Sulfate Production

307.2807	Mixed and Blend Fertilizer Production

SUBPART T:  PETROLEUM REFINING

Section

307.2901	Topping

307.2902	Cracking

307.2903	Petrochemical

307.2904	Lube

307.2905	Integrated

SUBPART U:  IRON AND STEEL MANUFACTURING

Section

307.3000	General Provisions

307.3001	Cokemaking

307.3002	Sintering

307.3003	Ironmaking

307.3004	Steelmaking

307.3005	Vacuum Degassing

307.3006	Continuous Casting

307.3007	Hot Forming

307.3008	Salt Bath Descaling

307.3009	Acid Pickling

307.3010	Cold Forming

307.3011	Alkaline Cleaning

307.3012	Hot Coating

307.3013	Other Operations

SUBPART V:  NONFERROUS METALS MANUFACTURING

Section

307.3100	General Provisions

307.3101	Bauxite Refining

307.3102	Primary Aluminum Smelting

307.3103	Secondary Aluminum Smelting

307.3104	Primary Copper Smelting

307.3105	Primary Electrolytic Copper Refining

307.3106	Secondary Copper

307.3107	Primary Lead

307.3108	Primary Zinc

307.3109	Metallurgical Acid Plants

307.3110	Primary Tungsten

307.3111	Primary Columbium-Tantalum

307.3112	Secondary Silver

307.3113	Secondary Lead

307.3114	Primary Antimony

307.3115	Primary Beryllium

307.3116	Primary and Secondary Germanium and Gallium

307.3117	Secondary Indium

307.3118	Secondary Mercury

307.3119	Primary Molybdenum and Rhenium

307.3120	Secondary Molybdenum and Vanadium

307.3121	Primary Nickel and Cobalt

307.3122	Secondary Nickel

307.3123	Primary Precious Metals and Mercury

307.3124	Secondary Precious Metals

307.3125	Primary Rare Earth Metals

307.3126	Secondary Tantalum

307.3127	Secondary Tin

307.3128	Primary and Secondary Titanium

307.3129	Secondary Tungsten and Cobalt

307.3130	Secondary Uranium

307.3131	Primary Zirconium and Hafnium

SUBPART X:  STEAM ELECTRIC POWER GENERATING

Section

307.3301	Steam Electric Power Generating

SUBPART Y:  FERROALLOY MANUFACTURING

Section

307.3401	Open Electric Furnaces With Wet Air Pollution Control Devices

307.3402	Covered Electric Furnaces and Other Smelting Operations with Wet Air Pollution Control Devices

307.3403	Slag Processing

307.3404	Covered Calcium Carbide Furnaces With Wet Air Pollution Control Devices

307.3405	Other Calcium Carbide Furnaces

307.3406	Electrolytic Manganese Products

307.3407	Electrolytic Chromium

SUBPART Z:  LEATHER TANNING AND FINISHING

Section

307.3500	General Provisions

307.3501	Hair Pulp, Chrome Tan, Retan-Wet Finish

307.3502	Hair Save, Chrome Tan, Retan-Wet Finish

307.3503	Hair Save or Pulp, Non-Chrome Tan, Retan-Wet Finish

307.3504	Retan-Wet Finish-Sides

307.3505	No Beamhouse

307.3506	Through-the-Blue

307.3507	Shearling

307.3508	Pigskin

307.3509	Retan-Wet Finish-Splits

307.3590	Potassium Ferricyanide Titration Method

SUBPART BA:  GLASS MANUFACTURING

Section

307.3601	Insulation Fiberglass

307.3602	Sheet Glass Manufacturing

307.3603	Rolled Glass Manufacturing

307.3604	Plate Glass Manufacturing

307.3605	Float Glass Manufacturing

307.3606	Automotive Glass Tempering

307.3607	Automotive Glass Laminating

307.3608	Glass Container Manufacturing

307.3610	Glass Tubing (Danner) Manufacturing

307.3611	Television Picture Tube Envelope Manufacturing

307.3612	Incandescent Lamp Envelope Manufacturing

307.3613	Hand Pressed and Blown Glass Manufacturing

SUBPART BB:  ASBESTOS MANUFACTURING

Section

307.3701	Asbestos-Cement Pipe

307.3702	Asbestos-Cement Sheet

307.3703	Asbestos Paper (Starch Binder)

307.3704	Asbestos Paper (Elastomeric Binder)

307.3705	Asbestos Millboard

307.3706	Asbestos Roofing

307.3707	Asbestos Floor Tile

307.3708	Coating or Finishing of Asbestos Textiles

307.3709	Solvent Recovery

307.3710	Vapor Absorption

307.3711	Wet Dust Collection

SUBPART BC:  RUBBER MANUFACTURING

Section

307.3801	Tire and Inner Tube Plants

307.3802	Emulsion Crumb Rubber

307.3803	Solution Crumb Rubber

307.3804	Latex Rubber

307.3805	Small-Sized General Molded, Extruded, and Fabricated Rubber Plants

307.3806	Medium-Sized General Molded, Extruded, and Fabricated Rubber Plants

307.3807	Large-Sized General Molded, Extruded, and Fabricated Rubber Plants

307.3808	Wet Digestion Reclaimed Rubber

307.3809	Pan, Dry Digestion, and Mechanical Reclaimed Rubber

307.3810	Latex-Dipped, Latex-Extruded, and Latex-Molded Rubber

307.3811	Latex Foam

SUBPART BD:  TIMBER PRODUCTS PROCESSING

Section

307.3900	General Provisions

307.3901	Barking

307.3902	Veneer

307.3903	Plywood

307.3904	Dry Process Hardboard

307.3905	Wet Process Hardboard

307.3906	Wood Preserving—Waterborne or Non-pressure

307.3907	Wood Preserving—Steam

307.3908	Wood Preserving—Boulton

307.3909	Wet Storage

307.3910	Log Washing

307.3911	Sawmills and Planing Mills

307.3912	Finishing

307.3913	Particleboard Manufacturing

307.3914	Insulation Board

307.3915	Wood Furniture and Fixture Production without Water Wash Spray Booths or without Laundry Facilities

307.3916	Wood Furniture and Fixture Production with Water Wash Spray Booths or with Laundry Facilities

SUBPART BE:  PULP, PAPER, AND PAPERBOARD

Section

307.4000	General Provisions

307.4001	Dissolving Kraft

307.4002	Bleached Papergrade Kraft and Soda

307.4003	Unbleached Kraft

307.4004	Dissolving Sulfite

307.4005	Papergrade Sulfite

307.4006	Semi-Chemical

307.4007	Mechanical Pulp

307.4008	Non-Wood Chemical Pulp

307.4009	Secondary Fiber Deink

307.4010	Secondary Fiber Non-Deink

307.4011	Fine and Lightweight Papers from Purchased Pulp

307.4012	Tissue, Filter, Non-Woven, and Paperboard from Purchased Pulp

307.4013	Groundwood-Thermo-Mechanical (Repealed)

307.4014	Groundwood-CMN Papers (Repealed)

307.4015	Groundwood-Fine Papers (Repealed)

307.4016	Soda (Repealed)

307.4017	Deink (Repealed)

307.4018	Nonintegrated-Fine Papers (Repealed)

307.4019	Nonintegrated-Tissue Papers (Repealed)

307.4020	Tissue From Wastepaper (Repealed)

307.4021	Papergrade Sulfite (Drum Wash) (Repealed)

307.4022	Unbleached Kraft and Semi-Chemical (Repealed)

307.4023	Wastepaper-Molded Products (Repealed)

307.4024	Nonintegrated-Lightweight Papers (Repealed)

307.4025	Nonintegrated-Filter and Nonwoven Papers (Repealed)

307.4026	Nonintegrated-Paperboard (Repealed)

SUBPART BF:  BUILDERS' PAPER AND BOARD MILLS

Section

307.4101	Builder's Paper and Roofing Felt (Repealed)

SUBPART BG:  MEAT PRODUCTS

Section

307.4201	Simple Slaughterhouse

307.4202	Complex Slaughterhouse

307.4203	Low-Processing Packinghouse

307.4204	High-Processing Packinghouse

307.4205	Small Processor

307.4206	Meat Cutter

307.4207	Sausage and Luncheon Meats Processor

307.4208	Ham Processor

307.4209	Canned Meats Processor

307.4210	Renderer

SUBPART BH:  METAL FINISHING

Section

307.4300	General Provisions

307.4301	Metal Finishing

SUBPART BJ:  OIL AND GAS EXTRACTION

Section

307.4503	Onshore Facility Standards

307.4508	Coalbed Methane Subcategory

SUBPART BL:  CENTRALIZED WASTE TREATMENT

Section

307.4700	General Provisions

307.4701	Metals Treatment and Recovery

307.4702	Oils Treatment and Recovery

307.4703	Organics Treatment and Recovery

307.4704	Multiple Waste Streams

SUBPART BN:  PHARMACEUTICAL MANUFACTURING

Section

307.4900	General Provisions

307.4901	Fermentation Products

307.4902	Extraction Products

307.4903	Chemical Synthesis Products

307.4904	Mixing/Compounding and Formulation

307.4905	Research (Repealed)

SUBPART BP:  DENTAL OFFICES

Section

307.5100	Dental Offices

SUBPART BQ:  TRANSPORTATION EQUIPMENT CLEANING

Section

307.5200	General Provisions

307.5201	Tank Trucks and Intermodal Tank Containers Transporting Chemical and Petroleum Cargos

307.5202	Rail Tank Cars Transporting Chemical and Petroleum Cargos

307.5203	Tank Barges and Ocean/Sea Tankers Transporting Chemical and Petroleum Cargos

307.5204	Tanks Transporting Food Grade Cargos

SUBPART BR:  PAVING AND ROOFING MATERIALS (TARS AND ASPHALT)

Section

307.5301	Asphalt Emulsion

307.5302	Asphalt Concrete

307.5303	Asphalt Roofing

307.5304	Linoleum and Printed Asphalt Felt

SUBPART BS:  WASTE COMBUSTORS

Section

307.5401	Commercial Hazardous Waste Combustor

SUBPART BT:  LANDFILLS

Section

307.5500	General Provisions

307.5501	RCRA Subtitle C Hazardous Waste Landfill

307.5502	RCRA Subtitle D Non-Hazardous Waste Landfill

SUBPART BU:  PAINT FORMULATING

Section

307.5601	Oil-Base Solvent Wash Paint

SUBPART BV:  INK FORMULATING

Section

307.5701	Oil-Base Solvent Wash Ink

SUBPART CD:  PESTICIDE CHEMICALS

Section

307.6500	General Provisions

307.6501	Organic Pesticide Chemicals Manufacturing

307.6502	Metallo-Organic Pesticides Chemicals Manufacturing

307.6503	Pesticide Chemicals Formulating and Packaging

307.6505	Repackaging of Agricultural Pesticides Performed at Refilling Establishments

SUBPART CG:  CARBON BLACK MANUFACTURING

Section

307.6801	Carbon Black Furnace Process

307.6802	Carbon Black Thermal Process

307.6803	Carbon Black Channel Process

307.6804	Carbon Black Lamp Process

SUBPART CJ:  BATTERY MANUFACTURING

Section

307.7100	General Provisions

307.7101	Cadmium

307.7102	Calcium

307.7103	Lead

307.7104	Leclanche

307.7105	Lithium

307.7106	Magnesium

307.7107	Zinc

SUBPART CL:  PLASTICS MOLDING AND FORMING

Section

307.7300	General Provisions

307.7301	Contact Cooling and Heating Water

307.7302	Cleaning Water

307.7303	Finishing Water

SUBPART CM:  METAL MOLDING AND CASTING

Section

307.7400	General Provisions

307.7401	Aluminum Casting

307.7402	Copper Casting

307.7403	Ferrous Casting

307.7404	Zinc Casting

SUBPART CN:  COIL COATING

Section

307.7500	General Provisions

307.7501	Steel Basis Material

307.7502	Galvanized Basis Material

307.7503	Aluminum Basis Material

307.7504	Canmaking

SUBPART CO:  PORCELAIN ENAMELING

Section

307.7600	General Provisions

307.7601	Steel Basis Material

307.7602	Cast Iron Basis Material

307.7603	Aluminum Basis Material

307.7604	Copper Basis Material

SUBPART CP:  ALUMINUM FORMING

Section

307.7700	General Provisions

307.7701	Rolling With Neat Oils

307.7702	Rolling With Emulsions

307.7703	Extrusion

307.7704	Forging

307.7705	Drawing With Neat Oils

307.7706	Drawing With Emulsions or Soaps

SUBPART CQ:  COPPER FORMING

Section

307.7800	General Provisions

307.7801	Copper Forming

307.7802	Beryllium Copper Forming

SUBPART CR:  ELECTRICAL AND ELECTRONIC COMPONENTS

Section

307.7901	Semiconductor

307.7902	Electronic Crystals

307.7903	Cathode Ray Tube

307.7904	Luminescent Materials

SUBPART CT:  NONFERROUS METALS FORMING AND METAL POWDERS

Section

307.8100	General Provisions

307.8101	Lead-Tin-Bismuth Forming

307.8102	Magnesium Forming

307.8103	Nickel-Cobalt Forming

307.8104	Precious Metals Forming

307.8105	Refractory Metals Forming

307.8106	Titanium Forming

307.8107	Uranium Forming

307.8108	Zinc Forming

307.8109	Zirconium-Hafnium Forming

307.8110	Metal Powders

307.APPENDIX A	References to Previous Rules (Repealed)

AUTHORITY:  Implementing Sections 7.2, 13, and 13.3 and authorized by Section 27 of the Environmental Protection Act [415 ILCS 5/7.2, 13, 13.3, and 27].

SOURCE:  Adopted in R70-5, March 31, 1971; amended in R70-8/R71-14/R71-20, March 7, 1972; amended in R74-3, October 30, 1975; amended in R74-15/R74-16 at 2 Ill. Reg. 44, p. 151, effective November 2, 1978; amended in R76-17 at 2 Ill. Reg. 45, p. 101, effective November 5, 1978; amended in R76-21 at 6 Ill. Reg. 563, effective December 24, 1981; codified at 6 Ill. Reg. 7818; amended in R82-5/R82-10 at 8 Ill. Reg. 1625, effective January 18, 1984; amended in R86-44 at 12 Ill. Reg. 2592, effective January 13, 1988; amended in R88-11 at 12 Ill. Reg. 13094, effective July 29, 1988; amended in R88-18 at 13 Ill. Reg. 1794, effective January 31, 1989; amended in R89-3 at 13 Ill. Reg. 19288, effective November 17, 1989; amended in R88-9 at 14 Ill. Reg. 3100, effective February 20, 1990; amended in R89-12 at 14 Ill. Reg. 7620, effective May 8, 1990; amended in R91-5 at 16 Ill. Reg. 7377, effective April 27, 1992; amended in R93-2 at 17 Ill. Reg. 19483, effective October 29, 1993; amended in R94-10 at 19 Ill. Reg. 9142, effective June 23, 1995; amended in R95-22 at 20 Ill. Reg. 5549, effective April 1, 1996; amended in R97-23 at 21 Ill. Reg. 11930, effective August 12, 1997; amended in R99-4 at 23 Ill. Reg. 4413, effective March 31, 1999; amended in R99-17 at 23 Ill. Reg. 8421, effective July 12, 1999; amended in R00-15 at 24 Ill. Reg. 11640, effective July 24, 2000; amended in R01-5 at 25 Ill. Reg. 1735, effective January 11, 2001; amended in R01-25 at 25 Ill. Reg. 10867, effective August 14, 2001; amended in R03-13 at 27 Ill. Reg. 15095, effective September 10, 2003; amended in R04-1 at 28 Ill. Reg. 3076, effective February 6, 2004; amended in R04-18 at 28 Ill. Reg. 10661, effective July 13, 2004; amended in R05-4/R05-15 at 29 Ill. Reg. 6921, effective April 26, 2005; amended in R06-13 at 30 Ill. Reg. 17811, effective October 26, 2006; amended in R08-5/R08-7/R08-13 at 32 Ill. Reg. 18986, effective November 26, 2008; amended in R13-7 at 37 Ill. Reg. 1936, effective February 4, 2013; amended in R16-9 and R17-8 at 41 Ill. Reg. 1129, effective January 23, 2017; amended in R18-6/R18-14 at 42 Ill. Reg. 10676, effective May 29, 2018; amended in R21-15 at 45 Ill. Reg. 8035, effective June 21, 2021; amended in R18-23 at 47 Ill. Reg. 4662, effective March 23, 2023; amended in R24-6 at 48 Ill. Reg. 8106, effective May 31, 2024.

### SUBPART A: GENERAL PROVISIONS

#### Section 307.101 	Preamble (Renumbered)

(Source: Section 307.101 renumbered to Section 307.1001 at 12 Ill.  Reg.  2592, effective January 13, 1988.)

#### Section 307.102 	General Requirements (Renumbered)

(Source: Section 307.102 renumbered to Section 307.1101 at 12 Ill.  Reg.  2592, effective January 13, 1988)

#### Section 307.103 	Mercury (Renumbered)

(Source: Section 307.103 renumbered to Section 307.1102 at 12 Ill.  Reg.  2592, effective January 13, 1988)

#### Section 307.104 	Cyanide (STORET number 00720) (Renumbered)

(Source: Section 307.104 renumbered to Section 307.1103 at 12 Ill.  Reg.  2592, effective January 13, 1988)

#### Section 307.105 	Pretreatment Requirements (Repealed)

(Source: Repealed at 12 Ill.  Reg.  2592, effective January 13, 1988)

Section 307.1001  Preamble

a)	This Part places certain restrictions on the types, concentrations, and quantities of contaminants that can be discharged into sewer systems in the State.

1)	Subpart B includes standards for the discharge of contaminants to sewer systems.  These apply to dischargers to publicly owned treatment works (POTWs) and to dischargers to other types of treatment works, as specified in each Section.

2)	Subparts F through CT include standards for the discharge of contaminants from certain industrial source categories into POTWs.

b)	35 Ill. Adm. Code 310 specifies requirements for pretreatment programs for POTWs.

c)	This Part incorporates federal regulations by reference.

1)	The incorporations include no later amendments or editions.

2)	Except where the contrary is clearly indicated, the Board intends to set forth all procedural requirements in full in this Part and 35 Ill. Adm. Code 310 and to use only the definitions, requirements, or standards from the incorporated material.

3)	Except where the contrary is clearly indicated, references to other federal regulations within incorporated material are to be construed as referencing Board rules derived from the referenced material rather than referencing the other federal regulation.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

#### Section 307.1002 	Definitions

a)	The definitions of 35 Ill.  Adm.  Code 301 do not apply to this Part.

b)	The definitions of 35 Ill.  Adm.  Code 310 apply to this Part.

(Source: Added at 12 Ill.  Reg.  2592, effective January 13, 1988)

Section 307.1003  Test Procedures for Measurement

The test procedures for measurement prescribed at 40 CFR 136 and incorporated by reference in 35 Ill. Adm. Code 310.107 apply to expressions of pollutant amounts, characteristics, or properties in pretreatment standards in this Part, unless otherwise noted or defined in individual Subparts.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.1005	Toxic Pollutants

a)	The Board incorporates by reference 40 CFR 401.15 (2005).  This incorporation includes no later amendments or editions.

b)	A “toxic pollutant” is one of the materials listed in 40 CFR 401.15 or in table II or III in appendix D to 40 CFR 122, incorporated by reference in 35 Ill. Adm. Code 310.107.

(Source: Amended at 30 Ill. Reg. 17811, effective October 26, 2006)

Section 307.1006  Electronic Reporting

The filing of any document under any provision of this Part as an electronic document is subject to 35 Ill. Adm. Code 310.106.

BOARD NOTE:  Derived from 40 CFR 3, as added, and 40 CFR 403.8(g) (2005), as amended at 70 Fed. Reg. 59848 (Oct. 13, 2005).

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART B: GENERAL AND SPECIFIC PRETREATMENT REQUIREMENTS

Section 307.1101  General and Specific Requirements

No person may introduce the following types of pollutants into a POTW:

a)	General Requirements

1)	Pollutants that pass through the POTW; or

2)	Pollutants that interfere with the operation or performance of the POTW.

b)	Specific Requirements

1)	Pollutants that create a fire or explosion hazard within the POTW, including, but not limited to, waste streams with a closed cup flashpoint of less than 60 °C (140 °F) using the test methods specified in 35 Ill. Adm. Code 721.121;

2)	Pollutants that would cause safety hazards to the personnel operating the treatment works;

3)	Pollutants that will cause corrosive damage to the POTW;

4)	Pollutants that would be injurious in any other way to sewers, treatment works, or structures;

5)	Discharges with a pH less than 5.0, unless the POTW is specifically designed to accommodate such discharges;

6)	Solid or viscous pollutants in amounts that will obstruct the flow in the POTW resulting in interference;

7)	Any pollutant, including oxygen-demanding pollutants, at a flow rate or concentration that will cause interference with the POTW;

8)	Heat in amounts that will inhibit biological activity in the POTW and interfere with the POTW;

9)	Heat in amounts that result in temperatures in the influent to the POTW treatment plant greater than 40 ºC (104 ºF), unless the Agency approves alternate temperature limits in the pretreatment plan;

10)	Pollutants that would cause the effluent from the treatment works to violate applicable effluent standards;

11)	Petroleum oil, non-biodegradable cutting oil, or products of mineral oil origin in amounts that will cause interference or pass through;

12)	Pollutants that result in the presence of toxic gases, vapors, or fumes within the POTW in a quantity that may cause acute worker health and safety problems; or

13)	Any trucked or hauled pollutants, except at discharge points designated by the POTW.

BOARD NOTE: Derived from 40 CFR 403.3 and 403.5 (2005).

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.1102  Mercury

a)	Except as provided in this Section, no person may cause or allow the concentration of mercury in any discharge to a publicly owned or publicly regulated sewer system to exceed 0.0005 mg/L, subject to the averaging rule contained in 35 Ill. Adm. Code 304.104(a).

b)	Subsection (a) does not apply if the discharge is to a publicly owned or publicly regulated sewer system that is required to meet a limitation less stringent than the 0.0005 mg/L mercury concentration, in which case the discharge limitation must be the same as that applicable to the publicly owned or regulated sewer system to which it discharges.

c)	Subsection (a) does not apply if all the following conditions are met:

1)	The discharger does not use mercury, the discharger uses mercury and this use cannot be eliminated, or the discharger uses mercury only in chemical analysis or in laboratory or other equipment and takes reasonable care to avoid contamination of wastewater;

2)	The discharge's mercury concentration is less than 0.003 mg/L, as determined by applying the averaging rules of 35 Ill. Adm. Code 304.104(a);

3)	The discharger is providing the best degree of treatment consistent with technological feasibility, economic reasonableness, and sound engineering judgment.  This may include no treatment for mercury; and

4)	The discharger has an inspection and maintenance program likely to reduce or prevent an increase in the level of mercury discharges.

d)	The discharge of wastes from medicinal or therapeutic use of mercury, exclusive of laboratory use, will be exempt from the limitations of subsection (a) if all the following conditions are met:

1)	The total plant discharge is less than 227 g (one-half pound) as Hg in any year;

2)	This discharge is to a public sewer system; and

3)	The discharge does not, alone or in conjunction with other sources, cause the effluent from the sewer system or treatment plant to exceed 0.0005 mg/L of mercury.

e)	No person may cause or allow any discharge of mercury to a publicly owned or publicly regulated sewer system that, alone or in combination with other sources, causes the sewer treatment plant discharge to violate the water quality standard of 35 Ill. Adm. Code 302 for mercury applicable in the receiving stream.

f)	For purposes of permit issuance, the Agency may consider applying the exception under subsection (b) or (c) to determine compliance with this Section.  The Agency may impose permit conditions necessary or required to assure continued application of the exception.  When subsection (b) or (c) applies, the Agency may impose an effluent limitation in the permit that allows the discharge of a concentration of mercury greater than 0.0005 mg/L but not more than 0.003 mg/L.

g)	The mercury standards of Section 307.1102 will not apply to the Reilly Industries, Inc., Granite City, which discharges to any publicly-owned treatment works that receives such a manufacturing facility's wastewater.  The amount of mercury discharged by any such manufacturing facility must not exceed a monthly average of 0.025 mg/L nor a maximum of 0.035 mg/L or 7.5 grams per day, subject to the Board's averaging rules during any one day.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.1103  Cyanide

a)	Waste to any public sewer system must not contain more than 10 mg/L total cyanide, and any sample tested must not release more than 2 mg/L of cyanide when tested at a pH of 4.5 and a temperature of 66 ºC (150 ºF) for 30 minutes, except as permitted by subsection (b).

b)	Upon application by a county, municipality, sanitary district, or public utility and approval by the Agency, based upon the Agency's determination that no violation of the effluent standards of 35 Ill. Adm. Code 304 or hazard to workers in that sewage works will result, limited additional amounts of cyanide exceeding the standards in subsection (a) may be discharged to the sewage works of that county, sanitary district, municipality, or public utility.

c)	Nothing in this Section may be construed as limiting the authority of any county, municipality, sanitary district, or public utility to impose any more stringent standards or limitations on cyanide discharges to its sewage works.

d)	Any actions undertaken under subsection (b) will be subject to the limitations of Section 307.2400(b)(7).

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART F: DAIRY PRODUCTS PROCESSING

Section 307.1501  Receiving Stations

a)	Applicability.  This Section applies to discharges resulting from the operation of receiving stations engaged in the assembly and reshipment of bulk milk for the use of manufacturing or processing plants.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 405.11 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 405.14 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 405.16 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 20, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.1502  Fluid Products

a)	Applicability.  This Section applies to discharges resulting from the manufacture of market milk (ranging from 3.5 percent fat to fat-free), flavored milk (chocolate and others), and cream (of various fat concentrations, plain, and whipped).

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 405.21 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 405.24 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 405.26 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 20, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.1503  Cultured Products

a)	Applicability.  This Section applies to discharges resulting from the manufacture of cultured products, including cultured skim milk (cultured buttermilk), yogurt, sour cream, and dips of various types.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 405.31 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 405.34 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 405.36 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	“New source” means any building, structure, facility, or installation whose construction commenced after December 20, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

#### Section 307.1504	Butter

a)	Applicability.  This Section applies to discharges resulting from the manufacture of butter, either by churning or continuous process.

b)	Specialized definitions.  The Board incorporates by reference 40 CFR 405.41 (2003).  This incorporation includes no later amendments or editions.

c)	Existing sources.

1)	The Board incorporates by reference 40 CFR 405.44 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) of this Section may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of such standards.

d)	New sources.

1)	The Board incorporates by reference 40 CFR 405.46 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) of this Section may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of such standards.

3)	“New source” means any building, structure, facility, or installation the construction of which commenced after December 20, 1973.

(Source:  Amended at 28 Ill. Reg. 3076, effective February 6, 2004)

Section 307.1504  Butter

a)	Applicability.  This Section applies to discharges resulting from the manufacture of butter, either by churning or continuous process.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 405.41 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 405.44 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 405.46 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 20, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.1505  Cottage Cheese and Cultured Cream Cheese

a)	Applicability.  This Section applies to discharges resulting from the manufacture of cottage cheese and cultured cream cheese.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 405.51 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 405.54 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 405.56 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 20, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.1506  Natural and Processed Cheese

a)	Applicability.  This Section applies to discharges resulting from the manufacture of natural cheese (hard curd) and processed cheese.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 405.61 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 405.64 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 405.66 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 20, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.1507  Fluid Mix for Ice Cream and Other Frozen Desserts

a)	Applicability.  This Section applies to discharges resulting from the manufacture of fluid mixes for ice cream and other frozen desserts for later freezing in other plants; it does not include freezing of the products as one of the affected operations.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 405.71 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 405.74 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 405.76 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 20, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.1508  Ice Cream, Frozen Desserts, Novelties, and Other Dairy Desserts

a)	Applicability.  This Section applies to discharges resulting from the manufacture of ice cream, ice milk, sherbet, water ices, stick confections, frozen novelties products, frozen desserts, mellorine, pudding, and other dairy product base desserts.  If fluid mixes prepared at another plant are employed, the appropriate values from Section 307.1507 should be deducted from the limitations.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 405.81 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 405.84 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 405.86 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 20, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.1509  Condensed Milk

a)	Applicability.  This Section applies to discharges resulting from the manufacture of condensed whole milk, condensed skim milk, sweetened condensed milk, and condensed buttermilk.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 405.91 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 405.94 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 405.96 (2003).  This incorporation includes no later amendments or editions.

2	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 20, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.1510  Dry Milk

a)	Applicability.  This Section applies to discharges resulting from the manufacture of dry whole milk, dry skim milk, and dry buttermilk.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 405.101 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 405.104 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 405.106 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 20, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.1511  Condensed Whey

a)	Applicability.  This Section applies to discharges resulting from the manufacture of condensed sweet whey and condensed acid whey.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 405.111 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 405.114 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 405.116 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 20, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.1512  Dry Whey

a)	Applicability.  This Section applies to discharges resulting from the manufacture of sweet or acid dry whey.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 405.121 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 405.124 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 405.126 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 20, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART G: GRAIN MILLS

Section 307.1601  Corn Wet Milling

a)	Applicability.  This Section applies to discharges resulting from the process in which shelled corn is steeped in a dilute solution of sulfurous acid and then processed by wet means into such products as animal feed, regular and modified starches, corn oil, corn syrup, and dextrose.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 406.11 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 406.14 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 406.16 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 4, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.1602  Corn Dry Milling

a)	Applicability.  This Section applies to discharges resulting from the process in which shelled corn is washed and subsequently milled by dry processes into such products as cornmeal, grits, flour, oil, and animal feed.  This Section does not apply to discharges from subsequent manufacturing operations to produce expanded or extruded feed or feed products.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 406.21 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 406.24 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 406.26 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 4, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.1603  Normal Wheat Flour Milling

a)	Applicability.  This Section applies to discharges resulting from the processes in which wheat and other grains are milled by dry processes into flour and millfeed.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 406.31 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 406.34 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 406.36 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 4, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.1604  Bulgur Wheat Flour Milling

a)	Applicability.  This Section applies to discharges resulting from the process in which wheat is parboiled, dried, and partially debranned in the production of bulgur.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 406.41 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 406.44 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 406.46 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 4, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.1605  Normal Rice Milling

a)	Applicability.  This Section applies to discharges resulting from the process in which rice is cleaned and milled by dry processes.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 406.51 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 406.54 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 406.56 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 4, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.1606  Parboiled Rice Milling

a)	Applicability.  This Section applies to discharges resulting from the process in which rice is cleaned, cooked, and dried before being milled.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 406.61 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 406.64 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 406.66 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 4, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.1607  Animal Feed

a)	Applicability.  This Section applies to discharges resulting from the manufacturing of animal feeds (formula feed concentrate) using primarily grain and grain by-products that may be supplemented by proteins, pharmaceuticals, vitamins, or mineral additives.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 406.71 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 406.76 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after September 17, 1974.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.1608  Hot Cereal

a)	Applicability.  This Section applies to discharges resulting from the production of various breakfast cereals from grains, principally wheat and oats, requiring cooking before normal human consumption.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 406.81 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 406.86 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after September 17, 1974.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.1609  Ready-to-Eat Cereal

a)	Applicability.  This Section applies to discharges resulting from the processing of various grains and other materials (whole grain wheat, rice, corn grits, oat flour, sugar, and minor ingredients) to produce various breakfast cereals normally available for human consumption without cooking.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 406.91 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 406.96 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after September 17, 1974.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.1610  Wheat Starch and Gluten

a)	Applicability.  This Section applies to discharges resulting from those industrial operations using wheat flour as a raw material for the production of wheat starch and gluten (protein) components through conventional processes of physical separation and subsequent refinement.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 406.101 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 406.106 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after September 17, 1974.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART H:  CANNED AND PRESERVED FRUITS AND VEGETABLES

#### Section 307.1700	General Provisions

Proration.  When a plant is subject to limitations covering more than one subcategory, the plant discharge limitation must be set by proration limitations for each subcategory based on the total raw material covered by each subcategory.

(Source:  Amended at 28 Ill. Reg. 3076, effective February 6, 2004)

Section 307.1701  Apple Juice

a)	Applicability.  This Section applies to discharges resulting from the processing of apples into apple juice or apple cider.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 407.11 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 407.14 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 407.16 (2003). This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after November 9, 1974.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.1702  Apple Products

a)	Applicability.  This Section applies to discharges resulting from the processing of apples into apple products.  The processing of apples into caustic-peeled or dehydrated products is specifically excluded.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 407.21 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 407.24 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 407.26 (2003). This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after November 9, 1974.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.1703  Citrus Products

a)	Applicability.  This Section applies to discharges resulting from the processing of citrus into citrus products.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 407.31 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 407.34 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 407.36 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after November 9, 1974.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.1704  Frozen Potato Products

a)	Applicability.  This Section applies to discharges resulting from the processing of white potatoes into frozen potato products.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 407.41 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 407.44 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 407.46 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after November 9, 1974.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.1705  Dehydrated Potato Products

a)	Applicability.  This Section applies to discharges resulting from the processing of white potatoes into dehydrated potato products.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 407.51 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 407.54 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 407.56 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after November 9, 1974.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.1706  Canned and Preserved Fruits

a)	Applicability.  This Section applies to discharges resulting from the processing of the following fruit products:  apricots; caneberries; sweet, sour, and brined cherries; cranberries; dried fruit; grape juice canning and pressing; olives; peaches; pears; fresh and processed pickles and pickle salting stations; pineapples; plums; raisins; strawberries; and tomatoes.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 407.61 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 407.64 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 407.66 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 21, 1975.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.1707  Canned and Preserved Vegetables

a)	Applicability.  This Section applies to discharges resulting from the processing of the following vegetable products:  beets, broccoli, carrots, canned and frozen corn, dehydrated onions and garlic, dehydrated vegetables, dry beans, lima beans, mushrooms, canned onions, peas, sauerkraut canning and cutting, snap beans, spinach, squash, and canned potatoes.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 407.71 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 407.74 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 407.76 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 21, 1975.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.1708  Canned and Miscellaneous Specialties

a)	Applicability.  This Section applies to discharges resulting from the processing of the following specialty products:  added ingredients; baby food; corn, potato, and tortilla chips; ethnic foods; jams and jellies; mayonnaise and dressings; soups; and tomato-starch-cheese canned specialties.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 407.81 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 407.84 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 407.86 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 21, 1975.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART I:  CANNED AND PRESERVED SEAFOOD

Section 307.1801  Farm-Raised Catfish

a)	Applicability.  This Section applies to discharges resulting from the processing of farm-raised catfish by existing facilities that process more than 1,362 kg (3,000 lb) of raw material per day on any day during a calendar year and all new sources.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 408.11 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 408.l4 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 408.16 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after February 6, 1974.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.1815  Fish Meal Processing Subcategory (Repealed)

(Source:  Repealed at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART J:  SUGAR PROCESSING

Section 307.1901  Beet Sugar Processing

a)	Applicability.  This Section applies to discharges resulting from any operation attendant to the processing of sugar beets for the production of sugar.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 409.11 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 409.14 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 409.16 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after August 22, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.1902  Crystalline Cane Sugar Refining

a)	Applicability.  This Section applies to discharges resulting from the processing of raw cane sugar into crystalline refined sugar.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 409.21 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 409.24 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 409.26 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 7, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.1903  Liquid Cane Sugar Refining

a)	Applicability.  This Section applies to discharges resulting from the processing of raw cane sugar into liquid refined sugar.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 409.31 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 409.34 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 409.36 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 7, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART K: TEXTILE MILLS

#### Section 307.2000	General Provisions

a)	Applicability.  This Subpart K applies to any textile mill or textile processing facility that introduces or may introduce process wastewater pollutants into a POTW.

b)	General definitions.  The Board incorporates by reference 40 CFR 410.01 (2003).  This incorporation includes no later amendments or editions.

(Source:  Amended at 28 Ill. Reg. 3076, effective February 6, 2004)

Section 307.2001  Wool Scouring

a)	Applicability.  This Section applies to discharges resulting from the following types of textile mills: wool scouring, topmaking, and general cleaning of raw wool.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 410.11 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 410.14 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 410.16 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 10, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2002  Wool Finishing

a)	Applicability.  This Section applies to discharges resulting from the following types of textile mills:  wool finishers, including carbonizing, fulling, dyeing, bleaching, rinsing, fireproofing, and other similar processes.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 410.21.  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 410.24 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 410.26 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 10, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2003  Low Water Use Processing

a)	Applicability.  This Section applies to discharges resulting from the following types of textile mills: yarn manufacture, yarn texturizing, unfinished fabric manufacture, fabric coating, fabric laminating, tire cord and fabric dipping, and carpet tufting and carpet backing.  Rubberized or rubber-coated fabrics regulated by 40 CFR 428 are specifically excluded.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 410.31 (2005).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 410.34 (2005).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 410.36 (2005).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 10, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2004  Woven Fabric Finishing

a)	Applicability.  This Section applies to discharges resulting from the following types of textile mills:  woven fabric finishers, which may include any or all of the following unit operations:  desizing, bleaching, mercerizing, dyeing, printing, resin treatment, waterproofing, flameproofing, soil repellency application, and a special finish application.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 410.41 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 410.44 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 410.46 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 10, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2005  Knit Fabric Finishing

a)	Applicability.  This Section applies to discharges resulting from the following types of textile mills:  knit fabric finishers, which may include any or all of the following unit operations:  bleaching, mercerizing, dyeing, printing, resin treatment, waterproofing, flameproofing, soil repellency application, and a special finish application.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 410.51 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 410.54 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 410.56 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 10, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2006  Carpet Finishing

a)	Applicability.  This Section applies to discharges resulting from the following types of textile mills:  carpet mills, which may include any or all of the following unit operations:  bleaching, scouring, carbonizing, fulling, dyeing, printing, resin treatment, waterproofing, flameproofing, soil repellency, looping, and backing with foamed and unfoamed latex and jute.  Carpet backing without other carpet manufacturing operations is included in Section 307.2003.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 410.61 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 410.64 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 410.66 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 10, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2007  Stock and Yarn Finishing

a)	Applicability.  This Section applies to discharges resulting from the following types of textile mills:  stock or yarn dyeing or finishing, which may include any or all of the following unit operations and processes:  cleaning, scouring, bleaching, mercerizing, dyeing, and special finishing.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 410.74 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 410.76 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 10, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2008  Nonwoven Manufacturing

a)	Applicability.  This Section applies to discharges resulting from facilities that primarily manufacture nonwoven textile products of wool, cotton, or synthetics, singly or as blends, by mechanical, thermal, or adhesive bonding procedures.  Nonwoven products produced by fulling and felting processes are covered in Section 307.2009.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 410.84 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 410.86 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 10, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2009  Felted Fabric Processing

a)	Applicability.  This Section applies to discharges resulting from facilities that primarily manufacture nonwoven products by employing fulling and felting operations as a means of achieving fiber bonding.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 410.94 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 410.96 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 10, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART L:  CEMENT MANUFACTURING

Section 307.2101  Nonleaching

a)	Applicability.  This Section applies to discharges resulting from the process in which several mineral ingredients (limestone or other natural sources of calcium carbonate, silica, alumina, and iron together with gypsum) are used in the manufacturing of cement and in which kiln dust is not contacted with water as an integral part of the process and water is not used in wet scrubbers to control kiln stack emissions.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 411.11 (2003).   This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 411.14 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 411.16 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after September 7, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2102  Leaching

a)	Applicability.  This Section applies to discharges resulting from the process in which several mineral ingredients (limestone or other natural sources of calcium carbonate, silica, alumina, and iron together with gypsum) are used in the manufacturing of cement and in which kiln dust is contacted with water as an integral part of the process or water is used in wet scrubbers to control kiln stack emissions.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 411.21 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 411.24 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 411.26 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after September 7, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2103  Materials Storage Piles Runoff

a)	Applicability.  This Section applies to discharges resulting from the runoff of rainfall that derives from the storage of materials including raw materials, intermediate products, finished products, and waste materials that are used in or derived from the manufacture of cement under either Section 307.2101 or 307.2102.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 411.31 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 411.34 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 411.36 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after September 7, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023

### SUBPART M:  CONCENTRATED ANIMAL FEEDING OPERATIONS

Section 307.2201   General

a)	Applicability.  This Section applies to manure, litter, or process wastewater discharges resulting from concentrated animal feeding operations (CAFOs).  Manufacturing or agricultural activities that may be subject to this Section are generally reported under one or more of the following SIC codes, as defined in the Standard Industrial Classification Manual, incorporated by reference in 35 Ill. Adm. Code 310.107:

1)	SIC Code 0211 (Beef Cattle Feedlots);

2)	SIC Code 0213 (Hogs);

3)	SIC Code 0214 (Sheep and Goats);

4)	SIC Code 0241 (Dairy Farms);

5)	SIC Code 0251 (Broiler, Fryer, and Roaster Chickens);

6)	SIC Code 0252 (Chicken Eggs);

7)	SIC Code 0253 (Turkeys and Turkey Eggs);

8)	SIC Code 0254 (Poultry Hatcheries);

9)	SIC Code 0259 (Poultry and Eggs, Not Elsewhere Classified); or

10)	SIC Code 0272 (Horses and Other Equines).

b)	General definitions.  The Board incorporates by reference 40 CFR 412.2 (2011).  This incorporation includes no later amendments or editions.

c)	General pretreatment standards.  The Board incorporates by reference 40 CFR 412.3 (2011).  This incorporation includes no later amendments or editions.

d)	A facility is determined a concentrated animal feeding operation (CAFO) according to the definitions and requirements of 40 CFR 122.23(b) and (c), incorporated by reference in 35 Ill. Adm. Code 310.107.

(Source:  Amended at 37 Ill. Reg. 1936, effective February 4, 2013)

Section 307.2202  Ducks

a)	Applicability.  This Section applies to discharges resulting from the production areas at dry lot and wet lot duck CAFOs.  This Section does not apply to such a CAFO with less than the following capacity:  5,000 ducks.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 412.21 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  USEPA removed the former standards applicable to existing sources at 68 Fed. Reg. 7176 (February 12, 2003).

d)	New Sources

1)	The Board incorporates by reference 40 CFR 412.26 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after September 7, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART N: ELECTROPLATING

Section 307.2300  General Provisions

a)	Applicability

1)	This Subpart N applies to any electroplating operations in which metal is electroplated on any basis material and to related metal finishing operations as identified in the various Sections, whether such operations are conducted in conjunction with electroplating, independently, or as part of some other operation.

2)	Operations similar to electroplating that are specifically excepted from coverage of this Subpart N include the following:

A)	Electrowinning and electrorefining conducted as a part of nonferrous metal smelting and refining (Subpart V);

B)	Metal surface preparation and conversion coating conducted as a part of coil coating (Subpart CN);

C)	Metal surface preparation and immersion plating or electroless plating conducted as a part of porcelain enameling (Subpart CO); and

D)	Electrodeposition of active electrode materials, electroimpregnation, and electroforming conducted as a part of battery manufacturing (Subpart CJ).

3)	Metallic platemaking and gravure cylinder preparation conducted within or for printing and publishing facilities, and continuous strip electroplating conducted within iron and steel manufacturing facilities are exempted from the pretreatment standards for existing sources in this Subpart N.

4)	Certain electroplating operations may be subject to the metal finishing standards of Subpart BH.

b)	General Definitions.  The Board incorporates by reference 40 CFR 413.02 (2003).  This incorporation includes no later amendments or editions.

c)	Monitoring Requirements.  The Board incorporates by reference 40 CFR 413.03 (2003).  This incorporation includes no later amendments or editions.

d)	Compliance Dates.  The Board incorporates by reference 40 CFR 413.01(a) (2003).  This incorporation includes no later amendments or editions.

e)	Integrated Facilities.  The Board incorporates by reference 40 CFR 413.04 (2003).  This incorporation includes no later amendments or editions.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2301  Electroplating of Common Metals

a)	Applicability.  This Section applies to discharges resulting from the process in which a ferrous or nonferrous basis material is electroplated with copper, nickel, chromium, zinc, tin, lead, cadmium, iron, aluminum, or any combination of these.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 413.11 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 413.14 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	Sources whose construction commenced after August 31, 1982 are subject to Subpart BH.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2302  Electroplating of Precious Metals

a)	Applicability.  This Section applies to discharges resulting from the process in which a ferrous or nonferrous basis material is plated with gold, silver, iridium, palladium, platinum, rhodium, ruthenium, or any combination of these.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 413.21 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 413.24 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of such standards.

d)	Sources whose construction commenced after August 31, 1982 are subject to Subpart BH.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2304  Anodizing

a)	Applicability.  This Section applies to discharges resulting from the anodizing of ferrous or nonferrous materials.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 413.41 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 413.44 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	Sources whose construction commenced after August 31, 1982 are subject to Subpart BH.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2305  Coatings

a)	Applicability.  This Section applies to discharges resulting from the chromating, phosphating, or immersion plating on ferrous or nonferrous materials.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 413.51 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 413.54 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	Sources whose construction commenced after August 31, 1982 are subject to Subpart BH.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2306  Chemical Etching and Milling

a)	Applicability.  This Section applies to discharges resulting from the chemical milling or etching of ferrous or nonferrous materials.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 413.61 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 413.64 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	Sources whose construction commenced after August 31, 1982 are subject to Subpart BH.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2307  Electroless Plating

a)	Applicability.  This Section applies to discharges resulting from the electroless plating of a metallic layer on a metallic or nonmetallic substrate.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 413.71 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 413.74 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	Sources whose construction commenced after August 31, 1982 are subject to Subpart BH.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2308  Printed Circuit Boards

a)	Applicability.  This Section applies to discharges resulting from the manufacture of printed circuit boards, including all manufacturing operations required or used to convert an insulating substrate to a finished printed circuit board.  The other Sections of this Subpart N do not apply to the manufacture of printed circuit boards.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 413.81 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 413.84 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	Sources whose construction commenced after August 31, 1982 are subject to Subpart BH.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART O:  ORGANIC CHEMICALS, PLASTICS, AND SYNTHETIC FIBERS

Section 307.2400  General Provisions

a)	General Definitions.  The Board incorporates by reference 40 CFR 414.10 (2005).  This incorporation includes no later amendments or editions.

b)	Applicability

1)	This Subpart O applies to process wastewater discharges from all establishments or portions of establishments that manufacture the organic chemicals, plastics, and synthetic fibers (OCPSF) products or product groups that are covered by Sections 307.2402 through 307.2408 and which are included in the following SIC major groups, as defined in the Standard Industrial Classification Manual, incorporated by reference in 35 Ill. Adm. Code 310.107:

A)	SIC 2821:  Plastic materials, synthetic resins, and nonvulcanizable elastomers.

B)	SIC 2823:  Cellulosic man-made fibers.

C)	SIC 2824:  Synthetic organic fibers, except cellulosic.

D)	SIC 2865:  Cyclic crudes and intermediates, dyes, and organic pigments.

E)	SIC 2869:  Industrial organic chemicals, not elsewhere classified.

2)	This Subpart O applies to wastewater discharges from OCPSF research and development, pilot plant, technical service, and laboratory bench-scale operations if such operations are conducted in conjunction with and related to existing OCPSF manufacturing activities at the plant site.

3)	Notwithstanding subsection (b)(1), this Subpart O does not apply to discharges resulting from the manufacture of OCPSF products if the products are included in the following SIC subgroups, as defined in the Standard Industrial Classification Manual, incorporated by reference in 35 Ill. Adm. Code 310.107, and if the products have in the past been reported by the establishment under these subgroups and not under the SIC groups listed in subsection (b)(1):

A)	SIC 2843085:  Bulk surface active agents.

B)	SIC 28914:  Synthetic resin and rubber adhesives.

C)	Chemicals and Chemical Preparations Not Elsewhere Classified:

i)	SIC 2899568:  Sizes, all types.

ii)	SIC 2899597:  Other industrial chemical specialties, including fluxes, plastic wood preparations, and embalming fluids.

D)	SIC 2911058:  Aromatic hydrocarbons manufactured from purchased refinery products.

E)	SIC 2911632:  Aliphatic hydrocarbons manufactured from purchased refinery products.

4)	Notwithstanding subsection (b)(1), this Subpart O does not apply to any discharges for which a different set of previously promulgated standards in this Part apply, unless the facility reports OCPSF products under SIC codes 2865, 2869, or 2821, as defined in the Standard Industrial Classification Manual, incorporated by reference in 35 Ill. Adm. Code 310.107, and the facility's OCPSF wastewaters are discharged separately to a POTW.

5)	This Subpart O does not apply to any process wastewater discharge from the manufacture of organic chemical compounds solely by extraction from plant and animal raw materials or by fermentation processes.

6)	Discharges of chromium, copper, lead, nickel, and zinc in “complexed metal-bearing waste streams,” listed in Section 307.2491, are not subject to this Subpart O.

7)	Non-Amenable Cyanide

A)	Discharges of cyanide in “cyanide-bearing waste streams,” listed in Section 307.2490, are not subject to the cyanide limitations of this Subpart O if both of the following occur:

i)	The Control Authority determines that the cyanide limitations are not achievable due to elevated levels of non-amenable cyanide (i.e., cyanide that is not oxidized by chlorine treatment) that result from the unavoidable complexing of cyanide at the process source of the cyanide-bearing waste stream, and

ii)	The control authority establishes an alternative total cyanide or amenable cyanide limitation that reflects the best available technology economically achievable.

B)	The control authority must base its determination made pursuant to subsection (b)(7)(A) on a review of the relevant engineering, production, and sampling and analytical information at its disposal, including measurements of both total and amenable cyanide in the waste stream.

C)	The control authority must set forth its determination made pursuant to subsection (b)(7)(A) in a written analysis of the extent of complexing in the waste stream and its impact on cyanide treatability, based on the information at its disposal.

D)	Alternative cyanide discharge limitation determinations made pursuant to this subsection (b)(7) are subject to the limitations of Section 307.1103.  However, Section 307.1103 may not be used to allow a discharge of total cyanide greater than that otherwise allowed by this subsection (b)(7).

8)	Allowances for Non-Metal-Bearing Waste Streams

A)	The control authority must establish discharge limitations for lead and zinc for waste streams not listed in Section 307.2490 and not otherwise determined to be “metal-bearing waste streams” if it determines that the wastewater metals contamination is due to background levels that are not reasonably avoidable, from such sources as intake water, corrosion of materials of construction, or contamination of raw materials.

B)	The control authority must base its determination made pursuant to subsection (b)(8)(A) on a review of relevant plant operating conditions, process chemistry, engineering, and sampling and analytical information.

C)	The control authority must set forth its determination made pursuant to subsection (b)(8)(A) in a written analysis of the sources and levels of the metals, based on the information at its disposal.

D)	The control authority may establish limitations for lead and zinc for non-metal-bearing waste streams for the purposes of subsection (b)(8)(A) between the following levels:

i)	The lowest level that the control authority determines, based on best professional judgment, can be reliably measured; and

ii)	The concentration of such metals present in the waste streams, but not to exceed the applicable limitations contained in Sections 307.2401 through 307.2407.

iii)	For zinc, the applicable limitations that the discharge must not exceed are those appearing in the tables in Sections 307.2401 through 307.2407, not the alternative limitations for rayon fiber manufacture by the viscose process, as set forth in footnote 2 to the table in 40 CFR 414.25, incorporated by reference at Section 307.2401(c)(1), or the alternative limitations for acrylic fiber manufacture by the zinc chloride/solvent process, as set forth in footnote 2 to the table in 40 CFR 414.35, each incorporated by reference at Section 307.2402(c)(1).

E)	The limitations for individual dischargers must be set on a mass basis, by multiplying the concentration allowance established by the control authority times the process wastewater flow from the individual waste streams in which incidental metals are present.

c)	Compliance Date.  All dischargers subject to a pretreatment standard for existing sources in this Subpart O must have complied with the standard by November 5, 1990.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2401  Rayon Fibers

a)	Applicability.  This Section applies to discharges of process wastewater resulting from the manufacture of rayon fiber by the viscose process only.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 414.25 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 414.26 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after March 21, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2402  Other Fibers

a)	Applicability.  This Section applies to discharges of process wastewater resulting from the manufacture of the products classified under SIC 2823, cellulosic man-made fibers and fiber groups, except rayon, and under SIC 2824, synthetic organic fibers and fiber groups, as defined in the Standard Industrial Classification Manual, incorporated by reference in 35 Ill. Adm. Code 310.107, listed in this subsection.  Product groups are indicated with an asterisk (*).

*Acrylic fibers (85% Polyacrylonitrile)

*Cellulose acetate fibers

*Fluorocarbon (Teflon) fibers

*Modacrylic fibers

*Nylon 6 fibers

Nylon 6 monofilament

*Nylon 66 fibers

Nylon 66 monofilament

*Polyamide fibers (Quiana)

*Polyaramid (Kevlar) resin-fibers

*Polyaramid (Nomex) resin-fibers

*Polyester fibers

*Polyethylene fibers

*Polypropylene fibers

*Polyurethane fibers (Spandex)

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 414.35 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 414.36 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after March 21, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2403  Thermoplastic Resins

a)	Applicability.  This Section applies to discharges of process wastewater resulting from the manufacture of the products classified under SIC 28213, thermoplastic resins and thermoplastic resin groups, as defined in the Standard Industrial Classification Manual, incorporated by reference in 35 Ill. Adm. Code 310.107, listed in this subsection.  Product groups are indicated with an asterisk (*).

*Abietic acid-derivatives

*ABS resins

*ABS-SAN resins

*Acrylate-methacrylate latexes

*Acrylic latex

*Acrylic resins

*Cellulose acetate butyrates

Cellulose acetate resin

*Cellulose acetates

*Cellulose acetates priopionates

Cellulose nitrate

*Ethylene-methacrylic acid copolymers

*Ethylene-vinyl acetate copolymers

*Fatty acid resins

*Fluorocarbon polymers

Nylon 11 resin

*Nylon 6-66 copolymers

*Nylon 6-Nylon 11 blends

Nylon 6 resin

Nylon 612 resin

Nylon 66 resin

*Nylons

*Petroleum hydrocarbon resins

*Polyvinyl pyrrolidone-copolymers

*Poly(alpha)olefins

Polyacrylic acid

*Polyamides

*Polyarylamides

*Polybutadiene

*Polybutenes

Polybutyl succinic anhydride

*Polycarbonates

*Polyester resins

*Polyester resins, Polybutylene terephthalate

*Polyester resins, Polyoxybenzoate

Polyethylene

*Polyethylene-ethyl acrylate resins

*Polyethylene-polyvinyl acetate copolymers

Polyethylene resin (HDPE)

Polyethylene resin (LDPE)

Polyethylene resin, scrap

Polyethylene resin, wax (low molecular weight)

Polyethylene resin, latex

Polyethylene resins

*Polyethylene resins, compounded

*Polyethylene, chlorinated

*Polyimides

*Polypropylene resins

Polystyrene (crystal)

Polystyrene (crystal) modified

*Polystyrene--copolymers

*Polystyrene-acrylic latexes

Polystyrene impact resins

Polystyrene latex

Polystyrene, expandable

Polystyrene, expanded

*Polysulfone resins

Polyvinyl acetate

*Polyvinyl acetate-PVC copolymers

*Polyvinyl acetate copolymers

*Polyvinyl acetate resins

Polyvinyl alcohol resin

Polyvinyl chloride

Polyvinyl chloride, chlorinated

*Polyvinyl ether-maleic anhydride

*Polyvinyl formal resins

*Polyvinylacetate-methacrylic copolymers

*Polyvinylacetate acrylic copolymers

*Polyvinylacetate-2-ethylhexylacrylate copolymers

Polyvinylidene chloride

*Polyvinylidene chloride copolymers

*Polyvinylidene-vinyl chloride resins

*PVC copolymers, acrylates (Latex)

*PVC copolymers, ethylene-vinyl chloride

*Rosin derivative resins

*Rosin modified resins

*Rosin resins

*SAN resins

*Silicones--silicone resin

*Silicones--silicone rubbers

*Styrene-maleic anhydride resins

Styrene polymeric residue

*Styrene-acrylic copolymer resins

*Styrene-acrylonitrile-acrylates copolymers

*Styrene-butadiene resins

*Styrene-butadiene resins (less than 50% butadiene)

*Styrene-butadiene resins (Latex)

*Styrene-divinylbenzene resins (ion exchange)

*Styrene-methacrylate terpolymer resins

*Styrene-methyl methacrylate copolymers

*Styrene-butadiene-vinyl toluene terpolymers

*Sulfonated styrene-maleic anhydride resins

*Unsaturated polyester resins

*Vinyl toluene resins

*Vinyl toluene-acrylate resins

*Vinyl toluene-butadiene resins

*Vinyl toluene-methacrylate resins

*Vinyl acetate-n-butyl acrylate copolymers

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 414.45 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 414.46 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after March 21, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2404  Thermosetting Resins

a)	Applicability.  This Section applies to discharges of process wastewater resulting from the manufacture of the products classified under SIC 28214, thermosetting resins and thermosetting resin groups, as defined in the Standard Industrial Classification Manual, incorporated by reference in 35 Ill. Adm. Code 310.107, listed in this subsection.  Product groups are indicated with an asterisk (*).

*Alkyd resins

Dicyanodiamide resin

*Epoxy resins

*Fumaric acid polyesters

*Furan resins

Glyoxal-urea formaldehyde textile resin

*Ketone-formaldehyde resins

*Melamine resins

*Phenolic resins

*Polyacetal resins

*Polyacrylamide

*Polyurethane prepolymers

*Polyurethane resins

*Urea formaldehyde resins

*Urea resins

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 414.55 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 414.56 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after March 21, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2405  Commodity Organic Chemicals

a)	Applicability.  This Section applies to discharges of process wastewater resulting from the manufacture of the products classified under SIC 2865 or 2869, commodity organic chemicals and commodity organic chemical groups, as defined in the Standard Industrial Classification Manual, incorporated by reference in 35 Ill. Adm. Code 310.107, listed in this subsection.  Product groups are indicated with an asterisk (*).

1)	Aliphatic Organic Chemicals

Acetaldehyde

Acetic acid

Acetic anhydride

Acetone

Acrylonitrile

Adipic acid

1,3-Butadiene

*Butylenes (Butenes)

Cyclohexane

1,2-Dichloroethane

Ethanol

Ethylene

Ethylene glycol

Ethylene oxide

Formaldehyde

Isopropanol

Methanol

Polyoxypropylene glycol

Propylene

Propylene oxide

Vinyl acetate

2)	Aromatic Organic Chemicals

Benzene

Cumene

Dimethyl terephthalate

Ethylbenzene

Phenol

*Pitch tar residues

Pyrolysis gasolines

Styrene

Terephthalic acid

Toluene

*Xylenes, mixed

o-Xylene

m-Xylene (impure)

p-Xylene

3)	Halogenated Organic Compounds

Vinyl chloride

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 414.65 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 414.66 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	For discharges of wastewater resulting from the manufacture of butadiene by any process that includes the oxidative dehydrogenation of butene, "new source" means any building, structure, facility, or installation whose construction commenced after December 17, 1973.  For other sources, "new source" means any building, structure, facility, or installation whose construction commenced after March 21, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2406  Bulk Organic Chemicals

a)	Applicability.  This Section applies to discharges of process wastewater resulting from the manufacture of the products classified under SIC 2865 or 2869, bulk organic chemicals and bulk organic chemical groups, as defined in the Standard Industrial Classification Manual, incorporated by reference in 35 Ill. Adm. Code 310.107, listed in this subsection.  Product groups are indicated with an asterisk (*).

1)	Aliphatic Organic Chemicals

*Acetic acid esters

*Acetic acid salts

Acetone cyanohydrin

Acetylene

Acrylic acid

*Acrylic acid esters

n-alkanes

*Alkylates

Butane (all forms)

1-Butene

1,4-Butanediol

2-Butene (cis and trans)

n-butyl acetate

n-butyl alcohol

sec-butyl alcohol

tert-butyl alcohol

n-butyraldehyde

n-butyric acid

n-butyric anhydride

C4 hydrocarbons (unsaturated)

Calcium stearate

Caprolactam

Carboxymethyl cellulose

Cellulose acetate butyrates

*Cellulose ethers

Cumene hydroperoxide

Cyclohexanol

Cyclohexanol-cyclohexanone (mixed)

Cyclohexanone

Cyclohexene

*C12-C18 primary alcohols (mixed)

*C5 concentrates

*C9 concentrates

Decanol

Diacetone alcohol

*Dicarboxylic acids-salts

Diethyl ether

Diethylene glycol

Diethylene glycol diethyl ether

Diethylene glycol dimethyl ether

Diethylene glycol monoethyl ether

Diethylene glycol monomethyl ether

*Dimer acids

Dioxane

Ethane

2-Ethylbutyraldehyde

Ethylene glycol monophenyl ether

*Ethoxylates, miscellaneous

Ethylene glycol dimethyl ether

Ethylene glycol monobutyl ether

Ethylene glycol monoethyl ether

Ethylene glycol monomethyl ether

2-Ethylhexanol

Glycerine (synthetic)

Glyoxal

Hexane

*Hexane and other C6 hydrocarbons

Isobutanol

Isobutyl acetate

Isobutylene

Isobutyraldehyde

Isophorone

Isophthalic acid

Isoprene

Isopropyl acetate

Ligninsulfonic acid, calcium salt

Maleic anhydride

Methacrylic acid

*Methacrylic acid esters

Methane

Methyl ethyl ketone

Methyl methacrylate

Methyl tert-butyl ether

Methyl isobutyl ketone

Nitrilotriacetic acid

Nylon salt

*alpha-olefins

Oxalic acid

*Oxo-aldehydes-alcohols

*n-parafins

Pentaerythritol

Pentane

1-Pentene

*Pentenes

*Petroleum sulfonates

Pine oil

Polyoxybutylene glycol

Polyoxyethylene glycol

Propane

PropionaldehydePropionic acid

n-propyl acetate

n-propyl alcohol

Propylene glycol

Sodium formate

Sorbitol

Stearic acid, calcium salt (wax)

2,2,4-Trimethyl-1, 3-pentanediol

2)	Amine and Amide Organic Chemicals

*Alkyl amines

Aniline

n-butylamine

sec-butylamine

tert-butylamine

Caprolactam, aqueous concentrate

2,4-Diaminotoluene

Diethanolamine

N,N-diethylaniline

2,6-Dimethylaniline

N,N-dimethylformamide

Diphenylamine

*Ethanolamines

Ethylamine

Ethylenediamine

Ethylenediaminetetraacetic acid

*Fatty amines

Hexamethylenediamine

4-(N-Hydroxyethylethylamino)-2-hydroxyethylaniline

Isopropylamine

Melamine

Melamine crystal

*Methylamines

4,4'-Methylenebis(N,N'-dimethyl) aniline

Methylenedianiline

4,4'-Methylenedianiline

*Nitroanilines

o-Phenylenediamine

1,4-Phenylenediamine dihydrochloride

Polymeric methylene dianiline

Toluenediamine (mixture)

m-Toluidine

*Toluidines

3)	Aromatic Organic Chemicals

*Alkyl benzenes

*Alkyl phenols

*Alkylbenzene sulfonic acids, salts

Aminobenzoic acid (meta and para)

Benzenedisulfonic acid

Benzoic acid

Bis(2-ethylhexyl)phthalate

Bisphenol A

BTX--benzene-toluene-xylene (mixed)

Butyl octyl phthalate

tert-butylphenol

Coal tar

*Coal tar products (miscellaneous)

Creosote

o-Cresol

m-Cresol

p-Cresol

*Cresols, mixed

Cyanuric acid

*Cyclic aromatic sulfonates

Dibutyl phthalate

Diisobutyl phthalate

Diisodecyl phthalate

Diisooctyl phthalate

Dimethyl phthalate

2,4-Dinitrotoluene

2,6-Dinitrotoluene

Dinitrotoluene (mixed)

Ditridecyl phthalate

Metanilic acid

Methylenediphenyldiisocyanate

alpha-methylstyrene

Naphthalene

beta-naphthalene sulfonic acid

*Naphthas, solvent

Nitrobenzene

Nitrotoluene

Nonylphenol

Phthalic acid

Phthalic anhydride

*Tars-pitches

*Toluenediisocyanates (mixture)

Trimellitic acid

1-Tetralol, 1-tetralone mix

4)	Halogenated Organic Chemicals

Allyl chloride

Benzyl chloride

Carbon tetrachloride

*Chlorinated paraffins, 35-44% chlorine

Chlorobenzene

*Chlorobenzenes (mixed)

Chlorodifluoroethane

Chloroform

*Chloromethanes

2-Chloro-5-methylphenol (6-Chloro-m-cresol)

*Chlorophenols

Chloroprene

Cyanogen chloride

Cyanuric chloride

1,1-Dichloroethane

2,4-Dichlorophenol

Dichloropropane

Epichlorohydrin

Ethyl chloride

*Fluorocarbons (Freons)

Methyl chloride

Methylene chloride

Pentachlorophenol

Phosgene

Tetrachloroethylene

1,1,1-Trichloroethane

Trichloroethylene

Trichlorofluoromethane

Vinylidene chloride

5)	Other Organic Chemicals

Adiponitrile

Carbon disulfide

Fatty nitriles

*Organo-tin compounds

*Phosphate esters

Tetraethyl lead

Tetramethyl lead

*Urethane prepolymers

*Waxes, emulsions-dispersions

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 414.75 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 414.76 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after March 21, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2407  Specialty Organic Chemicals

a)	Applicability.  This Section applies to discharges of process wastewater resulting from the manufacture of any SIC 2865 or 2869 organic chemicals and organic chemical groups, as defined in the Standard Industrial Classification Manual, incorporated by reference in 35 Ill. Adm. Code 310.107, that are not defined as commodity or bulk organic chemicals in Section 307.2405 or 307.2406.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 414.85 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 414.86 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after March 21, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2410  Indirect Discharge Point Sources

a)	Applicability.  The Board incorporates by reference 40 CFR 414.11 (2005).  This incorporation includes no later amendments or editions.  This Section applies to the discharge of process wastewater resulting from the manufacture of the OCPSF products and product groups defined by 40 CFR 414.11 from any indirect discharge point source.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 414.111 (2005).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources.  All sources are treated as existing sources.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

#### Section 307.2490	Non-Complexed Metal-Bearing and Cyanide-Bearing Waste Streams

The Board incorporates by reference 40 CFR 414, Appendix A (2003).  This incorporation includes no later amendments or editions.

(Source:  Amended at 28 Ill. Reg. 3076, effective February 6, 2004)

#### Section 307.2491	Complexed Metal-Bearing Waste Streams

The Board incorporates by reference 40 CFR 414, Appendix B (2003).  This incorporation includes no later amendments or editions.

(Source:  Amended at 28 Ill. Reg. 3076, effective February 6, 2004)

### SUBPART P:  NORGANIC CHEMICALS MANUFACTURING

#### Section 307.2500	General Provisions

Compliance dates.  The Board incorporates by reference 40 CFR 415.01 (1986).  This incorporation includes no later amendments or editions.

(Source: Added at 12 Ill. Reg. 2592, effective January 13, 1988)

Section 307.2501  Aluminum Chloride Production

a)	Applicability.  This Section applies to discharges resulting from the production of aluminum chloride.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 415.14 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources.  All sources are regulated as existing sources.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2502  Aluminum Sulfate Production

a)	Applicability.  This Section applies to discharges resulting from the production of aluminum sulfate.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 415.24 (2003).  This incorporation includes no later amendments or editions.

2	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.26 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after July 24, 1980.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2503  Calcium Carbide Production

a)	Applicability.  This Section applies to discharges resulting from the production of calcium carbide in uncovered furnaces.

b)	Specialized Definitions.  None.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.36 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after July 24, 1980.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2504  Calcium Chloride Production

a)	Applicability.  This Section applies to discharges resulting from the production of calcium chloride by the brine extraction process.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 415.41 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.46 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after July 24, 1980.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2505  Calcium Oxide Production

a)	Applicability.  This Section applies to discharges resulting from the production of calcium oxide.

b)	Specialized Definitions.  None.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.56 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after July 24, 1980.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2506  Chlor-Alkali Process (Chlorine and Sodium or Potassium Hydroxide

Production)

a)	Applicability.  This Section applies to discharges resulting from the production of chlorine and sodium or potassium hydroxide by the diaphragm cell process or by the mercury cell process.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 415.61 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 415.64 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.66 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after July 24, 1980.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2508  Hydrofluoric Acid Production

a)	Applicability.  This Section applies to discharges resulting from the production of hydrofluoric acid.

b)	Specialized Definitions.  None.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.86 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after July 24, 1980.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2509  Hydrogen Peroxide Production

a)	Applicability.  This Section applies to discharges resulting from the production of hydrogen peroxide by the electrolytic process or by the oxidation of alkyl hydroanthraquinones.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 415.91 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources.  All sources are regulated as existing sources.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2511  Potassium Metal Production

a)	Applicability.  This Section applies to discharges resulting from the production of potassium metal.

b)	Specialized Definitions.  None.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.116 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after July 24, 1980.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2512  Potassium Dichromate Production

a)	Applicability.  This Section applies to discharges resulting from the production of potassium dichromate.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 415.124 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.126 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after July 24, 1980.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2513  Potassium Sulfate Production

a)	Applicability.  This Section applies to discharges resulting from the production of potassium sulfate.

b)	Specialized Definitions.  None.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.136 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after July 24, 1980.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2514  Sodium Bicarbonate Production

a)	Applicability.  This Section applies to discharges resulting from the production of sodium bicarbonate.

b)	Specialized Definitions.  None.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.146 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after July 24, 1980.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2516  Sodium Chloride Production

a)	Applicability.  This Section applies to discharges resulting from the production of sodium chloride by the solution brine-mining process or by the solar evaporation process.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 415.161 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.166 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after July 24, 1980.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2517  Sodium Dichromate and Sodium Sulfate Production

a)	Applicability.  This Section applies to discharges resulting from the production of sodium dichromate and by-product sodium sulfate.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 415.171 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.176 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after July 24, 1980.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2520  Sodium Sulfite Production

a)	Applicability.  This Section applies to discharges resulting from the production of sodium sulfite by reacting sulfur dioxide with sodium carbonate.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 415.201 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.206 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 25, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2522  Titanium Dioxide Production

a)	Applicability.  This Section applies to discharges resulting from the production of titanium dioxide by the sulfate process, the chloride process, or the simultaneous beneficiation-chlorination (chloride-ilmenite) process.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 415.221 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.226 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after July 24, 1980.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2523  Aluminum Fluoride Production

a)	Applicability.  This Section applies to discharges resulting from the production of aluminum fluoride.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 415.231 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources.  All sources are regulated as existing sources.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2524  Ammonium Chloride Production

a)	Applicability.  This Section applies to discharges resulting from the production of ammonium chloride by the reaction of anhydrous ammonia with hydrogen chloride gas or by the recovery process from Solvay process wastes.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 415.241 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources.  All sources are regulated as existing sources.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2527  Borax Production

a)	Applicability.  This Section applies to discharges resulting from the production of borax by the ore mining process or by the Trona process.

b)	Specialized Definitions.  None.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources.  All sources are regulated as existing sources.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2528  Boric Acid Production

a)	Applicability.  This Section applies to discharges resulting from the production of boric acid from ore-mined borax or from borax produced by the Trona process.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 415.281 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources.  All sources are regulated as existing sources.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2529  Bromine Production

a)	Applicability.  This Section applies to discharges resulting from the production of bromine by the brine-mining process or by the Trona process.

b)	Specialized Definitions.  None.

c)	Existing Sources. These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.296 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 25, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2530  Calcium Carbonate Production

a)	Applicability.  This Section applies to discharges resulting from the production of calcium carbonate by the milk of lime process or by the recovery process from Solvay process wastes.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 415.301 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources.  All sources are regulated as existing sources.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2531  Calcium Hydroxide Production

a)	Applicability.  This Section applies to discharges resulting from the production of calcium hydroxide by the lime slaking process.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 415.311 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.316 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 25, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2533  Carbon Monoxide and Byproduct Hydrogen Production

a)	Applicability.  This Section applies to discharges resulting from the production of carbon monoxide and by-product hydrogen by the reforming process.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 415.331 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources.  All sources are regulated as existing sources.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2534  Chrome Pigments Production

a)	Applicability.  This Section applies to discharges resulting from the production of chrome pigments.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 415.341 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 415.344 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.346 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after July 24, 1980.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2535  Chromic Acid Production

a)	Applicability.  This Section applies to discharges resulting from the production of chromic acid in facilities that also manufacture sodium dichromate.

b)	Specialized Definitions.  None.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.356 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 25, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2536  Copper Salts Production

a)	Applicability.  This Section applies to discharges resulting from the production of copper salts, including the following:

1)	Copper sulfate, copper chloride, copper iodide, and copper nitrate; and

2)	Copper carbonate.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 415.361 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 415.364 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.366 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 25, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2538  Ferric Chloride Production

a)	Applicability.  This Section applies to discharges resulting from the production of ferric chloride from pickle liquor.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 415.381 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 415.384 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.386 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 25, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2540  Fluorine Production

a)	Applicability.  This Section applies to discharges resulting from the production of fluorine by the liquid hydrofluoric acid electrolysis process.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 415.401 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.406 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 25, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2541  Hydrogen Production

a)	Applicability.  This Section applies to discharges resulting from the production of hydrogen as a refinery by-product.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 415.411 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources.  All sources are regulated as existing sources.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2542  Hydrogen Cyanide Production

a)	Applicability.  This Section applies to discharges resulting from the production of hydrogen cyanide by the Andrussow process.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 415.421 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.426 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after July 24, 1980.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2543  Iodine Production

a)	Applicability.  This Section applies to discharges resulting from the production of iodine.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 415.431 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.436 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 25, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2544  Lead Monoxide Production

a)	Applicability.  This Section applies to discharges resulting from the production of lead monoxide.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 415.441 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 415.444 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.446 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 25, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2545  Lithium Carbonate Production

a)	Applicability.  This Section applies to discharges resulting from the production of lithium carbonate by the Trona process or from spodumene ore.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 415.451 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources.  All sources are regulated as existing sources.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2547  Nickel Salts Production

a)	Applicability.  This Section applies to discharges resulting from the production of nickel salts, including the following:

1)	Nickel sulfate, nickel chloride, nickel nitrate, and nickel fluoborite; and

2)	Nickel carbonate.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 415.471 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 415.474 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.476 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 25, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2549  Oxygen and Nitrogen Production

a)	Applicability.  This Section applies to discharges resulting from the production of oxygen and nitrogen by air liquification.

b)	Specialized Definitions.  None.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources.  All sources are regulated as existing sources.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2550  Potassium Chloride Production

a)	Applicability.  This Section applies to discharges resulting from the production of potassium chloride by the Trona process or by the mining process.

b)	Specialized Definitions.  None.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.506 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 25, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2551  Potassium Iodide Production

a)	Applicability.  This Section applies to discharges resulting from the production of potassium iodide.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 415.511 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and special pretreatment requirements of Subpart B.

d)	New Sources.  All sources are regulated as existing sources.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2553  Silver Nitrate Production

a)	Applicability.  This Section applies to discharges resulting from the production of silver nitrate.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 415.531 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 415.534 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources.  All sources are regulated as existing sources.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2554  Sodium Bisulfite Production

a)	Applicability.  This Section applies to discharges resulting from the production of sodium bisulfite.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 415.541 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.546 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after July 24, 1980.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2555  Sodium Fluoride Production

a)	Applicability.  This Section applies to discharges resulting from the production of sodium fluoride by the anhydrous neutralization process or by the silico-fluoride process.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 415.551 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 415.554 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.556 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 25, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2560  Stannic Oxide Production

a)	Applicability.  This Section applies to discharges resulting from the production of stannic oxide by the reaction of tin metal with air or oxygen.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 415.601 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.606 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 25, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2563  Zinc Sulfate Production

a)	Applicability.  This Section applies to discharges resulting from the production of zinc sulfate.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 415.631 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.636 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 25, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2564  Cadmium Pigments and Salts Production

a)	Applicability.  This Section applies to discharges resulting from the production of cadmium pigments and salts, including cadmium chloride, cadmium nitrate, and cadmium sulfate.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 415.641 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 415.644 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.646 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 25, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2565  Cobalt Salts Production

a)	Applicability.  This Section applies to discharges resulting from the production of cobalt salts.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 415.651 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 415.654 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.656 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 25, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2566  Sodium Chlorate Production

a)	Applicability.  This Section applies to discharges resulting from the production of sodium chlorate.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 415.661 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.666 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 25, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2567  Zinc Chloride Production

a)	Applicability.  This Section applies to discharges resulting from the production of zinc chloride.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 415.671 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 415.674 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 415.676 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 25, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART R:  SOAP AND DETERGENTS

Section 307.2701  Soap Manufacturing by Batch Kettle

a)	Applicability.  This Section applies to discharges resulting from operations in which neat soap is produced through saponification of animal and vegetable fats and oils by boiling in kettles.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 417.11 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 417.14 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 417.16 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 26, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2702  Fatty Acid Manufacturing by Fat Splitting

a)	Applicability.  This Section applies to discharges resulting from the splitting of fats to fatty acids by hydrolysis and the subsequent processing of the fatty acids (e.g., refining and hydrogenation) to produce a suitable feed material for the manufacture of soap by fatty acid neutralization.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 417.21 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 417.24 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 417.26 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 26, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2703  Soap Manufacturing by Fatty Acid Neutralization

a)	Applicability.  This Section applies to discharges resulting from the manufacturing of neat soap by neutralizing refined fatty acids with an alkaline material in approximately stoichiometric amounts in batch or continuous operations.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 417.31 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 417.34 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 417.36 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 26, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2704  Glycerine Concentration

a)	Applicability.  This Section applies to discharges resulting from the concentration of sweet water from saponification or fat splitting to approximately 60 to 80 percent crude glycerine content.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 417.41 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 417.44 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 417.46 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 26, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2705  Glycerine Distillation

a)	Applicability.  This Section applies to discharges resulting from the production of finished glycerine of various grades (e.g., USP) through concentration from crude glycerine by means of distillation.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 417.51 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 417.54 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 417.56 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 26, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2706  Manufacture of Soap Flakes and Powders

a)	Applicability.  This Section applies to discharges resulting from all operations associated with the manufacture of soap flakes and powders, commencing with the drying of the neat soap to and including packaging of the finished flakes and powders.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 417.61 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 417.64 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 417.66 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 26, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2707  Manufacture of Bar Soaps

a)	Applicability.  This Section applies to discharges resulting from all operations associated with the conversion of neat soap to finished bar soaps, including drying, milling, plodding, stamping, and packaging.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 417.71 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 417.74 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 417.76 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 26, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2708  Manufacture of Liquid Soaps

a)	Applicability.  This Section applies to discharges resulting from the blending of ingredients employed in the manufacture of liquid soaps and the packaging of the finished products.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 417.81 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 417.84 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 417.86 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 26, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2709  Oleum Sulfonation and Sulfation

a)	Applicability.  This Section applies to discharges resulting from the manufacture of sulfonic acid and sulfuric acid esters using sulfonation and sulfation of raw materials, including but not limited to petroleum-derived alkyls, employing oleum in either continuous or batch processes.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 417.91 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 417.94 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 417.96 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 26, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2710  Air-Sulfur Trioxide Sulfation and Sulfonation

a)	Applicability.  This Section applies to discharges resulting from the manufacture of sulfonic acid and sulfuric acid esters by means of sulfation and sulfonation employing air and sulfur trioxide in either continuous or batch processes.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 417.101 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 417.104 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 417.106 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 26, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2711  Sulfur Trioxide Solvent and Vacuum Sulfonation

a)	Applicability.  This Section applies to discharges resulting from the operations in which undiluted sulfur trioxide and organic reactant are fed through a mixing nozzle into a vacuum reactor where the sulfonation of the organic reactant takes place.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 417.111 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 417.114 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 417.116 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 26, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2712  Sulfamic Acid Sulfation

a)	Applicability.  This Section applies to discharges resulting from operations in which sulfamic acid is employed as the sulfating agent.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 417.121 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 417.124 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 417.126 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 26, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2713  Chlorosulfonic Acid Sulfation

a)	Applicability.  This Section applies to discharges resulting from sulfation of alcohols, alkylphenols, and alcohol ethoxylates using chlorosulfonic acid as the sulfating agent.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 417.131 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 417.134 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 417.136 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 26, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2714  Neutralization of Sulfuric Acid Esters and Sulfonic Acids

a)	Applicability.  This Section applies to discharges resulting from the continuous or batch neutralization of sulfated and sulfonated alkylbenzenes, alcohols, and other materials to convert them to neutral salts.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 417.141 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 417.144 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 417.146 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 26, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2715  Manufacture of Spray-Dried Detergents

a)	Applicability.  This Section applies to discharges resulting from all operations associated with the manufacture of spray-dried detergents, including but not limited to assembly and storage of raw materials, crutching, spray drying, blending (including tumble spraying or additives), and packaging.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 417.151 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 417.156 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 26, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2716  Manufacture of Liquid Detergents

a)	Applicability.  This Section applies to discharges resulting from all operations associated with the manufacture of liquid detergents, commencing with the blending of ingredients, to and including bottling or packaging finished products.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 417.161 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 417.166 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 26, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2717  Manufacturing of Detergents by Dry Blending

a)	Applicability.  This Section applies to discharges resulting from operations associated with the manufacture of detergents by blending dry ingredients, including blending and subsequent packaging.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 417.171 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 417.176 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 26, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2718  Manufacture of Drum-Dried Detergents

a)	Applicability.  This Section applies to discharges resulting from operations associated with the manufacture of detergents by drum drying, including drying of formulations on heated drums or rollers, conversion of dried detergents to powders or flakes, and packaging of finished products.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 417.181 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 417.186 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 26, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2719  Manufacture of Detergent Bars and Cakes

a)	Applicability.  This Section applies to discharges resulting from operations associated with the manufacture of detergent bars and cakes, including, but not limited to, drying, milling, plodding, stamping, and packaging.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 417.191 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 417.194 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 417.196 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 26, 1973.

### (Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART S: FERTILIZER MANUFACTURING

Section 307.2801  Phosphate

a)	Applicability.  This Section applies to discharges resulting from the manufacture of sulfuric acid by sulfur burning, wet-process phosphoric acid, normal superphosphate, triple superphosphate, and ammonium phosphate.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 418.11 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 418.16 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 7, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2802  Ammonia

a)	Applicability.  This Section applies to discharges resulting from the manufacture of ammonia.  Discharges attributable to shipping losses and cooling tower blowdown are excluded.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 418.21 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 418.26 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 7, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2803  Urea

a)	Applicability.  This Section applies to discharges resulting from the manufacture of urea.  Discharges attributable to shipping losses and precipitation runoff from outside the battery limits of the urea manufacturing operations and cooling tower blowdown are excluded.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 418.31 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 418.36 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 7, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2804  Ammonium Nitrate

a)	Applicability.  This Section applies to discharges resulting from the manufacture of ammonium nitrate.

b)	These sources must comply with the general and specific pretreatment requirements of Subpart B.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2805  Nitric Acid

a)	Applicability.  This Section applies to discharges resulting from the production of nitric acid in concentrations up to 68 percent.  Discharges from shipping losses are excluded.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 418.51 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 418.56 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 7, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2806  Ammonium Sulfate Production

a)	Applicability.  This Section applies to discharges resulting from the production of ammonium sulfate by the synthetic process or by coke oven byproduct recovery.  This Section does not apply to ammonium sulfate produced as a byproduct of caprolactam production.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 418.61 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 418.66 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 7, 1974.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2807  Mixed and Blend Fertilizer Production

a)	Applicability.  This Section applies to discharges resulting from the production of mixed fertilizer and blend fertilizer.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 418.71 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 418.76 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 7, 1974.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART T: PETROLEUM REFINING

Section 307.2901  Topping

a)	Applicability. This Section applies to discharges from any facility that produces petroleum products by the use of topping and catalytic reforming, whether or not the facility includes any other process in addition to topping and catalytic reforming.  This Section does not apply to facilities that include thermal processes (coking, vis-breaking, etc.) or catalytic cracking.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 419.11 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 419.15 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 419.17 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 21, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2902  Cracking

a)	Applicability.  This Section applies to discharges from any facility that produces petroleum products by the use of topping and cracking, whether or not the facility includes any process in addition to topping and cracking.  This Section does not apply, however, to facilities that include the processes specified in Sections 307.2903, 307.2904, and 307.2905.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 419.21 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 419.25 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 419.27 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 21, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2903  Petrochemical

a)	Applicability.  This Section applies to discharges from any facility that produces petroleum products by the use of topping, cracking, and petrochemical operations whether or not the facility includes any process in addition to topping, cracking, and petrochemical operations.  This Section does not apply, however, to facilities that include the processes specified in Section 307.2904 or 307.2905.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 419.31 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 419.35 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 419.37 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 21, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2904  Lube

a)	Applicability.  This Section applies to discharges from any facility that produces petroleum products by the use of topping, cracking, and lube oil manufacturing processes, whether or not the facility includes any process in addition to topping, cracking, and lube oil manufacturing processes.  The Section does not apply, however, to facilities that include the processes specified in Sections 307.2903 and 307.2905.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 419.41 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 419.45 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 419.47 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 21, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.2905  Integrated

a)	Applicability.  This Section applies to discharges from any facility that produces petroleum products by the use of topping, cracking, lube oil manufacturing processes, and petrochemical operations, whether or not the facility includes any process in addition to topping, cracking, lube oil manufacturing processes, and petrochemical operations.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 419.51 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 419.55 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 419.57 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 21, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART U: IRON AND STEEL MANUFACTURING

Section 307.3000	General Provisions

a)	Applicability.

1)	This Subpart U applies to the introduction of pollutants into a POTW resulting from production operations in the iron and steel category.

2)	The Board incorporates by reference 40 CFR 420.01(b) (2002).  This incorporation includes no later amendments or editions.

b)	General definitions.  The Board incorporates by reference 40 CFR 420.02 (2002), as amended at 67 Fed. Reg. 64216 (October 17, 2002).  This incorporation includes no later amendments or editions.

c)	Compliance date.  The Board incorporates by reference 40 CFR 420.05 (2002).  This incorporation includes no later amendments or editions.

d)	Calculation of pretreatment standards.  The Board incorporates by reference 40 CFR 420.04 (2002).  This incorporation includes no later amendments or editions.

e)	Non-process wastewater and storm water.  The Board incorporates by reference 40 CFR 420.08, as added at 67 Fed. Reg. 64216 (October 17, 2002).

f)	Removal credits for phenols.  The control authority may grant removal credits pursuant to Subpart C of 35 Ill. Adm. Code 310 for phenols limited in this Subpart U when used as an indicator or surrogate pollutant.

(Source:  Amended at 27 Ill. Reg. 15095, effective September 10, 2003)

Section 307.3001  Cokemaking

a)	Applicability.  This Section applies to discharges resulting from byproduct and other cokemaking operations.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 420.11 (2005).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 420.15 (2005).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 420.16 (2005), as amended at 70 Fed. Reg. 73623 (December 13, 2005).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after January 7, 1981.

e)	Compliance Date.  The Board incorporates by reference 40 CFR 420.18 (2005).

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3002  Sintering

a)	Applicability.  This Section applies to discharges resulting from sintering operations conducted by the heating of iron-bearing wastes (mill scale and dust from blast furnaces and steelmaking furnaces) together with fine iron ore, limestone, and coke fines in an ignition furnace to produce an agglomerate for charging to the blast furnace.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 420.21 (2005).

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 420.25 (2005).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 420.26 (2005), as amended at 70 Fed. Reg. 73623 (December 13, 2005).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after January 7, 1981.

e)	Compliance Date.  The Board incorporates by reference 40 CFR 420.28 (2005).

f)	Point of Compliance Monitoring.  The Board incorporates by reference 40 CFR 420.29(b) (2005).

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3003  Ironmaking

a)	Applicability.  This Section applies to discharges resulting from ironmaking operations in which iron ore is reduced to molten iron in a blast furnace.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 420.31 (2002), as amended at 67 Fed. Reg. 64216 (October 17, 2002).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 420.35 (2002), as amended at 67 Fed. Reg. 64216 (October 17, 2002).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 420.36 (2002), as amended at 67 Fed. Reg. 64216 (October 17, 2002).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after January 7, 1981.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3004  Steelmaking

a)	Applicability.  This Section applies to discharges resulting from steelmaking operations conducted in basic oxygen or electric arc furnaces.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 420.41 (2002), as amended at 67 Fed. Reg. 64216 (October 17, 2002).  This incorporation includes no later amendments or editions.

c)	Existing Sources.

1)	The Board incorporates by reference 40 CFR 420.45 (2002), as amended at 67 Fed. Reg. 64216 (October 17, 2002).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 420.46 (2002), as amended at 67 Fed. Reg. 64216 (October 17, 2002).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after January 7, 1981.

e)	Compliance Date.  The Board incorporates by reference 40 CFR 420.48, as added at 67 Fed. Reg. 64216 (October 17, 2002).

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3005  Vacuum Degassing

a)	Applicability.  This Section applies to discharges resulting from vacuum degassing operations conducted by applying a vacuum to molten steel.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 420.55 (2002).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 420.56 (2002).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source"  means any building, structure, facility, or installation whose construction commenced after January 7, 1981.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3006  Continuous Casting

a)	Applicability.  This Section applies to discharges resulting from the continuous casting of molten steel into intermediate or semi-finished steel products through water-cooled molds.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 420.65 (2002).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 420.66 (2002).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after January 7, 1981.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3007  Hot Forming

a)	Applicability.  This Section applies to discharges resulting from hot forming operations conducted in primary, section, flat, and pipe and tube mills.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 420.71 (2002).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 420.75 (2002).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources.  These sources must comply with the standards for existing sources.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3008  Salt Bath Descaling

a)	Applicability.  This Section applies to discharges resulting from oxidizing or reducing salt bath descaling operations.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 420.81 (2002).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 420.85 (2002).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 420.86 (2002).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after January 7, 1981.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3009  Acid Pickling

a)	Applicability.  This Section applies to discharges resulting from sulfuric acid, hydrochloric acid, or combination acid pickling operations.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 420.91 (2002).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 420.95 (2002).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 420.96 (2002).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after January 7, 1981.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3010  Cold Forming

a)	Applicability.  This Section applies to discharges resulting from cold rolling and cold working pipe and tube operations in which unheated steel is passed through rolls or otherwise processed to reduce its thickness, produce a smooth surface, or develop controlled mechanical properties in the steel.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 420.101 (2002).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 420.105 (2002).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 420.106 (2002).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after January 7, 1981.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3011  Alkaline Cleaning

a)	Applicability.  This Section applies to discharges resulting from operations in which steel or steel products are immersed in alkaline cleaning baths to remove mineral or animal fats or oils from the steel, and the rinse operations following that immersion.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 420.111 (2002).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources.  All sources are regulated as existing sources.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3012  Hot Coating

a)	Applicability.  This Section applies to discharges resulting from operations in which steel is coated with zinc, terne metal, or other metals by the hot-dip process, and the rinsing operations associated with that process.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 420.121 (2002).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 420.125 (2002).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 420.126 (2002).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after January 7, 1981.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3013  Other Operations

a)	Applicability.  This Section applies to discharges resulting from the production of direct-reduced iron and from briquetting and forging operations.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 420.131, as added at 67 Fed. Reg. 64216 (October 17, 2002).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 420.135, as added at 67 Fed. Reg. 64216 (October 17, 2002).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 420.136, as added at 67 Fed. Reg. 64216 (October 17, 2002).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after November 18, 2002.

e)	Compliance Date.  An existing source must comply with the requirements of this Section by October 17, 2005.  A new source must comply with the requirements of this Section on the date it begins discharging.

BOARD NOTE:  Subsections (d)(3) and (e) derive from the preamble at 67 Fed. Reg. 64216 (October 17, 2002).

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART V:  NONFERROUS METALS MANUFACTURING

#### Section 307.3100	General Provisions

a)	Applicability.  This Subpart V applies to any facility producing primary metals from ore concentrates or recovering secondary metals from recycle wastes that introduces or may introduce pollutants into a POTW.  This Subpart V applies only to alloying or casting of hot metal directly from the nonferrous metals manufacturing process without cooling.  Remelting followed by alloying or cooling is included in the aluminum forming, nonferrous metals forming, or metal molding and casting categories.

b)	This subsection corresponds to 40 CFR 421.2, marked “reserved” by USEPA.  This statement maintains structural consistency with the corresponding federal regulations.

c)	Monitoring requirements.  The Board incorporates by reference 40 CFR 421.3 (2003).  This incorporation includes no later amendments or editions.

d)	Compliance dates.  The Board incorporates by reference 40 CFR 421.4 (2003).  This incorporation includes no later amendments or editions.

e)	Removal credits.  The control authority may grant removal credits pursuant to 35 Ill. Adm. Code 310.300 et seq. for toxic metals limited in this Subpart V when used as indicator pollutants.

(Source:  Amended at 28 Ill. Reg. 3076, effective February 6, 2004)

Section 307.3101  Bauxite Refining

a)	Applicability.  This Section applies to discharges resulting from the refining of bauxite to alumina by the Bayer process or by the combination process.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 421.11 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 421.16 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after June 27, 1984.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3102  Primary Aluminum Smelting

a)	Applicability.  This Section applies to discharges resulting from the production of aluminum from alumina by the Hall-Herloult process.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 421.21 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 421.26 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after February 17, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3103  Secondary Aluminum Smelting

a)	Applicability.  This Section applies to discharges resulting from the recovery, processing, and remelting of aluminum scrap to produce metallic aluminum alloys.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 421.31 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 421.35 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 421.36 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after February 17, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3104  Primary Copper Smelting

a)	Applicability.  This Section applies to discharges resulting from the primary smelting of copper from ore or ore concentrates.  Primary copper smelting includes, but is not limited to, roasting, converting, leaching if preceded by a pyrometallurgical step, slag granulation and dumping, fire refining, and the casting of products from these operations.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 421.41 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 421.46 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after February 17, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3105  Primary Electrolytic Copper Refining

a)	Applicability.  This Section applies to discharges resulting from the electrolytic refining of primary copper, including anode casting performed at refineries that are not located on-site with a smelter, product casting, and by-product recovery.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 421.51 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 421.56 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after February 17, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3106  Secondary Copper

a)	Applicability.  This Section applies to discharges resulting from the recovery, processing, and remelting of new and used copper scrap and residues to produce copper metal and copper alloys; but does not apply to continuous rod casting.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 421.61 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 421.65 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 421.66 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after February 17, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3107  Primary Lead

a)	Applicability.  This Section applies to discharges resulting from the production of lead at primary lead smelters and refineries.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 421.71 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 421.75 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 421.76 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after February 17, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3108  Primary Zinc

a)	Applicability.  This Section applies to discharges resulting from the production of zinc by either electrolytic or pyrolytic means.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 421.81 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 421.85 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 421.86 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after February 17, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3109  Metallurgical Acid Plants

a)	Applicability.  This Section applies to discharges resulting from or associated with the manufacture of by-product sulfuric acid at primary smelters, primary copper smelters, primary zinc facilities, primary lead facilities, or primary molybdenum facilities, including associated air pollution control or gas-conditioning systems for sulfur dioxide off-gases from pyrometallurgical operations.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 421.91 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 421.95 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 421.96 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after February 17, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3110  Primary Tungsten

a)	Applicability.  This Section applies to discharges resulting from the production of tungsten at primary tungsten facilities.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 421.101 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 421.105 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 421.106 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after February 17, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3111  Primary Columbium-Tantalum

a)	Applicability.  This Section applies to discharges resulting from the production of columbium or tantalum by primary columbium-tantalum facilities.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 421.111 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 421.115 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 421.116 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after February 17, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3112  Secondary Silver

a)	Applicability.  This Section applies to discharges resulting from the production of silver from secondary silver facilities processing photographic and nonphotographic raw materials.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 421.121 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 421.125 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 421.126 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after February 17, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3113  Secondary Lead

a)	Applicability.  This Section applies to discharges resulting from the production of lead by secondary lead facilities.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 421.131 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 421.135 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 421.136 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after February 17, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3114  Primary Antimony

a)	Applicability.  This Section applies to discharges resulting from the production of antimony at primary antimony facilities.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 421.141 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 421.146 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after June 27, 1984.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3115  Primary Beryllium

a)	Applicability.  This Section applies to discharges resulting from the production of beryllium by primary beryllium facilities processing beryllium ore concentrates or beryllium hydroxide raw materials.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 421.151 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 421.156 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after June 27, 1984.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3116  Primary and Secondary Germanium and Gallium

a)	Applicability.  This Section applies to discharges resulting from the production of germanium or gallium from primary or secondary germanium or gallium facilities.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 421.181 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 421.185 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 421.186 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after June 27, 1984.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3117  Secondary Indium

a)	Applicability.  This Section applies to discharges resulting from the production of indium at secondary indium facilities processing spent electrolyte solutions and scrap indium raw materials.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 421.191 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 421.195 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 421.196 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after June 27, 1984.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3118  Secondary Mercury

a)	Applicability.  This Section applies to discharges resulting from the production of mercury from secondary mercury facilities processing recycled mercuric oxide batteries and other mercury-containing scrap raw materials.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 421.201 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 421.206 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after June 27, 1984.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3119  Primary Molybdenum and Rhenium

a)	Applicability.  This Section applies to discharges resulting from the production of molybdenum and rhenium facilities.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 421.211 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 421.216 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after June 27, 1984.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3120  Secondary Molybdenum and Vanadium

a)	Applicability.  This Section applies to discharges resulting from the production of molybdenum or vanadium by secondary molybdenum or vanadium facilities.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 421.221 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 421.226 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after June 27, 1984.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3121  Primary Nickel and Cobalt

a)	Applicability.  This Section applies to discharges resulting from the production of nickel or cobalt by primary nickel or cobalt facilities processing ore concentrate raw materials.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 421.231 (2005).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 421.236 (2005).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after June 27, 1984.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3122  Secondary Nickel

a)	Applicability.  This Section applies to discharges resulting from the production of nickel by secondary nickel facilities processing slag, spent acids, or scrap metal raw materials.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 421.241 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 421.245 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 421.246 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after June 27, 1984.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3123  Primary Precious Metals and Mercury

a)	Applicability.  This Section applies to discharges resulting from the production of gold, silver, or mercury by primary precious metals and mercury facilities.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 421.251 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 421.256 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after June 27, 1984.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3124  Secondary Precious Metals

a)	Applicability.  This Section applies to discharges resulting from the production of precious metals at secondary precious metals facilities.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 421.261 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 421.265 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 421.266 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after June 27, 1984.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3125  Primary Rare Earth Metals

a)	Applicability.  This Section applies to discharges resulting from the production of rare earth metals and mischmetal by primary rare earth metals facilities processing rare earth metal oxides, chlorides, or fluorides.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 421.271 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 421.275 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 421.276 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after June 27, 1984.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3126  Secondary Tantalum

a)	Applicability.  This Section applies to discharges resulting from the production of tantalum at secondary tantalum facilities.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 421.281 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 421.286 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after June 27, 1984.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3127  Secondary Tin

a)	Applicability.  This Section applies to discharges resulting from the production of tin at secondary tin facilities using either pyrometallurgical or hydrometallurgical processes to recover tin from secondary materials.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 421.291 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 421.295 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 421.296 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after June 27, 1984.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3128  Primary and Secondary Titanium

a)	Applicability.  This Section applies to discharges resulting from the production of titanium at primary or secondary titanium facilities.  Facilities that practice only vacuum distillation for sponge purification and which do not practice electrolytic recovery of magnesium are exempt from this Section.  All other primary and secondary titanium facilities are covered by this Section.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 421.301 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 421.305 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 421.306 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after June 27, 1984.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3129  Secondary Tungsten and Cobalt

a)	Applicability.  This Section applies to discharges resulting from the production of tungsten or cobalt at secondary tungsten or cobalt facilities processing tungsten or tungsten carbide raw materials.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 421.311 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 421.315 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 421.316 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after June 27, 1984.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3130  Secondary Uranium

a)	Applicability.  This Section applies to discharges resulting from the production of uranium (including depleted uranium) by secondary uranium facilities.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 421.321 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 421.326 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after June 27, 1984.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3131  Primary Zirconium and Hafnium

a)	Applicability.  This Section applies to discharges resulting from the production of zirconium or hafnium at primary zirconium or hafnium facilities.  There are two levels of pretreatment standards for existing and new sources.  Facilities that produce zirconium or zirconium/nickel alloys only by magnesium reduction of zirconium dioxide are subject only to Subpart B.  All other facilities are subject to this Section.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 421.331 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B

d)	New Sources

1)	The Board incorporates by reference 40 CFR 421.336 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after June 27, 1984.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART X:  STEAM ELECTRIC POWER GENERATING

Section 307.3301  Steam Electric Power Generating

a)	Applicability.  This Section applies to discharges resulting from the operation of a generating unit by an establishment whose generation of electricity is the predominant source of revenue or principal reason for operation, and whose generation of electricity results primarily from a process using fossil-type fuel (coal, oil, or gas), fuel derived from fossil fuel (e.g., petroleum coke, synthesis gas), or nuclear fuel in conjunction with a thermal cycle employing the steam water system as the thermodynamic medium.  This Section applies to discharges associated with both the combustion turbine and steam turbine portions of a combined cycle generating unit.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 423.11 (2020), as amended at 85 Fed. Reg. 64650 (Oct. 13, 2020).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 423.13(g)(3)(i) and 423.16 (2020), as amended at 85 Fed. Reg. 64650 (Oct. 13, 2020); 40 CFR 423.13(k)(3) and (o), 423.18, and 423.19, as added at 85 Fed. Reg. 64650 (Oct. 13, 2020); 40 CFR 423.19(f)(1) as amended at 88 Fed. Reg. 18440 (March 29, 2023); and appendix A to 40 CFR 423 (2020).  These incorporations include no later amendments or editions.

A)	Flue Gas Desulfurization (FGD) Wastewater Discharges by Electric Generating Units (EGUs) Seeking Voluntary Incentives Participation (VIP).  The owner or operator of an EGU opting into VIP may seek to operate under alternative standards for discharges of FGD wastewater, as defined in 40 CFR 423.11(n), incorporated by reference in subsection (b), as provided in 40 CFR 423.13(g)(3)(i) after fulfilling the certification requirements in 40 CFR 423.19(a), (b), and (h).

BOARD NOTE:  This subsection (c)(1)(A) derives from 40 CFR 423.13(g)(3)(i) and 423.19(h).  USEPA calls VIP “Voluntary Incentives Program,” and the Board evokes enough of that name as is evocative but stops short of using the same name.  USEPA stated that its Voluntary Incentives Program applies only to direct discharges and is not finalized as to indirect discharges.  85 Fed. Reg. 64650, 64660, 64675 (Oct. 13, 2020).  USEPA’s rules, however, expressly contemplate applying its elements to indirect discharges.  See 40 CFR 423.19(h)(3).

B)	Flue Gas Desulfurization (FGD) Wastewater Discharges by EGUs Initiating Cessation of Coal Burning.  The owner or operator of an EGU that will undergo permanent cessation of coal combustion, as defined in 40 CFR 423.11(w), incorporated by reference in subsection (b), may seek to operate under alternative standards for discharges of FGD wastewater, as defined in 40 CFR 423.11(n), incorporated by reference in subsection (b), as provided in 40 CFR 423.16(e)(1) after fulfilling the certification requirements in 40 CFR 423.19(a), (b), and (f).

BOARD NOTE:  This subsection (c)(1)(B) derives from 40 CFR 423.16(e)(1) and 423.19(f).

C)	Notice of Material Delay.  An EGU that will undergo permanent cessation of coal combustion or one that opted into VIP under alternative standards under subsection (c)(1)(A) operating under alternative standards under subsection (c)(1)(B) must submit a notice of significant delay as required by 40 CFR 423.19(j).

BOARD NOTE:  This subsection (c)(1)(C) derives from 40 CFR 423.19(j).

D)	FGD Wastewater Discharges by Low Utilization EGUs (LUEGUs).  The owner or operator of an EGU that qualifies as a LUEGU, as defined in 40 CFR 423.11(z), incorporated by reference in subsection (b), may seek to operate under the alternative standards for discharges of FGD wastewater, as defined in 40 CFR 423.11(n), incorporated by reference in subsection (b), provided in 40 CFR 423.16(e)(2) after fulfilling the certification requirements in 40 CFR 423.19(a), (b), and (e).

BOARD NOTE:  This subsection (c)(1)(D) derives from 40 CFR 423.16(e)(2) and 423.19(e).

E)	Bottom Ash (BA) Transport Water Discharges.  Discharge of BA transport water, as defined in 40 CFR 423.11(p), incorporated by reference in subsection (b), is prohibited and only allowed as provided in 40 CFR 423.16(g) after complying with the best management practices requirements of 40 CFR 423.13(k)(3) and fulfilling the certification requirements in 40 CFR 423.19(a) through (d).

BOARD NOTE:  This subsection (c)(1)(E) derives from 40 CFR 423.13(k)(3), 423.16(g), and 423.19(c) and (d).

F)	EGUs Seeking to Transfer Between Applicable Discharge Limitations.  Subject to the limitations in subsection (c)(1)(G), the owner or operator of an EGU may seek to transfer applicable discharge standards, as provided in 40 CFR 423.13(o), after fulfilling the certification requirements in 40 CFR 423.19(a), (b), and (i).  The permissible transfers are the following:

i)	Before December 31, 2023, from limitations applicable to an EGU initiating cessation of coal combustion under subsection (c)(1)(B) to those applicable to a LUEGU under subsection (c)(1)(D);

ii)	Before December 31, 2023, from VIP limitations under subsection (c)(1)(A) to those applicable to a LUEGU under subsection (c)(1)(D);

iii)	Before December 31, 2025, from VIP limitations under subsection (c)(1)(A) to those applicable to an EGU initiating cessation of coal combustion under subsection (c)(1)(D);

iv)	Before December 31, 2025, from limitations applicable to an EGU initiating cessation of coal combustion under subsection (c)(1)(B) to VIP limitations under subsection (c)(1)(A);

v)	Before December 31, 2025, from limitations applicable to a LUEGU under subsection (c)(1)(D) to generally applicable limitations for discharges of FGD wastewater and BA transport water under 40 CFR 423.16;

vi)	Before December 31, 2025, from limitations applicable to a LUEGU under subsection (c)(1)(D) to VIP limitations under subsection (c)(1)(A); and

vii)	Before December 31, 2025, from limitations applicable to a LUEGU under subsection (c)(1)(D) to those applicable to an EGU initiating cessation of coal combustion under subsection (c)(1)(B).

BOARD NOTE:  This subsection (c)(1)(F) derives from 40 CFR 423.13(o) and 423.19(i).  USEPA calls VIP “Voluntary Incentives Program,” and the Board evokes enough of that name as is evocative but stops short of using the same name.  USEPA stated that its Voluntary Incentives Program applies only to direct discharges and is not finalized as to indirect discharges.  85 Fed. Reg. 64650, 64708 n. 166 (Oct. 13, 2020).  USEPA’s rules, however, expressly contemplate applying its elements to indirect discharges.  See 40 CFR 423.19(i)(1).

G)	Conditions for Transfer Between Applicable Discharge Limitations.  Conditions apply to transfer between applicable discharge limitations:

i)	An EGU must comply with all currently applicable requirements before filing a notice under 40 CFR 423.19(i) seeking transfer to other applicable discharge limitations, as provided in 40 CFR 423.13(o)(2); and

ii)	An EGU seeking a transfer described in subsections (c)(1)(F)(iii) through (c)(1)(F)(vii) must comply with more stringent limitations that already apply, instead of the less stringent limitations sought.

BOARD NOTE:  This subsection (c)(1)(G) derives from 40 CFR 423.13(o)(2) and (o)(3).

H)	An EGU that would otherwise qualify as a LUEGU or as ceasing combustion of coal before December 31, 2028 will continue to qualify if the conditions in 40 CFR 423.18 are true and after the owner or operator fulfills the certification requirements in 40 CFR 423.19(a), (b), and (g).

BOARD NOTE:  This subsection (c)(1)(H) derives from 40 CFR 423.16(e)(1) and (e)(2), 423.18, and 423.19(g).  The requirements of 40 CFR 423.18 directly apply to conditions in NPDES permits.  The certification requirement of 40 CFR 423.19(g) allows the discharge standards for the LUEGU or EGU ceasing coal combustion subcategory continue to continue to apply to indirect dischargers under the same circumstances and conditions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 423.17 and appendix A to 40 CFR 423 (2020).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 14, 1980.

(Source:  Amended at 48 Ill. Reg. 8106, effective May 31, 2024)

### SUBPART Y:  FERROALLOY MANUFACTURING

Section 307.3401  Open Electric Furnaces With Wet Air Pollution Control Devices

a)	Applicability.  This Section applies to discharges resulting from the smelting of ferroalloys in open electric furnaces with wet air pollution control devices.  This subcategory includes those electric furnaces of such construction or configuration that the furnace off-gases are burned above the furnace charge level by air drawn into the system.  After combustion, the gases are cleaned in a wet air pollution control device, such as a scrubber or an electrostatic precipitator with water or another aqueous spray.  The provisions of this Section do not apply to noncontact cooling water or to those electric furnaces that are covered, closed, sealed, or semi-covered and in which the furnace off-gases are not burned before collection (regulated in 35 Ill. Adm. Code 301.3402).

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 424.11 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 424.16 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 18, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3402  Covered Electric Furnaces and Other Smelting Operations with Wet Air

Pollution Control Devices

a)	Applicability.  This Section applies to discharges resulting from the smelting of ferroalloys in covered electric furnaces or other smelting operations, not elsewhere included in this Subpart, with wet air pollution control devices.  This subcategory includes electric furnaces of such construction or configuration (known as covered, closed, sealed, semi-covered, or semi-closed furnaces) that the furnace off-gases are not burned before collection and cleaning, and which off-gases are cleaned after collection in a wet air pollution control device such as a scrubber or a "wet" baghouse.  This subcategory also includes non-electric furnace smelting operations, such as exothermic (i.e., aluminothermic or silicothermic) smelting or ferromanganese refining, when these are controlled for air pollution by wet air pollution control devices. This subcategory does not include noncontact cooling water or furnaces that use dry dust collection techniques, such as dry baghouses.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 424.21 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 424.26 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 18, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3403  Slag Processing

a)	Applicability.  This Section applies to discharges resulting from slag processing in which either of the following occurs:

1)	The residual metal values in the furnace slag are recovered via concentration for return to the furnace; or

2)	The slag is "shotted" for further use.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 424.31 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 424.36 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 18, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3404  Covered Calcium Carbide Furnaces With Wet Air Pollution Control Devices

a)	Applicability.  This Section applies to discharges resulting from the production of calcium carbide in covered electric furnaces that use wet air pollution control devices.  This subcategory includes electric furnaces of such construction or configuration (known as covered, closed, sealed, semi-covered, or semi-closed furnaces) that the furnace off-gases are not burned before collection and cleaning, and which off-gases are cleaned after collection in a wet air pollution control device such as a scrubber or “wet” baghouse.  This subcategory does not include noncontact cooling water or furnaces that use dry dust collection techniques, such as dry baghouses.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 424.41 (2005).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources.  All sources are regulated as existing sources.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3405  Other Calcium Carbide Furnaces

a)	Applicability.  This Section applies to discharges resulting from the production of calcium carbide in covered furnaces that do not use wet air pollution control methods.  Covered calcium carbide furnaces using wet air pollution control devices are regulated in Section 307.3404.  Open (uncovered) calcium carbide furnaces are regulated in Subpart P, Inorganic Chemicals Manufacturing.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 424.51 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources.  All sources are regulated as existing sources.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3406  Electrolytic Manganese Products

a)	Applicability.  This Section applies to discharges resulting from the manufacture of electrolytic manganese products, such as electrolytic manganese metal or electrolytic manganese dioxide.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 424.61 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources.  All sources are regulated as existing sources.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3407  Electrolytic Chromium

a)	Applicability.  This Section applies to discharges resulting from the manufacture of chromium metal by the electrolytic process.  It does not apply to the manufacture of chromium metal by aluminothermic or other methods.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 424.71 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources.  All sources are regulated as existing sources.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART Z:  LEATHER TANNING AND FINISHING

#### Section 307.3500	General Provisions

a)	Applicability.  This Subpart Z applies to any leather tanning and finishing facility that introduces or may introduce process wastewater pollutants into a POTW.

b)	General definitions.  The Board incorporates by reference 40 CFR 425.02 (2003).  This incorporation includes no later amendments or editions.

c)	Sulfide analytical method.  The Board incorporates by reference 40 CFR 425.03 (2003).  This incorporation includes no later amendments or editions.

d)	Applicability of sulfide standards.  The Board incorporates by reference 40 CFR 425.04 (2003).  This incorporation includes no later amendments or editions.

e)	Compliance dates.  The Board incorporates by reference 40 CFR 425.05 (2003).  This incorporation includes no later amendments or editions.

f)	Monitoring requirements.  The Board incorporates by reference 40 CFR 425.06 (2003).  This incorporation includes no later amendments or editions.

(Source:  Amended at 28 Ill. Reg. 3076, effective February 6, 2004)

Section 307.3501  Hair Pulp, Chrome Tan, Retan-Wet Finish

a)	Applicability.  This Section applies to discharges resulting from any tannery that, either exclusively or in addition to other unhairing and tanning operations, processes raw or cured cattle or cattle-like hides into finished leather by chemically dissolving the hide hair, chrome tanning, and retan-wet finishing.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 425.15 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 425.16 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after July 2, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3502  Hair Save, Chrome Tan, Retan-Wet Finish

a)	Applicability.  This Section applies to discharges resulting from any tannery that processes raw or cured cattle or cattle-like hides into finished leather by hair save unhairing, chrome tanning, and retan-wet finishing.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 425.25 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 425.26 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after July 2, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3503  Hair Save or Pulp, Non-Chrome Tan, Retan-Wet Finish

a)	Applicability.  This Section applies to discharges resulting from any tannery that processes raw or cured cattle or cattle-like hides into finished leather by hair save or pulp unhairing, vegetable tanning, or alum, syntans, oils, and other agents for tanning and retan-wet finishing.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 425.35 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 425.36 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after July 2, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3504  Retan-Wet Finish-Sides

a)	Applicability.  This Section applies to discharges resulting from any tannery that processes previously tanned hides and skins (grade side only) into finished leather by retan-wet finishing.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 425.45 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 425.46 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after July 2, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3505  No Beamhouse

a)	Applicability.  This Section applies to discharges resulting from any tannery that processes cattle hides, sheepskins, or splits (hair previously removed and pickled) into finished leather by chrome or non-chrome tanning, and retan-wet finishing.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 425.55 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 425.56 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after July 2, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3506  Through-the-Blue

a)	Applicability.  This Section applies to discharges resulting from any tannery that processes raw or cured cattle or cattle-like hides through the blue tanned state by hair pulp unhairing and chrome tanning; no retan-wet finishing is performed.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 425.65 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 425.66 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	New source means any building, structure, facility, or installation whose construction commenced after July 2, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3507  Shearling

a)	Applicability.  This Section applies to discharges resulting from any tannery that processes raw or cured sheep or sheep-like skins with the wool or hair retained into finished leather by chrome tanning and retan-wet finishing.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 425.75 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 425.76 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after July 2, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3508  Pigskin

a)	Applicability.  This Section applies to discharges resulting from any tannery that processes raw or cured pigskins into finished leather by chemically dissolving or pulping the hair and tanning with chrome, then retan-wet finishing.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 425.85 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 425.86 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	New source means any building, structure, facility, or installation whose construction commenced after July 2, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3509  Retan-Wet Finish-Splits

a)	Applicability.  This Section applies to discharges resulting from any tannery that processes previously unhaired and tanned splits into finished leather by retan-wet finishing.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 425.95 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 425.96 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after July 2, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

#### Section 307.3590	Potassium Ferricyanide Titration Method

The Board incorporates by reference 40 CFR 425, Appendix A, as adopted at 53 Fed.  Reg.  9181, March 21, 1988.  This incorporation includes no later amendments or editions.

(Source: Added at 13 Ill.  Reg.  1794, effective January 31, 1989)

### SUBPART BA:	GLASS MANUFACTURING

Section 307.3601  Insulation Fiberglass

a)	Applicability.  This Section applies to discharges resulting from the production of insulation fiberglass in which molten glass is either directly or indirectly made, continuously fiberized, and chemically bonded into a wool-like material.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 426.11 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 426.16 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after August 22, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3602  Sheet Glass Manufacturing

a)	Applicability.  This Section applies to discharges resulting from the process in which several mineral ingredients (sand, soda ash, limestone, dolomite, cullet, and other ingredients) are mixed, melted in a furnace, and drawn vertically from a melting tank to form sheet glass.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 426.21 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 426.24 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 426.26 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 17, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3603  Rolled Glass Manufacturing

a)	Applicability.  This Section applies to discharges resulting from the process in which several mineral ingredients (sand, soda ash, limestone, dolomite, cullet, and other ingredients) are mixed, melted in a furnace, and cooled by rollers to form rolled glass.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 426.31 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 426.34 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 426.36 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 17, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3604  Plate Glass Manufacturing

a)	Applicability.  This Section applies to discharges resulting from the process in which several mineral ingredients (sand, soda ash, limestone, dolomite, cullet, and other ingredients) are melted in a furnace, pressed between rollers, and finally ground and polished to form plate glass.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 426.41 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 426.44 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 426.46 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 17, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3605  Float Glass Manufacturing

a)	Applicability.  This Section applies to discharges resulting from the process in which several mineral ingredients (sand, soda ash, limestone, dolomite, cullet, and other ingredients) are mixed, melted in a furnace, and floated on a molten tin bath to produce float glass.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 426.51 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 426.56 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 17, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3606  Automotive Glass Tempering

a)	Applicability.  This Section applies to discharges resulting from the processes in which glass is cut and then passed through a series of processes that grind and polish the edges, bend the glass, and then temper the glass to produce side and back windows for automobiles.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 426.61 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 426.64 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 426.66 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 17, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3607  Automotive Glass Laminating

a)	Applicability.  This Section applies to discharges resulting from the processes that laminate a plastic sheet between two layers of glass, or that prepare the glass for lamination such as cutting, bending, and washing, to produce automobile windshields.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 426.71 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 426.76 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 17, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3608  Glass Container Manufacturing

a)	Applicability.  This Section applies to discharges resulting from the process by which raw materials are melted in a furnace and mechanically processed into glass containers.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 426.81 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 426.86 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after August 21, 1974.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3610  Glass Tubing (Danner) Manufacturing

a)	Applicability.  This Section applies to discharges resulting from the process by which raw materials are melted in a furnace and glass tubing is mechanically drawn from the furnace horizontally using the Danner process, which requires the intermittent quenching of cullet.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 426.101 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 426.106 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after August 21, 1974.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3611  Television Picture Tube Envelope Manufacturing

a)	Applicability.  This Section applies to discharges resulting from the process by which raw materials are melted in a furnace and processed into television picture tube envelopes.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 426.111 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 426.116 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after August 21, 1974.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3612  Incandescent Lamp Envelope Manufacturing

a)	Applicability.  This Section applies to discharges resulting from the processes by which either of the following occurs:

1)	Raw materials are melted in a furnace and mechanically processed into incandescent lamp envelopes; or

2)	Incandescent lamp envelopes are etched with hydrofluoric acid to produce frosted envelopes.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 426.121 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 426.126 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after August 21, 1974.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3613  Hand Pressed and Blown Glass Manufacturing

a)	Applicability.  This Section applies to discharges resulting from the process by which raw materials are melted in a furnace and processed by hand into pressed or blown glassware.  This includes plants that:

1)	Produce leaded glass using hydrofluoric acid finishing techniques;

2)	Produce non-leaded glass using hydrofluoric acid finishing techniques; or

3)	Produce leaded or non-leaded glass without using hydrofluoric acid finishing techniques.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 426.131 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 426.136 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after August 21, 1974.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART BB:  ASBESTOS MANUFACTURING

Section 307.3701  Asbestos-Cement Pipe

a)	Applicability.  This Section applies to discharges resulting from the process in which asbestos, Portland cement, silica, and other ingredients are used in the manufacturing of asbestos-cement pipe.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 427.11 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 427.14 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 427.16 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 30, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3702  Asbestos-Cement Sheet

a)	Applicability.  This Section applies to discharges resulting from the process in which asbestos, Portland cement, silica, and other ingredients are used in the manufacturing of asbestos-cement sheets.  Discharges resulting from the manufacture of asbestos-cement sheet laboratory tops are specifically excluded from the provisions of this Section.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 427.21 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 427.24 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 427.26 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 30, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3703  Asbestos Paper (Starch Binder)

a)	Applicability.  This Section applies to discharges resulting from the process in which asbestos, starch binders, and other ingredients are used in the manufacture of asbestos paper (starch binder).

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 427.31 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 427.34 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 427.36 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 30, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3704  Asbestos Paper (Elastomeric Binder)

a)	Applicability.  This Section applies to discharges resulting from the process in which asbestos, elastomeric binder, and other ingredients are used in the manufacture of asbestos paper (elastomeric binder).

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 427.41 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 427.44 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 427.46 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 30, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3705  Asbestos Millboard

a)	Applicability.  This Section applies to discharges resulting from the process in which asbestos in combination with various other materials such as cement, starch, clay, lime, and mineral wool is used in the manufacture of asbestos millboard.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 427.51 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 427.54 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 427.56 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 30, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3706  Asbestos Roofing

a)	Applicability.  This Section applies to discharges resulting from the process in which asbestos paper is saturated with asphalt or coal tar with the subsequent application of various surface treatments to produce asbestos roofing products.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 427.61 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 427.64 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 427.66 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 30, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3707  Asbestos Floor Tile

a)	Applicability.  This Section applies to discharges resulting from the process in which asbestos, polyvinyl chloride resin, chemical stabilizers, limestone, and other fillers are used in the manufacture of asbestos floor tile.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 427.71 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 427.74 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 427.76 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 30, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3708  Coating or Finishing of Asbestos Textiles

a)	Applicability.  This Section applies to discharges resulting from the process of coating or impregnating asbestos textiles with materials that impart specific desired qualities to the finished product.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 427.81 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 427.86 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after August 29, 1974.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3709  Solvent Recovery

a)	Applicability.  This Section applies to discharges resulting from the process of solvent recovery in the manufacture of asbestos products.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 427.91 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 427.96 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after August 29, 1974.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3710  Vapor Absorption

a)	Applicability.  This Section applies to discharges resulting from the removal of volatilized organic materials from atmospheric emissions using wet scrubbers.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 427.101 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B

d)	New Sources

1)	The Board incorporates by reference 40 CFR 427.106 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after August 29, 1974.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3711  Wet Dust Collection

a)	Applicability.  This Section applies to discharges resulting from the removal of dust (particulates) from atmospheric emissions using wet scrubbers.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 427.111 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 427.116 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after August 29, 1974.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART BC:	RUBBER MANUFACTURING

Section 307.3801  Tire and Inner Tube Plants

a)	Applicability.  This Section applies to discharges resulting from the production of pneumatic tires and inner tubes in tire and inner tube plants.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 428.11 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 428.16 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 18, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3802  Emulsion Crumb Rubber

a)	Applicability.  This Section applies to discharges resulting from the manufacture of emulsion crumb rubber, other than acrylonitrile-butadiene rubber.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 428.21 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources.  All sources are regulated as existing sources.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3803  Solution Crumb Rubber

a)	Applicability.  This Section applies to discharges resulting from the manufacture of crumb rubber.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 428.31 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources.  All sources are regulated as existing sources.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3804  Latex Rubber

a)	Applicability.  This Section applies to discharges resulting from the manufacture of latex rubber.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 428.41 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 428.46 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 18, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3805  Small-Sized General Molded, Extruded, and Fabricated Rubber Plants

a)	Applicability.  This Section applies to discharges resulting from the production of molded, extruded, or fabricated rubber products, foam rubber backing, rubber cement-dipped goods, or retreaded tires by small-sized plants.  Specifically excluded from this Section are discharges resulting from the production of latex-based products and of tires and inner tubes, and discharges from textile plants subject to Subpart K.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 428.51 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 428.56 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 18, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3806  Medium-Sized General Molded, Extruded, and Fabricated Rubber Plants

a)	Applicability.  This Section applies to discharges resulting from the production of molded, extruded, or fabricated rubber products, foam rubber backing, rubber cement-dipped goods, or retreaded tires by medium-sized plants.  Specifically excluded from this Section are discharges resulting from the production of latex-based products and of tires and inner tubes, and discharges from textile plants subject to the provisions of Subpart K.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 428.61 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 428.66 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 18, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3807  Large-Sized General Molded, Extruded, and Fabricated Rubber Plants

a)	Applicability.  This Section applies to discharges resulting from the production of molded, extruded, or fabricated rubber products, foam rubber backing, rubber cement-dipped goods, or retreaded tires by large-sized plants.  Specifically excluded from this Section are discharges resulting from the production of latex-based products and of tires and inner tubes, and discharges from textile plants subject to Subpart K.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 428.71 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 428.76 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 18, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3808  Wet Digestion Reclaimed Rubber

a)	Applicability.  This Section applies to discharges resulting from the production of reclaimed rubber by use of the wet digestion process.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 428.81 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 428.86 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 18, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3809  Pan, Dry Digestion, and Mechanical Reclaimed Rubber

a)	Applicability.  This Section applies to discharges resulting from the production of reclaimed rubber, except when produced by the wet digestion process.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 428.91 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 428.96 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source"means any building, structure, facility, or installation whose construction commenced after December 18, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3810  Latex-Dipped, Latex-Extruded, and Latex-Molded Rubber

a)	Applicability.  This Section applies to discharges resulting from the manufacture of latex-dipped, latex-extruded, or latex-molded products, except for discharges from textile plants subject to Subpart K.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 428.101 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 428.106 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 18, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3811  Latex Foam

a)	Applicability.  This Section applies to discharges resulting from the manufacture of latex foam, except for discharges from textile plants subject to Subpart K.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 428.111 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 428.116 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after December 18, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

#### SUBPART BD:  TIMBER PRODUCTS PROCESSING

Section 307.3900	General Provisions

a)	Applicability.  This Subpart BD applies to any timber products processing operation, and any plant producing insulation board with wood as the major raw material, that introduces or may introduce process wastewater pollutants into a POTW.

b)	General definitions.  The Board incorporates by reference 40 CFR 429.11 (2004), as amended at 69 Fed. Reg. 45944, 46045 (July 30, 2004).  This incorporation includes no later amendments or editions.

(Source:  Amended 29 Ill. Reg. 6921, effective April 26, 2005)

Section 307.3901  Barking

a)	Applicability.  This Section applies to discharges resulting from the barking of logs by plants in SIC major group 24 or by plants producing insulation board (SIC group 2661), as defined in the Standard Industrial Classification Manual, incorporated by reference in 35 Ill. Adm. Code 310.107.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 429.25 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 429.26 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 31, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3902  Veneer

a)	Applicability.  This Section applies to discharges resulting from any plant that manufactures veneer and does not store or hold raw materials in wet storage conditions.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 429.35 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 429.36 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 31, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3903  Plywood

a)	Applicability.  This Section applies to discharges resulting from any plywood production plant that does not store or hold raw materials in wet storage conditions.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 429.45 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 429.46 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 31, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3904  Dry Process Hardboard

a)	Applicability.  This Section applies to discharges resulting from any plant that produces hardboard products using the dry matting process for forming the board mat.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 429.55 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 429.56 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 31, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3905  Wet Process Hardboard

a)	Applicability.  This Section applies to discharges resulting from any plant that produces hardboard products using the wet matting process for forming the board mat.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 429.65 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 429.66 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 31, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3906  Wood Preserving—Waterborne or Non-Pressure

a)	Applicability.  This Section applies to discharges resulting from all non-pressure wood preserving treatment processes or all pressure wood preserving treatment processes employing waterborne inorganic salts.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 429.75 (2011).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 429.76 (2011).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 31, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3907  Wood Preserving—Steam

a)	Applicability.  This Section applies to discharges resulting from wood-preserving processes that use direct steam impingement on wood or the vapor drying process as the predominant conditioning method, direct steam conditioning processes that use the same retort to treat both salt and oil type preservatives, and steam conditioning processes that apply both salt type and oil type preservatives to the same stock.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 429.85 (2011).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 429.86 (2011).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 31, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3908  Wood Preserving—Boulton

a)	Applicability.  This Section applies to discharges resulting from wood preserving operations that use the Boulton process as the predominant method of conditioning stock.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 429.95 (2011).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 429.96 (2011).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 31, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3909  Wet Storage

a)	Applicability.  This Section applies to discharges resulting from the storage of unprocessed wood, i.e., the storage of logs or roundwood before or after removal of bark in self-contained bodies of water (mill ponds or log ponds) or the storage of logs or roundwood on land during which water is sprayed or deposited intentionally on the logs (wet decking).

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 429.105 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 429.106 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 31, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3910  Log Washing

a)	Applicability.  This Section applies to discharges resulting from the log washing process in which water under pressure is applied to logs to remove foreign material from the surface of the log before further processing.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 429.115 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 429.116 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 31, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3911  Sawmills and Planing Mills

a)	Applicability.  This Section applies to discharges resulting from the timber products processing procedures that include all or part of the following operations:  bark removal (other than hydraulic barking as defined in Section 307.3901), sawing, resawing, edging, trimming, planing, and machining.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 429.125 (2003). This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 429.126 (2003). This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 31, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3912  Finishing

a)	Applicability.  This Section applies to discharges resulting from the drying, planing, dipping, staining, end coating, moisture proofing, fabrication, or by-product utilization timber processing operations not otherwise covered by specific guidelines and standards.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 429.135 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 429.136 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 31, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3913  Particleboard Manufacturing

a)	Applicability.  This Section applies to discharges resulting from any plant that manufactures particleboard.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 429.145 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 429.146 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 31, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3914  Insulation Board

a)	Applicability.  This Section applies to discharges resulting from plants that produce insulation board using wood as the primary raw material.  Specifically excluded from this Section is the manufacture of insulation board from the primary raw material bagasse.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 429.155 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 429.156 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 31, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3915  Wood Furniture and Fixture Production without Water Wash Spray Booths or without Laundry Facilities

a)	Applicability.  This Section applies to discharges resulting from the manufacture of wood furniture and fixtures at the following establishments:

1)	Those that do not use water wash spray booths to collect and contain the overspray from spray application of finishing materials; and

2)	Those that do not maintain on-site laundry facilities for fabric used in various finishing operations.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 429.165 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 429.166 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 31, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.3916  Wood Furniture and Fixture Production with Water Wash Spray Booths or with Laundry Facilities

a)	Applicability.  This Section applies to discharges resulting from the manufacture of wood furniture and fixtures at establishments that do either of the following:

1)	Those that use water wash spray booths to collect and contain the overspray from spray application of finishing materials; or

2)	Those that use on-site laundry facilities for fabric used in various finishing operations.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 429.175 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 429.176 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 31, 1979.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

SUBPART BE:  PULP, PAPER AND PAPERBOARD

Section 307.4000   General Provisions

a)	Applicability.  This Subpart BE applies to any pulp, paper, or paperboard mill that introduces or may introduce process wastewater pollutants into a publicly owned treatment works (POTW).

b)	General definitions.  The Board incorporates by reference 40 CFR 430.01 (2012).  This incorporation includes no later amendments or editions.

c)	Monitoring requirements.  The Board incorporates by reference 40 CFR 430.02 (2012).  This incorporation includes no later amendments or editions.

d)	Best management practices.  The Board incorporates by reference 40 CFR 430.03 (2012).  This incorporation includes no later amendments or editions.

(Source:  Amended at 37 Ill. Reg. 1936, effective February 4, 2013)

Section 307.4001  Dissolving Kraft

a)	Applicability.  This Section applies to discharges resulting from the production of dissolving pulp at kraft mills.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 430.11 (2002).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 430.16 (2002).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 430.17 (2002).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after January 6, 1981.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.4002  Bleached Papergrade Kraft and Soda

a)	Applicability.  This Section applies to discharges resulting from:  the production of market pulp, the integrated production of paperboard, coarse paper, and tissue paper, the integrated production of pulp and fine papers at bleached kraft mills; and the integrated production of pulp and paper at soda mills.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 430.21 (2002).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 430.26 (2002).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 430.27 (2002).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after January 6, 1981.

e)	Best Management Practices.  The Board incorporates by reference 40 CFR 430.28 (2002).  This incorporation includes no later amendments or editions.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.4003  Unbleached Kraft

a)	Applicability.  This Section applies to discharges resulting from:  the production of pulp and paper at unbleached kraft mills; the production of pulp and paper at unbleached kraft-neutral sulfite semi-chemical (cross recovery) mills; and the production of pulp and paper at combined unbleached kraft and semi-chemical mills whose spent semi-chemical cooking liquor is burned within the unbleached kraft chemical recovery system.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 430.31 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 430.36 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 430.37 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after January 6, 1981.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.4004  Dissolving Sulfite

a)	Applicability.  This Section applies to discharges resulting from the production of pulp at dissolving sulfite mills.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 430.41 (2002).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 430.46 (2002).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 430.47 (2002).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after January 6, 1981.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.4005  Papergrade Sulfite

a)	Applicability.  This Section applies to discharges resulting from:  the integrated production of pulp and paper at papergrade sulfite mills where blow pit pulp washing techniques are used; and the integrated production of pulp and paper at papergrade sulfite mills where vacuum or pressure drums are used to wash pulp.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 430.51 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 430.56 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 430.57 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after January 6, 1981.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.4006  Semi-Chemical

a)	Applicability.  This Section applies to discharges resulting from the integrated production of pulp and paper at semi-chemical mills.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 430.61 (2002).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 430.66 (2002).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 430.67 (2002).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after January 6, 1981.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.4007  Mechanical Pulp

a)	Applicability.  This Section applies to discharges resulting from:  the production of pulp and paper at groundwood chemi-mechanical mills; the production of pulp and paper at groundwood mills through the application of the thermo-mechanical process; the integrated production of pulp and coarse paper, molded pulp products, and newsprint at groundwood mills; and the integrated production of pulp and fine paper at groundwood mills.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 430.71 (2002).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 430.76 (2002).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 430.77 (2002).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after January 6, 1981.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.4008  Non-Wood Chemical Pulp

a)	Applicability.  This Section applies to discharges resulting from the production of pulp and paper at non-wood chemical pulp mills.  This subcategory includes mills producing non-wood pulps from chemical pulping processes such as kraft, sulfite, or soda.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 430.81 (2002).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 430.86 (2002).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 430.87 (2002).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after January 6, 1981.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.4009  Secondary Fiber Deink

a)	Applicability.  This Section applies to discharges resulting from the integrated production of pulp and paper at deink mills.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 430.91 (2002).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 430.96 (2002).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 430.97 (2002).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after January 6, 1981.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.4010  Secondary Fiber Non-Deink

a)	Applicability.  This Section applies to discharges resulting from the production of:  paperboard from wastepaper; tissue paper from wastepaper without deinking at secondary fiber mills; molded products from wastepaper without deinking at secondary fiber mills; and builders’ paper and roofing felt from wastepaper.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 430.101 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 430.106 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 430.107 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after January 6, 1981.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.4011  Fine and Lightweight Papers from Purchased Pulp

a)	Applicability.  This Section applies to discharges resulting from the production of:  fine paper at non-integrated mills; and lightweight paper at non-integrated mills.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 430.111 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 430.116 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 430.117 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after January 6, 1981.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.4012  Tissue, Filter, Non-Woven, and Paperboard from Purchased Pulp

a)	Applicability.  This Section applies to discharges resulting from the production of tissue papers at non-integrated mills, filter and non-woven papers at non-integrated mills, and paperboard at non-integrated mills.  The production of electrical grades of board and matrix board is not included in this Subpart BE.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 430.121 (2002).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources.  All sources are regulated as existing sources.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

#### Section 307.4013	Groundwood-Thermo-Mechanical (Repealed)

(Source:  Repealed at 23 Ill.  Reg. 4413, effective March 31, 1999)

#### Section 307.4014	Groundwood-CMN Papers (Repealed)

(Source:  Repealed at 23 Ill.  Reg. 4413, effective March 31, 1999)

#### Section 307.4015	Groundwood-Fine Papers (Repealed)

(Source:  Repealed at 23 Ill.  Reg. 4413, effective March 31, 1999)

#### Section 307.4016	Soda (Repealed)

(Source:  Repealed at 23 Ill.  Reg. 4413, effective March 31, 1999)

#### Section 307.4017	Deink (Repealed)

(Source:  Repealed at 23 Ill.  Reg. 4413, effective March 31, 1999)

#### Section 307.4018	Nonintegrated-Fine Papers (Repealed)

(Source:  Repealed at 23 Ill.  Reg. 4413, effective March 31, 1999)

Section 307.4019	Nonintergrated-Tissue Papers (Repealed)

(Source:  Repealed at 23 Ill.  Reg. 4413, effective March 31, 1999)

Section 307.4020	Tissue From Wastepaper (Repealed)

(Source:  Repealed at 23 Ill.  Reg. 4413, effective March 31, 1999)

Section 307.4021	Papergrade Sulfite (Drum Wash) (Repealed)

(Source:  Repealed at 23 Ill.  Reg. 4413, effective March 31, 1999)

#### Section 307.4022	Unbleached Kraft and Semi-Chemical (Repealed)

(Source:  Repealed at 23 Ill.  Reg. 4413, effective March 31, 1999)

#### Section 307.4023	Wastepaper-Molded Products (Repealed)

(Source:  Repealed at 23 Ill.  Reg. 4413, effective March 31, 1999)

#### Section 307.4024	Nonintegrated-Lightweight Papers (Repealed)

(Source:  Repealed at 23 Ill.  Reg. 4413, effective March 31, 1999)

#### Section 307.4025	Nonintegrated-Filter and Nonwoven Papers (Repealed)

(Source:  Repealed at 23 Ill.  Reg. 4413, effective March 31, 1999)

#### Section 307.4026	Nonintegrated-Paperboard (Repealed)

(Source:  Repealed at 23 Ill.  Reg. 4413, effective March 31, 1999)

### SUBPART BF: UILDERS' PAPER AND BOARD MILLS (Repealed)

#### Section 307.4101	Builder's Paper and Roofing Felt (Repealed)

(Source:  Repealed at 23 Ill.  Reg. 4413, effective March 31, 1999)

### SUBPART BG:  MEAT PRODUCTS

Section 307.4201  Simple Slaughterhouse

a)	Applicability.  This Section applies to discharges resulting from the production of red meat carcasses, in whole or part, by simple slaughterhouses.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 432.11 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 432.14 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 432.16 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 29, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.4202  Complex Slaughterhouse

a)	Applicability.  This Section applies to discharges resulting from the production of red meat carcasses, in whole or part, by complex slaughterhouses.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 432.21 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 432.24 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 432.26 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 29, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.4203  Low-Processing Packinghouse

a)	Applicability.  This Section applies to discharges resulting from the production of red meat carcasses in whole or part, by low-processing packinghouses.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 432.31 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 432.34 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 432.36 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 29, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.4204  High-Processing Packinghouse

a)	Applicability.  This Section applies to discharges resulting from the production of red meat carcasses, in whole or part, by high-processing packinghouses.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 432.41 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 432.44 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 432.46 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after October 29, 1973.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.4205  Small Processor

a)	Applicability.  This Section applies to discharges resulting from the production of finished meat products such as fresh meat cuts, smoked products, canned products, hams, sausages, luncheon meats, or similar products by a small processor.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 432.51 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 432.56 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after August 28, 1974.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.4206  Meat Cutter

a)	Applicability.  This Section applies to discharges resulting from the fabrication or manufacture of fresh meat cuts such as steaks, roasts, chops, etc. by a meat cutter.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 432.61 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 432.66 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after August 28, 1974.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.4207  Sausage and Luncheon Meats Processor

a)	Applicability.  This Section applies to discharges resulting from the manufacture of fresh meat cuts, sausage, bologna, and other luncheon meats by a sausage and luncheon meat processor.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 432.71 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 432.76 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after August 28, 1974.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.4208  Ham Processor

a)	Applicability.  This Section applies to discharges resulting from the manufacture of hams alone or in combination with other finished products by a ham processor.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 432.81 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 432.86 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after August 28, 1974.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.4209  Canned Meats Processor

a)	Applicability.  This Section applies to discharges resulting from the manufacture of canned meats, alone or in combination with any other finished products, by a canned meats processor.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 432.91 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 432.96 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after August 28, 1974.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.4210  Renderer

a)	Applicability.  This Section applies to discharges resulting from the manufacture of meat meal, dried animal by-product residues (tankage), animal oils, grease, and tallow, perhaps including hide curing, by a renderer.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 432.101 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 432.106 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after August 28, 1974.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART BH: METAL FINISHING

Section 307.4300  General Provisions

a)	Applicability.  Except as provided in subsection (a)(3) or (a)(4), this Subpart BH applies to discharges resulting from plants that perform on any basis material any of the six metal finishing operations listed in subsection (a)(1).  If any of those six operations are present, then this Subpart BH applies also to discharges from the forty process operations specified in subsection (a)(2).

1)	Six metal finishing operations.  Electroplating, electroless plating, anodizing, coating (chromating, phosphating, and coloring), chemical etching and milling, and printed circuit board manufacture.

2)	Forty additional process operations.  Cleaning, machining, grinding, polishing, tumbling, burnishing, impact deformation, pressure deformation, shearing, heat treating, thermal cutting, welding, brazing, soldering, flame spraying, sandblasting, other abrasive jet machining, electric discharge machining, electrochemical machining, electron beam machining, laser beam machining, plasma arc machining, ultrasonic machining, sintering, laminating, hot-dip coating, sputtering, vapor plating, thermal infusion, salt bath descaling, solvent degreasing, paint stripping, painting, electrostatic painting, electropainting, vacuum metalizing, assembly, calibration, testing, and mechanical plating.

3)	Exclusions.  This Subpart BH does not apply if there are pretreatment standards that are effective and applicable for the following industrial categories:

A)	Nonferrous metal smelting and refining (Subpart V;

B)	Coil coating (Subpart CN);

C)	Porcelain enameling (Subpart CO);

D)	Battery manufacturing (Subpart CJ);

E)	Iron and steel (Subpart U);

F)	Metal casting foundries (Subpart CM);

G)	Aluminum forming (Subpart CP);

H)	Copper forming (Subpart CQ);

I)	Plastic molding and forming (Subpart CL);

J)	Nonferrous forming (Subpart CT); and

K)	Electrical and electronic components (Subpart CR.

4)	This Subpart BH does not apply to the following:

A)	Metallic platemaking and gravure cylinder preparation conducted within or for printing and publishing facilities; and

B)	Existing indirect discharging job shops and independent printed circuit board manufacturers that are covered by Subpart N.

b)	General Definitions.  The Board incorporates by reference 40 CFR 433.11 (2003).  This incorporation includes no later amendments or editions.

c)	Monitoring Requirements.  The Board incorporates by reference 40 CFR 433.12 (2003).  This incorporation includes no later amendments or editions.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.4301  Metal Finishing

a)	Existing Sources

1)	The Board incorporates by reference 40 CFR 433.15 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (a)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

b)	New Sources

1)	The Board incorporates by reference 40 CFR 433.17 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (b)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after August 31, 1982.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

SUBPART BJ:  OIL AND GAS EXTRACTION

Section 307.4503  Onshore Facility Standards

a)	Applicability.  This Section applies to facilities engaged in the production, field exploration, drilling, well completion, and well treatment in the oil and gas extraction industry that are not included within subpart F of 40 CFR 435 (2016) (Stripper Subcategory), incorporated by reference.  This incorporation includes no later amendments or editions.

BOARD NOTE:  Subsection (a) is derived from 40 CFR 435.30 (2016).

b)	Specialized Definitions.  For the purpose of this Section, the general definitions, abbreviations, and methods of analysis set forth in 40 CFR 401 (2016), incorporated by reference, apply to this Section.  This incorporation includes no later amendments or editions.

BOARD NOTE:  Subsection (b) is derived from 40 CFR 435.31 (2016).

c)	Pretreatment Standards for Existing Sources (PSES).

1)	PSES for Wastewater from Unconventional Oil and Gas Extraction.  Except as provided in 40 CFR 403.7 and 403.13 (2016), incorporated by reference, any existing source subject to this Section must achieve the following PSES.  This incorporation by reference includes no later amendments or editions.

A)	There must be no discharge of wastewater pollutants associated with production, field exploration, drilling, well completion, or well treatment for unconventional oil and gas extraction (including, but not limited to, drilling muds, drill cuttings, produced sand, or produced water) into publicly owned treatment works.

B)	For the purposes of this subsection (c) and subsection (d), the following definitions apply:

i)	“Unconventional oil and gas” means crude oil and natural gas produced by a well drilled into a shale or tight formation (including, but not limited to, shale gas, shale oil, tight gas, or tight oil).

ii)	“Drill cuttings” means the particles generated by drilling into subsurface geologic formations and carried out from the wellbore with the drilling fluid.

iii)	“Drilling mud” means the circulating fluid (mud) used in the rotary drilling of wells to clean and condition the hole and to counterbalance formation pressure.

iv)	“Produced sand” means the slurried particles used in hydraulic fracturing, the accumulated formation sands, and scale particles generated during production.  Produced sand also includes desander discharge from the produced water waste stream and blowdown of the water phase from the produced water treating system.

v)	“Produced water” means the fluid brought up from the hydrocarbon-bearing strata during the extraction of oil and gas, and includes, where present, formation water, injection water, and any chemicals added downhole or during the oil/water separation process.

C)	Compliance Deadline for Existing Sources.  Existing sources lawfully discharging into publicly owned treatment works on or between April 7, 2015 and June 28, 2016 must comply with the PSES before August 29, 2019.  All other existing sources must immediately comply.

BOARD NOTE:  Corresponding 40 CFR 435.33(a)(3) provides, “All other existing sources shall comply by August 29, 2016.”

2)	PSES for Wastewater from Conventional Oil and Gas Extraction.  The pretreatment standards of this subsection (c) do not apply to conventional oil and gas extraction.  This subsection (c)(2) corresponds with 40 CFR 435.33(b), which USEPA marked “reserved.”

BOARD NOTE:  Subsection (c) is derived from 40 CFR 435.33 (2016), as amended at 81 Fed. Reg. 67191 (Sept. 30, 2016).

d)	Pretreatment Standards for New Sources (PSNS).

1)	Wastewater from Unconventional Oil and Gas Extraction.  Except as provided in 40 CFR 403.7 and 403.13 (2016), incorporated by reference, any new source with discharges subject to this Section must achieve the following PSNS.  This incorporation by reference includes no later amendments or editions.  There must be no discharge of wastewater pollutants associated with production, field exploration, drilling, well completion, or well treatment for unconventional oil and gas extraction (including, but not limited to, drilling muds, drill cuttings, produced sand, or produced water) into publicly owned treatment works.

2)	PSNS for Wastewater from Conventional Oil and Gas Extraction.  The pretreatment standards of this subsection (d) do not apply to conventional oil and gas extraction.  This subsection (d)(2) corresponds with 40 CFR 435.34(b), which USEPA marked “reserved.”

BOARD NOTE:  Subsection (d) is derived from 40 CFR 435.34 (2016).

(Source:  Added at 41 Ill. Reg. 1129, effective January 23, 2017)

Section 307.4508  Coalbed Methane Subcategory

The pretreatment standards of Section 307.4503 do not apply to coalbed methane.  This Section corresponds with subpart H of 40 CFR 435, which USEPA marked “reserved”.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART BL:  CENTRALIZED WASTE TREATMENT

Section 307.4700  General Provisions

a)	Applicability

1)	The Board incorporates by reference 40 CFR 437.1 (2003).  This incorporation includes no later amendments or editions.

2)	This Subpart BL applies to the portion of wastewater discharges from a centralized waste treatment facility resulting from any of the following activities, as defined in the materials incorporated by reference in subsection (a)(1):

A)	Treatment and recovery of hazardous or non-hazardous industrial metal-bearing wastes, oily wastes, and organic-bearing wastes received from off-site; and

B)	The treatment of centralized waste treatment wastewater.

b)	General Definitions.  The Board incorporates by reference 40 CFR 437.2 (2003).  This incorporation includes no later amendments or editions.

c)	General Pretreatment Standards.  Any source subject to this Subpart BL that introduces process wastewater pollutants into a publicly owned treatment works (POTW) must comply with Subpart B of 35 Ill. Adm. Code 307 and 35 Ill. Adm. Code 310.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.4701  Metals Treatment and Recovery

a)	Applicability.  This Section applies to the portion of the discharge of wastewater from a centralized waste treatment facility resulting from the treatment of, or recovery of metals from, both metal-bearing wastes received from off-site and other centralized waste treatment wastewater associated with the treatment of, or recovery of metal-bearing wastes.  The Board incorporates by reference 40 CFR 437.10 (2003).  This incorporation includes no later amendments or editions.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 437.15 (2003), as amended at 68 Fed. Reg. 71014 (December 22, 2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 437.16 (2003), as amended at 68 Fed. Reg. 71014 (December 22, 2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.4702  Oils Treatment and Recovery

a)	Applicability.  This Section applies to the portion of the discharge of wastewater from a centralized waste treatment facility resulting from the treatment of, or recovery of oil from both oily wastes received from, off-site and other centralized waste treatment wastewater associated with the treatment of, or recovery of, oily wastes.  The Board incorporates by reference 40 CFR 437.20 (2003).  This incorporation includes no later amendments or editions.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 437.25 (2003), as amended at 68 Fed. Reg. 71014 (December 22, 2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 437.26 (2003), as amended at 68 Fed. Reg. 71014 (December 22, 2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.4703  Organics Treatment and Recovery

a)	Applicability.  This Section applies to the portion of the discharge of wastewater from a centralized waste treatment facility resulting from the treatment of, or recovery of organic material from both organic wastes received from, off-site and other centralized waste treatment wastewater associated with the treatment of, or recovery of, organic wastes.  The Board incorporates by reference 40 CFR 437.30 (2003).  This incorporation includes no later amendments or editions.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 437.35 (2003), as amended at 68 Fed. Reg. 71014 (December 22, 2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources.

1)	The Board incorporates by reference 40 CFR 437.36 (2003), as amended at 68 Fed. Reg. 71014 (December 22, 2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.4704  Multiple Waste Streams

a)	Applicability.  Facilities that treat wastes subject to more than one of the previous Sections in this Subpart BL must comply with either the provisions of this Section or the applicable provisions of Section 307.4701, 307.4702, or 307.4703.  This Section applies to the portion of the discharge of wastewater from a centralized waste treatment facility resulting from mixing any combination of treated or untreated waste otherwise subject to Section 307.4701, 307.4702, or 307.4703.  The Board incorporates by reference 40 CFR 437.40 (2003).  This incorporation includes no later amendments or editions.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 437.46 (2003), as amended at 68 Fed. Reg. 71014 (December 22, 2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 437.47 (2003), as amended at 68 Fed. Reg. 71014 (December 22, 2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART BN:  PHARMACEUTICAL MANUFACTURING

Section 307.4900	General Provisions

a)	Applicability.  This Subpart BN applies to any pharmaceutical manufacturing facility that introduces or may introduce process wastewater pollutants into a POTW.  The Board incorporates by reference 40 CFR 439.0 (2004).  This incorporation includes no later amendments or editions.

b)	General definitions.  The Board incorporates by reference 40 CFR 439.1 (2004).  This incorporation includes no later amendments or editions.

c)	Monitoring requirements.  The Board incorporates by reference 40 CFR 439.2 and Appendix A (2004).  This incorporation includes no later amendments or editions.

d)	General pretreatment standards.  The Board incorporates by reference 40 CFR 439.3 (2004).  This incorporation includes no later amendments or editions.

e)	General limitation or standard for pH.  The Board incorporates by reference 40 CFR 439.4 (2004).  This incorporation includes no later amendments or editions.

(Source:  Amended 29 Ill. Reg. 6921, effective April 26, 2005)

Section 307.4901  Fermentation Products

a)	Applicability.  This Section applies to discharges resulting from the manufacture of pharmaceuticals by fermentation.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 439.11 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 439.16 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 439.17 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after November 26, 1982.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.4902  Extraction Products

a)	Applicability.  This Section applies to discharges of wastewater resulting from the manufacture of pharmaceuticals by extraction into the collection system of a POTW.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 439.21 (2004).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 439.26 (2004).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 439.27 (2004).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after November 26, 1982.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.4903  Chemical Synthesis Products

a)	Applicability.  This Section applies to discharges of wastewater resulting from the manufacture of pharmaceuticals by chemical synthesis into the collection system of a POTW.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 439.31 (2004).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 439.36 (2004).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 439.37 (2004).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after November 26, 1982.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.4904  Mixing/Compounding and Formulation

a)	Applicability.  This Section applies to discharges resulting from mixing/compounding and formulation operations of pharmaceutical products.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 439.41 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 439.46 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 439.47 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after November 26, 1982.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

#### Section 307.4905	Research (Repealed)

(Source: Repealed at 23 Ill. Reg. 8421, effective July 12, 1999)

SUBPART BP:  DENTAL OFFICE

Section 307.5100	Dental Offices

a)	Applicability.

1)	Except as provided in subsections (a)(3), (a)(4), and (a)(5), this Subpart BP applies to dental dischargers.

2)	Unless otherwise designated by the control authority, a dental discharger subject to this Subpart BP is not a significant industrial user, as defined in 35 Ill. Adm. Code 310, and is not a categorical industrial user or industrial user subject to categorical pretreatment standards as a result of applicability of this Subpart BP.

3)	This Subpart BP does not apply to dental dischargers that exclusively practice one or more of the following dental specialties:  oral pathology, oral and maxillofacial radiology, oral and maxillofacial surgery, orthodontics, periodontics, or prosthodontics.

4)	This Subpart BP does not apply to wastewater discharges from a mobile unit operated by a dental discharger.

5)	This Subpart BP does not apply to dental dischargers that do not discharge any amalgam process wastewater to a POTW, such as dental dischargers that collect all dental amalgam process wastewater for transfer to a centralized waste treatment facility, as defined in 40 CFR 437.2(c).

6)	A dental discharger that does not place dental amalgam and does not remove amalgam, except in limited emergency or unplanned, unanticipated circumstances, and which certifies these facts to the control authority, as required in subsection (d), is exempt from any further requirements of this Subpart BP.

b)	General Definitions.  The Board incorporates by reference 40 CFR 441.20 (2017), as added at 82 Fed. Reg. 30997 (July 5, 2017).  This incorporation includes no later amendments or editions.

c)	Pretreatment Standards for Existing Sources (PSES).  The Board incorporates by reference 40 CFR 441.30 (2017), as added at 82 Fed. Reg. 30997 (July 5, 2017).  This incorporation includes no later amendments or editions.

d)	Pretreatment Standards for New Sources (PSNS).  The Board incorporates by reference 40 CFR 441.40 (2017), as added at 82 Fed. Reg. 30997 (July 5, 2017).  This incorporation includes no later amendments or editions.

e)	Reporting and Recordkeeping Requirements.  The Board incorporates by reference 40 CFR 441.50 (2017), as added at 82 Fed. Reg. 30997 (July 5, 2017).  This incorporation includes no later amendments or editions.

### (Source:  Added at 42 Ill. Reg. 10676, effective May 29, 2018)

### SUBPART BQ:  TRANSPORTATION EQUIPMENT CLEANING

Section 307.5200  General Provisions

a)	Applicability

1)	The Board incorporates by reference 40 CFR 442.1 (2003).  This incorporation includes no later amendments or editions.

2)	This Subpart BQ applies to discharges resulting from cleaning the interior of tanks used to transport chemical, petroleum, or food-grade cargos, as defined in the materials incorporated by reference in subsection (a)(1).

b)	General Definitions.  The Board incorporates by reference 40 CFR 442.2 (2003).  This incorporation includes no later amendments or editions.

c)	General Pretreatment Standards.  Any source subject to this Subpart BQ that introduces process wastewater pollutants into a publicly owned treatment works (POTW) must comply with Subpart B of 35 Ill. Adm. Code 307 and 35 Ill. Adm. Code 310.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.5201  Tank Trucks and Intermodal Tank Containers Transporting Chemical and Petroleum Cargos

a)	Applicability.  This Section applies to discharges of wastewater resulting from the cleaning of tank trucks and intermodal tank containers that have been used to transport chemical or petroleum cargos into the collection system of a POTW.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 442.15 (2004).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 442.16 (2004), as amended at 70 Fed. Reg. 5058 (Feb. 1, 2005).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.5202  Rail Tank Cars Transporting Chemical and Petroleum Cargos

a)	Applicability.  This Section applies to discharges resulting from the cleaning of rail tank cars that have been used to transport chemical or petroleum cargos.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 442.25 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 442.26 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.5203  Tank Barges and Ocean/Sea Tankers Transporting Chemical and Petroleum Cargos

a)	Applicability.  This Section applies to discharges resulting from the cleaning of tank barges or ocean/sea tankers that have been used to transport chemical or petroleum cargos.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 442.35 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 442.36 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.5204  Tanks Transporting Food Grade Cargos

This Section applies to discharges resulting from the cleaning of tank trucks, intermodal tank containers, rail tank cars, tank barges, and ocean/sea tankers that have been used to transport food-grade cargos.  If wastewater generated from cleaning tanks used to transport food grade cargos is mixed with wastewater resulting from cleaning tanks used to transport chemical or petroleum cargos, then the combined wastewater is subject to the provisions established for the corresponding tanks in Sections 307.5201, 307.5202, or 307.5203.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART BR:  PAVING AND ROOFING MATERIALS (TARS AND ASPHALT)

Section 307.5301  Asphalt Emulsion

a)	Applicability.  This Section applies to discharges resulting from the production of asphalt paving and roofing emulsions.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 443.11 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 443.16 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after January 10, 1975.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.5302  Asphalt Concrete

a)	Applicability.  This Section applies to discharges resulting from the production of asphalt concrete.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 443.21 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 443.26 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after January 10, 1975.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.5303  Asphalt Roofing

a)	Applicability.  This Section applies to discharges resulting from the production of asphalt roofing materials.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 443.31 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 443.36 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after January 10, 1975.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.5304  Linoleum and Printed Asphalt Felt

a)	Applicability.  This Section applies to discharges resulting from the production of linoleum and printed asphalt felt floor coverings.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 443.41 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 443.46 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after January 10, 1975.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART BS:  WASTE COMBUSTORS

Section 307.5401  Commercial Hazardous Waste Combustor

a)	Applicability

1)	The Board incorporates by reference 40 CFR 444.10 (2003).  This incorporation includes no later amendments or editions.

2)	This Section applies to discharges of wastewater that are associated with Commercial Hazardous Waste Combustor wastewater, as defined in the materials incorporated by reference in subsection (a)(1).

b)	Definitions.  The Board incorporates by reference 40 CFR 444.11 (2003).  This incorporation includes no later amendments or editions.

c)	Monitoring Requirements.  The Board incorporates by reference 40 CFR 444.12 (2003).  This incorporation includes no later amendments or editions.

d)	Existing Sources

1)	The Board incorporates by reference 40 CFR 444.16 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

e)	New Sources

1)	The Board incorporates by reference 40 CFR 444.18 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (e)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART BT:  LANDFILLS

Section 307.5500  General Provisions

a)	Applicability

1)	The Board incorporates by reference 40 CFR 445.1 (2003).  This incorporation includes no later amendments or editions.

2)	This Section applies to discharges of wastewater from landfill units, as defined in the materials incorporated by reference in subsection (a)(1).

b)	General Definitions.  The Board incorporates by reference 40 CFR 445.2 (2003).  This incorporation includes no later amendments or editions.

c)	General Pretreatment Standards.  Any source subject to this Section that introduces wastewater pollutants into a publicly owned treatment works (POTW) must comply with Subpart B and 35 Ill. Adm. Code 310.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.5501  RCRA Subtitle C Hazardous Waste Landfill

a)	Applicability.  Except as provided in Section 307.5500, this Section applies to discharges of wastewater from landfills subject to the provisions of Subpart N of 35 Ill. Adm. Code 724 and Subpart N (landfills) of 35 Ill. Adm. Code 725.

b)	Existing Sources.  Any source subject to this Section that introduces wastewater pollutants into a publicly owned treatment works (POTW) must comply with Subpart B of 35 Ill. Adm. Code 307 and 35 Ill. Adm. Code 310.

c)	New Sources.  Any source subject to this Section that introduces wastewater pollutants into a publicly owned treatment works (POTW) must comply with Subpart B of 35 Ill. Adm. Code 307 and 35 Ill. Adm. Code 310.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.5502  RCRA Subtitle D Non-Hazardous Waste Landfill

a)	Applicability.  Except as provided in Section 307.5500, this Section applies to discharges of wastewater from landfills subject to the provisions of 35 Ill. Adm. Code 810 and 811 and 40 CFR 257.

b)	Existing Sources.  Any source subject to this Section that introduces wastewater pollutants into a publicly owned treatment works (POTW) must comply with Subpart B of 35 Ill. Adm. Code 307 and 35 Ill. Adm. Code 310.

c)	New Sources.  Any source subject to this Section that introduces wastewater pollutants into a publicly owned treatment works (POTW) must comply with Subpart B of 35 Ill. Adm. Code 307 and 35 Ill. Adm. Code 310.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

#### SUBPART BU:  PAINT FORMULATING

Section 307.5601  Oil-Base Solvent Wash Paint

a)	Applicability.  This Section applies to discharges resulting from the production of oil-base paint where the tank cleaning is performed using solvents.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 446.11 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources.

1)	The Board incorporates by reference 40 CFR 446.16 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after February 26, 1975.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART BV:  INK FORMULATING

Section 307.5701  Oil-Base Solvent Wash Ink

a)	Applicability.  This Section applies to discharges resulting from the production of oil-base ink where the tank washing system uses solvents.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 447.11 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 447.16 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after February 26, 1975.

### (Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART CD: PESTICIDE CHEMICALS

#### Section 307.6500	General Provisions

a)	General definitions.  The Board incorporates by reference 40 CFR 455.10 (2003).  This incorporation includes no later amendments or editions.

b)	Compliance date.  The Board incorporates by reference 40 CFR 455.11 (2003).  This incorporation includes no later amendments or editions.

(Source:  Amended at 28 Ill. Reg. 3076, effective February 6, 2004)

Section 307.6501  Organic Pesticide Chemicals Manufacturing

a)	Applicability

1)	The Board incorporates by reference 40 CFR 455.20 (2003).  This incorporation includes no later amendments or editions.

2)	This Section applies to discharges resulting from any plant that manufactures organic pesticide chemicals, as defined in the materials incorporated by reference in subsection (a)(1).

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 455.21 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 455.26 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 455.27 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.6502  Metallo-Organic Pesticides Chemicals Manufacturing

a)	Applicability.  This Section applies to discharges resulting from the manufacture of metallo-organic active ingredients containing mercury, cadmium, arsenic, or copper.  The manufacture of the intermediates used to manufacture the active ingredients is excluded from this Section.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 455.31 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B

d)	New Sources.  All sources are regulated as existing sources.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.6503  Pesticide Chemicals Formulating and Packaging

a)	Applicability

1)	The Board incorporates by reference 40 CFR 455.40 (2005).  This incorporation includes no later amendments or additions.

2)	This Section applies to discharges resulting from all pesticide formulating and packaging operations, as defined in the materials incorporated in subsection (a)(1).

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 455.41 (2005).  This incorporation includes no later amendments or additions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 455.46 (2005).  This incorporation includes no later amendments or additions.

2)	No person subject to the pretreatment standards incorporated by reference in section subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 455.47 (2005).  This incorporation includes no later amendments or additions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	New source means any building, structure, facility, or installation whose construction commenced after April 14, 1994.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.6505  Repackaging of Agricultural Pesticides Performed at Refilling Establishments

a)	Applicability

1)	The Board incorporates by reference 40 CFR 455.60 (2005).  This incorporation includes no later amendments or additions.

2)	This Section applies to discharges resulting from all pesticide formulating and packaging operations, as defined in the materials incorporated in subsection (a)(1).

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 455.61 (2005).  This incorporation includes no later amendments or additions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 455.66 (2005).  This incorporation includes no later amendments or additions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 455.67 (2005).  This incorporation includes no later amendments or additions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	New source means any building, structure, facility, or installation whose construction commenced after April 14, 1994.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART CG:  CARBON BLACK MANUFACTURING

Section 307.6801  Carbon Black Furnace Process

a)	Applicability.  This Section applies to discharges resulting from the production of carbon black by the furnace process.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 458.11 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 458.16 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after May 18, 1976.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.6802  Carbon Black Thermal Process

a)	Applicability.  This Section applies to discharges resulting from the production of carbon black by the thermal process.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 458.21 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 458.26 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after May 18, 1976.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.6803  Carbon Black Channel Process

a)	Applicability.  This Section applies to discharges resulting from the production of carbon black by the channel process.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 458.31 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources.

1)	The Board incorporates by reference 40 CFR 458.36 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after May 18, 1976.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.6804  Carbon Black Lamp Process

a)	Applicability.  This Section applies to discharges resulting from the production of carbon black by the lamp process.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 458.41 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 458.46 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after May 18, 1976.

### (Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART CJ: BATTERY MANUFACTURING

Section 307.7100  General Provisions

a)	Applicability.  This Subpart CJ applies to any battery manufacturing plant that introduces or may introduce process wastewater pollutants into a POTW.  Battery manufacturing operations regulated under this Subpart CJ are not subject to Subpart N or AH.

b)	General Definitions.  The Board incorporates by reference 40 CFR 461.2 (2003).  This incorporation includes no later amendments or editions.

c)	Monitoring Requirements.  The Board incorporates by reference 40 CFR 461.3 (2003).  This incorporation includes no later amendments or editions.

d)	Compliance Dates.  The Board incorporates by reference 40 CFR 461.4 (2003).  This incorporation includes no later amendments or editions.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.7101  Cadmium

a)	Applicability.  This Section applies to discharges resulting from the manufacturing of cadmium anode batteries.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 461.14 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 461.15 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after November 10, 1982.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.7102  Calcium

a)	Applicability.  This Section applies to discharges resulting from manufacturing calcium anode batteries.

b)	Specialized Definitions.  None.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 461.25 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after November 10, 1982.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.7103  Lead

a)	Applicability.  This Section applies to discharges resulting from the manufacturing of lead anode batteries.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 461.34 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 461.35 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after November 10, 1982.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.7104  Leclanche

a)	Applicability.  This Section applies to discharges resulting from manufacturing Leclanche-type batteries (zinc anode batteries with acid electrolyte).

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 461.44 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 461.45 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after November 10, 1982.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.7105  Lithium

a)	Applicability.  This Section applies to discharges resulting from the manufacturing of lithium anode batteries.

b)	Specialized Definitions.  None.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 461.55 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after November 10, 1982.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.7106  Magnesium

a)	Applicability.  This Section applies to discharges resulting from the manufacturing of magnesium anode batteries.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 461.64 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 461.65 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after November 10, 1982.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.7107  Zinc

a)	Applicability.  This Section applies to discharges resulting from the manufacturing of zinc anode batteries.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 461.74 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 461.75 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after November 10, 1982.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART CL: PLASTICS MOLDING AND FORMING

Section 307.7300  General Provisions

a)	Applicability

1)	This Subpart CL applies to any plastics molding and forming process that introduces or may introduce process wastewater pollutants into a POTW.  Plastics molding and forming processes include processes that blend, mold, form, or otherwise process plastic materials into intermediate or final plastic products.  They include commonly recognized processes such as extrusion, molding, coating and laminating, thermoforming, calendering, casting, foaming, cleaning, and finishing.

2)	Plastics molding and forming processes (e.g., extrusion and pelletizing) used by plastics resin manufacturers to process crude intermediate plastic material for shipment off-site are excluded from this Subpart and regulated under the organic chemicals, plastics, and synthetic fibers category.  Plastics molding and forming processes used by plastic resin manufacturers to process crude intermediate plastic materials, which are further processed on-site into intermediate or final plastics products in molding and forming processes, are controlled by this Subpart CL.

3)	Processes that coat a plastic material onto a substrate may fall within the definition of electroplating and metal finished as defined in Subpart N or AH.  These coating processes are excluded from the electroplating and metal finishing point source categories and are subject to this Subpart CL.

4)	Coating of plastic material onto a formed metal substrate is also covered by this Subpart and is not covered by the specific metal forming rules such as aluminum forming, copper forming and nonferrous metals forming, (Subparts CP, CQ, and CT, respectively).  This Subpart CL applies only to the coating process; the metal forming operations are subject to the specific metal forming rules.

5)	Research and development laboratories that produce plastic products using a plastics molding and forming process are subject to this Subpart CL if the plastics molding and forming process discharges process water.  The mass of plastic product produced in the plastics molding and forming process is not considered when determining the applicability of this Subpart CL to plastics molding and forming processes at research and development laboratories.

6)	Chemical and thermal reticulation processes for polyurethane foam are not subject to this Subpart CL.  Water used in those processes is not considered to be process water as defined in 40 CFR 463.2 (2003), incorporated by reference in subsection (b).  Processes used to further mold or form the reticulated foam are subject, however, to this Subpart if they discharge process water.

7)	Processes used to regenerate cellulose and to produce a product (e.g., rayon) from the regenerated cellulose are not subject to the limitations and standards in this Subpart.  Processes that mold or form cellulose derivatives (e.g., cellulose acetate) are subject to this Subpart CL if they discharge process water.

b)	General Definitions.  The Board incorporates by reference 40 CFR 463.2 (2003).  This incorporation includes no later amendments or editions.

c)	Monitoring Requirements.  The Board incorporates by reference 40 CFR 463.3 (2003).  This incorporation includes no later amendments or editions.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.7301  Contact Cooling and Heating Water

a)	Applicability.  This Section applies to the introduction into POTWs of pollutants resulting from processes in the contact cooling and heating water subcategory.  Processes in the contact cooling and heating water subcategory are processes where process water comes in contact with plastic materials or plastic products for the purpose of heat transfer during plastics molding and forming.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 463.11 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 463.15 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 463.16 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after February 15, 1984.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.7302  Cleaning Water

a)	Applicability.  This Section applies to the introduction into POTWs of pollutants resulting from processes in the cleaning water subcategory.  Processes in the cleaning water subcategory are processes where water comes in contact with the plastic product for the purpose of cleaning the surface of the product and where water comes in contact with shaping equipment, such as molds and mandrels, that contact the plastic material for the purpose of cleaning the equipment surfaces.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 463.21 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 463.25 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 463.26 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after February 15, 1984.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.7303  Finishing Water

a)	Applicability.  This Section applies to the introduction into POTWs of pollutants resulting from processes in the finishing water subcategory.  Processes in the finishing water subcategory are processes where water comes in contact with the plastic product during finishing.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 463.31 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 463.35 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 463.36 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after February 15, 1984.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART CM: METAL MOLDING AND CASTING

#### Section 307.7400	General Provisions

a)	Applicability.  This Subpart CM applies to metal molding and casting facilities that introduce or may introduce process wastewater pollutants into a POTW.

b)	General definitions.  The Board incorporates by reference 40 CFR 464.02 (2003).  This incorporation includes no later amendments or editions.

c)	Monitoring requirements.  The Board incorporates by reference 40 CFR 464.03 (2003).  This incorporation includes no later amendments or editions.

d)	Compliance dates.  The Board incorporates by reference 40 CFR 464.04 (2003).  This incorporation includes no later amendments or editions.

(Source:  Amended at 28 Ill. Reg. 3076, effective February 6, 2004)

SUBPART CM: METAL MOLDING AND CASTING

Section 307.7401  Aluminum Casting

a)	Applicability.  This Section applies to discharges resulting from aluminum casting operations, as defined in 40 CFR 464.02, incorporated by reference in Section 307.7400(b).

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 464.11 (2005).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 464.15 (2005).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 464.16 (2005).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after November 15, 1982.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.7402  Copper Casting

a)	Applicability.  This Section applies to discharges resulting from copper casting operations.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 464.21 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 464.25 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 464.26 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after November 15, 1982.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.7403  Ferrous Casting

a)	Applicability.  This Section applies to discharges resulting from ferrous casting operations.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 464.31 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 464.35 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 464.36 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after November 15, 1982.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.7404  Zinc Casting

a)	Applicability.  This Section applies to discharges resulting from zinc casting operations.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 464.41 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 464.45 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 464.46 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after November 15, 1982.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART CN: COIL COATING

#### Section 307.7500	General Provisions

a)	Applicability.  This Subpart CN applies to any coil coating facility or to any canmaking facility that introduces or may introduce process wastewater pollutants into a POTW.

b)	General definitions.  The Board incorporates by reference 40 CFR 465.02 (2007).  This incorporation includes no later amendments or editions.

c)	Monitoring requirements.  The Board incorporates by reference 40 CFR 465.03 (2007).  This incorporation includes no later amendments or editions.

d)	Compliance dates.  The Board incorporates by reference 40 CFR 465.04 (2007).  This incorporation includes no later amendments or editions.

(Source:  Amended at 32 Ill. Reg. 18986, effective November 26, 2008))

#### Section 307.7501	Steel Basis Material

a)	Applicability.  This Section applies to discharges resulting from coil coating of steel basis material coils.

b)	Specialized definitions.  None.

c)	Existing sources.

1)	The Board incorporates by reference 40 CFR 465.14 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) of this Section may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of such standards.

d)	New sources.

1)	The Board incorporates by reference 40 CFR 465.15 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) of this Section may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of such standards.

3)	“New source” means any building, structure, facility, or installation the construction of which commenced after January 12, 1981.

(Source:  Amended at 28 Ill. Reg. 3076, effective February 6, 2004)

#### Section 307.7502	Galvanized Basis Material

a)	Applicability.  This Section applies to discharges resulting from coil coating of galvanized basis material coils.

b)	Specialized definitions.  None.

c)	Existing sources.

1)	The Board incorporates by reference 40 CFR 465.24 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) of this Section may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of such standards.

d)	New sources.

1)	The Board incorporates by reference 40 CFR 465.25 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) of this Section may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of such standards.

3)	“New source” means any building, structure, facility, or installation the construction of which commenced after January 12, 1981.

(Source:  Amended at 28 Ill. Reg. 3076, effective February 6, 2004)

#### Section 307.7503	Aluminum Basis Material

a)	Applicability.  This Section applies to discharges resulting from coil coating of aluminum basis material coils.

b)	Specialized definitions.  None.

c)	Existing sources.

1)	The Board incorporates by reference 40 CFR 465.34 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) of this Section may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of such standards.

d)	New sources.

1)	The Board incorporates by reference 40 CFR 465.35 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) of this Section may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of such standards.

3)	“New source” means any building, structure, facility, or installation the construction of which commenced after January 12, 1981.

(Source:  Amended at 28 Ill. Reg. 3076, effective February 6, 2004)

#### Section 307.7504	Canmaking

a)	Applicability.  This Section applies to discharges resulting from the manufacturing of seamless can bodies, which are washed.

b)	Specialized definitions.  None.

c)	Existing sources.

1)	The Board incorporates by reference 40 CFR 465.44 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) of this Section may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of such standards.

d)	New sources.

1)	The Board incorporates by reference 40 CFR 465.45 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) of this Section may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of such standards.

3)	“New source” means any building, structure, facility, or installation the construction of which commenced after February 10, 1983.

(Source:  Amended at 28 Ill. Reg. 3076, effective February 6, 2004)

### SUBPART CO: PORCELAIN ENAMELING

Section 307.7600  General Provisions

a)	Applicability

1)	Except as provided in subsection (a)(2) or (a)(3), this Subpart CO applies to any porcelain enameling facility that introduces or may introduce process wastewater pollutants into a POTW.

2)	Any existing porcelain enameling facility that prepares or coats less than 1600 square meters per day and which introduces less than 60,000 liters per day of wastewater into a POTW is not controlled by the pretreatment standards for existing sources in this Subpart.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

3)	This Subpart CO does not apply to porcelain enameling on precious metal basis material.

b)	General Definitions.  The Board incorporates by reference 40 CFR 466.02 (2003).  This incorporation includes no later amendments or editions.

c)	Monitoring Requirements.  The Board incorporates by reference 40 CFR 466.03 (2003).  This incorporation includes no later amendments or editions.

d)	Compliance Dates.  The Board incorporates by reference 40 CFR 466.04 (2003).  This incorporation includes no later amendments or editions.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.7601  Steel Basis Material

a)	Applicability.  This Section applies to discharges resulting from porcelain enameling on steel basis materials.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 466.14 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 466.15 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after January 27, 1981.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.7602  Cast Iron Basis Material

a)	Applicability.  This Section applies to discharges resulting from porcelain enameling of cast iron basis materials.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 466.24 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 466.25 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after January 27, 1981.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.7603  Aluminum Basis Material

a)	Applicability.  This Section applies to discharges resulting from porcelain enameling of aluminum basis materials.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 466.34 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 466.35 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after January 27, 1981.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.7604  Copper Basis Material

a)	Applicability.  This Section applies to discharges resulting from porcelain enameling of copper basis materials.

b)	Specialized Definitions.  None.

c)	Existing Sources. These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 466.45 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after January 27, 1981.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART CP: ALUMINUM FORMING

Section 307.7700  General Provisions

a)	Applicability

1)	Aluminum forming includes commonly recognized forming operations such as rolling, drawing, extruding, and forging, and related operations such as heat treatment, casting, and surface treatments.  Surface treatment of aluminum is any chemical or electrochemical treatment applied to the surface of the aluminum.  Such surface treatment is considered to be a part of aluminum forming whenever it is performed as an integral part of aluminum forming.  For this Subpart CP, surface treatment of aluminum is considered to be an integral part of aluminum forming whenever it is performed at the same plant site at which aluminum is formed and such operations are not considered for regulation under the electroplating and metal finishing provisions of Subparts N and AH.  Casting aluminum when performed as an integral part of aluminum forming and located on-site at an aluminum forming plant is considered an aluminum forming operation and is covered under this Subpart CP.  When aluminum forming is performed on the same site as primary aluminum reduction the casting must be regulated by Subpart CT (nonferrous metals) if there is no cooling of the aluminum before casting.  If the aluminum is cooled before casting, then the casting must be regulated by this Subpart CP.

2)	This Subpart CP applies to any aluminum forming facility, except for plants identified under subsection (a)(3), that introduces or may introduce pollutants into a POTW.

3)	This Subpart CP applies to indirect discharging aluminum forming plants that extrude less than three million pounds of product per year and draw, with emulsions or soaps, less than one million pounds per year.

b)	General Definitions.  The Board incorporates by reference 40 CFR 467.02 (2003).  This incorporation includes no later amendments or editions.

c)	Monitoring Requirements.  The Board incorporates by reference 40 CFR 467.03 (2003).  This incorporation includes no later amendments or editions.

d)	Compliance Dates.  The Board incorporates by reference 40 CFR 467.04 (2003).  This incorporation includes no later amendments or editions.

e)	Removal Credits.  The control authority may grant removal credits under Subpart C of 35 Ill. Adm. Code 310 for toxic metals limited in this Subpart CP when used as indicator pollutants.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.7701  Rolling With Neat Oils

a)	Applicability.  This Section applies to discharges resulting from the core and the ancillary operations of the rolling with neat oils subcategory.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 467.11 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 467.15 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 467.16 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after November 22, 1982.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.7702  Rolling With Emulsions

a)	Applicability.  This Section applies to discharges resulting from the core and the ancillary operations of the rolling with emulsions subcategory.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 467.21 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 467.25 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 467.26 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after November 22, 1982.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.7703  Extrusion

a)	Applicability.  This Section applies to discharges resulting from the core and the ancillary operations of the extrusion subcategory.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 467.31 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 467.35 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 467.36 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after November 22, 1982.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.7704  Forging

a)	Applicability.  This Section applies to discharges resulting from the core of the forging subcategory and the ancillary operations.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 467.41 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 467.45 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 467.46 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after November 22, 1982.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.7705  Drawing With Neat Oils

a)	Applicability.  This Section applies to discharges resulting from the core of the drawing with neat oils subcategory and the ancillary operations.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 467.51 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 467.55 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 467.56 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after November 22, 1982.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.7706  Drawing With Emulsions or Soaps

a)	Applicability.  This Section applies to discharges resulting from the core and the ancillary operations of the drawing with emulsions or soaps subcategory.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 467.61 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 467.65 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 467.66 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after November 22, 1982.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART CQ: COPPER FORMING

Section 307.7800  General Provisions

a)	Applicability

1)	This Subpart CQ applies to discharges resulting from the manufacture of formed copper and copper alloy products.  The forming operations covered are hot rolling, cold rolling, drawing, extrusion, and forging.  This Subpart CQ does not regulate the forming of precious metals.  (See Subpart CT.)  The casting of copper and copper alloys is not covered by this Subpart CQ.  (See Subpart CM.)

2)	The discharge allowance for drawing spent lubricant of 40 CFR 468.14(c) and 468.15(c), incorporated by reference in Section 307.7801(d)(1), applies only to those plants that actually discharge the drawing spent lubricant waste stream at copper forming sites.  No discharge allowance is applicable or allowable when these wastewaters are hauled off-site for disposal or are otherwise not discharged at copper forming sites.

b)	General Definitions.  The Board incorporates by reference 40 CFR 468.02 (2003).  This incorporation includes no later amendments or editions.

c)	Monitoring Requirements.  The Board incorporates by reference 40 CFR 468.03 (2003).  This incorporation includes no later amendments or editions.

d)	Compliance Dates.  The Board incorporates by reference 40 CFR 468.04 (2003).  This incorporation includes no later amendments or editions.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.7801  Copper Forming

a)	Applicability.  This Section applies to discharges resulting from the forming of copper and copper alloys except beryllium copper alloys.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 468.14 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 468.15 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after November 12, 1982.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.7802  Beryllium Copper Forming

a)	Applicability.  This Section applies to discharges resulting from the forming of beryllium copper alloys.

b)	Specialized Definitions.  None.

c)	Existing Sources.  These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources.  All sources are regulated as existing sources.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART CR: ELECTRICAL AND ELECTRONIC COMPONENTS

Section 307.7901  Semiconductor

a)	Applicability.  This Section applies to discharges resulting from all process operations associated with the manufacture of semiconductors, except sputtering, vapor deposition, and electroplating.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 469.12 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 469.16 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 469.18 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after August 8, 1982.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.7902  Electronic Crystals

a)	Applicability.  This Section applies to discharges resulting from the manufacture of electronic crystals.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 469.22 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 469.26 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 469.28 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after August 8, 1982.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.7903  Cathode Ray Tube

a)	Applicability.  This Section applies to discharges resulting from the manufacture of cathode ray tubes.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 469.31 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 469.34 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 469.36 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after March 9, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.7904  Luminescent Materials

a)	Applicability.  This Section applies to discharges resulting from the manufacture of luminescent materials.

b)	Specialized Definitions.  The Board incorporates by reference 40 CFR 469.41 (2003).  This incorporation includes no later amendments or editions.

c)	Existing Sources. These sources must comply with the general and specific pretreatment requirements of Subpart B.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 469.43 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after March 9, 1983.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

### SUBPART CT: NONFERROUS METALS FORMING AND METAL POWDERS

Section 307.8100  General Provisions

a)	Applicability

1)	This Subpart CT applies to the introduction of pollutants into a POTW from the forming of nonferrous metals (including nonferrous metal alloys), except beryllium, copper, and aluminum, and their alloys.  Aluminum alloys are defined as any alloy in which aluminum is the major constituent in percent by weight.  Copper alloys are defined as any alloy in which copper is the major constituent in percent by weight except when copper is alloyed with precious metals.  Any copper-precious metal alloy containing 30 percent or greater precious metal is considered a precious metal alloy for this Subpart CT.  Beryllium alloys are any alloy in which beryllium is present at 0.1 percent or greater.  This Subpart CT applies to the following:

A)	Forming operations, including rolling (both hot and cold), extruding forging, drawing, swaging, cladding, and tube reducing, and

B)	Ancillary operations performed as an integral part of the forming of these metals, including casting for subsequent forming, heat treatment, surface treatment, alkaline cleaning, solvent degreasing, product testing, surface coating, sawing, grinding, tumbling, burnishing, and wet air pollution control.

2)	This Subpart CT also applies to the introduction of pollutants into a POTW from mechanical metal powder production operations, the forming of parts from metal powders, and associated ancillary operations of the following:

A)	Iron, copper, and aluminum and their alloys; and

B)	The nonferrous metal and their alloys described in subsection (a)(1).  This Subpart CT does not regulate the production of metal powders by chemical means such as precipitation.  The production of metal powder as the final step in refining metal is regulated under nonferrous metals manufacturing, Subpart V.

3)	Surface treatment includes any chemical or electrochemical treatment applied to the surface of the metal.  For this Subpart CT, surface treatment of metals is considered to be an integral part of the forming of metals whenever it is performed at the same plant site at which the metals are formed.  These surface treatment operations are not regulated under the electroplating or metal finishing rules in Subpart N or AH.

4)	Casting is covered by this Subpart CT when it is performed as an integral part of the metal forming process and takes place at the same plant site at which metals are formed.  Such casting is not regulated under the provisions of metal molding and casting (Subpart CM).

5)	This Subpart CT does not apply to the forming of the metals cadmium, chromium, gallium, germanium, indium, lithium, manganese, neodymium, or praseodymium.

b)	General Definitions.  The Board incorporates by reference 40 CFR 471.02 (2003).  This incorporation includes no later amendments or editions.

c)	Compliance Dates.  The Board incorporates by reference 40 CFR 471.03 (2003).  This incorporation includes no later amendments or editions.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.8101  Lead-Tin-Bismuth Forming

a)	Applicability.  This Section applies to discharges resulting from the process operations of the lead-tin bismuth forming subcategory.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 471.14 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 471.15 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after March 5, 1984.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.8102  Magnesium Forming

a)	Applicability.  This Section applies to discharges resulting from the process operations of the magnesium forming subcategory.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 471.24 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 471.25 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after March 5, 1984.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.8103  Nickel-Cobalt Forming

a)	Applicability.  This Section applies to discharges resulting from the process operations of the nickel-cobalt forming subcategory.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 471.34 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 471.35 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after March 5, 1984.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.8104  Precious Metals Forming

a)	Applicability.  This Section applies to discharges resulting from the process operations of the precious metals forming subcategory.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 471.44 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 471.45 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after March 5, 1984.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.8105  Refractory Metals Forming

a)	Applicability.  This Section applies to discharges resulting from the process operations of the refractory metals forming subcategory.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 471.54 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 471.55 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after March 5, 1984.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.8106  Titanium Forming

a)	Applicability.  This Section applies to discharges resulting from the process operations of the titanium forming subcategory.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 471.64 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 471.65 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after March 5, 1984.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.8107  Uranium Forming

a)	Applicability.  This Section applies to discharges resulting from the process operations of the uranium forming subcategory.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 471.74 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 471.75 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after March 5, 1984.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.8108  Zinc Forming

a)	Applicability.  This Section applies to discharges resulting from the process operations of the zinc forming subcategory.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 471.84 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 471.85 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after March 5, 1984.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.8109  Zirconium-Hafnium Forming

a)	Applicability.  This Section applies to discharges resulting from the process operations of the zirconium-hafnium forming subcategory.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 471.94 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 471.95 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after March 5, 1984.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

Section 307.8110  Metal Powders

a)	Applicability.  This Section applies to discharges resulting from the process operations of the metal powders subcategory.

b)	Specialized Definitions.  None.

c)	Existing Sources

1)	The Board incorporates by reference 40 CFR 471.104 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (c)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

d)	New Sources

1)	The Board incorporates by reference 40 CFR 471.105 (2003).  This incorporation includes no later amendments or editions.

2)	No person subject to the pretreatment standards incorporated by reference in subsection (d)(1) may cause, threaten, or allow the discharge of any contaminant to a POTW in violation of those standards.

3)	"New source" means any building, structure, facility, or installation whose construction commenced after March 5, 1984.

(Source:  Amended at 47 Ill. Reg. 4662, effective March 23, 2023)

#### APPENDIX A	References to Previous Rules (Repealed)

(Source: Repealed at 12 Ill.  Reg.  2592, effective January 13, 1988)