# TITLE 35: ENVIRONMENTAL PROTECTION

# SUBTITLE B: AIR POLLUTION

## CHAPTER I: POLLUTION CONTROL BOARD

## SUBCHAPTER c: EMISSION STANDARDS AND LIMITATIONS

## FOR STATIONARY SOURCES

### PART 216

### CARBON MONOXIDE EMISSIONS

SUBPART A: GENERAL PROVISIONS

Section

| 216.100 | Scope and Organization |
| --- | --- |
| 216.101 | Measurement Methods |
| 216.102 | Abbreviations and Conversion Factors |
| 216.103 | Definitions |
| 216.104 | Incorporations by Reference |

SUBPART B: FUEL COMBUSTION EMISSION SOURCES

Section

| 216.121 | Fuel Combustion Emission Sources |
| --- | --- |
| 216.122 | Exception, Midwest Grain Products |

SUBPART C: INCINERATORS

Section

| 216.141 | Incinerators |
| --- | --- |
| 216.142 | Exceptions |

SUBPART N: PETROLEUM REFINING AND CHEMICAL MANUFACTURE

Section

| 216.361 | Petroleum and Petrochemical Processes |
| --- | --- |
| 216.362 | Polybasic Organic Acid Partial Oxidation Manufacturing Processes |

SUBPART O: PRIMARY AND FABRICATED METAL PRODUCTS

Section

| 216.381 | Cupolas |
| --- | --- |
| 216.382 | Exception, General Motor's Ferrous Foundry in Vermilion County |

| Appendix A | Rule into Section Table |
| --- | --- |
| Appendix B | Section into Rule Table |
| Appendix C | Compliance Dates |

AUTHORITY: Implementing Section 10 and authorized by Section 27 of the Environmental Protection Act (Ill.  Rev.  Stat.  1991, ch.  111 1/2, pars.  1010 and 1027).

SOURCE: Adopted as Chapter 2: Air Pollution, Rule 206: Carbon Monoxide Emissions, R71-23, 4 PCB 191, April 13, 1972, filed and effective April 14, 1972; amended at 3 Ill.  Reg.  47, p.  92, effective November 8, 1979; amended at 4 Ill.  Reg.  24, p.  514, effective June 4, 1980; codified at 7 Ill.  Reg.  13607; amended in R87-18 at 12 Ill.  Reg.  20774, effective December 6, 1988; amended in R90-23 at 16 Ill.  Reg.  18075, effective November 13, 1992.

### SUBPART A: GENERAL PROVISIONS

#### Section 216.100	Scope and Organization

a)	This Part contains standards and limitations for carbon monoxide emissions from stationary sources.

b)	Permits for sources subject to this Part may be required pursuant to 35 Ill.  Adm.  Code 201.

c)	Notwithstanding the provisions of this Part, the air quality standards contained in 35 Ill.  Adm.  Code 243 may not be violated.

d)	This Part includes Subparts arranged as follows:

1)	Subpart A: General Provisions;

2)	Subparts B-J: Fuel Combustion Sources and Incinerators;

3)	Subparts K-M: Reserved for Emission Process Sources;

4)	Subparts N-End: Industry and Site-specific rules.

e)	Rules have been grouped for convenience of the public; the scope of each is determined by its language and history.

#### Section 216.101	Measurement Methods

Carbon Monoxide concentrations in an effluent stream shall be measured by the non-dispersive infrared method or by other methods approved by the Illinois Environmental Protection Agency (Agency) according to the provisions of 35 Ill.  Adm.  Code 201.

#### Section 216.102	Abbreviations and Conversion Factors

a)	The following abbreviations are used in this Part:

|  | btu | British thermal unit (60F) |
| --- | --- | --- |
|  | CO | carbon monoxide |
|  | oC | degrees Centigrade |
|  | oF | degrees Fahrenheit |
|  | kg | kilograms |
|  | lbs | pounds |
|  | mmbtu/hr | million btu per hour |
|  | MW | Megawatts; one million watts |
|  | ppm | parts per million |

b)	The following conversion factors have been used in this Part:

|  | English | Metric |
| --- | --- | --- |
|  | 1.0 mmbtu/hr | 0.293 MW |
|  | 2.205 lbs | 1 kg |

#### Section 216.103	Definitions

The definitions contained in 35 Ill.  Adm.  Code 201 and 211 apply to this Part.

#### Section 216.104	Incorporations by Reference

The following materials are incorporated by reference: non-dispersive infrared method, 40 CFR 60, Appendix A, Method 10 (1982).

### SUBPART B: FUEL COMBUSTION EMISSION SOURCES

#### Section 216.121	Fuel Combustion Emission Sources

No person shall cause or allow the emission of carbon monoxide (CO) into the atmosphere from any fuel combustion emission source with actual heat input greater than 2.9 MW (10 mmbtu/hr) to exceed 200 ppm, corrected to 50 percent excess air.

#### Section 216.122	Exception, Midwest Grain Products

The standard for carbon monoxide of Section 216.121 does not apply to emissions from the fluidized bed combustion boiler of Midwest Grain Products of Illinois, located in Pekin, Illinois, where the emission of carbon monoxide shall not exceed 700 parts per million, corrected to 50 percent excess air.  Compliance shall be based upon a one-hour average.

(Source: Added at 12 Ill.  Reg.  20774, effective December 6, 1988)

### SUBPART C: INCINERATORS

#### Section 216.141	Incinerators

No person shall cause or allow the emission of carbon monoxide into the atmosphere from any incinerator to exceed 500 ppm, corrected to 50 percent excess air.

#### Section 216.142	Exceptions

Section 216.141 shall not apply to:

a)	Existing incinerators burning less than 907 kg (2000 lbs) of refuse per hour which are in compliance with 35 Ill.  Adm.  Code 212.181(c).

b)	Existing small explosive waste incinerators which meet the conditions of 35 Ill.  Adm.  Code 212.184(a).

(Source: Added at 4 Ill.  Reg.  24, p.  514, effective June 4, 1980)

### SUBPART N: PETROLEUM REFINING AND CHEMICAL MANUFACTURE

#### Section 216.361	Petroleum and Petrochemical Processes

a)	No person shall cause or allow the emission of a carbon monoxide waste gas stream into the atmosphere from a petroleum or petrochemical process unless such waste gas stream is burned in a direct flame afterburner or carbon monoxide boiler so that the resulting concentration of carbon monoxide in such waste gas stream is less than or equal to 200 ppm corrected to 50 percent excess air, or such waste gas stream is controlled by other equivalent air pollution control equipment approved by the Agency according to the provisions of 35 Ill.  Adm.  Code 201.

b)	Notwithstanding subsection (a), any existing petroleum or petrochemical process using catalyst regenerators of fluidized catalytic converters equipped for in situ combustion of carbon monoxide, may emit a carbon monoxide waste gas stream into the atmosphere if the carbon monoxide concentration of such waste gas stream is less than or equal to 750 ppm corrected to 50 percent excess air.

c)	Notwithstanding subsection (a), any new petroleum or petrochemical process using catalyst regenerators of fluidized catalytic converters equipped for in situ combustion of carbon monoxide, may emit a carbon monoxide waste gas stream into the atmosphere if the carbon monoxide concentration of such waste gas stream is less than or equal to 350 ppm corrected to 50 percent excess air.

#### Section 216.362	Polybasic Organic Acid Partial Oxidation Manufacturing Processes

No person shall cause or allow the emission of any gases containing carbon monoxide into the atmosphere from any polybasic organic acid partial oxidation manufacturing process unless the total fuel value of the waste gas stream is less than 30 percent of that required for flame incineration of the waste gas stream at 793°C (1460°F) without heat exchange.  Polybasic organic acid partial oxidation manufacturing processes not meeting the above conditions shall burn such waste gas stream in a direct flame afterburner to achieve a resulting concentration of carbon monoxide in such waste gas stream of less than or equal to 200 ppm or shall employ such other equivalent control method or equipment as may be approved by the Agency according to the provisions of 35 Ill. Adm. Code 201.

### SUBPART O: PRIMARY AND FABRICATED METAL PRODUCTS

#### Section 216.381	Cupolas

No person shall cause or allow the emission of gases containing carbon monoxide into the atmosphere from any cupola with a manufacturer's rated melt rate in excess of 5 tons per hour, unless such gases are burned in a direct flame afterburner so that the resulting concentration of carbon monoxide in such gases is less than or equal to 200 ppm corrected to 50 percent excess air or such gas streams are controlled by other equivalent pollution control equipment approved by the Agency according to the provisions of 35 Ill. Adm. Code 201.

#### Section 216.382	Exception, General Motor's Ferrous Foundry in Vermilion County

The standard for carbon monoxide of 35 Ill. Adm. Code 216.381 shall not apply to the existing ferrous foundry located adjacent to Interstate 74 at G Street in Vermilion County, owned by General Motors Corporation on the effective date of this regulation.  The emission of carbon monoxide from this foundry shall not exceed 2,000 ppm corrected to 50 percent excess air.

(Source: Added at 16 Ill.  Reg.  18075, effective November 13, 1992)

### Appendix A

### Rule into Section Table

|  | RULE | SECTION |
| --- | --- | --- |
|  | 206(a) | 216.121 |
|  | 206(b) | 216.141 |
|  | 206(b)(1) and (2) | 216.142 |
|  | 206(c) | 216.361 |
|  | 206(d) | Deleted |
|  | 206(e) | 216.381 |
|  | 206(f) | 216.101 |
|  | 206(g) | Appendix C |
|  | 206(h) | 216.362 |

### Appendix B

### Section into Rule Table

|  | SECTION | RULE |
| --- | --- | --- |
|  | 216.100 | --- |
|  | 216.101 | 206(f) |
|  | 216.102 | --- |
|  | 216.103 | --- |
|  | 216.104 | --- |
|  | 216.121 | 206(a) |
|  | 216.141 | 206(b) |
|  | 216.142 | 206(b)(1) and (2) |
|  | 216.361 | 206(c) |
|  | 216.362 | 206(h) |
|  | 216.381 | 206(e) |
|  | Appendix C | 206(g) |

### Appendix C

### Compliance Dates

Every owner or operator of a new emission source was required to comply with the standards and limitations of this Part by April 14, 1972.

Every owner or operator of an existing emission source was required to comply with the standards and limitations of this Part by December 31, 1973.