TITLE 35:  ENVIRONMENTAL PROTECTION

SUBTITLE C:  WATER POLLUTION

CHAPTER II:  ENVIRONMENTAL PROTECTION AGENCY

PART 355

DETERMINATION OF AMMONIA NITROGEN WATER QUALITY BASED EFFLUENT LIMITS

FOR DISCHARGES TO GENERAL USE WATERS

SUBPART A:  INTRODUCTION

Section

355.101   Purpose, Scope and Application

355.103   Definitions

SUBPART B:  AMMONIA NITROGEN (as N) WATER QUALITY STANDARDS AND WQBELs

Section

355.201   Introduction

355.203   Conversion of Total Ammonia and Un-ionized Ammonia Nitrogen

355.205   Estimation of Projected Effluent Quality

355.207   Mixing Allowance

355.209   Calculation of Preliminary Effluent Limitation

355.211   Summary of the Results for a Reasonable Potential Analysis and

the Determination of Ammonia Nitrogen WQBELs

SUBPART C:  EFFLUENT MODIFIED WATERS

Section

355.301   Introduction

355.303   EMW Application Requirements

355.305   Evaluation of EMW Applications

355.307   Determination of EMW Designation

355.309   Procedures for Delineating an EMW

355.311   Ammonia Nitrogen Decay Equation

355.313   Restrictions Applicable to Discharges with EMWs

355.315   Publication of EMWs

AUTHORITY:  Implementing and authorized by Section 39 of the Illinois

Environmental Protection Act [415 ILCS 5/39].

SOURCE:  Adopted at 23 Ill. Reg. 7267, effective June 9, 1999.

NOTE:  In this Part, unless the context clearly indicates otherwise,

superscript numbers or letters are denoted by parentheses; subscript are

denoted by brackets.

SUBPART A:  INTRODUCTION

Section 355.101  Purpose, Scope and Application

a)  This Part contains procedures to determine water quality based

effluent limits for ammonia nitrogen (as N) (ammonia nitrogen

WQBELs) that are necessary to prevent waters of the State from

exceeding water quality standards pursuant to 40 CFR 122.44(d)(1)

and 35 Ill. Adm. Code 309.141(d)(3).  Ammonia nitrogen WQBELs must

be sufficient to ensure compliance with the water quality

standards for ammonia nitrogen found in the Illinois Pollution

Control Board (IPCB) regulations at 35 Ill. Adm. Code 302.202,

302.212, 302.213 and 304.122.

b)  Ammonia nitrogen WQBELs  are applicable to the general use waters

of the State.

c)  There shall be an opportunity for compliance with the ammonia

nitrogen water quality standards as provided by the IPCB

regulations through application of allowed mixing, mixing zones

and zones of initial dilution at 35 Ill. Adm. Code 302.102 and

302.213.

d)  In addition to water quality based effluent limits, the discharge

of ammonia nitrogen from a facility may be limited based on other

provisions in the Environmental Protection Act [415 ILCS 5] (Act)

and regulations adopted thereunder or the Federal Water Pollution

Control Act, 33 USC 1251-1387 (FWPCA) and regulations adopted

thereunder.

Section 355.103  Definitions

All terms in this Part shall have the meanings set forth in the

Environmental Protection Act and in the IPCB regulations under 35 Ill. Adm.

Code 301 and 302 except, for purposes of this Part, the following

definitions apply:

"7Q10" means the average daily flow of the lowest total flow for a

seven day period that occurs once in a 10 year period.

"AWQMN" or "Ambient Water Quality Monitoring Network" means the

network of sampling stations maintained by the Agency and located

on streams throughout the State.

"Agency" means the Illinois Environmental Protection Agency.

"Ammonia decay" refers to the cumulative effect of nitrification,

volatilization, plant uptake, and other processes that reduce the

concentration of ammonia nitrogen in waters by natural means.

"cfs" means cubic feet per second.

"DAF" means design average flow.

"DMR" means discharge monitoring report.

"EMW" or "Effluent Modified Water" means those waters or portions

of waters that the Agency has determined, pursuant to 35 Ill. Adm.

Code 302.213, are not subject to the chronic ammonia nitrogen

standards of 35 Ill. Adm. Code 302.212(b).

"IPCB" means the Illinois Pollution Control Board.

"ISWS" means the Illinois State Water Survey, a part of the Office

of Scientific Research and Analysis in the Illinois Department of

Natural Resources.

"Kjeldahl" means the total of organic nitrogen and ammonia

nitrogen.

"MGD" means million gallons per day.

"PEL" or "Preliminary Effluent Limitation" is an estimate of an

allowable discharge concentration taking into consideration

allowed mixing or dilution.

"PEQ" or "Projected Effluent Quality" is the maximum contaminant

concentration estimated to be discharged by a facility or activity

taking into account statistical analysis of the discharge or

activity.

"Reasonable Potential Analysis" or "Reasonable Potential to

Exceed" means the procedure to predict whether an existing or

future discharge may cause or contribute to a violation of water

quality standards, criteria or values.

"Summer" means the months of April through October, inclusive.

"USEPA" means the United States Environmental Protection Agency.

"USGS" means the United States Geological Survey.

"WQBEL" or "Water Quality Based Effluent Limit" means an NPDES

permit limit that ensures that applicable water quality standards

and criteria are met in waters where such standards and criteria

apply.

"Winter" means the months of November through March, inclusive.

SUBPART B:  AMMONIA NITROGEN (as N) WATER QUALITY STANDARDS AND WQBELs

Section 355.201  Introduction

The need for an ammonia nitrogen (as N) WQBEL is based on the reasonable

potential of a discharge to cause or contribute to a violation of the

applicable ammonia nitrogen water quality standard.  During the NPDES

permit review process, the Agency shall conduct an analysis of the

reasonable potential for ammonia to exceed or contribute to excursions

above the ammonia nitrogen water quality standard that may occur in the

receiving water.  This analysis shall be conducted for both acute and

chronic winter and summer ammonia nitrogen water quality standards.  The

Agency may subdivide summer or winter periods into quarterly or monthly

segments with analysis of reasonable potential corresponding to those

smaller time segments in individual permit applications.

a)  The first step in the reasonable potential analysis is to compare

the Projected Effluent Quality (PEQ), as provided in Section

355.205, to the water quality standard as converted to total

ammonia nitrogen as provided in Section 355.203.  If the PEQ is

less than or equal to the water quality standard as converted to

total ammonia nitrogen as provided in Section 355.203, then no

reasonable potential to exceed the standard exists and no effluent

limitation will be established in the permit unless otherwise

warranted under subsection (c) of this Section.

b)  If the PEQ exceeds the applicable water quality standard as

converted to total ammonia nitrogen as provided in Section

355.203, the analysis shall proceed to the second step as provided

in Section 355.207.

c)  If the wastewater prior to treatment contains total Kjeldahl

nitrogen at levels in which a reasonable potential to exceed water

quality standards as converted to total ammonia nitrogen as

provided in Section 355.203 exists, then the discharge of ammonia

nitrogen shall be limited in the NPDES permit by an ammonia

nitrogen WQBEL.  Reasonable potential to exceed water quality

standards will be determined consistent with Sections 355.203

through 355.211 of this Part.  Even if there appears to be no

potential to exceed the water quality standards based on the

effluent quality analysis in subsection (a) or (b), an ammonia

nitrogen WQBEL shall be established.

Section 355.203  Conversion of Total Ammonia and Un-ionized Ammonia

Nitrogen

The numeric water quality standards for ammonia nitrogen in 35 Ill. Adm.

Code 302.212 are established as the un-ionized fraction of the total

ammonia nitrogen present, since the un-ionized component more closely

relates to the toxicology information utilized in deriving the ammonia

nitrogen standard.  However, most discharge monitoring data used in

deriving a PEQ will be in the form of total ammonia nitrogen.  WQBELs will

be set as total ammonia nitrogen concentrations. The conversion formula

contained in 35 Ill. Adm. Code 302.212 shall be used to estimate the

portion of total ammonia nitrogen that exists in the un-ionized condition.

The primary variables effecting the equilibrium between ionized and

un-ionized fractions are temperature and pH.  Both stream temperature and

pH can be expected to be different than discharge temperature and pH;

therefore, the conversion calculation will be based on conditions  expected

to exist downstream of the discharge.

a)  Where stream specific data is available, that data shall be the

basis for the selection of temperature and pH values to be used in

converting total ammonia nitrogen to un-ionized ammonia nitrogen.

When stream specific data is not available, data from the closest

representative Agency water quality monitoring station during the

most recent five years will be used in this conversion formula.

The temperature will be set at the 75th percentile (75 percent of

the values are less than).  The pH value will be set at the 75th

percentile (75 percent of the values are less than) for

determination of both acute and chronic conditions.  If the 75th

percentile pH value results in a permit limit for chronic exposure

conditions (monthly average ammonia permit limit) less than 1.5

mg/L summer limit or 4.0 mg/L winter limit, the values will be

recalculated based on a 50th percentile pH value (half the values

are less than).  The permit limit will then be set at the value

derived with a 50th percentile pH as long as that value does not

exceed 1.5 mg/L for summer months and 4.0 mg/L for winter months.

If a 50th percentile pH value would allow a higher summer limit

than 1.5 mg/L, the limit will be set at 1.5 mg/L.  If a 50th

percentile pH would allow a higher winter limit than 4.0 mg/L, the

winter limit would be set at 4.0 mg/L.

b)  When sufficient stream specific information is available with

simultaneous measurements of total ammonia, pH, and temperature, a

conversion relationship reflecting the dynamic interaction between

pH, temperature and ammonia equilibrium may be developed instead

of the approach presented in subsection (a) above.

Section 355.205  Estimation of Projected Effluent Quality

The Projected Effluent Quality (PEQ) is the estimation of the maximum

expected effluent concentration.  Individual PEQs shall be estimated for

both summer and winter acute and chronic exposure periods.

a)  The PEQ shall be derived from representative facility specific

data to reflect a 95 percent confidence level for the 95th

percentile value.  These data will be presumed to adhere to a

lognormal distribution pattern with a coefficient of variation of

0.6 unless the facility's effluent data demonstrates a different

distribution pattern.  If facility specific data in excess of 10

data values is available, a facility specific coefficient of

variation that is the ratio of the standard deviation to the

arithmetic average may be calculated.  The PEQ is derived as the

upper bound of a 95 percent confidence bracket around the 95th

percentile value through a multiplier from the following table

applied to the maximum value in the data set that has its quality

assured consistent with subsection (e) as appropriate for acute

and chronic data sets.

PEQ = (maximum data point)(statistical multiplier)

Coefficient of Variation

No. of Samples    0.1   0.2   0.3   0.4   0.5   0.6   0.7

1                 1.4   1.9   2.6   3.6   4.7   6.2   8.0

2                 1.3   1.6   2.0   2.5   3.1   3.8   4.6

3                 1.2   1.5   1.8   2.1   2.5   3.0   3.5

4                 1.2   1.4   1.7   1.9   2.2   2.6   2.9

5                 1.2   1.4   1.6   1.8   2.1   2.3   2.6

6                 1.1   1.3   1.5   1.7   1.9   2.1   2.4

7                 1.1   1.3   1.4   1.6   1.8   2.0   2.2

8                 1.1   1.3   1.4   1.6   1.7   1.9   2.1

9                 1.1   1.2   1.4   1.5   1.7   1.8   2.0

10                1.1   1.2   1.3   1.5   1.6   1.7   1.9

11                1.1   1.2   1.3   1.4   1.6   1.7   1.8

12                1.1   1.2   1.3   1.4   1.5   1.6   1.7

13                1.1   1.2   1.3   1.4   1.5   1.6   1.7

14                1.1   1.2   1.3   1.4   1.4   1.5   1.6

15                1.1   1.2   1.2   1.3   1.4   1.5   1.6

16                1.1   1.1   1.2   1.3   1.4   1.5   1.6

17                1.1   1.1   1.2   1.3   1.4   1.4   1.5

18                1.1   1.1   1.2   1.3   1.3   1.4   1.5

19                1.1   1.1   1.2   1.3   1.3   1.4   1.5

20                1.1   1.1   1.2   1.2   1.3   1.4   1.4

30                1.0   1.1   1.1   1.1   1.2   1.2   1.2

40                1.0   1.0   1.1   1.1   1.1   1.1   1.1

50                1.0   1.0   1.0   1.0   1.0   1.0   1.0

60 or greater     1.0   1.0   1.0   1.0   1.0   1.0   1.0

Coefficient of Variation

No. of Samples    0.8   0.9   1.0   1.1   1.2   1.3

1                 10.1  12.6  15.5  18.7  22.3  26.4

2                 5.4   6.4   7.4   8.5   9.7   10.9

3                 4.0   4.6   5.2   5.8   6.5   7.2

4                 3.3   3.7   4.2   4.6   5.0   5.5

5                 2.9   3.2   3.6   3.9   4.2   4.5

6                 2.6   2.9   3.1   3.4   3.7   3.9

7                 2.4   2.6   2.8   3.1   3.3   3.5

8                 2.3   2.4   2.6   2.8   3.0   3.2

9                 2.1   2.3   2.4   2.6   2.8   2.9

10                2.0   2.2   2.3   2.4   2.6   2.7

11                1.9   2.1   2.2   2.3   2.4   2.5

12                1.9   2.0   2.1   2.2   2.3   2.4

13                1.8   1.9   2.0   2.1   2.2   2.3

14                1.7   1.8   1.9   2.0   2.1   2.2

15                1.7   1.8   1.8   1.9   2.0   2.1

16                1.6   1.7   1.8   1.9   1.9   2.0

17                1.6   1.7   1.7   1.8   1.9   1.9

18                1.6   1.6   1.7   1.7   1.8   1.9

19                1.5   1.6   1.6   1.7   1.8   1.8

20                1.5   1.5   1.6   1.6   1.7   1.7

30                1.3   1.3   1.3   1.3   1.4   1.4

40                1.1   1.2   1.2   1.2   1.2   1.2

50                1.1   1.1   1.1   1.1   1.1   1.1

60 or greater     1.0   1.0   1.0   1.0   1.0   1.0

1)  If the PEQ determined in this Section is less than or equal

to the applicable water quality standard, there is no

reasonable potential and no WQBEL will be established in the

permit unless otherwise warranted under Section 355.201(c).

2)  If the PEQ as determined in this Section exceeds the

applicable water quality standard but does not exceed the PEL

determined through Section 355.209, there is no reasonable

potential and no WQBEL will be established unless otherwise

warranted under Section 355.201(c).

b)  The Agency shall compare monthly average effluent data values,

when available, with the chronic water quality standard to

evaluate the need for monthly average WQBEL.  If a monthly average

WQBEL is included in an NPDES permit, the Agency will also include

a daily maximum WQBEL to enforce the acute water  quality

standard.

c)  The Agency may apply other scientifically defensible statistical

methods for calculating PEQ at the 95 percent upper confidence

level for use in the reasonable potential analysis.  For new or

existing discharges where no prior operating record is available,

PEQ shall be estimated based on knowledge of the tributary

wastewater characteristics and treatment facility capabilities.

For existing sources where the PEQ for the term of the permit

cannot be accurately characterized by historical performance data

as specified in subsection (a) of this Section due to significant

changes in tributary loading, plant operating parameters or other

factors affecting treatment efficiency during the term covered by

the permit, a PEQ representative of the future permit term may be

estimated by analysis of the historical data consistent with

subsection (a) with adjustment of the historical value to reflect

the change expected from the anticipated loading or operating

changes.

d)  Regardless of the statistical procedure used, if the PEQ for

ammonia nitrogen (as N) is less than or equal to the water quality

standard, the Agency shall deem the discharge not to have a

reasonable potential to exceed and a WQBEL shall not be required

unless otherwise required under Section 355.201.

e)  Data Requirements

The derivation of PEQ is based on the effluent quality

demonstrated by self-monitoring data as required by the NPDES

permit or Agency-generated data, such as effluent sampling or

facility-related stream studies.  Effluent data used in the

derivation of PEQ shall be representative of the concentration and

variability of ammonia nitrogen in the discharge anticipated for

the applicable period of the  NPDES permit.  Data shall be

collected and analyzed in accordance with USEPA or Agency approved

sampling and analytical methods (40 CFR 136).  The following

criteria shall be followed in data selection:

1)  the most recent five years of data shall be used unless the

Agency determines that an alternative period better

represents the time period for which effluent quality is

being projected.  Such  alternative time periods may include,

but are not limited to, shorter periods that reflect changed

discharge characteristics resulting from changes in

manufacturing activities or wastewater treatment systems; and

2)  data anomalies resulting from collection, analysis or

recording errors or atypical plant operating conditions may

be eliminated from the data.

Section 355.207  Mixing Allowance

If the PEQ for ammonia nitrogen (as N) is greater than the water quality

standard, the Agency shall assess the level of treatment being provided by

the discharger.  If the discharger is providing (or will be providing) a

level of treatment consistent with the best degree of treatment required by

35 Ill. Adm. Code 304.102(a), the PEQ derived under Section 355.205 shall

be compared to the PEL determined by applying allowed dilution to the

discharge consistent with Section 355.209.

Section 355.209  Calculation of Preliminary Effluent Limitation

a)  The preliminary effluent limitation (PEL) is calculated in a mass

balance approach reflecting allowed dilution as referenced in

Section 355.207:

WQS = [(Q[e])(PEL) + (Q[d])(C[d])]/(Q[e] + Q[d])

or

PEL = [WQS(Q[e] + Q[d]) - (Q[d])(C[d])]/Q[e]

where:

WQS  = applicable water quality standard as converted to total

ammonia nitrogen pursuant to Section 355.203

Q[e] = effluent flow rate

Q[d] = allowed mixing flow rate as determined in accordance with

the mixing zone provisions of 35 Ill. Adm. Code 302.102 and

implementation procedures adopted thereunder

C[d] = background ammonia nitrogen (as N) concentration in mixing

water

Effluent flow rate shall be selected to coincide with the critical

stream flow condition used to quantify allowed dilution.

Typically this will be estimated to be the average of the lowest

three months average flow rate during the previous year for

domestic wastewater sources.  For industrial and other wastewater

sources where flow rates are not directly correlated to climatic

patterns, Q[e] will be estimated as the average of the highest

three monthly average flow rates.  With either approach, Q[e]

shall be modified when future flows are expected to vary

significantly from historical data.

b)  The reasonable potential analysis shall be completed separately

for the winter and summer seasons and for acute and chronic water

quality standards.  The Agency may subdivide summer or winter

periods into quarterly or monthly segments with analysis of

reasonable potential corresponding to those smaller time segments

in individual permit applications. WQBELs based on the acute water

quality standard shall be expressed as a daily maximum.   WQBELs

based on the chronic water quality standard shall be expressed as

a monthly average.

Section 355.211  Summary of the Results for a Reasonable Potential Analysis

and the Determination of Ammonia Nitrogen WQBELs

a)  If the PEQ determined in Section 355.205 is less than or equal to

the applicable water quality standard, there is no reasonable

potential and no WQBEL will be established in the permit unless

otherwise warranted under Section 355.201(c).

b)  If the PEQ exceeds the applicable water quality standard but does

not exceed the PEL determined through Section 355.209, there is no

reasonable potential and no WQBEL shall be established unless

otherwise warranted under Section 355.201(c).

c)  If the PEQ exceeds the PEL determined through Section 355.209,

there is reasonable potential to exceed the standard and the PEL

shall be established as the WQBEL.

d)  If a WQBEL is warranted under Section 355.201(c), the WQBEL shall

be set at the PEL as determined through Section 355.209.

SUBPART C:  EFFLUENT MODIFIED WATERS

Section 355.301  Introduction

IPCB regulations at 35 Ill. Adm. Code 302.202, 302.212, 302.213, and

304.122 establish provisions for designating waters as EMWs.  EMWs are

subject to all general use water quality standards  except for the chronic

ammonia nitrogen water quality standards of 302.212(b).  This Section

provides for the designation of an EMW wherein the chronic portion of the

un-ionized standard is inapplicable.  In lieu of the chronic standard, the

IPCB has established discharge restrictions at 35 Ill. Adm. Code 304.122(d)

for any discharge tributary to an EMW.  These restrictions include limits

on discharges at 1.5 mg/L total ammonia nitrogen during the April through

October summer season and 4.0 mg/L total ammonia nitrogen during November

through March as monthly averages.  Beyond these monthly average limits,

there is also a provision to assure continuation of the existing level of

performance and adherence to the nondegradation provision of 35 Ill. Adm.

Code 302.105.  The criteria for designation of an EMW include two specific

provisions:  the water body must have the potential to exceed the chronic

standard due to a permitted discharge; and the elevated chronic ammonia

nitrogen concentration will not adversely impact designated uses of the

affected stretch of the water body.  EMW status shall be designated in the

receiving water body if:

a)  aquatic life expected to exist in the receiving waters is known to

be tolerant of the projected ammonia nitrogen concentrations

resulting from the treatment plant effluent in conjunction with

ambient conditions.  Determination of the aquatic community

expected to inhabit the receiving waters shall be consistent with

stream morphology, particularly physical features and hydrologic

regimes of the water body;

b)  the receiving stream does not exceed the acute water quality

standard of 35 Ill. Adm. Code 302.212(b); and

c)  the discharger demonstrates a reasonable potential to exceed the

chronic ammonia nitrogen standard pursuant to Subpart B of this

Part.

If an EMW cannot be granted, then monthly average effluent limits in the

NPDES permit shall be determined from the procedures for establishing

ammonia nitrogen WQBELs pursuant to Subpart B of this Part.   If necessary,

a schedule to attain compliance with these limits shall also be included in

the discharger's NPDES permit.

Section 355.303  EMW Application Requirements

The Agency shall consider designating a portion of the receiving waterbody

as an EMW upon receipt of a valid application for an EMW and when the

provisions of this Subpart are met.

a)  All applicants shall provide:

1)  the name, address and design average flow of the facility;

2)  all instream ammonia nitrogen, pH and water temperature data

collected by or available to the applicant;

3)  a physical description of the receiving stream including

information on depth, substrate, instream cover, average

width, percent canopy, riffle-pool sequence, stream gradient

and other pertinent factors that the discharger wishes to be

considered; and

4)  any other information concerning the receiving waterbody that

the applicant believes is relevant.

Receiving stream information must be collected from the reach

anticipated to constitute the requested EMW and continuing

downstream for an additional distance comprising 33% of the

requested EMW length.

b)  Applicants having one or more of the following characteristics

shall supply, in addition to the information in subsection (a),

information required under subsection (c) below:

1)  a DAF larger than 0.25 MGD;

2)  a receiving stream with springs, or other sources of

permanent flow constituting 7Q10 flows of greater than zero

(excluding the applicant's discharge), upstream or within the

reach of the anticipated EMW; or

3)  a discharge location on a receiving stream evaluated under

the Agency's Biological Stream Characterization (BSC) program

and having received an "A" or "B" rating, provided the

discharge is located no more than four stream miles upstream

of the furthest upstream BSC monitored site.

c)  Dischargers applying for EMW status and having one or more of the

characteristics of subsection (b) above must also supply the

following information:

1)  Stream survey data that assesses ammonia nitrogen impact to

the aquatic life of the receiving stream.   Generally, data

collected within the past five years that are reflective of

current loading, stream flow, and physical conditions are

preferred.  If none of these factors have significantly

changed, older data may suffice.  However, any additional

data concerning the aquatic life community of the receiving

stream must be included in the application as it becomes

known to the discharger. The Agency may have previously

conducted such studies and these may satisfy this

requirement; and

2)  Data concerning the presence of sensitive species including

threatened and endangered federally or State listed aquatic

species, self-sustaining populations of cold water species or

species of special significance regarding their sensitivity

to ammonia nitrogen.  Such data may be available from one or

more of the following sources or other local or regional

sources:

A)  the Illinois Department of Natural Resources Division of

Natural Resources Review & Coordination;

B)  the report "Biologically Significant Illinois Streams",

a publication of the INHS (Center for Biological

Diversity Technical Report 1992(1)); or

C)  local colleges and universities.

Section 355.305  Evaluation of EMW Applications

The Agency shall evaluate EMW applications based on all information

provided pursuant to Section 355.303, as well as information available from

the Agency's monitoring programs.  Additionally, the Agency shall seek and

obtain information from other Illinois natural resource agencies.  Such

information shall include the following:

a)  biological studies conducted on the receiving water;

b)  ammonia nitrogen, pH, and temperature data from ambient, intensive

basin, or facility-related stream surveys;

c)  ammonia nitrogen, pH and temperature effluent data;

d)  physical instream habitat data; or

e)  total ammonia nitrogen loading and related information attributed

to other sources in the affected reach.

Section 355.307  Determination of EMW Designation

Upon evaluating the EMW application and any additional information

available, the Agency shall determine whether the receiving stream can be

designated as an EMW based on the provisions of 35 Ill. Adm. Code 302.213,

304.122, and 355.305.  Existing EMW designations are subject to review as

to whether requirements for such designations continue to be met at the

time of an NPDES permit renewal or modification.

a)  If the Agency determines that a receiving stream cannot be

designated as an EMW, the Agency shall notify the applicant in

writing as to its decision and the basis for that decision.

b)  If a receiving stream can be designated as an EMW, the Agency

shall issue a public notice that contains:

1)  determination of the length of the EMW, and

2)  summary of the ecological analysis used in the EMW

designation process.

Section 355.309  Procedures for Delineating an EMW

The methodology for determining the length of a water body to be designated

as EMW shall be based on the chronic total ammonia nitrogen (as N) water

quality standard for winter conditions and a decay coefficient representing

colder ambient conditions as indicated in Section 355.311.  Winter

conditions depict the "worst-case" ammonia nitrogen decay rates and are to

be used when calculating the reach of a water body to be designated as EMW.

This modeling shall be performed in the following manner:

a)  Downstream waters shall be subdivided into segments where

discharge and stream cross-sectional area are uniform.  Segments

will typically begin at confluences with other streams or where

additional point sources of total ammonia nitrogen (as N) enter

the receiving water.

b)  The DAF for domestic wastewater treatment plants and the maximum

flow for industrial plants and other point sources of ammonia

nitrogen (as N) downstream will be used as effluent flow rates in

the analysis.  A 7Q10 flow rate shall be determined for each

segment.  Discharge rates under 7Q10 conditions are to be obtained

from maps generated by the ISWS unless the Agency has previously

approved an alternate 7Q10 discharge rate.

c)  The average velocity for each segment shall be derived for 7Q10

discharge conditions.  In the absence of field measurements,

velocity shall be determined from hydraulic geometry equations

derived by the ISWS.  These equations are published in the

University of Illinois Water Resources Center publication, "WRC

Research Report No. 15, Hydraulic Geometry of Illinois Streams"

(July 1968), which is hereby incorporated by reference and

includes no further editions or amendments.  A minimum velocity of

0.2 ft/sec will be used unless field measurements  indicate that a

different velocity exists during 7Q10 conditions.

d)  The chronic water quality standard shall be converted to total

ammonia nitrogen (as N) as outlined in Section 355.203.

e)  The concentrations of ammonia nitrogen in the effluents shall be

the same as the monthly average winter ammonia nitrogen permit

limit for the point source.   If no monthly average winter ammonia

nitrogen permit limit exists, then a value of 4.0 mg/L shall be

used.

f)  The ammonia nitrogen concentration at the end of each segment

shall be calculated using the equations contained in Section

355.311.  The point at which the water quality standard will be

met shall be the downstream terminus of the EMW.  The length of

the EMW shall equal the sum of all segment lengths, but in no case

shall be less than 100 yards in length.

g)  The permittee has the opportunity to submit field measurements to

be used in this analysis.

Section 355.311  Ammonia Nitrogen Decay Equation

A decay equation shall be used to predict instream ammonia nitrogen

concentrations at locations downstream of the outfall, thereby determining

the linear extent of the EMW.

a)  Modeling of the decay (conversion to nitrite/nitrate) of ammonia

nitrogen from a discharge and predicting the levels of ammonia

nitrogen at points downstream from the discharge shall follow the

decay equation:

C[nf] = [(Q[n] C[n] + Q[n-1] C[n-1])/(Q[n-1] + Q[n])] x e-(kt)

where the parameters used in the decay equation are defined as

follows:

C[nf]   =   ammonia nitrogen concentration at the end of segment

"n"

t       =   travel time to point "n" (days)

Q[n]    =   additional flow introduced into segment "n" (cfs) (see

Section 355.309(b) for initial segment)

C[n]    =   ammonia nitrogen concentration introduced into segment

"n" (monthly average effluent limit for initial

segment)

Q[n-1]  =   upstream 7Q10 flow rate or flow rate entering segment

"n" from previous segment (cfs)

C[n-1]  =   upstream ammonia nitrogen concentration entering

segment "n" from previous segment

k       =   first order decay coefficient used in determining the

natural biological, physical, and chemical degradation

of ammonia nitrogen that occurs. The value of "k" may

vary as a function of the receiving stream

characteristics.  In the absence of stream specific

data, a representative value shall be selected from

studies of streams with similar characteristics and

shall be used in calculations as a default value

representative of winter ammonia nitrogen decay.

b)  Where no upstream flow is available for mixing and no additional

sources of ammonia nitrogen are present downstream, the equation

reduces to the following:

C[nf] = C[n] e(-kt)

where:

C[nf] = the applicable winter chronic water quality standard

C[n]  = monthly average winter effluent limit

Section 355.313  Restrictions Applicable to Discharges with EMWs

When the Agency issues a publication of a draft NPDES permit designating an

EMW, effluent limits for ammonia nitrogen shall be protective of the

aquatic community expected to exist in the EMW as provided in 35 Ill. Adm.

Code 304.122.

a)  In no instance shall these effluent limits exceed 30 day average

concentrations of 1.5 mg/L total ammonia nitrogen (as N) during

the months of April through October, and 4.0 mg/L total ammonia

nitrogen (as N) during the months of November through March.

b)  When uses are at risk of impact due to increased concentrations of

ammonia nitrogen, more stringent 30 day average effluent limits

shall be incorporated.

c)  The draft permit shall also include daily maximum effluent limits

for total ammonia nitrogen (as N) and these shall be determined by

applying the acute water quality standards of 35 Ill. Adm. Code

302.212(b) pursuant to Section 355.203.

d)  The Agency shall take final action as to the designation of an EMW

concurrent with final NPDES permit issuance.

Section 355.315  Publication of EMWs

The Agency shall compile the number and length of EMWs and report the

information in each edition of the Illinois Water Quality Report pursuant

to Section 305(b) of the Federal Clean  Water Act, as amended, 33 USC

1315(b), and in the Illinois Register on a semi-annual basis.