TITLE 35:  ENVIRONMENTAL PROTECTION

SUBTITLE C:  WATER POLLUTION

CHAPTER II:  ENVIRONMENTAL PROTECTION AGENCY

PART 375

COMBINED SEWER OVERFLOW EXCEPTION CRITERIA

AND FIRST FLUSH DETERMINATION

SUBPART A:  INTRODUCTION

Section

375.101   Policy

375.102   Definitions

375.103   Prohibitions

SUBPART B:  WATER QUALITY INVESTIGATION

375.201   Policy

375.202   Phase I - Background Information

375.203   Phase II - Preliminary Stream Inspection

375.204   Phase III - Detailed Water Quality Evaluation

375.205   Special Analysis for Congested Areas

375.206   Time Schedule

SUBPART C:  ASSESSMENT OF CONTROL OPTIONS

Section

375.301   Water Quality Evaluation

375.302   Control Alternatives

375.303   Benefits Assessment

SUBPART D:  FIRST FLUSH DETERMINATION

375.401   Variables in First Flush Determination

375.402   Conditions for Determination

375.403   Field Studies

375.404   Modeling

APPENDIX A      First Flush Graph

AUTHORITY:  Implementing Section 12(a) and authorized by  Section 4 of the

Environmental Protection Act (Ill.  Rev. Stat. 1981, ch. 111 1/2, pars.

1012(a) and 1004) and by  35 Ill. Adm. Code 306.352.

SOURCE:  Adopted and Codified at 7 Ill. Reg. 10808, effective  August 23,

1983; amended at 8 Ill. Reg. 19436, effective September 26, 1984.

SUBPART A:  INTRODUCTION

<BSection 375.101  Policy>>

This policy provides guidelines for:

a)  Determination of water quality impacts attributable  to combined

sewer overflows and treatment plant bypasses;

b)  Estimation of potential water quality improvements;

c)  Selection of a combined sewer overflow (CSO) control  program that

can be justified on the basis of local  environmental conditions

and fiscal responsibility;

d)  Evaluation and review of discharger's submittals  pursuant to 35

Ill. Adm. Code 306.351; and

e)  Determination of first flush as referenced in 35 Ill. Adm. Code

306.305(a).

(Source:  Amended at 8 Ill. Reg. 19436, effective September 26, 1984.

<BSection 375.102  Definitions>>

All definitions in 35 Ill. Adm. Code 301 shall apply here.  No

incorporation by reference in this Part contains any later amendment or

addition.

"Combined Sewer" means a sewer designed and constructed to

receive both wastewater and land runoff.

"Dry Weather Flow" means the flowrate consisting of wastewater

and non excessive sewer infiltration in the absence of inflow

sources attributable to direct precipitation or snowmelt and  is

not restricted to meteorologic drought conditions.  Nonexcessive

sewer infiltration is defined at 40 CFR 35.2005(1984).

"First Flush" is defined as that volume of water and associated

flow rate needed to carry solids or BOD concentrations is excess

of the dry weather flow.  This volume is shown on the "cross

hatched" area in Appendix A.

35 Ill. Adm. Code 306.305(a) uses the term "first flush to

describe those flows requiring complete treatment.  When

sewage is flowing through a sewer system at fairly uniform

lower velocities, which occur during dry weather conditions,

some of the solids present in the waste are deposited in the

pipes.  In addition, solids and some organic material that

may be washed into the combined sewer systems tend to build

up on streets, parking lots, etc.

When a storm event occurs, the volume of wastewater carried

by the sewer increases and the velocities in the sewer also

increase.  The high volume of storm water causes material to

be washed from the streets into the sewer and the sludge

deposited in the sewers to be scoured from the pipes.  This

material is carried to the sewage treatment plant where it

exerts an additional organic load on the biological system

and an additional solids load on the physical systems.  The

maximum impact of this phenonmenon tends to occur at the

beginning of a storm event and taper off as the storm

continues and there are no additional solids to remove.

"Human Contact Activities" means wading, swimming or any other

activity involving partial or total immersion of the human  body

in the subject waterway whether for recreational or other

purpose.

"Wet Weather Flow" means a flowrate condition, elevated above  dry

weather flow, attributable to land runoff as the result of

snowmelt or a precipitation event.

(Source:  Amended at 8 Ill. Reg. 19436, effective September 26, 1984)

<BSection 375.103  Prohibitions>>

a)  Dry weather overflows are expressly prohibited under  existing

regulations and are not eligible for consideration  under the

provisions of this rule.  Combined sewer overflow  programs that

will result in continued or future dry weather  overflows or

treatment plant bypasses will not be approved by  the Agency.

b)  This policy applies to overflow from combined sewer  systems and

treatment plant bypasses resulting from wet  weather flow

conditions.  Overflows from separate sanitary  sewers and from

portions of combined sewer communities served  by separate

sanitary sewers are not subject to this rule.

SUBPART B:  WATER QUALITY INVESTIGATION

<BSection 375.201  Policy>>

It is the intent of this policy that water quality  investigations be no

more extensive than is necessary:

a)  to identify the degree of CSO control appropriate  for a specific

community as mutually agreed to by discharger  and Agency, and

b)  to substantiate the appropriateness of the proposed  control

program under the scrutiny of the Board in a  proceeding pursuant

to 35 Ill. Adm. Code 306:  Subpart D.

<BSection 375.202  Phase I - Background Information>>

a)  A community seeking to pursue alternate CSO control  requirements

under provisions of 35 Ill. Adm. Code 306:   Subpart D and this

Part shall review existing documents  including facility planning

reports, sewer maintenance  records, and Discharge Monitoring

Reports to assemble  pertinent background information.  To the

extent available in  existing resources, pertinent background

information shall  include the following:

1)  a map of the sewer system depicting:

A)  portions of the community served by  combined sewers and

separate sanitary sewers; and

B)  location of each overflow and receiving  water body;

2)  size of watershed of receiving water body at  point of each

overflow;

3)  drainage area and population tributary to each  overflow;

4)  land use, zoning classification and projected  growth

patterns in the vicinity of each overflow using the

following classifications:  residential, commercial,

industrial, recreational, agricultural;

5)  actual or potential use of the affected  waterway for human

contact activities;

6)  history of complaints regarding the stream and  surrounding

environment and downstream of overflows;

7)  history of other complaints concerning the  sewer system;

8)  description of structural and physical  condition of sewer

system including age of system, incidence  of sewer

collapses, bottlenecks in the system; and

9)  list of industrial and other sewer users  tributary to

overflows which contains substances in wastewater  or storm

drainage in concentrations likely to create hazardous  or

toxic conditions at the point of combined sewer overflow.

b)  Submittals:

1)  A written report of review conducted in  subsection (a) above

shall be submitted to the Agency along  with a request for

Agency participation on an advisory and  review basis in a

CSO impact/evaluation project.

2)  Submittals shall be addressed to:

Illinois Environmental Protection Agency

Construction Grant Section

Division of Water Pollution Control

2200 Churchill Road

Springfield, Illinois  62706

c)  Upon receipt and review of discharger's report, an  Agency

representative will be designated as project  coordinator and

contact person for the Agency's participation  in the project.

The Agency Project Coordinator will contact  the Applicant's

representative to schedule a conference  regarding second phase

activities.

d)  After Phase I, conferences shall be held between the  Agency and

discharger's representatives to discuss and agree  upon the nature

and extent of each subsequent phase of  investigation prior to

commencing that phase.

<BSection 375.203  Phase II - Preliminary Stream Inspection>>

This phase requires a physical inspection of the receiving  water body and

near stream property.  Additionally, any  necessary elements missing from

the Phase I background report  shall be addressed in this step.  Factors to

be considered  include:

a)  Inspection of stream in and around vicinity of each  overflow for

sludge deposits, sewage related odors, floating  debris of

sanitary sewage origin, and any other visible signs  of pollution

impact.  Inspection techniques shall be selected  so as to assure

maximum reliability of the results including:

1)  probing, disturbance and extraction of bottom  sediments in

pools and other quiescent portions of stream to  note color,

texture, odor and other aspects of sediments that  are

indicative of sewage sludge:

2)  estimates of extent and severity of sludge  deposition in

terms of depth of deposition, area of stream  affected and

percentage of stream bed within affected area  that contains

sludge deposits; and

3)  inspection of shoreline vegetation, logjams and  other

obstructions likely to retain floating sewage debris.

b)  Stream Hydraulics:

1)  Stream hydraulic factors shall be identified  including:

average width of stream channel from bank to bank,  height of

stream banks above thalweg, stream bed gradient.

2)  For receiving streams with a 7Q10 (average  seven day low

flow which occurs once in 10 years) flow greater  than 10

cubic feet per second this information can be omitted,

substituting the 7Q10 value.

c)  Stream morphological factors including:

1)  substrate type;

2)  variation of structure via natural meandering,  pool and

riffle sequence;

3)  degree of dredging, channelization or other  alteration of

natural stream character; and

4)  accumulation of logjams and other naturally  occurring

vegetative debris.

d)  Description of stream side property including:

1)  topography;

2)  land cover including forrested, agricultural  row crop,

marsh, grass buffer strip, residential lawn, and

3)  land use if not already identified in Section  375.202(a)(4)

including a specific determination if affected  area is

utilized by or accessible to children for recreational

activities.

<BSection 375.204  Phase III - Detailed Water Quality Evaluation>>

Areas to be addressed in a detailed water quality survey shall  include,

consistent with 35 Ill. Adm. Code 306.361 (d):

a)  water column chemistry -

1)  upstream/downstream and overflow sampling;

2)  wet weather/dry weather sampling;

3)  diurnal sampling for dissolved oxygen (D.O.);

4)  parameters generally will include BOD5,  suspended solids,

volatile suspended solids, ammonia.  Other  parameters may

become important depending on the nature of the  overflow;

and

5)  other parameters as deemed appropriate relative  to 35 Ill.

Adm. Code 375.302(a)(8).

b)  sediment analysis -

1)  sediment oxygen demand;

2)  percent volatile matter;

3)  particle size distribution; and

4)  other parameters as deemed appropriate relative  to 35 Ill.

Adm. Code 375.302(a)(8).

c)  biological survey -

1)  benthic macroinvertebrates;

2)  fishes; and

3)  plant productivity assessment (algal blooms).

d)  hydraulic and hydrologic factors including storm  hydrographs for

receiving stream and overflow.

<BSection 375.205  Special Analysis for Congested Areas>>

In small drainage basins receiving combined sewer overflows  from

collection systems belonging to several separate  governing bodies the

significance of the aggregate CSO load  may well dictate a basin wide

evaluation of alternate control  levels in lieu of community by community

evaluation.   Resolution of CSO issues in such basins may require

evaluation  of all CSO sources concurrently with cooperative data

collection efforts involving several communities.  This is  especially

applicable in major metropolitan areas.

<BSection 375.206  Time Schedule>>

a)  Once a water quality investigation is initiated,  every effort

shall be made to expedite each step of the  activity, completing

the investigation in the shortest  possible time without

compromising the validity of the  findings.  Effective field

activities are dependent upon  proper seasonal, hydrologic and

meteorologic conditions which  will require flexibility in

scheduling.  Nevertheless the  following time table is suggested

as a general guideline.

b)  Phase I - 30 to 60 days for completion plus 30 to 60  days for

Agency review and planning of Phase II.

c)  Phase II - 30 to 60 days for completion plus 30 to  60 days for

Agency review and planning of Phase III.

d)  Phase III - 6 months to 1 year and generally  containing the

summer months of June, July and August.

SUBPART C:  ASSESSMENT OF CONTROL OPTIONS

<BSection 375.301  Water Quality Evaluation>>

It is intended that water quality evaluation performed under  Subpart C of

this Part will be undertaken in conjunction with  or subsequent to first

flush analysis and CSO studies  consistent with Subpart D.  Control options

being  evaluated shall not be limited to alternatives resulting in  full

compliance with 35 Ill. Adm. Code 306.305; lesser control  options of

varying degrees shall be evaluated also.

(Source:  Amended at 8 Ill. Reg. 19436, effective September 26, 1984)

<BSection 375.302  Control Alternatives>>

Alternatives resulting in noncompliance with existing water  quality

standards and requiring regulatory relaxation of water  quality standards

may also be considered by the discharger. Water quality revisions being

considered shall be consistent  with the purpose and objectives of the Act

and the CWA.

<BSection 375.303  Benefits Assessment>>

Benefits shall not be assessed solely on a pound pollutant removed basis

but also on instream improvement of a documented pollutional impact or

reduction in a public health risk. Acceptable techniques for estimating

water quality improvements will vary depending on individual circumstances

but may include:

a)  consideration of receiving water body assimilative  capabilities

and strategic relocation of overflows;

b)  selection of size of storm to design controls for  based on stream

hydrograph, available dilution, turbulance and  velocity under

storm conditions and their affect on  assimilation (since CSO's

are dominated by suspended rather  than soluble pollutants, major

emphasis will be placed on  prevention of instream deposition);

and

c)  mathematic modeling.

SUBPART D:  FIRST FLUSH DETERMINATION

<BSection 375.401  Variables in First Flush Determination>>

The intensity and duration of first flush are a function of certain

variables including:

a)  Physical and hydraulic features of the sewer system and tributary

watershed.

b)  Amount of sediment accumulated in the sewer system and on

impervious surfaces of the watershed.

c)  Intensity and duration of the storm event causing the flushing.

(Source:  Added at 8 Ill. Reg. 19436, effective September 26, 1984)

<BSection 375.402  Conditions for Determination>>

For any given sewer system, or portion of a sewer system, first flush

volume subject to full treatment requirements of 35 Ill. Adm. Code

306.305(a) and associated flow rate shall be determined for the following

condition(s):

a)  A storm of 1.2 inch per hour intensity with a duration of 60

minutes, which approximates a one year-one-hour storm for most

sections of the state.

b)  There shall be sufficient time between the storm event chosen to

determine first flush and any previous event to allow for adequate

solids deposition in the sewers and on the streets to demonstrate

first flush under this Part.  In general a ten day period should

be sufficient.

(Source:  Added at 8 Ill. Reg. 19436, effective September 26, 1984)

<BSection 375.403  Field Studies>>

a)  First flush volumes may be determined through direct field

measurement at a point of overflow, modeling of the collection

system or equally accurate techniques.  Either approach is

dependent on a field monitoring program as described in (c) below.

Commonly, first flush for the design storm is determined by direct

measurement of flush rate and volume for several storms of varying

intensity and extrapolating to the design storm (1.2 in/hr.).

b)  Detailed monitoring may be limited to less than the total number

of overflow locations in the system when monitoring all locations

would be impossible or would not be helpful in demonstrating the

capabilities of the system.  Sampling sites should be selected in

order to monitor the largest discharges in terms of loading.

Consideration shall also be given to the different land use

patterns, population densities, and sewer system characteristics

tributary to a given overflow site within the study area.

c)  General guidance for field measurement includes:

1)  A minimum of 3 storms should be monitored for each sampling

site.

2)  There shall be a least one rain gauge located within the

study area.  For systems serving an area of over 2 squares

miles, multiple gauges shall be used and located to achieve

gauge spacing of 2 miles or less throughout the study area.

Rain gauges should be on the continuous recording type,

otherwise manual readings should be recorded on intervals of

10 minutes or less.  If only one gauge is used it should be

located near the center of the study area;  multiple gauges

shall be located so as to identify variations in storm

intensity across the study area.

3)  Flow monitoring shall be initiated at the onset of a storm

and continue through and beyond the sewer flushing period.

Flow meters may be located in interceptor manholes, diversion

structures, outfall sewers or other locations where dependale

opertion is assured.  Caution should be taken to protect

instrumentation from flooding, corrosion, excess humidity,

vandalism or other factors which jeopardize either the

equipment or reliability of the data.

4)  Water samples should be collected at the onset of the storm

and at 10 or 15 minute intervals thereafter.  In most

instances a moderate to heavy storm will fully flush the

system within two hours.  Most automatic discrete samplers

have adequate capacity to contain the entire flush period

with 10 minutes sampling frequency.  Again, caution should be

taken to protect against flooding, corrosion, or excess

humidity that may cause malfunctioning or failure of the

mechanical or electronic components of the sampler.  Samples

should be analyzed for BOD as defined in 35 Ill. Adm. Code

304.120(a), total and volatile suspended solids with sample

preservation, and analysis consistent with Standard Methods

15th Edition (1980).

5)  Overflow points that are not subject to direct flow and

quality monitoring should be inspected during the storm

period to determine if the overflows are functioning and to

estimate the rate and duration of overflow.

(Source:  Added at 8 Ill. Reg. 19436, effective September 26, 1984)

<BSection 375.404  Modeling>>

a)  If all of the overflow locations to be studied cannot be sampled

and monitored, or field conditions were not available through

direct field measurement as described in Section 375.403, then

other methods, including modeling of section (s) of the study

area, shall be required to determine first flush.  These methods

should be used to:

1)  Predict the quality and quantity of loads and concentrations.

2)  Identify significant sources.

3)  Analyze hydraulics of the tributary system.

b)  Data supplied for the modeling must be based on factors particular

to a given system or study area and should include:

1)  Rainfall data

2)  Size of drainage area

3)  Land use

4)  Population density

5)  Hydraulics of area

6)  Percentage of combined and/or separate sewers

c)  The type of modeling to be used for a given collection system or

study area shall be addressed in the Plan of Study for the CSO

Analysis.  References, such as texbooks, technical papers, etc.,

for the modeling methods to be used shall be listed.

d)  Verification that the model(s) is valid shall be provided by

comparing the output obtained from the model(s) with actual

sampling and monitoring data from overflow points specified in

accordance with Section 375. 403 for two or more storms.

(Source:  Added at 8 Ill. Reg. 19436, effective September 26, 1984)

<BSection 375.APPENDIX A  First Flush Graph>>