TITLE 35:  ENVIRONMENTAL PROTECTION

SUBTITLE C:  WATER POLLUTION

CHAPTER II:  ENVIRONMENTAL PROTECTION AGENCY

PART 366

PROCEDURES AND REQUIREMENTS FOR

DETERMINING LOAN PRIORITIES FOR MUNICIPAL WASTEWATER

TREATMENT WORKS

SUBPART A:  INTRODUCTION

Section

366.101   Purpose

366.102   Definitions

366.103   Incorporations by Reference

366.104   Priority System and Project Priority List

366.105   Funding Allocations

366.106   Pre-applications

366.107   Facility Planning

SUBPART B:  PROCEDURE FOR CALCULATING THE LOAN PRIORITY INDEX OF SERVICE

CONTINUATION PROJECTS

Section

366.201   Formula for Computing the Loan Priority Index for Service

Continuation Projects

366.202   A1 Factor (Financial Impact)

366.203   A2 Factor (Water Quality)

366.204   A3 Factor (Organic Load)

366.205   A4 Factor (Assessment of Existing Facilities)

366.206   A5 Factor (Operational Excellence)

SUBPART C:  PROCEDURE FOR CALCULATING THE LOAN PRIORITY INDEX OF SERVICE

EXPANSION PROJECTS

Section

366.301   Formula for Computing the Loan Priority Index for Service

Expansion Projects

366.302   B1 Factor (Financial Impact)

366.303   B2 Factor (Water Quality)

366.304   B3 Factor (Economic Benefit)

366.305   B4 Factor (Existing Utilization)

366.306   B5 Factor (Operational Excellence)

366.307   B6 Factor (Health Hazard)

SUBPART D:  PROCEDURE FOR CALCULATING LOAN PRIORITY INDEX FOR NEW SERVICE PROJECTS

Section

366.401   Formula for Computing the Loan Priority Index for New Service

Projects

366.402   C1 Factor (Financial Impact)

366.403   C2 Factor (Water Quality)

366.404   C3 Factor (Organic Load)

366.405   C4 Factor (Health Hazard)

SUBPART E:  PROCEDURE FOR CALCULATING LOAN PRIORITY INDEX FOR COMBINED

SEWER SERVICE PROJECTS

Section

366.501   Formula for Computing the Loan Priority Index for Combined Sewer

Projects

366.502   D1 Factor (Financial Impact)

366.503   D2 Factor (Drainage Area)

366.504   D3 Factor (Flooding Frequency)

366.505   D4 Factor (Basement Backups)

366.506   D5 Factor (Percentage of Basements Affected)

SUBPART F:  PROCEDURE FOR APPLICATION OF SCORING CONVENTIONS

Section

366.601   Scoring Conventions

APPENDIX A      Waterbody Specific Information

APPENDIX B      Service Continuation A4 Factor Scoring Review Sheet

APPENDIX C      Excellence of Operation Scoring Review Sheet For Local

Government Units That Own Wastewater Treatment Facilities

APPENDIX D      Excellence of Operation Scoring Review Sheet For Local

Government Units That Own Only Wastewater Collection

Facilities

AUTHORITY:  Implementing and authorized by Sections 19.1 through 19.8 of

the Environmental Protection Act [415 ILCS 5/19.1-19.8]

SOURCE:  Adopted at 14 Ill. Reg. 8121, effective May 14, 1990; amended at

20 Ill. Reg. 15598, effective November 26, 1996.

SUBPART A:  INTRODUCTION

<BSection 366.101  Purpose>>

This Part sets forth the procedures and requirements established by the

Illinois Environmental Protection Agency (IEPA) for determining priorities

in awarding financial assistance for the construction of municipal

wastewater treatment works under Sections 19.1 through 19.9 of the

Environmental Protection Act (the Act) [415 ILCS 5/19.1-19.9] and Title VI

of the Federal Clean Water Act (33 U.S.C. 1281 et seq.).

(Source:  Amended at 20 Ill. Reg. 15598, effective November 26, 1996)

<BSection 366.102  Definitions>>

a)  Unless specified otherwise, all terms shall have the meanings set

forth in the Environmental Protection Act [415 ILCS 5], the

Federal Clean Water Act (33 U.S.C. 1281 et seq.) and regulations

adopted under these Acts, including 35 Ill. Adm. Code 365.

b)  For purposes of these rules, the following definitions apply:

"Agency" -- Illinois Environmental Protection Agency.

"Combined Sewer Service Projects" -- Projects constructed in

a combined sewer service area which are intended to reduce or

eliminate street, area and basement flooding.

"Fund" -- The Water Pollution Control Revolving Fund as

authorized by P.A. 85-1135, effective September 1, 1988.

"Intended Use Plan" -- A plan which includes a description of

the short and long term goals and objectives of the Fund,

project categories, discharge requirements, terms of

financial assistance and the communities to be served.

"Monitoring Reports" -- Reports submitted in response to

permits issued under the authority of the Federal Clean Water

Act (33 U.S.C. 1281 et seq.), the Environmental Protection

Act [415 ILCS 5], and regulations adopted under these Acts,

including discharge (NPDES) permits and State operating

permits.

"New Service Project" -- Projects which will provide

wastewater collection, transportation or treatment for an

unsewered local government unit.

"P.E. BOD" -- A term used to evaluate the impact of

industrial or other waste on a treatment works or streams in

terms of five day biochemical oxygen demand. One P.E. BOD

equals 0.17 pounds (77g).

"Permits" -- National Pollutant Discharge Elimination System

(NPDES) permits and State operating permits as described in

35 Ill. Adm. Code 309.

"Priority System" -- A methodology used to rank projects for

inclusion on the project priority list.

"Project Priority List" -- An ordered listing of projects

which the Agency has determined are eligible to receive

financial assistance from the Fund.

"Service Continuation Project" -- Projects for the

improvement, upgrade, rehabilitation, renovation, and/or

replacement of wastewater treatment works.

"Service Expansion Project" -- Projects to expand capacity of

existing wastewater treatment works.

"Title VI" -- Title VI of the Federal Clean Water Act (33

U.S.C. 1281 et seq.).

"USEPA Reach File" -- Hydrologic Nomenclature System

developed by USEPA to identify and locate specific

waterbodies.

(Source:  Amended at 20 Ill. Reg. 15598, effective November 26, 1996)

<BSection 366.103  Incorporations by Reference>>

U.S. Department of Commerce, Economics and Statistics Administration,

Bureau of the Census: 1990 Census of Population and Housing: Summary

Social, Economic, and Housing Characteristics, Illinois, Table 10, 1990

CPH-5-15 (no later editions or amendments).

(Source:  Added at 20 Ill. Reg. 15598, effective November 26, 1996)

<BSection 366.104  Priority System and Project Priority List>>

a)  Financial assistance will be provided from the Fund only to

projects which are identified on the project priority list.

b)  This Part sets forth a priority system to be used to rank projects

for inclusion on the project priority list.  The ranking of a

project is as calculated under Subparts B, C, D and E of this

Part.

c)  The project priority list shall be published annually in the

preliminary Water Pollution Control Program Plan developed in

accordance with Section 106 of the Federal Clean Water Act (33

U.S.C. 1256).  After the public hearing is held to discuss the

Program Plan, the Agency shall evaluate and consider any public

comments received concerning the project priority list. The final

project priority lists shall be published in the final Water

Pollution Control Program Plan.

d)  The Agency will develop a priority list with four (4) separate

classes of projects:

1)  Service Continuation Projects

2)  Service Expansion Projects

3)  New Service Projects

4)  Combined Sewer Service Projects

e)  Data provided in the applicant's pre-application will determine

the appropriate class for each project for which assistance is

requested from the Fund.

(Source:  Section 366.104 renumbered from Section 366.103 and amended

at 20 Ill. Reg. 15598, effective November 26, 1996)

<BSection 366.105  Funding Allocations>>

a)  In the development of its priority list, the Agency will allocate

available loan funds to the four major classes of projects in

proportion to the relative needs of the State for each project

class, subject to the limitations of Section 602(b)(6) of the

Clean Water Act (33 U.S.C. 1382(b)(6)).

b)  Annual allocations of available loan funds to each class shall

initially be made on the basis of State wastewater needs as

identified in the pre-applications for projects that qualify for

inclusion on the Intended Use Plan for that fiscal year in

accordance with the requirements of 35 Ill. Adm. Code 365.410(c).

c)  After January 1 of each fiscal year, the Agency may adjust its

allocations of available funds among project classes in the

Intended Use Plan (see 35 Ill. Adm. Code 365.420) to reflect the

relative needs contained in completed loan applications, subject

to the overall availability of loan funds for that fiscal year.

d)  Loan funds available from State and federal appropriations during

the capitalization period authorized by Section 607 of the Clean

Water Act to capitalize the Water Pollution Control Revolving Fund

will be subject to an equal division between the service area of

the Metropolitan Water Reclamation District of Greater Chicago and

the area which is comprised of the geographical balance of the

State, to the extent that projects in either area in any fiscal

year have qualified to receive loan assistance and are ready to

proceed in accordance with the criteria for loan award (see 35

Ill. Adm. Code 365.430).

e)  If sufficient projects in either area are not able to complete a

loan application in any fiscal year to permit an equal division of

the above funds, loans will be made to those projects which are

able to complete a loan application to the extent that the

appropriated funds are available.

f)  Any imbalance in the division of the total loan funds appropriated

during the capitalization period authorized by the Clean Water Act

to capitalize the Water Pollution Control Revolving Fund shall be

carried forward from year to year and shall be applied as projects

are able to complete a loan application to achieve an

accumulatively equal distribution subject to the constraints of

Section 366.106 of this Part.

(Source:  Section 366.105 renumbered from Section 366.104 and amended

at 20 Ill. Reg. 15598, effective November 26, 1996)

<BSection 366.106  Pre-applications>>

a)  A local government unit may submit a pre-application at any time.

The pre-application must identify the class of the project, the

discharge location point, the scope of the project, the population

tributary to the project, a cost estimate and schedule for

completion of the project.

b)  An applicant is required to renew its pre-application annually.

c)  Pre-applications must be received by March 31st of the preceding

fiscal year to be included on the Intended Use Plan.

d)  A project with approved facility planning may be added to the

priority list at any time by the submission of a pre-application.

(Source:  Section 366.106 renumbered from Section 366.105 at 20 Ill.

Reg. 15598, effective November 26, 1996)

<BSection 366.107  Facility Planning>>

a)  A project's priority will be adjusted to reflect completed and

approved facility planning (see 35 Ill. Adm. Code 365:  Subpart

E).

b)  Projects may be split into more than one project, deleted or

modified at the option of the applicant on the priority list as a

result of the approval of the facility planning.

(Source:  Section 366.107 renumbered from Section 366.106 at 20 Ill.

Reg. 15598, effective November 26, 1996)

SUBPART B:  PROCEDURE FOR CALCULATING THE LOAN PRIORITY INDEX OF SERVICE

CONTINUATION PROJECTS

<BSection 366.201  Formula for Computing the Loan Priority Index for Service

Continuation Projects>>

The Loan Priority Index (LPI) is a number that is the product of five

factors.  The LPI is calculated as follows:  A1 x A2 x A3 x A4 x A5 = LPI.

<BSection 366.202  A1 Factor (Financial Impact)>>

A1 is a factor which evaluates the financial impact of wastewater

improvements on the residents of the municipality.  The A1 factor is based

on median household income for the smallest governmental entity that

encompasses the applicant's service area as presented in the 1990 Census of

Population and Housing:  Summary Social, Economic, and Housing

Characteristics, Illinois, Table 10, as published by the U.S. Department of

Commerce, Economics and Statistics Administration, Bureau of the Census, or

the applicant will provide a determination of the mid-point of the

distribution of the annual incomes of at least 80% of the households in the

project service area (commonly known as median household income). The A1

factor is based on median household income and is calculated as follows:

Greater than $0

but less than

$15,000                                   1.10

$15,000 - $19,999                         1.09

$20,000 - $24,999                         1.08

$25,000 - $29,999                         1.07

$30,000 - $34,999                         1.06

$35,000 - $39,999                         1.05

$40,000 - $44,999                         1.04

$45,000 - $49,999                         1.03

$50,000 - $54,999                         1.02

$55,000 - $59,999                         1.01

Greater than $59,999                      1.00

(Source:  Amended at 20 Ill. Reg. 15598, effective November 26, 1996)

<BSection 366.203  A2 Factor (Water Quality)>>

A2 is a factor that evaluates the water quality of the receiving waterbody

or proposed receiving waterbody.  These water quality evaluations are based

on current waterbody specific information.  A combination of biotic

integrity information and abiotic data from three categories are used in A2

calculations when available.  These categories include the Index of Biotic

Integrity (IBI), the Predicted Index of Biotic Integrity (PIBI), and the

Degree of Use Support (DUS).  As a result, those stream reaches resulting

in the highest priority are those with the highest quality and potential

for improvement (see Appendix A).  A2 calculations for stream reaches (as

defined by USEPA REACH file) are derived by:

A2 =       <Psum of the points from IBI, PIBI, DUS categories>>  divided by

100        5

sum of the maximum possible points for each category

<BSection 366.204  A3 Factor (Organic Load)>>

A3 is a factor that evaluates the existing organic load that is tributary

to the proposed project.  It is calculated as the square root of log10 of

the tributary waste load in P.E. BOD.  Where the municipality has provided

monitoring report data, it will be used for the previous calendar year.  In

cases where the monitoring report data is not available for the previous

calendar year, the approved facilities planning will provide estimated

existing organic load.

<BSection 366.205  A4 Factor (Assessment of Existing Facilities)>>

A4 is a factor that evaluates the need for the project in terms of its

importance to the treatment works.  It is calculated as the product of

three elements: (the objective assessment of the existing facilities) x

(the degree of utilization) x (frequency of permit violations) = A4.

a)  The objective assessment will be completed based on the approved

facility planning and the Agency's verification of the facilities

planning's finding in terms of the adequacy, age, structural

and/or mechanical reliability of the existing treatment units.

The objective assessment for the particular project will be

completed by the Agency prior to the completion of the facility

planning.  (See Appendix B.)

b)  The degree of utilization will be calculated as follows:

1)  For wastewater treatment facilities, ratio of the existing

load to the design load will be calculated for both hydraulic

and organic load as follows:

<PAverage organic load>> or 3 <Pmonths low flow average>>

organic design          hydraulic design

The larger of the two ratios will be used in the A4

calculation.

2)  For wastewater transportation facilities:  <Pexisting peak flow>>

design peak

3)  Where relief sewer capacity is proposed, the degree of

utilization will be calculated at the point in the treatment

works where the greatest reserve capacity exists.

c)  The permit exceedance element will be calculated as follows:

1)  For wastewater treatment facilities, the permit exceedance

element shall be based on monitoring report data for the

previous calendar year as follows:

<Pnumber of months with a permit exceedance event>> +1

number of months reported

Where:

A)  BOD, suspended solids or phosphorus exceed the monthly

limits or;

B)  Ammonia nitrogen, chlorine or toxics exceed the monthly

or maximum limit as specified in the permit.

The permit exceedance element shall be based on the single

parameter that is addressed by the project with the greatest

ratio of permit exceedance; or

2)  For sewer system improvements an alternate calculation will

be utilized as follows based on the information in the

approved facility planning:

For overflow and/or bypass events, number of occurrences

in last year:

0-5        = 1.2

6-10       = 1.4

11-15      = 1.6

16-20      = 1.8

Greaterthan 20    =2.0

or

3)  For basement back-ups the frequency and the number of

affected basements will be used (number x frequency).

A)  Average number of basements affected per occurrence:

0-10       = 1.1

11-20      = 1.2

21-50      = 1.3

51-100     = 1.4

Greater

than 100    =1.5.

B)  Annual frequency of occurrence of basement backups:

0-5 times    = 1.1

6-10 times  =   1.2

11-15 times =    1.3

16-20 times =    1.4

Greater than

20 times    = 1.5.

<BSection 366.206  A5 Factor (Operational Excellence)>>

A5 is a factor which evaluates the operation of the existing facilities and

provides a bonus for excellence of operation.  For mechanical treatment and

sewage collection facilities that have exhibited excellence in operation

and maintenance by receiving a score of 15 or greater out of a possible

composite score of 20, or lagoon treatment facilities that have exhibited

excellence in operation and maintenance by receiving a score of 13.5 or

greater out of a possible composite score of 20, 1.5 points will be

awarded.  All others will be 1.0.  This factor will be calculated by the

Agency using the criteria in Appendix C or D.

(Source:  Amended at 20 Ill. Reg. 15598, effective November 26, 1996)

SUBPART C:  PROCEDURE FOR CALCULATING THE LOAN PRIORITY INDEX OF SERVICE

EXPANSION PROJECTS

<BSection 366.301  Formula for Computing the Loan Priority Index for Service

Expansion Projects>>

The Loan Priority Index (LPI) is a number that is the product of six (6)

factors.  The LPI is calculated as follows:  B1 x B2 x B3 x B4 x B5 x B6 =

LPI.

<BSection 366.302  B1 Factor (Financial Impact)>>

B1 is factor which addresses the financial ability of the community as in

A1 above.

<BSection 366.303  B2 Factor (Water Quality)>>

B2 is a factor that evaluates the existing quality of the receiving stream

in accordance with the procedure outlined in A2, and modifies that ranking

so as not to encourage additional waste load to high quality streams.  The

B2 factor is calculated by multiplying the Water Quality Factor as

calculated in A2 by the high quality water factor for the stream class (see

Appendix A):

0.60 for A streams (Unique Aquatic Resource);

0.75 for B streams (Highly Valued Aquatic Resource);

1.00 for C streams (Moderate Aquatic Resource);

1.00 for D Streams (Limited Aquatic Resource);

1.00 for E streams (Restricted Aquatic Resource).

<BSection 366.304  B3 Factor (Economic Benefit)>>

a)  B3 is a factor that evaluates the potential for economic benefit.

Additional points will be awarded for having an annual

unemployment percentage above the State average as determined by

the Department of Employment Security pursuant to Section 43a.08

of the Civil Administrative Code of Illinois (Code) [20 ILCS

5/43a.08] and 20 CFR 634 (1989) (no subsequent dates or editions).

Information concerning a municipality's unemployment rate can be

obtained from:  Economic Information and Analysis Section,

Department of Employment Security, 555 S. Pasfield, Floor 2,

Springfield, Illinois 62704 (217) 785-4624.

b)  B3 is calculated as (unemployment rate factor) + 1.

Where the applicant's unemployment rate is above the State

average, unemployment rate points will be awarded as follows:

less than 1% above the State average = 0.0

1-2.9% above the State average = 0.1

3.0-5% above the State average = 0.2

5% above State average = 0.3

(Source:  Amended at 20 Ill. Reg. 15598, effective November 26, 1996)

<BSection 366.305  B4 Factor (Existing Utilization)>>

B4 is a factor representative of the existing loadings or the degree of

utilization of the existing capacities.  This factor will be calculated for

each type of project (i.e. wastewater treatment expansion, collection

system extension or additional sewer capacity) using different factors as

follows:

a)  Wastewater Treatment Expansion

The wastewater treatment expansion factor will be based on the

ratio of the existing load to the design capability.  The larger

of the two ratios will be used as follows:

1)  <Paverage organic load>>    +1

organic design              or

2)  <P3 months low flow average>>+1

hydraulic design

b)  Collection System Extension

0-50 P.E. = 1.1; for each additional 50 P.E., 0.1 point will be

added to a maximum of 2.0.

c)  Collection System Capacity Expansion

<Pexisting flow + outstanding state construction permits>> + 1

permitted capacity

<BSection 366.306  B5 Factor (Operational Excellence)>>

B5 is a factor that evaluates the operation of the existing facilities as

in A5 above.

<BSection 366.307  B6 Factor (Health Hazard)>>

B6 is a health hazard factor for use with failing septic systems in

unsewered areas to be served by collection system extensions.  Projects

that are determined by the responsible public health agency to be necessary

to correct an existing public health hazard will be assigned a value of 1.1

and all others will be assigned a value of 1.0.

(Source:  Amended at 20 Ill. Reg. 15598, effective November 26, 1996)

SUBPART D:  PROCEDURE FOR CALCULATING LOAN

PRIORITY INDEX FOR NEW SERVICE PROJECTS

<BSection 366.401  Formula for Computing the Loan Priority Index for New

Service Projects>>

The Loan Priority Index (LPI) for new service projects is a number that is

the product of four (4) factors.  The LPI is calculated as follows:  C1 x

C2 x C3 x C4 = LPI.

<BSection 366.402  C1 Factor (Financial Impact)>>

C1 is a factor that addresses the financial ability of the unit of local

government as in A1 above.

<BSection 366.403  C2 Factor (Water Quality)>>

C2 is a factor representative of the existing receiving stream water

quality as in B2 above.

<BSection 366.404  C3 Factor (Organic Load)>>

C3 is a factor that evaluates the existing organic load tributary to the

proposed project and is calculated as in A3 above.

<BSection 366.405  C4 Factor (Health Hazard)>>

C4 is a factor to denote an existing health hazard and is applied as in B6

above.

SUBPART E:  PROCEDURE FOR CALCULATING THE LOAN PRIORITY INDEX FOR COMBINED

SEWER SERVICE PROJECTS

<BSection 366.501  Formula for Computing the Loan Priority Index for Combined

Sewer Projects>>

The Loan Priority Index (LPI) number for Combined Sewer Service projects is

the product of a 5 factors.  The LPI is calculated as follows:  D1 x D2 x

D3 x D4 x D5 = LPI.

<BSection 366.502  D1 Factor (Financial Impact)>>

D1 is a factor that addresses the financial ability of the unit of local

government as in A1 above.

<BSection 366.503  D2 Factor (Drainage Area)>>

D2 is a factor that represents the size of the drainage area tributary to

the project.  The factor will be based on the size of the drainage area:

0 acres                =1.00

Greater than 0

but less than

or

equal to 50

acres                  =1.01

Greater than 50

but less than

or

equal to 100

acres                  =1.02

Greater than

100

but less than

or

equal to 150

acres                  =1.03

Greater than

150

but less than

or

equal to 200

acres                  =1.04

Greater than

200

acres                  =1.05

<BSection 366.504  D3 Factor (Flooding Frequency)>>

D3 is a factor that considers the frequency of street or area flooding

caused by inadequate combined sewer transport capacity.  The factor will be

based on the annual frequency of flooding events as follows:

0 times per        =    1.00

year

1-5 times per      =    1.01

year

6-10 times per     =    1.02

year

11-15 times per    =    1.03

year

16-20 times per    =    1.04

year

More than 20       =    1.05

times per year

<BSection 366.505  D4 Factor (Basement Backups)>>

D4 is a factor that represents the frequency of basement flooding.  D4 is

calculated as follows:  (frequency of basement flooding) x (average number

of basements affected) = D4.

a)  The frequency of basement flooding element will be calculated as

follows:

0 times per        =    1.00

year

1-5 times per      =    1.01

year

6 to 10 times      =    1.02

per year

11 to 15 times     =    1.03

per year

16 to 20 times     =    1.04

per year

More than 20       =    1.05

times per year

b)  The number of basements affected will be calculated as follows

using the average number of basements affected by storm events

during the previous year:

0 basements        =    1.0

1-10 basements     =    1.1

11-25 basements    =    1.2

26-50 basements    =    1.3

51-100             =    1.4

basements

More than 100      =    1.5

basements

<BSection 366.506  D5 Factor (Percentage of Basements Affected)>>

D5 is a factor that expresses the average number of basements affected in

the project drainage area as a percent of the basements in the project

drainage area as follows:

D5 = 100 <Paverage # of basements affected in drainage area>>

# of basements in drainage area

SUBPART F:  PROCEDURE FOR APPLICATION OF SCORING CONVENTIONS

<BSection 366.601  Scoring Conventions>>

a)  For integrally related projects, such as an expansion of

wastewater treatment capacity and a project to add additional load

to the treatment facilities (i.e. relief interceptors, new

interceptors, collection system extensions, new service regional

projects, etc.), the wastewater treatment facility expansion

project will proceed at the LPI of the expansion project or at the

LPI of the project adding load, whichever results in the more

favorable priority ranking.

b)  Projects for Inflow/Infiltration correction serving more than one

municipality will be scored for priority using the A2 and A4

factors for the receiving wastewater treatment works. The A1, A3

and A5 factors will be determined based on the data for the

municipality where the project is proposed.

c)  At the time of facility planning approval, all projects will be

rescored based on the approved planning.  As a result of this

rescoring, projects may be split into a number of projects,

deleted or moved to an appropriate class.

d)  For relief or replacement interceptor sewers that provide capacity

in addition to the relief capacity required for the existing

service area, the Agency will assign segments of the project

different priority numbers or place segments in different project

classes based on the percent of capacity utilization for each

segment as determined during facility planning approval.

e)  Where the project encompasses more than one area of review (i.e.

wastewater treatment plant and lift stations or wastewater

collection system and lift stations), the objective assessment

portion of the A4 factor will be averaged for the areas of review

included in the project.

f)  The loan applicant may provide monitoring data for a pending

permit requirement in order to allow the Agency to calculate a

permit exceedance element as identified in 366.205(c), provided

that the Agency has formally notified the applicant of a pending

new permit requirement.

g)  Where adequate data is not available to calculate any factor, a

value of 1.0 will be assigned to allow completion of the LPI

calculation.

<BSection 366.APPENDIX A  Waterbody Specific Information>>

The waterbody specific information used by the Agency for the priority

system is contained in the Agency's biennial reports required under Section

305(b) of the Clean Water Act (33 U.S.C. 1315(b)).  The calculations for

river reaches range on a possible scale of 0-100, where 100 indicates the

most highly valued water quality resources and results in the highest

priority for protection or preservation.  Calculations for lakes and

reservoirs are based on the Degree of Use Support Assessment (DUS) and

expressed as the DUS points as a percentage of the maximum DUS points.

The component categories of IBI, PIBI, and DUS are determined based on the

following:

a)  IBI - Index of Biotic Integrity

The Index of Biotic Integrity (IBI) is the priority metric of the

Biological Stream Characterization (BSC) stream classification

system.  IBI values ranging from 12 to 60 constitute the primary

basis of the five stream classes as follows:

When quality stream fishery data are lacking for IBI

determinations, BSC ratings may be derived by a subjective

evaluation of narrative fishery criteria.  In the absence of

fishery data, aquatic macroinvertebrate data may be used for

Limited or Restricted Use Aquatic Resource ratings (Class C

and D respectively) for stream segments five miles in length

or longer.  The actual index is used in calculating A2.  The

maximum number of points when IBI is used is 60.

b)  PIBI - Predicted Index of Biotic Integrity

The Predicted Index of Biotic Integrity (PIBI) was developed to

predict biotic potential (as measured by IBI) from habitat

metrics.

The index is determined from the following relationships of four

(4) stream habitat variables:

1.    Percent substrate as silt-mud

2.    Percent substrate as claypan

3.    Mean stream width

4.    Percent pool

The following equation is utilized for prediction of biotic

potential as defined by a predicted IBI (PIBI) value:

Predicted IBI = 40.1 - (0.126 silt-mud) - (0.123 claypan)

+ (0.0424 pool) + (0.0916 width)

For purposes of deriving A2 calculations, PIBI values are divided

by one half.  The maximum number for the PIBI value is 30.

c)  DUS - Degree of Use Support -- Streams

Section 305(b) of the Federal Clean Water Act (33 U.S.C. 1315(b))

requires each state to prepare a biennial report which addresses,

among other items, the water quality of its surface water

resources and the extent to which these waters meet objectives of

the Act.  Surface water resources are described in terms of the

degree to which they are attaining designated uses.  The Degree of

Use Support (DUS) for Illinois streams is described in terms as

follows:

Full = Fully supporting aquatic life uses

Partial/Minor = Partially supporting aquatic life uses with

minor

impairment

Partial/Moderate = Partially supporting aquatic life uses

with

moderate impairment

Nonsupport = Not supporting aquatic life uses

The DUS is reported at 2 assessment levels:  monitored and

evaluated.  The monitored assessment level is based on

current water chemistry, sediment chemistry, biological, and

habitat data collected from various Agency monitoring

programs.

The evaluated assessment level is based primarily on historic

data (5 years or older) or similarity of the area to

monitored waters within the same basins or geographic region.

For purposes of A2 calculations, DUS assessments are

incorporated as follows:

Full or Full/Threatened       =       50

Partial Minor, impact P       =       45

greater than  NP

Partial Moderate, impact P    =       40

greater than NP

Nonsupport, impact P          =       35

greater than NP

Partial Minor, impact P       =       30

less than NP

Partial Moderate, impact P    =       25

less than  NP

Nonsupport, impact P less     =       20

than NP

Partial Minor, impact NP      =       15

only

Partial Moderate, impact      =       10

NP only

Nonsupport, impact NP only    =        5

Note:  P = point source

NP = nonpoint source

The maximum number of points is 50.  Aquatic life use

impairments resulting primarily from point sources are given

a higher priority.  Full or Full/Threatened and Partial Minor

assessments which are based on an evaluated level of

assessment will be assigned 40 points.

For purposes of B2 and C2 calculations, the Stream Class A,

B, C, D or E is assigned from the Biological Stream

Characterization Summary as follows:

Biological Stream Characterization (BSC) summary.

STREAM

CLASS     BSC CATEGORY              BIOTIC RESOURCE QUALITY DESCTIPTION

<P                                                                           >>

A         Unique Aquatic Resource   EXCELLENT.  Comparable to the best

situations without human

disturbance.

B         Highly Valued Aquatic     GOOD.  Good fishery for important

Resource                  gamefish species (sauger, walleye,

northern pike, black bass, panfish

and

catfish); species richness

may be somewhat below expectations

for stream size or geographic region.

C         Moderate Aquatic          FAIR.  Fishery consists

Resource                  predominantly of bullheads

(Ictalurus spp.), sunfish

(Lepomis spp.), and carp

(Cyprinus carpio).  Species

diversity and number of intolerant

fish reduced.  Trophic structure

skewed with increased frequency of

omnivores, green sunfish or tolerant

species.

D         Limited Aquatic           POOR.  Fishery predominantly

Resource

for carp; fish community dominated by

omnivores and tolerant forms.

Intolerant macroinvertebrates rare or

absent; moderate, facultative and

tolerant organisms dominate benthic

community.  Species richness may be

notably lower than expected for

geographic area, stream size or

available habitat.

E         Restricted Aquatic        VERY POOR.  Few fish of any species

Resource                  present; no sport fishery exists.

Intolerant macroinvertebrates absent;

benthic community consists of

essentially tolerant forms, or no

aquatic life may be present.  Species

richness may be restricted to a few

oligochaete or chironomid taxa.

d)  DUS -- Degrees of Use Support -- Lakes & Reservoirs

Index of Biotic Integrity (IBI) and Predicted Index of Biotic

Integrity (PIBI) information is not applicable to lakes and

reservoirs.  The A2 factor will be determined solely from the

Degree of Use Support classification and point allocation

contained in paragraph (c) above.

<BSection 366.APPENDIX B  Service Continuation A4 Factor Scoring Review Sheet>>

Scoring Elements:

A.  <PWASTEWATER TREATMENT PLANTS:>>

NPDES Permit exceedance (if applicable)                        BOD

.....................................................................<P      >>

SS....<P      >>

Ammonia/Nitrogen...<P      >>

Phosphorus...<P      >>

Dechlorination/Toxics....<P      >>

Overflows/Bypasses.....<P      >>

Cause of exceedance<P                                                    >>

<P                                                                       >>

<P                                                                       >>

Work necessary to correct exceedance<P                                   >>

<P                                                                       >>

<P                                                                       >>

1. Age of wastewater treatment facilities since last upgrade...

<P    >> Years.  (0.5 point for each 5 years over 20 years old).        <P    >>

............................................................

2. Is plant concrete sound? ...  Y <P  >> N <P  >>.  Extensive spauling

of concrete must be evident to be classed unsound.  (0.1

point

for each 10% of the concrete thickness that is gone.)  .....        <P    >>

3. If yes, is the reinforcing steel exposed? ... Y <P  >> N <P  >> (1.0

point if steel is exposed.)  ...............................        <P    >>

4. Does the condition of the concrete pose a threat to the

structural integrity of any unit process, building, or

mechanical equipment in the plant or pose a safety hazard to

operating personnel?......................   Y <P    >> N <P    >> <P >>

(If yes, 1.0 point.)  ......................................        <P    >>

5. Is the present condition of various plant concrete

structures

contributing to or causing effluent violations? Y <P  >> N <P  >>

(If yes, 1.0 point.)........................................        <P    >>

6. Would a failure of any of the concrete structures which are

in

poor condition cause a discharge to the waters of the state         <P    >>

or an effluent violation? Y <P  >> N <P  >> (If yes, 1.0 point.)....

7. Is corrosion of metal structures (bridges, walkways, control

panels, valve vaults, handrails, etc.) at the point where a

potential threat exists to continued operation of plant

units or a safety threat exists for plant personnel? Y <P  >> N

<P  >>

(If yes, 1.0 point.) .......................................        <P    >>

8. Number of mechanical equipment failures during the past five

years.......<P    >>.  Causes:  <P                                >>

(0.1 point for each occurrence that resulted in one unit

process being out of service for at least one day; 3.0

points maximum.)............................................        <P    >>

9. Number of mechanical equipment failures during the past year

<P    >>.    Causes:  <P                                          >>

<P                                                               >>

10.Were the mechanical failures in any way related to improper

maintenance?.. Y <P    >> N <P    >> (If no, then 0.1 point for each        <P    >>

event in which a unit process was out of operation of at

least one day; 2.0 points maximum.).........................

11.Did any of the mechanical failures result in a raw or

partially

treated sewage discharge to waters of the state?... Y <P    >> N

<P    >>

(If yes, 2.0 points.) ......................................        <P    >>

12.Did any of these mechanical failures result in an actual or

potential safety hazard to plant personnel?... Y <P    >> N <P    >>

(If yes, 1.0 point.)........................................        <P    >>

13.Are there any portions of the plant which are permanently

out

of service due to mechanical failure or lack of availability

of

replacement parts due to equipment age?  (0.5 point for each

tank

or functional unit that is no longer operational, 4 points          <P    >>

maximum.)...................................................

14.Were these mechanical failures due to equipment design or

application problems? .. Y <P    >> N <P    >> (If yes, subtract 1.0

point.).....................................................        <P    >>

15.Have all warranties expired? ......... Y <P    >> N <P    >> (If no,

subtract 1 point for each piece of equipment that is not

operational for which the warranty is still in effect.).....        <P    >>

TOTAL SCORE, WASTE TREATMENT FACILITIES........................        <P    >>

B.  <PSEWER COLLECTION SYSTEMS:>>

1. Age of the original sewer system in years..<P      >> (1.0 point

for each 10 years or fraction thereof over 50 years old; 3.0

points maximum.) ...........................................        <P    >>

2. Material of construction:  vitrified clay pipe (VCP) <P    >>;

reinforced concrete pipe (RCP) <P    >>; Brick <P    >>;

cast-in-place concrete <P    >> (brick = one point; RCP = 0.5

point;

cast-in-place = 0.5 point.).............................<P    >>

3. If concrete, is there crown corrosion? ....... Y <P    >> N <P    >>

(If yes, 1.0 point.) .......................................        <P    >>

4. If yes, what percent of the pipe thickness at the crown is

gone? ......................................................        <P    >>

(1.0 point for each 25% of pipe thickness lost.)............        <P    >>

5. If pipe is brick, percent of joint material remaining.......

(1.0 point for each 25% of joint material lost.)............        <P    >>

6. If pipe is reinforced concrete pipe, or cast-in-place

concrete,

is the reinforcing steel exposed? ...... Y <P    >> N <P    >>.  (If

steel

is exposed, 1.0 point.)  ...................................        <P    >>

7. Have there been any cave-ins on the system in the past five

years?........Y <P    >> N <P    >> (If yes, then 0.1 point for each

cave-in

event; 3.0 points maximum.) ................................        <P    >>

8. Have there been any cave-ins in the past year?.Y <P    >> N <P    >>

(If yes, add one additional point for each event;

3.0 points maximum.) .......................................        <P    >>

9. Have there been any discharges to waters of the state or

basement backups as a result of any of these cave-ins? Y

<P    >>

N <P    >>. (0.1 additional point for each overflow or basement

backup event caused by pipe failures; 3.0 points maximum.)

............................................................        <P    >>

10.Number of system overflows over the last 5 years due to dry

weather surcharging <P        >> (associated with a non-pipe

failure event i.e. tree roots, pipe settled with grit, etc.)

(0.1 point for each overflow; 3.0 points maximum.) .........        <P    >>

11.Number of homes experiencing basement backups over the last

5 years due to dry weather surcharging <P        >> (non-pipe

failure event); (0.5 point for each 50 homes or fraction

thereof, 5.0 points maximum.) ..............................        <P    >>

12.Percent of system that is combined sewers: <P        >>%  (For

sewer separation projects, 1.0 point for each 10% of the

entire

system or fraction thereof that will be separated by the

proposed project; 5.0 points maximum.)......................        <P    >>

TOTAL SCORE SEWER COLLECTION SYSTEM............................        <P    >>

C. <PCOLLECTION SYSTEM LIFT STATIONS:>>

1. Percent of lift stations on the system that are over 20

years

old <P        >>.  (1.0 point for each 25% of the total stations

or

fraction thereof over 20 years old; 4.0 points maximum.) ...        <P    >>

2. Are all stations equipped with alarms? Y <P    >> N <P    >>.

(No = 1.0 point.) ..........................................        <P    >>

3. Number of homes experiencing backups over the last 5 years

due

to lift station failures or power outages <P        >>.  (1.0

point

for each 50 homes or fraction thereof; 4.0 points maximum.) .       <P    >>

4. Number of system overflows over the last five years due to

lift

station failures or power outages <P        >>.  (0.1 point for

each overflow event; 4.0 points maximum ....................        <P    >>

5. Percent of total station pumping capacity that is out of

service due to obsolete equipment <P        >>.  (1.0 point for

each 10% of total capacity that is out because of inability

to

get replacement equipment; 2.0 points maximum.) ............        <P    >>

6. Do all pumping stations have standby power or alternate

means

of pumping during power failures as required? Y <P    >> N <P    >>.

(If no, 0.5 point for each station not properly equipped

that

will be upgraded by this project; 2.0 points maximum.) .....        <P    >>

7. How many lift stations have equipment or structural

deterioration problems which contribute to operational

problems or safety hazards to operating personnel? <P        >>

(0.5 point for each station that will have these problems

corrected by the

proposed project; maximum 2.0 points.)......................        <P    >>

TOTAL SCORE FOR LIFT STATIONS..................................        <P    >>

Reviewer  <P                    >>

Date

<BSection 366.APPENDIX C  Excellence of Operation Scoring Review Sheet For

Local Government Units That Own Wastewater Treatment Facilities>>

This scoring sheet will be used to rate those projects which have

demonstrated excellence in the maintenance and operation of existing

wastewater treatment facilities.  The scoring factors are as follows:

A.  <PPROCESS PERFORMANCE AND CONTROL:>>

1. Plant performance compared to permit limit requirements.

(If

both BOD/SS are consistently 20% below permit limits, 2.0

points; if 40% or more below limits, 3.0 points.)...........        <P    >>

2. How long has the plant been producing an effluent no greater

than 80% of its permit limits?  (If at least 5 years, 1.0

point; if 10 or more

years, 2.0 points.)  .......................................        <P    >>

3. Current plant loading as a percentage of the design

capacity.

(If the current average daily load is 80 to 90% of design

capacity, 0.5 point; if 90 to 100%; 2.0 points; if over 100%

3.0 points.)................................................        <P    >>

4. Does the operator use laboratory data to make appropriate

process control adjustments?  (If yes, 1.0 point.) .........        <P    >>

5. Is the effluent quality consistent during the entire range

of

plant flows?  (If it is within permit limits at all flows,

1.0

point.) ....................................................        <P    >>

6. Is the sludge quality acceptable for land application?  (If

a

land application permit has been issued, 1.0 point.) .......        <P    >>

7. Is the process subject to process upsets?  (If there have

been

no process upsets due to discharges into the sewer system in

the last 5 years, 0.5 point.)  .............................        <P    >>

8. For processes using the activated sludge process, is

microscopic analysis used for process monitoring?  (If yes,

0.5

point.) ....................................................        <P    >>

TOTAL PROCESS PERFORMANCE AND CONTROL..........................        <P    >>

B. <PMAINTENANCE:>>

1. How long have mechanical equipment and structures been in

service without a failure that affected plant performance?

(Over 5 years, 0.5 point; over 10 years, 1.0 point;

20 or more years, 2.0 points.)..............................        <P    >>

2. Are the plant grounds, buildings, and equipment

well-maintained

(grass cut, equipment and buildings painted, etc.)? (If yes,

0.5 point.)  ...............................................        <P    >>

3. Does the plant have a routine preventative maintenance

program?

(If yes, 1.0 point.) .......................................        <P    >>

4. Does the plant have a spare parts inventory?  (If yes, 0.5

point.) ....................................................        <P    >>

TOTAL MAINTENANCE..............................................        <P    >>

C. ADMINISTRATION:

1. Does the operating agency control industrial discharges into

the sewer system that may adversely affect the treatment

process, sludge or effluent quality or pose a safety hazard

to system workers?  (If a local ordinance exists, 0.5 point;

if there is an approved local pretreatment program, 1.0

point.)

............................................................        <P    >>

2. Are all self-monitoring reports and other reports required

by

permit conditions submitted on time?  (If they are, 0.5

point.)

............................................................        <P    >>

3. Are all financial reporting requirements submitted in

accordance with permit conditions?  (If they are, 0.5               <P    >>

point.) ....................................................

4. Does the chief operator have the proper level of

certification

required by Title 35, Subtitle C of the Illinois Pollution

Control Board Regulations?  (If yes, 0.5 point.) ...........        <P    >>

5. Has the plant been issued an Agency safety certificate

during

the past year?  (If yes, 0.5 point.) .......................        <P    >>

6. Does the plant have an emergency plan to respond to

hazardous

material emergencies?  (If yes, 0.5 point.) ................        <P    >>

7. Does the plant manager prepare an annual report to the board

or

council on annual facility performance which includes budget

needs for the coming year?  (If yes, 0.5 point.) ...........        <P    >>

8. Is revenue being accumulated for annual 0 & M needs and

equipment replacement? (If yes, 0.5 point.).................        <P    >>

TOTAL ADMINISTRATION...........................................        <P    >>

TOTAL ALL CATEGORIES...........................................        <P    >>

TOTAL                 <P    >>

Reviewer <P                  >>  Date <P      >>

<BSection 366.APPENDIX D  Excellence of Operation Scoring Review Sheet For

Local Government Units That Own Only Wastewater Collection Facilities>>

This scoring sheet will be used to rate those projects which have

demonstrated excellence in the maintenance and operation of existing

wastewater treatment facilities.  The scoring factors are as follows:

A.  COLLECTION SYSTEM PERFORMANCE AND CONTROL:

1. When was the original collection system installed?<P        >>.

(0.2 point will be awarded for each 10 years over 20 years

old

for the original system to a maximum of 1.0 point.)  .......        <P    >>

2. If there have been no basement backups reported in the last

year, 1.0 point; in the last five years, 2.0 points; in the

last 10 years, 3.0 points.  ................................        <P    >>

3. If there have been no sewer system overflows to waters of

the

State (i.e.; wet well overflows at lift stations, manhole

overflow, etc.) 0.4 point per year to a 4.0 point maximum.  .       <P    >>

4. If there are lift stations on the collection system, are

they

all equipped with a standby generator or alternate means of

pumping in case of a power outage? (If yes, 1.0 point.)  ...        <P    >>

5. If all lift station wet wells are equipped with high water

alarms, 1.0 point.  ........................................        <P    >>

6. Do any portions of the collection system flow at full

capacity

during wet weather periods?  (If no, then 1.0 point.)  .....        <P    >>

TOTAL FOR COLLECTION SYSTEM PERFORMANCE........................        <P    >>

B. MAINTENANCE:

1. If there is a program for systematic inspection and cleaning

of

all sewer system lines, 1.0 point.  ........................        <P    >>

2. Have there been any television inspections of the system

during

the past five years?  (If so, 1.0 point.)  .................        <P    >>

3. If there is a systematic preventative maintenance program

for

all lift stations on the system, 1.0 point.  ...............        <P    >>

4. If there is a spare parts inventory for all lift stations,

1.0

point  .....................................................        <P    >>

TOTAL FOR SEWER MAINTENANCE....................................        <P    >>

C. ADMINISTRATION

1. Does the operating agency maintain a current sewer atlas?

(If yes, 1.0 point.)  ......................................        <P    >>

2. Does the operating agency have a program to prohibit

downspouts, footing drains, and other clean water

connections

to the sanitary sewer system? (If yes, 1.0 point)  .........        <P    >>

3. Is there a program for local inspection and enforcement of

sewer use ordinances to prohibit downspout and footing or

area

drain connections? (If yes, 1.0 point.) ....................        <P    >>

4. Does the operating agency have an ordinance to regulate the

types of wastes discharged into its collection system? (If

yes,

1.0 point.) ................................................        <P    >>

5. Does the operating agency have a safety program for its

collection system maintenance crews? (If so, 1.0 point.) ...        <P    >>

TOTAL FOR SYSTEM ADMINISTRATION  ..............................        <P    >>

TOTAL SCORE FOR ALL CATEGORIES  ...............................        <P    >>

Reviewer  <P                  >>  Date  <P          >>